Microsoft®
Memphis Developer's Release Device Driver Kit

SONYDCR
IEEE 1394 Bus Class Sample Minidriver Design Notes

This document is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Microsoft Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to the patents, trademarks, copyrights, or other intellectual property rights except as expressly provided in any written license agreement from Microsoft Corporation.

Microsoft does not make any representation or warranty regarding specifications in this document or any product or item developed based on these specifications. Microsoft disclaims all express and implied warranties, including but not limited to the implied warranties or merchantability, fitness for a particular purpose and freedom from infringement. Without limiting the generality of the foregoing, Microsoft does not make any warranty of any kind that any item developed based on these specifications, or any portion of a specification, will not infringe any copyright, patent, trade secret or other intellectual property right of any person or entity in any country. It is your responsibility to seek licenses for such intellectual property rights where appropriate. Microsoft shall not be liable for any damages arising out of or in connection with the use of these specifications, including liability for lost profit, business interruption, or any other damages whatsoever. Some states do not allow the exclusion or limitation of liability or consequential or incidental damages; the above limitation may not apply to you.

ActiveMovie, ActiveVRML, ActiveX, BackOffice, Developer Studio, Direct3D, DirectDraw, DirectInput, DirectMusic, DirectPlay, DirectSound, DirectX, Microsoft, NetMeeting, OpenType, Visual Basic, Visual C++, Win32, Windows, and Windows NT are trademarks or registered trademarks of Microsoft Corporation in the United States and/or other countries. Other product and company names mentioned herein may be the trademarks of their respective owners.

© 1997 Microsoft Corporation. All rights reserved.

Revision History

| Date | Description |
| --- | --- |
| 3/29/97 | March 97 Developer's Release draft |

Contents

Overview	3

Background Notes	3

Anatomy of the Sample Minidriver	4

Event-driven Walkthrough of the Sample Minidriver	4

Reference to Other Microsoft-supplied Samples	5

- Overview

The purpose of this Design Note is to document SONYDCR, which is a sample IEEE 1394 bus class minidriver that is provided by Microsoft, in source-code form, to give the user assistance in writing their own IEEE 1394 bus class minidriver. To be of maximum value, this document requires the user to have the following additional materials close at hand.

- IEEE 1394 Bus Driver Design Notes and Reference, which is another of the documents in the Memphis DDK. All 1394 bus class service calls made by this sample minidriver are documented here.

- WDM Design Notes and Reference, which is another of the documents in the Memphis DDK. References for I/O Request Packets (IRPs) that are used by this sample minidriver are documented here.

- The SONYDCR source-code files, which are a part of the Memphis DDK.

SONYDCR is a skeleton driver for the Sony VX-1000 Camcorder. This sample demonstrates how a minidriver gets loaded, and how it submits requests to the IEEE 1394 bus driver.

There are additional device-specific sample minidrivers located on this Memphis DDK. See the section titled “Reference to Other Microsoft-provided Samples” for a list of additional sample minidrivers provided by Microsoft to give the user assistance in writing their own minidriver. The focus of this Design Note is on the IEEE 1394 bus class sample minidriver. No attempt is made here to explain the IEEE 1394 bus class driver — a “and then a miracle occurs” attitude is taken with regard to that driver.

This Design Note is organized as follows:

Background Notes

Provides important information relevant to writing IEEE 1394 bus class minidrivers.
Clarifies the minidriver’s role in a layered architecture, by showing what lives above and below it.
Describes the nature of resources available to the minidriver.

Anatomy of the Sample Minidriver

Provides a table that illustrates the anatomy of this sample minidriver.

Event-driven Walkthrough of the Sample Minidriver

Provides helpful commentary on the sample source code.
Identifies those portions of the minidriver that the user needs to adapt to meet their particular device requirements.

References

References to material helpful toward understanding this sample minidriver.
A reference to other Microsoft-provided sample minidrivers.

- Background Notes

A minidriver is a hardware-specific DLL that uses a Microsoft-provided class driver to accomplish most actions through function calls, and provides only device-specific controls. Under WDM, the minidriver registers each adapter with the class driver, and the class driver creates a device object to represent each adapter that registers. The registration process is described in WDM Design Notes and Reference. The minidriver uses the class driver’s device object to make system calls.

There will be a DeviceObject created for every device that is found on the IEEE 1394 bus. The DeviceObject created by the bus driver is provided to the device driver associated with the discovered hardware component at the device driver’s load time.

The device driver communicates with the IEEE 1394 bus driver by creating (or receiving) an IRP and calling IoCallDriver with this IRP and the DeviceObject (provided at attach time) as input parameters. This has the effect of calling the parent device that created this DeviceObject (that is, the IEEE 1394 bus driver). In this way, devices do not need to have any embedded knowledge of who their parent device is. It is the responsibility of the IEEE 1394 bus driver to ensure that a DeviceObject remains constant while its underlying addressing information may change in the case of a bus reset.

This section has described general issues relevant to writing IEEE 1394 bus class minidrivers. Now you are ready to start looking at the SONYDCR source code. The following section is an “anatomy” of the sample minidriver code in terms of C modules and driver entry points.

- Anatomy of the Sample Minidriver

After you install the WDM DDK, the following source code and header files are available:

| sonydcr.c | Shows how a driver gets loaded, and how it submits requests to the 1394 bus driver. |
| --- | --- |
| sonydcr.h | Contains structures that are samples of information that could be used in a device extension structure. |

These sample files are commented, to aid the user in understanding their purpose and their relationship to the stream class driver. Documentation for the DDIs referenced below may be found in IEEE 1394 Bus Driver Design Notes and Reference, in WDM Design Notes and Reference, or in the source code files for the sample minidriver, listed above. Documentation for the class driver DDIs can be quickly found by looking to the Table of Contents for these documents.

| Minidriver DDI | Description |
| --- | --- |
| NTSTATUS DriverEntry( IN PDRIVER_OBJECT DriverObject, IN PUNICODE_STRING RegistryPath ); | Initial entry point for the minidriver. |
| NTSTATUS SonyDcrPnPAddDevice( IN PDRIVER_OBJECT DriverObject, IN PDEVICE_OBJECT PhysicalDeviceObject ); | Called by the Operating System to create a new instance of a Sony 1394 Desktop camera. |
| NTSTATUS SonyDcrPnP( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp ); | Receives PnP messages. |
| NTSTATUS SonyDcrSubmitIrpSynch( PSONYDCR_EXTENSION DeviceExtension, PIRP Irp, PIRB Irb ); | Submits an IRP synchronously to the bus driver. |
| NTSTATUS SonyDcrSynchCompletionRoutine( IN PDEVICE_OBJECT DeviceObject, IN PIRP Irp, IN PKEVENT Event ); | For use with synchronous IRP processing. Signals an event, so the driver knows it can continue. |

- Event-driven Walkthrough of the Sample Minidriver

The following steps briefly describe loading and request submission for the sample IEEE 1394 bus class minidriver. The sample minidriver routines mentioned here are described in greater detail in the comments of the sample source code and in routine-specific notes that follow.

- Plug and Play loads the minidriver and the minidriver's DriverEntry routine is called. Two DriverObject structure members are initialized with the addresses of minidriver-supplied functions.

- The AddDevice entry point is called by the PnP manager to inform the 1394 bus driver that a new device instance has arrived. Since AddDevice is initialized with the address of the SonyDcrPnPAddDevice function, it is that function which responds. SonyDcrPnPAddDevice obtains a handle to the 1394 bus driver (although it is not yet safe to submit requests using this handle).

- The MajorFunction entry point receives PnP messages. Since MajorFunction[IRP_MJ_PNP] is initialized with the address of the SonyDcrPnP function, it is that function which responds.

- SonyDcrPnP is the sample minidriver function that receives PnP messages and extracts MinorFunction from the IRP stack. It is now safe to submit IRPs to the 1394 bus driver. A switch statement within SonyDcrPnP causes the code associated with the IRP to execute.

- IRP_MN_START_DEVICE is used to initialize the device. As part of the process, two minidriver-supplied routines are called to facilitate the submission of requests to the 1394 bus driver.

- SonyDcrSynchCompletionRoutine is called to signal an event, so the driver knows it can continue.

- SonyDcrSubmitIrpSynch is called to submit an IRP synchronously to the 1394 bus driver.

- IRP_MN_STOP_DEVICE is used to stop the device. No minidriver-supplied code is called.

- IRP_MN_REMOVE_DEVICE is used to remove the device. No minidriver-supplied code is called.

- IRP_MN_BUS_RESET provides notification that a bus reset has occurred. No minidriver-supplied code is called.

- Reference to Other Microsoft-supplied Samples

The following Microsoft-supplied minidrivers are available as part of this Memphis DDK.

SCSAMPLE: Stream Class Sample Minidriver Design Notes documents the SCSAMPLE sample minidriver, which is a skeleton minidriver in the sense that it simply demonstrates the relationship between the Stream class driver and a generic Stream class minidriver. No device-specific issues are covered.

TESTCAP: Stream Class Video Capture Sample Minidriver Design Notes documents the TESTCAP sample minidriver. It requires no hardware to operate; it simply synthesizes and streams images. It implements a number of property sets. TESTCAP is designed to act as a testbed for further development, and attempts to illustrate the basic technique of writing a WDM Stream class video capture driver.

WDMSB16: Stream Class Audio Sample Minidriver Design Notes documents the WDMSB16 sample minidriver. WDMSB16 demonstrates the basic path from Stream class to SB16 PCM audio device output.

I82930: USB Device Sample Minidriver Design Notes documents the I82930 sample minidriver. I82930 is a driver for the Intel 82930 USB test board and demonstrates how a minidriver gets loaded, and how it submits requests to the USB Driver Interface (USBDI).

HIDMINI: HID Class Sample Minidriver Design Notes documents the HIDMINI sample minidriver. HIDMINI demonstrates how to use the lower-edge interface of the HIDCLASS bus driver. A developer uses this interface to implement a minidriver for a USB device that must appear in the system as a HID device.