Microsoft®
Memphis Developer's Release Device Driver Kit

TESTCAP
Stream Class Video Capture Sample Minidriver Design Notes

This document is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Microsoft Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to the patents, trademarks, copyrights, or other intellectual property rights except as expressly provided in any written license agreement from Microsoft Corporation.

Microsoft does not make any representation or warranty regarding specifications in this document or any product or item developed based on these specifications. Microsoft disclaims all express and implied warranties, including but not limited to the implied warranties or merchantability, fitness for a particular purpose and freedom from infringement. Without limiting the generality of the foregoing, Microsoft does not make any warranty of any kind that any item developed based on these specifications, or any portion of a specification, will not infringe any copyright, patent, trade secret or other intellectual property right of any person or entity in any country. It is your responsibility to seek licenses for such intellectual property rights where appropriate. Microsoft shall not be liable for any damages arising out of or in connection with the use of these specifications, including liability for lost profit, business interruption, or any other damages whatsoever. Some states do not allow the exclusion or limitation of liability or consequential or incidental damages; the above limitation may not apply to you.

ActiveMovie, ActiveVRML, ActiveX, BackOffice, Developer Studio, Direct3D, DirectDraw, DirectInput, DirectMusic, DirectPlay, DirectSound, DirectX, Microsoft, NetMeeting, OpenType, Visual Basic, Visual C++, Win32, Windows, and Windows NT are trademarks or registered trademarks of Microsoft Corporation in the United States and/or other countries. Other product and company names mentioned herein may be the trademarks of their respective owners.

© 1997 Microsoft Corporation. All rights reserved.

Revision History

| Date | Description |
| --- | --- |
| 3/29/97 | March 97 Developer's Release draft |

Contents

Overview	3

Background Notes	4

Notes on Writing Stream Class Minidrivers	4

Anatomy of the Sample Minidriver	5

Event-driven Walkthrough of the Sample Minidriver	7

Reference to Other Microsoft-provided Samples	9

- Overview

The purpose of this Design Note is to document TESTCAP, which is a sample stream class video capture minidriver that is provided by Microsoft, in source-code form, to give the user assistance in writing their own stream class minidriver. To be of maximum value, this document requires the user to have the following additional materials close at hand.

- Stream Class Driver and Minidriver Design Notes and Reference, which is another of the documents in the Memphis DDK. All stream class service calls made by this sample minidriver are documented here.

- Stream Class Video Capture Minidriver Design Notes and Reference, which is another of the documents in the Memphis DDK. References for property sets, which are used by this sample minidriver, are documented here.

- The TESTCAP source-code files, which are a part of the Memphis DDK.

TESTCAP is a sample WDM Stream Class video capture driver.    It requires no hardware to operate; it simply synthesizes and streams images.    It is designed to act as a testbed for further development, and attempts to illustrate the basic technique of writing a WDM Stream Class video capture driver.

Property sets are implemented in this sample. For the most part, these property sets are provided as boilerplate code. Since TESTCAP does not utilize hardware, the property sets don’t actually affect the image in any way.

- VideoProcAmp – for adjusting image quality such as brightness, contrast, and hue.

- TVTuner – for selecting TV channels

- Crossbar – for selecting input sources

The Crossbar property set is actually functional.    As different inputs on the crossbar are “selected”, TESTCAP synthesizes different image streams.    The image streams that may be selected include NTSC 100% and 75% saturation color bars, black, white, and grayscale image streams.

There are additional device-specific sample stream class minidrivers located on this Memphis DDK. See the section titled “Reference to Other Microsoft-provided Samples” for a list of additional sample minidrivers provided by Microsoft to give the user assistance in writing their own stream class minidriver. The focus of this Design Note is on the Stream class video capture minidriver. No attempt is made here to explain the stream class driver — a “and then a miracle occurs” attitude is taken with regard to that driver.

This Design Note is organized as follows:

Background Notes

Provides important information relevant to writing stream class minidrivers for any type of device, as well as information specific to video capture.
Clarifies the minidriver’s role in a layered architecture, by showing what lives above and below it.
Describes the nature of resources available to the minidriver.

Anatomy of the Sample Minidriver

Provides a table that illustrates the anatomy of this sample minidriver.

Event-driven Walkthrough of the Sample Minidriver

Provides helpful commentary on the sample source code.
Identifies those portions of the minidriver that the user needs to adapt to meet their particular device requirements.

References

References to material helpful toward understanding this sample minidriver.
A reference to other Microsoft-provided sample minidrivers.

- Background Notes

A minidriver is a hardware-specific DLL that uses a Microsoft-provided class driver to accomplish most actions through function calls, and provides only device-specific controls. Under WDM, the minidriver registers each adapter with the class driver, and the class driver creates a device object to represent each adapter that registers. The registration process is described in Stream Class Driver and Minidriver Design Notes and Reference. The minidriver uses the class driver’s device object to make system calls.

Some design tradeoffs in the stream class driver/minidriver definition are outlined below, and apply to this sample.

- The minidriver does not create a device object, but “shares” the class driver’s device object as necessary. This choice was made to save system resources.

- Only one device object is created per adapter. Multiple subdevices (called “streams”), supported by the adapter, are represented by CSA pins. TESTCAP can be compiled to generate either one or two streams of RGB video data.

The Stream class internal interface is primarily a set of function calls between the two subdrivers. The Microsoft-provided Stream class driver controls the request flow, calling the user’s minidriver when access to the hardware is necessary. The Stream class driver is responsible for multiprocessor and interrupt synchronization. Once both subdrivers are initialized, the user’s minidriver is passive and is called only by the Stream class driver. Most of the function calls from the user’s minidriver to the Stream class driver are low-level service requests.

Since TESTCAP does not use hardware interrupts, a timer is utilized to determine when to synthesize the next image.    TESTCAP is non-reentrant, when a thread is executing in the minidriver, no calls will be made to any other function within the minidriver.

Each subfunction of the adapter is referred to as a stream in this document. Note that a stream is identical to a CSA Pin.

The streaming minidriver is a peer of the class driver, and can call Windows NT system services as necessary. The minidriver does not allocate a device object, however, and therefore utilizes the class driver’s device object to make system calls. Most minidrivers will not need to make Windows NT system calls, as all necessary functionality should be available from the class driver. Users who discover missing functionality in class drivers are encouraged to contact http://ihv@microsoft.com.

Minidrivers that make Windows NT system calls must be aware (unless the TurnOffSynchronization Boolean in the HW_INITIALIZATION_DATA structure is set to TRUE) that all minidriver entry points are called at IRQL > DISPATCH level, except for the StreamClassCallAtNewPriority function, which permits being called at IRQL = DISPATCH level or PASSIVE level, depending on the priority specified.

- Notes on Writing Stream Class Minidrivers

- Minidrivers must run on non Intel x86 processors, and therefore must be written in ‘C’ (or other high-level language). Minidrivers shall not contain assembly language source code.

- Some hardware requires that firmware be loaded before it will function. Minidrivers shall embed this firmware in the minidriver binary rather than trying to load it from disk.

- Hardware resources (port or memory addresses) shall be accessed via the macros found in Wdm.h. For example, to write to a word port the macro WRITE_PORT_USHORT shall be used. To write a buffer to a port, WRITE_PORT_BUFFER_USHORT shall be used, etc.

- Minidrivers that need to access PCI configuration space shall use HalGetBusByOffset and HalSetBusDataByOffset functions. Note that these functions must be called at DISPATCH priority or lower.

- Minidrivers shall not have unprotected spin loops; meaning that if the minidriver needs to “spin” waiting for the value of a port to change, the loop must be protected by a loop counter.

- Minidrivers that need to synchronously wait for a small period shall use the KeStallExecutionProcessor routine. This routine shall be used sparingly.

- Minidrivers that need to wait for a long period of time (more than a few microseconds) for some operation to complete shall either be interrupt-driven or use the StreamClassScheduleTimer function to schedule a timed callback. Minidrivers shall not “spin” for a long time waiting for a status change, as this is deleterious to overall system performance.

This section has described general issues relevant to writing stream class minidrivers. Now you are ready to start looking at the TESTCAP source code. The following section is an “anatomy” of the sample minidriver code in terms of C modules and driver entry points.

- Anatomy of the Sample Minidriver

After you install the WDM DDK, the following source code files are available.

| capmain.c | Contains code for minidriver initialization, command processing, and uninitialization. |
| --- | --- |
| capirq.c | Contains code for minidriver interrupt processing. |
| capvideo.c | Contains code for processing individual commands. |
| capprop.c | Contains code related to property sets. |
| capxfer.c | Contains code for the synthesis of images. |

After you install the WDM DDK, the following header files are available.

| capmain.h | Contains structures that are samples of information that could be used in a device extension structure. |
| --- | --- |
| capprop.h | Contains definitions for property sets supported by the minidriver. |

These sample files are commented, to aid the user in understanding their purpose and their relationship to the Stream class driver. Documentation for the DDIs referenced below may be found in Stream Class Driver and Minidriver Design Notes and Reference, in Stream Class Video Capture Minidriver Design Notes and Reference, or in the source code files for the sample minidriver, listed above. Documentation for the class driver DDIs can be quickly found by looking to the Table of Contents for these documents. When DDIs are part of the sample minidriver source code, the file locations are noted in parentheses.

Note: functions are listed in the order they appear in the source code.

| Module | Minidriver DDI | Description |
| --- | --- | --- |
| capmain.c | ULONG DriverEntry( PVOID Context1, PVOID Context2 ); | Initial entry point for the minidriver. Initialization, then registration with the stream class driver happen here. |
| capmain.c | NTSTATUS HwInitialize( IN OUT PHW_STREAM_REQUEST_BLOCK pSrb ); | Initializes an adapter accessed through the information provided in the ConfigInfo structure. |
| capmain.c | BOOLEAN HwUnInitialize( PVOID DeviceExtension ); | Releases all resources and cleans up the hardware. |
| capmain.c | BOOLEAN HwQueryUnload( PHW_STREAM_REQUEST_BLOCK pSrb ); | Called when the class driver wants to unload the driver. |
| capmain.c | VOID AdapterOpenStream( PHW_STREAM_REQUEST_BLOCK pSrb ); | Opens the requested stream. |
| capmain.c | VOID AdapterCloseStream( PHW_STREAM_REQUEST_BLOCK pSrb ); | Close the requested stream. |
| capmain.c | VOID AdapterStreamInfo( PHW_STREAM_REQUEST_BLOCK pSrb ); | Returns information of all the streams that are supported by the minidriver. |
| capmain.c | VOID AdapterReceivePacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Main entry point for receiving adapter-based request SRBs. |
| capmain.c | VOID AdapterCancelPacket( PHW_STREAM_REQUEST_BLOCK pSrb ); | Request to cancel a packet that is currently in process in the minidriver. |
| capmain.c | VOID AdapterTimeoutPacket( PHW_STREAM_REQUEST_BLOCK pSrb ); | Called when a packet has been in the minidriver for too long. The adapter must decide what to do with the packet. |
| capmain.c | VOID CompleteStreamSRB( IN PHW_STREAM_REQUEST_BLOCK pSrb, STREAM_MINIDRIVER_STREAM_NOTIFICATION _TYPE NotificationType1, BOOL fUseNotification2, STREAM_MINIDRIVER_STREAM_NOTIFICATION _TYPE NotificationType2 ); | Called when a packet is being completed. Optional second notification type is used to indicate ReadyForNext. |
| capmain.c | VOID CompleteDeviceSRB( IN STREAM_MINIDRIVER_DEVICE _NOTIFICATION_TYPE NotificationType, BOOL fReadyForNext ); | Called when a packet is being completed. |
| capmain.c | BOOL AdapterVerifyFormat( PKS_DATAFORMAT_VIDEOINFOHEADER pKSDataFormatToVerify, int StreamNumber ); | Checks the validity of a format request by walking through the array of supported KSDATA_RANGEs for a given stream. |
| capirq.c | BOOLEAN HwInterrupt( IN PHW_DEVICE_EXTENSION pHwDevEx ); | Called when an interrupt at the IRQ level specified by the ConfigInfo structure, which was passed to the HwInitialize routine, is received. |
| capvideo.c | VOID VideoReceiveDataPacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Receives video data packet commands. |
| capvideo.c | VOID VideoReceiveCtrlPacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Receives packet commands that control the video stream. |
| capvideo.c | VOID VideoGetProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | Processes video property requests. |
| capvideo.c | VOID VideoTimerRoutine( PVOID Context ); | Routine to capture video frames based on a timer. |
| capvideo.c | VOID VideoSetState( PHW_STREAM_REQUEST_BLOCK pSrb ); | Sets the current video state of the requested stream. |
| capprop.c | VOID AdapterSetCrossbarProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | Selects the input to use. TESTCAP uses this to select an image to synthesize. |
| capprop.c | VOID AdapterGetCrossbarProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | To be provided. |
| capprop.c | VOID AdapterSetTunerProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | To be provided. |
| capprop.c | VOID AdapterGetTunerProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | To be provided. |
| capprop.c | VOID AdapterSetVideoProcAmpProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | To be provided. |
| capprop.c | VOID AdapterGetVideoProcAmpProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | To be provided. |
| capprop.c | VOID AdapterSetProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | Routing point to individual property set handlers when settting properties. |
| capprop.c | VOID AdapterGetProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | Routing point to individual property set handlers when retrieving properties. |
| capxfer.c | VOID ImageSynth( IN OUT PHW_STREAM_REQUEST_BLOCK pSrb, IN ImageXferCommands Command ); | Synthesizes images based on the Crossbar input selected. |

- Event-driven Walkthrough of the Sample Minidriver

The following steps briefly describe initialization, use, and uninitialization for the sample stream class minidriver. The sample minidriver routines mentioned here are described in greater detail in the comments of the sample source code and in routine-specific notes that follow.

- The adapter supported by the minidriver is detected by the PCI (or other) enumerator.

- Plug and Play loads the minidriver and calls the minidriver’s DriverEntry routine (in capmain.c), where a HW_INITIALIZATION_DATA structure is allocated and initialized. DriverEntry then calls the stream class driver’s StreamClassRegisterAdapter function. StreamClassRegisterAdapter takes two private structures, used by the stream class driver to find the resources for this adapter, and the address of the HW_INITIALIZATION_DATA structure as arguments, and then registers with the stream class driver.
Note: During the initialization of the HW_INITIALIZATION_DATA structure, some members of that structure are initialized with the addresses of minidriver-supplied functions (in capmain.c, DriverEntry function).

- AdapterReceivePacket is the sample minidriver function that receives Stream Request Block (SRB) command codes for the device. This sample does not utilize all the supported device/instance-specific SRBs. A switch statement within AdapterReceivePacket causes the appropriate minidriver DDI to execute.
Note: During the initialization of the HW_INITIALIZATION_DATA structure that took place in the DriverEntry routine, the address of the minidriver’s AdapterReceivePacket routine was assigned to HwInitData.HwReceivePacket (in capmain.c). It is in this way that SRBs posted to the HwReceivePacket field of the HW_INITIALIZATION_DATA structure are executed by the minidriver’s AdapterReceivePacket routine.

- SRB_INITIALIZE_DEVICE results in a call to HwInitialize (in capmain.c) to initialize the adapter hardware.

- SRB_UNINITIALIZE_DEVICE results in a call to HwUnInitialize (in capmain.c) to uninitialize the adapter hardware.

- SRB_OPEN_STREAM results in a call to AdapterOpenStream (in capmain.c) to open the specified stream. The stream opened is specified by the HW_STREAM_OBJECT structure, which is pointed to by the StreamObject field in the SRB.

- SRB_CLOSE_STREAM results in a call to AdapterCloseStream (in capmain.c) to open the specified stream. The stream closed is specified by the HW_STREAM_OBJECT structure, which is pointed to by the StreamObject field in the SRB.

- SRB_GET_STREAM_INFO results in a call to AdapterStreamInfo (in capmain.c) to return information on the streams it supports.

- SRB_QUERY_UNLOAD results in a call to HwQueryUnload (in capmain.c) to safely unload the driver.

- SRB_GET_DEVICE_PROPERTY results in a call to AdapterGetProperty (in capprop.c) to retrieve properties that apply to the adapter as a whole.    TESTCAP does not utilize stream property sets. However, notice the use of GUIDs to identify property sets and the use of calls to other minidriver-supplied functions.

- SRB_SET_DEVICE_PROPERTY results in a call to AdapterSetProperty (in capprop.c) to set properties that apply to the adapter as a whole.    TESTCAP does not utilize stream property sets. However, notice the use of GUIDs to identify property sets and the use of calls to other minidriver-supplied functions.

- VideoReceiveDataPacket (in capvideo.c) is the sample minidriver function that receives SRB command codes for video data streams opened by AdapterOpenStream. A switch statement within the function causes the appropriate code to execute.

- SRB_READ_DATA results in a check to determine the current device state and, if appropriate, calls CompleteStreamSRB.

- VideoReceiveCtrlPacket (in capvideo.c) is the sample minidriver function that receives SRB command codes for control of streams opened by AdapterOpenStream. These functions are used to get and set properties and other control information for the stream. A switch statement within the function causes the appropriate code to execute.

- SRB_SET_STREAM_STATE results in a call to VideoSetState (in capvideo.c) to set the current state of the requested stream. In this sample, KSSTATE_STOP, KSSTATE_PAUSE, and KSSTATE_RUN are supported. A switch statement within the function causes the appropriate code to execute.

- SRB_GET_STREAM_PROPERTY results in a call to VideoGetProperty (in capvideo) to process video property requests. In this sample, code to determine which property set is being requested has not been filled in. It simply returns STATUS_SUCCESS.

- Reference to Other Microsoft-provided Samples

The following Microsoft-supplied minidrivers are available as part of this Memphis DDK.

SCSAMPLE: Stream Class Sample Minidriver Design Notes documents the SCSAMPLE sample minidriver, which is a skeleton minidriver in the sense that it simply demonstrates the relationship between the Stream class driver and a generic Stream class minidriver. No device-specific issues are covered.

SONYDCR: IEEE 1394 Bus Class Sample Minidriver Design Notes documents the SONYDCR sample minidriver. SONYDCR is a skeleton driver for the Sony VX-1000 Camcorder. It demonstrates how a minidriver gets loaded, and how it submits requests to the Microsoft-provided IEEE 1394 bus driver.

WDMSB16: Stream Class Audio Sample Minidriver Design Notes documents the WDMSB16 sample minidriver. WDMSB16 demonstrates the basic path from Stream class to SB16 PCM audio device output.

I82930: USB Device Sample Minidriver Design Notes documents the I82930 sample minidriver. I82930 is a driver for the Intel 82930 USB test board and demonstrates how a minidriver gets loaded, and how it submits requests to the USB Driver Interface (USBDI).

HIDMINI: HID Class Sample Minidriver Design Notes documents the HIDMINI sample minidriver. HIDMINI demonstrates how to use the lower-edge interface of the HIDCLASS bus driver. A developer uses this interface to implement a minidriver for a USB device that must appear in the system as a HID device.