Microsoft®
Memphis Developer's Release Device Driver Kit

SCSAMPLE
Stream Class Sample Minidriver Design Notes

This document is for informational purposes only. MICROSOFT MAKES NO WARRANTIES, EXPRESS OR IMPLIED, IN THIS DOCUMENT.

Microsoft Corporation may have patents or pending patent applications, trademarks, copyrights, or other intellectual property rights covering subject matter in this document. The furnishing of this document does not give you any license to the patents, trademarks, copyrights, or other intellectual property rights except as expressly provided in any written license agreement from Microsoft Corporation.

Microsoft does not make any representation or warranty regarding specifications in this document or any product or item developed based on these specifications. Microsoft disclaims all express and implied warranties, including but not limited to the implied warranties or merchantability, fitness for a particular purpose and freedom from infringement. Without limiting the generality of the foregoing, Microsoft does not make any warranty of any kind that any item developed based on these specifications, or any portion of a specification, will not infringe any copyright, patent, trade secret or other intellectual property right of any person or entity in any country. It is your responsibility to seek licenses for such intellectual property rights where appropriate. Microsoft shall not be liable for any damages arising out of or in connection with the use of these specifications, including liability for lost profit, business interruption, or any other damages whatsoever. Some states do not allow the exclusion or limitation of liability or consequential or incidental damages; the above limitation may not apply to you.

ActiveMovie, ActiveVRML, ActiveX, BackOffice, Developer Studio, Direct3D, DirectDraw, DirectInput, DirectMusic, DirectPlay, DirectSound, DirectX, Microsoft, NetMeeting, OpenType, Visual Basic, Visual C++, Win32, Windows, and Windows NT are trademarks or registered trademarks of Microsoft Corporation in the United States and/or other countries. Other product and company names mentioned herein may be the trademarks of their respective owners.

© 1997 Microsoft Corporation. All rights reserved.

Revision History

| Date | Description |
| --- | --- |
| 3/20/97 | March 97 Developer's Release draft |

Contents

Overview	3

Background Notes	3

Notes on Writing Stream Class Minidrivers	4

Anatomy of the Sample Minidriver	5

Event-driven Walkthrough of the Sample Minidriver	7

Reference to Other Microsoft-provided Samples	8

- Overview

The purpose of this Design Note is to document SCSAMPLE, which is a sample Stream class minidriver that is provided by Microsoft, in source-code form, to give the user assistance in writing their own Stream class minidriver. To be of maximum value, this document requires the user to have the following additional materials close at hand.

- Stream Class Driver and Minidriver Design Notes and Reference, which is another of the documents in the Memphis DDK. All Stream class service calls made by this sample minidriver are documented here.

- The SCSAMPLE source-code files, which are a part of the Memphis DDK.

SCSAMPLE is a skeleton minidriver in the sense that it simply demonstrates the relationship between the Stream class driver and a generic Stream class minidriver. No device-specific issues are covered here. There are device-specific sample Stream class minidrivers located on this Memphis DDK. See the section titled “Reference to Other Microsoft-supplied Samples” for a list of additional sample minidrivers provided by Microsoft to give the user assistance in writing their own Stream class minidriver. The focus of this Design Note is on the Stream class minidriver. No attempt is made here to explain the Stream class driver — a “and then a miracle occurs” attitude is taken with regard to that driver.

This Design Note is organized as follows:

Background Notes

Provides important information relevant to writing Stream class minidrivers for any type of device.
Clarifies the minidriver’s role in a layered architecture, by showing what lives above and below it.
Describes the nature of resources available to the minidriver.

Anatomy of the Sample Minidriver

Provides a table that illustrates the anatomy of this sample minidriver.

Event-driven Walkthrough of the Sample Minidriver

Provides helpful commentary on the sample source code.
Identifies those portions of the minidriver that the user needs to adapt to meet their particular device requirements.

References

References to material helpful toward understanding this sample minidriver.
A reference to other Microsoft-provided sample minidrivers.

- Background Notes

A minidriver is a hardware-specific DLL that uses a Microsoft-provided class driver to accomplish most actions through function calls, and provides only device-specific controls. Under WDM, the minidriver registers each adapter with the class driver, and the class driver creates a device object to represent each adapter that registers. The registration process is described in Stream Class Driver and Minidriver Design Notes and Reference. The minidriver uses the class driver’s device object to make system calls.

Some design tradeoffs in the Stream class driver/minidriver definition are outlined below, and apply to this sample.

- The minidriver does not create a device object, but “shares” the class driver’s device object as necessary. This choice was made to save system resources.

- Only one device object is created per adapter. Multiple subdevices (called streams), supported by the adapter, are represented by CSA pins.

The Stream class internal interface is primarily a set of function calls between the two subdrivers. The Microsoft-provided Stream class driver controls the request flow, calling the user’s minidriver when access to the hardware is necessary. The Stream class driver is responsible for multiprocessor and interrupt synchronization. Once both subdrivers are initialized, the user’s minidriver is passive and is called only by the Stream class driver. Most of the function calls from the user’s minidriver to the Stream class driver are low-level service requests.

In this sample, all streaming minidriver functions are synchronized with the adapter’s Interrupt Service Routine (ISR), making this class of minidrivers nonreentrant. In other words, when a thread is executing in the minidriver, no calls will be made to any other function within the minidriver, including the ISR. This nonreentrancy holds true even on multiprocessor Windows NT systems, making the minidrivers easier to write. Nonreentrancy is accomplished by masking off the IRQ of the streaming adapter (and all lower priority IRQs) when code is executing in any of the minidriver’s routines.

Each subfunction of the adapter is referred to as a stream in this document. Note that a stream is identical to a CSA Pin.

The streaming minidriver is a peer of the class driver, and can call Windows NT system services as necessary. The minidriver does not allocate a device object, however, and therefore utilizes the class driver’s device object to make system calls. Most minidrivers will not need to make Windows NT system calls, as all necessary functionality should be available from the class driver. Users who discover missing functionality in class drivers are encouraged to contact http://ihv@microsoft.com.

Minidrivers that make Windows NT system calls must be aware (unless the TurnOffSynchronization Boolean in the HW_INITIALIZATION_DATA structure is set to TRUE) that all minidriver entry points are called at IRQL > DISPATCH level, except for the StreamClassCallAtNewPriority function, which permits being called at IRQL = DISPATCH level or PASSIVE level, depending on the priority specified.

- Notes on Writing Stream Class Minidrivers

- Minidrivers must run on non Intel x86 processors, and therefore must be written in ‘C’ (or other high-level language). Minidrivers shall not contain assembly language source code.

- Some hardware requires that firmware be loaded before it will function. Minidrivers shall embed this firmware in the minidriver binary rather than trying to load it from disk.

- Hardware resources (port or memory addresses) shall be accessed via the macros found in Wdm.h. For example, to write to a word port the macro WRITE_PORT_USHORT shall be used. To write a buffer to a port, WRITE_PORT_BUFFER_USHORT shall be used, etc.

- Minidrivers that need to access PCI configuration space shall use HalGetBusByOffset and HalSetBusDataByOffset functions. Note that these functions must be called at DISPATCH priority or lower.

- Minidrivers shall not have unprotected spin loops, meaning that if the minidriver needs to spin waiting for the value of a port to change, the loop must be protected by a loop counter.

- Minidrivers needing to synchronously wait for a small period shall use the KeStallExecutionProcessor routine. This routine shall be used sparingly.

- Minidrivers that need to wait for a long period of time (more than a few microseconds) for some operation to complete shall either be interrupt-driven or use the StreamClassScheduleTimer function to schedule a timed callback. Minidrivers shall not spin for a long time waiting for a status change, as this is deleterious to overall system performance.

This section has described general issues relevant to writing Stream class minidrivers. Now you are ready to start looking at the SCSAMPLE source code. The following section is an “anatomy” of the sample minidriver code in terms of C modules and driver entry points.

- Anatomy of the Sample Minidriver

After you install the WDM DDK, the following source code files are available.

| mpinit.c | Contains code related to minidriver initialization and uninitialization. |
| --- | --- |
| mpirq.c | Contains code related to minidriver interrupt processing. |
| sccmd.c | Contains code related to minidriver command processing. |
| mpaudio.c | Contains code related to processing individual commands. |
| mpvideo.c | Contains code related to processing individual commands. |

After you install the WDM DDK, the following header files are available.

| mpinit.h | Contains structures that are samples of information that could be used in a device extension structure. |
| --- | --- |
| strmid.h | Contains header definitions for stream types supported by the minidriver. |

These sample files are heavily commented, to aid the user in understanding their purpose and their relationship to the Stream class driver. Documentation for the DDIs referenced below may be found either in Stream Class Driver and Minidriver Design Notes and Reference, or in the source code files for the sample minidriver, listed above. Documentation for the class driver DDIs can be quickly found by looking to the Table of Contents for Stream Class Driver and Minidriver Design Notes and Reference. When DDIs are part of the sample minidriver source code, the file locations are noted in parentheses.

| Module | Minidriver DDI | Description |
| --- | --- | --- |
| mpinit.c | ULONG DriverEntry( PVOID Context1, PVOID Context2 ); | Initial entry point for the minidriver. Initialization, then registration with the stream class driver happen here. |
| mpinit.c | NTSTATUS HwInitialize( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Initializes an adapter accessed through the information provided in the ConfigInfo structure. |
| mpinit.c | BOOLEAN HwUnInitialize( IN PVOID DeviceExtension ); | Release all resources and clean up the hardware. |
| mpirq.c | BOOLEAN HwInterrupt( IN PHW_DEVICE_EXTENSION pHwDevEx ); | Called when an interrupt at the IRQ level specified by the ConfigInfo structure, which was passed to the HwInitialize routine, is received. |
| sccmd.c | VOID AdapterCancelPacket( PHW_STREAM_REQUEST_BLOCK pSrb ); | Request to cancel a packet that is currently in process in the minidriver. |
| sccmd.c | VOID AdapterTimeoutPacket( PHW_STREAM_REQUEST_BLOCK pSrb ); | Called when a packet has been in the minidriver for too long. The adapter must decide what to do with the packet. |
| sccmd.c | VOID AdapterReceivePacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Main entry point for receiving adapter-based request SRBs. |
| sccmd.c | VOID AdapterStreamInfo( PHW_STREAM_REQUEST_BLOCK pSrb ); | Returns information of all the streams that are supported by the minidriver. |
| sccmd.c | VOID AdapterOpenStream( PHW_STREAM_REQUEST_BLOCK pSrb ); | Open the requested stream. |
| sccmd.c | VOID AdapterCloseStream( PHW_STREAM_REQUEST_BLOCK pSrb ); | Close the requested stream. |
| sccmd.c | VOID AudioReceiveDataPacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Receives audio data packet commands. |
| sccmd.c | VOID VideoReceiveDataPacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Receives video data packet commands. |
| sccmd.c | VOID VideoReceiveCtrlPacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Receives packet commands that control the video stream. |
| sccmd.c | VOID AudioReceiveCtrlPacket( IN PHW_STREAM_REQUEST_BLOCK pSrb ); | Receives packet commands that control the audio stream. |
| mpaudio.c | VOID GetAudioProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | Routine to process audio property requests. |
| mpaudio.c | VOID GetAudLvl( PHW_STREAM_REQUEST_BLOCK pSrb ); | Completes the audio buffer level property. |
| mpaudio.c | VOID SetAudioState( PHW_STREAM_REQUEST_BLOCK pSrb ); | Sets the current audio state of the requested stream. |
| mpvideo.c | VOID StartDataPacket( PHW_STREAM_REQUEST_BLOCK pSrb ); | Routine to initialize the stream data packet handling. |
| mpvideo.c | VOID VideoPacketStub( PHW_STREAM_OBJECT pstrm ); | Stub routine for changing the priority of the data copy for video. |
| mpvideo.c | VOID AudioPacketStub( PHW_STREAM_OBJECT pstrm ); | Stub routine for changing the priority of the data copy for audio. |
| mpvideo.c | VOID ProcessDataPacket( PHW_STREAM_OBJECT pstrm ); | Process the audio or video data packets. |
| mpvideo.c | VOID GetVideoProperty( PHW_STREAM_REQUEST_BLOCK pSrb ); | Processes video property requests. |
| mpvideo.c | VOID GetVidLvl( PHW_STREAM_REQUEST_BLOCK pSrb ); | Completes the video buffer level property. |
| mpvideo.c | VOID SetVideoState( PHW_STREAM_REQUEST_BLOCK pSrb ); | Sets the current video state of the requested stream. |
| mpvideo.c | VOID EnableIRQ( PHW_STREAM_OBJECT pstrm ); | Enables all IRQs associated with the minidriver. |
| mpvideo.c | VOID DisableIRQ( PHW_STREAM_OBJECT pstrm ); | Disables all IRQs associated with the minidriver. |

- Event-driven Walkthrough of the Sample Minidriver

The following steps briefly describe initialization, use, and uninitialization for the sample Stream class minidriver. The sample minidriver routines mentioned here are described in greater detail in the comments of the sample source code and in routine-specific notes that follow.

- The adapter supported by the minidriver is detected by the PCI (or other) enumerator.

- Plug and Play loads the minidriver and calls the minidriver’s DriverEntry routine (in mpinit.c), where a HW_INITIALIZATION_DATA structure is allocated and initialized. DriverEntry then calls the Stream class driver’s StreamClassRegisterAdapter function. StreamClassRegisterAdapter takes two private structures, used by the Stream class driver to find the resources for this adapter, and the address of the HW_INITIALIZATION_DATA structure as arguments, and then registers with the Stream class driver.
Note: During the initialization of the HW_INITIALIZATION_DATA structure, some members of that structure are initialized with the addresses of minidriver-supplied functions (in mpinit.c, DriverEntry function).

- AdapterReceivePacket is the sample minidriver function that receives Stream request block (SRB) command codes for the device. A switch statement within AdapterReceivePacket causes the appropriate minidriver DDI to execute.
Note: During the initialization of the HW_INITIALIZATION_DATA structure that took place in the DriverEntry routine, the address of the minidriver’s AdapterReceivePacket routine was assigned to HwInitData.HwReceivePacket (in mpinit.c). It is in this way that SRBs posted to the HwReceivePacket field of the HW_INITIALIZATION_DATA structure are executed by the minidriver’s AdapterReceivePacket routine.

- SRB_INITIALIZE_DEVICE results in a call to HwInitialize (in mpinit.c) to initialize the adapter hardware.

- SRB_GET_STREAM_INFO results in a call to AdapterStreamInfo (in sccmd.c) to return information on the streams it supports.

- SRB_OPEN_STREAM results in a call to AdapterOpenStream (in sccmd.c) to open the specified stream. The stream opened is specified by the HW_STREAM_OBJECT structure, which is pointed to by the StreamObject field in the SRB.

- SRB_CLOSE_STREAM results in a call to AdapterCloseStream (in sccmd.c) to open the specified stream. The stream closed is specified by the HW_STREAM_OBJECT structure, which is pointed to by the StreamObject field in the SRB.

- SRB_UNINITIALIZE_DEVICE results in a call to HwUnInitialize (in mpinit.c) to uninitialize the adapter hardware.

- AudioReceiveDataPacket and VideoReceiveDataPacket are the sample minidriver functions that receive SRB command codes for data streams.
Note: In the sample minidriver’s AdapterOpenStream function, the addresses of the sample minidriver’s AudioReceiveDataPacket and VideoReceiveDataPacket functions (both in sccmd.c) are assigned (using a switch statement) to pSrb->StreamObject->ReceiveDataPacket (in sccmd.c). In this way, AudioReceiveDataPacket and VideoReceiveDataPacket receive their respective data packet commands.

- SRB_READ_DATA or SRB_WRITE_DATA results in a call to either AudioReceiveDataPacket or VideoReceiveDataPacket. A switch statement within each function causes the appropriate code to execute (in sccmd.c).

- AudioReceiveCtrlPacket and VideoReceiveCtrlPacket are the sample minidriver functions that receive SRB command codes for control of streams. These functions are used to get and set properties and other control information for the stream.
Note: In the sample minidriver’s AdapterOpenStream function, the addresses of the sample minidriver’s AudioReceiveCtrlPacket and VideoReceiveCtrlPacket functions (both in sccmd.c) are assigned (using a switch statement) to pSrb->StreamObject->ReceiveControlPacket (in sccmd.c). In this way, AudioReceiveCtrlPacket and VideoReceiveCtrlPacket receive their respective control packet commands.

- SRB_GET_STREAM_STATE or SRB_SET_STREAM_STATE results in a call to either AudioReceiveCtrlPacket or VideoReceiveCtrlPacket. A switch statement within each function causes the appropriate code to execute (in sccmd.c).

- SRB_GET_STREAM_PROPERTY or SRB_SET_STREAM_PROPERTY results in a call to either AudioReceiveCtrlPacket or VideoReceiveCtrlPacket. A switch statement within each function causes the appropriate code to execute (in sccmd.c).

- Reference to Other Microsoft-provided Samples

The following Microsoft-supplied minidrivers are available as part of this Memphis DDK.

TESTCAP: Stream Class Video Capture Sample Minidriver Design Notes documents the TESTCAP sample minidriver. It requires no hardware to operate; it simply synthesizes and streams images. It implements a number of property sets. TESTCAP is designed to act as a testbed for further development, and attempts to illustrate the basic technique of writing a WDM Stream class video capture driver.

SONYDCR: IEEE 1394 Bus Class Sample Minidriver Design Notes documents the SONYDCR sample minidriver. SONYDCR is a skeleton driver for the Sony VX-1000 Camcorder. It demonstrates how a minidriver gets loaded, and how it submits requests to the Microsoft-provided IEEE 1394 bus driver.

WDMSB16: Stream Class Audio Sample Minidriver Design Notes documents the WDMSB16 sample minidriver. WDMSB16 demonstrates the basic path from Stream class to SB16 PCM audio device output.

I82930: USB Device Sample Minidriver Design Notes documents the I82930 sample minidriver. I82930 is a driver for the Intel 82930 USB test board and demonstrates how a minidriver gets loaded, and how it submits requests to the USB Driver Interface (USBDI).

HIDMINI: HID Class Sample Minidriver Design Notes documents the HIDMINI sample minidriver. HIDMINI demonstrates how to use the lower-edge interface of the HIDCLASS bus driver. A developer uses this interface to implement a minidriver for a USB device that must appear in the system as a HID device.