DATA OUT PHASE

1) The target requests the INITIATOR in the DATA OUT phase to send a number of bytes over the DATA BUS.

2) SCSI command to target determines the number of bytes that can be received by the target.The transfer length in CDB specifies this field.

3) This phase is indicated by the TARGET driving the C/D,I/O and MSG signal inactive.

DATA IN PHASE

1) The TARGET requests the INITIATOR  in the DATA IN phase to receive a number of bytes over the data bus.

2) DATA IN phase is indicated by the TARGET driving the I/O signal active,and the C/D and MSG inactive.

3) The number of bytes to be sent are determined by the SCSI command previously sent to TARGET.

INITIATOR

TARGET

First TARGET requests the INITIATOR.

Initiator posts a INQUIRY(12h) request on the SCSI bus and this INQUIRY command does the following tasks

a) Sets the number of bytes in the Command Descriptor Block(CDB) that will be transfered to the INITIATOR.

b) By analysing this CDB,the TARGET transfers the requested bytes to the INITIATOR.

The most simplified use of this architecture is scanning of scsi bus for different scsi devices installed on your system.

COMMAND PHASE

The target requests commands from the initiator in the command phase.

The command phase is indicated by the target driving  C/D signal active and the I/O and MSG signals inactive.

The COMMAND bytes specify the type of operation the TARGET is to perform.

COMMAND STRUCTURE

A SCSI command is a multibyte data structure known as a command descriptor block or CDB.The first byte of a CDB always contains an operation code.Operation code (7,6,5 bits are Group Code and 5,4,3,2,1,0 are Command Code).

Three bits provides for eight groups of command codes.Group 0 commands use a single byte to specify the transfer length for a value of 1 to 256 bytes. The following is first byte of a CDB.

7 	            6              5	4	3	2	1	   0

Group Code 			Command Code

Commands for scanner devices

Command name                       	 	Operation code                Type

CHANGE DEFINITION                   	 40h       	 		O

COMPARE                                        	39h     			O

COPY                                	   	18h     		 	O

COPY AND VERIFY                         	3Ah         		O

GET DATA BUFFER STATUS         	34h       			O

GET WINDOW                                   	25h  		 	O

INQUIRY                                 		12h      			M

LOG SELECT                                		4Ch        		O

LOG SENSE                               		4Dh        		O

MODE SELECT(6)                          	15h        			O

MODE SELECT(10)                         	55h        			O

MODE SENSE(6)                           	1Ah        		O

MODE SENSE(10)                         	5Ah        		O

OBJECT POSITION                         	31h        			O

READ                                    		28h        			M

READ BUFFER                             		3Ch       			 O

RECEIVE DIAGNOSTIC RESULTS          1Ch        		 O

RELEASE UNIT                            		17h       			M

REQUEST SENSE                           	03h        			M

RESERVE UNIT                          	 	16h       			 M

SCAN                                    		1Bh        		O

SET WINDOW                              		24h        			M

SEND                                   		2Ah        		O

SEND DIAGNOSTIC                        	1Dh        		M

TEST UNIT READY                         	00h        			M

WRITE BUFFER                            		3Bh        		O

Key:    M = command implementation is mandatory

O = command implementation is optional.

A command is communicated by sending a command descriptor block to the target.The CDB always has an operation code as its first byte and a control byte as its last byte.For all commands,if there is an invalid parameter in the CDB,then target shall terminate the command.

Mandatory commands

INQUIRY    (12h)

READ          (28h)

RELEASE UNIT    (17h)

REQUEST SENSE (03h)

RESERVE UNIT    (16 h)

SET WINDOW       (24h)

SEND DIAGNOSTIC (1Dh)

TEST UNIT READY  (00h)

Commands implemented by all SCSI devices

There are four commands that all SCSI-2 targets implement

a)  INQUIRY.

b) REQUESTS SENSE.

c) SEND DIAGNOSTIC.

d)TEST UNIT READY.

a) The INQUIRY command is  used to determine the configuration of the SCSI bus.Target devices respond with information that includes their type and standard level .It is recommended that SCSI targets be capable of returning this info.

b) Whenever a contingent allegiance condition is established,the initiator that received the error should issue a REQUESTS SENSE command to receive the sense data describing what caused the contingent allegiance condition.If the initiator issues some other command,the sense data is lost.

Contingent allegiance condition : The contingent allegiance condition exist following the return of CHECK CONDITION or COMMAND TERMINATED.

c) The SEND DIAGNOSTIC command provides a means to request the target to perform a self test.While the test is target specific,the means of requesting the test is standardized and the response is simply  GOOD status if all is well or CHECK CONDITION status if the test fails.

d) The TEST UNIT READY command is useful in that it allows an initiator to poll a logical unit until it is ready without the need to allocate space for returned data.Targets are expected to respond promptly to indicate the current status of the device.

READ Command

The READ command requests the target transfer data to the initiator.

SCSI Status Phase

When the command is completed,the target will drive the bus into the STATUS phase and will send a status byte to the INITIATOR (to indicate whether the command failed or succeeded without error).When the target receives a BUS DEVICE RESET,an ABORT message,or a hard RESET,the target will go to BUS FREE phase and will not issuse a status byte.

A SCSI status code is an 8-bit value with four bits for defining the standard status code,three bits for defining vendor unique usage,and one reserve bit.

7         6         543       2          1             0

Reserved   Vendor Unique      Status Byte Code          V

Message System

The message system allows the INITIATOR and TARGET to communicate in order to manage the data flow and physical path between both devices.All SCSI devices are required to support the COMMAND COMPLETE message,but for increased flexibility,it is recommended that most of the optional,non-extended messages should be supported.

Writable Control Store

ESBC also contains a 128 word by 30-bit WCS memory.The user can program bus phases according to his requirements and this bus  sequence is complied by the Adaptec provided DOS assembler .The WCS is normally  started by specifying the address(0-127) which is the starting point of  a particular command.

This address is [14. SQADR = 00] in normal page.

WCS Memory(32*9)

SCSI Bus

CDB

supppose wcs sequence  as programmed by the user

wcs[0] = selection

wcs[1] = check phase

wcs[2] = GetCDB

wcs[3] = stop  At this point wcs generates an interrupt.

f/w polls the interrupt register

and fetches the CDB and proceeds further by analysing this CDB.

FIRMWARE

Right now our target is not capable of returning info to the initiator bcoz of the missing command and message  phase.When initiator posts a  command(Command Descriptor Block) onto the SCSI bus,that CDB is accepted by the WCS , WCS stops itself and generates an interrupt.The f/w(firmware) polls the interrupt register and knows that wcs has been stoped,f/w fetches the CDB,analyse it and proceeds as per the intructions in the CDB.

SCANNING SCSI BUS FOR INSTALLED HOST ADAPTERS AND ATTACHED SCSI DEVICES FOR Win32.

ASPI Manager for Win-32 is implemented as a Dynamic Link Library(winaspi32.dll).So use winaspi32.dll if you are using dynamic linking.Use wnaspi32.lib if using static linking.

We are using two standard functions provided by adaptec library(wnaspi32.lib).

1.DWORD GetASPI32SupportInfo()

This function returns the number of SCSI host adapters installed and ensures that the ASPI manager is properly initialized.The user must call this function once at initialization time before the SendASPI32Command is accessed.

2. DWORD  SendASPI32Command()

This function handles requests for SCSI I/O.It is called with a pointer to a SCSI Request Block structure.Following commands for SCSI I/O operation.

1) SC_HA_INQUIRY

2) SC_GET_DEV_TYPE

3) SC_EXEC_SCSI_CMD

4) SC_RESET_DEV

SRB_HAInquiry structure

// ASPI command code

SRB_Cmd    		=  [in] SC_HA_INQUIRY

//value returned by ASPI manager,command status byte

SRB_Status  		=  [out]

// this field specifies which installed host adapter the request is intended for. Host adapter numbers are always assigned by the SCSI manager layer begining with zero.

SRB_HaId		=  [in]

// Reserved

SRB_Flags		=  [in]

// Reserved

SRB_Hdr_Rsvd   	=  0

// //  Number of host adapters present. To determine the total number of host adapters in the system,the SRB_HaId should bet set to zero.

HA_Count           		=   [out]

// SCSI ID of the host adapter. The ASPI manager sets this field with the SCSI ID of the given host adapter.

HA_SCSI_ID      		=   [out]

// The ASPI manager fills this buffer with the string “ASPI for Win32”.

HA_ManagerId[16]	=   [out]

// The ASPI manager fills this buffer with a string describing the host adapter.

HA_Identifier[16]              =   [out]

HA_Unique[16]                 =   [out]

HA_Unique[1]   =   0	  No boundary requirements

=   1	  WORD Alignment required

=   3	  DWORD Alignment required

=   8	  2 DWORD Alignment required

HA_Unique[2]   =   2 	 Residual Byte Count Supported

HA_Unique[3]   =   8          Indicates the maximum number of SCSI targets(SCSI IDs 0 to 7)

These feature are evaluated by this command(SC_HA_INQUIRY)

1) Residual Byte Count

2) Buffer Alignment Mask

3) Maximum number of SCSI targets.

4) ASPI for which platform like Win32,Win16,DOS etc.

5) The host adapter Miniport driver.

6) The status of the ASPI command.

7) Number of host adapters.

8) Maximum transfer length.

9) SCSI ID assigned to a particular host adapter by ASPI manager.

Execute SCSI I/O Command[SC_EXEC_SCSI_CMD]

Once an ASPI client has initialized properly,all I/O is performed with this command.

SRB_Cmd 		= [in]    SC_EXEC_SCSI_CMD

SRB_Status                        = [out]

SRB_HaId                          = [in]

SRB_Flags                         = [in]

Suppose you are performing the SCSI_INQUIRY command.The data transfer will take place from the target to the initiator means data in.So we will set this field with SRB_DIR_IN.

SRB_DIR_IN | SRB_POSTING | SRB_ENABLE_RESIDUAL_COUNT

INITIATOR

TARGET

SRB_DIR_OUT | SRB_POSTING | SRB_ENABLE_RESIDUAL_COUNT

SRB_BufPointer

Start location of returned info from target

SCSI ID of the target device

SRB_Target                      = [in]

SRB_Lun                          = 0 [in]

The target transfers data to initiator on request.This field indicates the number of bytes to be transfered.If residual byte count is supported ,this field is returned with the residual number of bytes(usually 0).e.g in scsi inquiry command the target returns its product name in this field which starts from the 8th location means &SRB_BufPointer[8].

SRB_BufLen                    =  32

//Pointer to data buffer

SRB_BufPointer              = malloc(32)

// SENSE_LEN  Default sense buffer length 14

// [IN] A request sense is automatically generated if a check condition is presented at the end of a SCSI command.

SRB_SenseLen                =  SENSE_LEN

// the length of CDB in bytes either 6 or 10

SRB_CDBLen                 =  6

// host adapter status

SRB_HaStat                    = [out]

//the status of the target

SRB_TargStat                 = [out]

//if posting(SRB_POSTING) is enabled ,this represent the pointer to function.ASPI for Win32 calls this function upon completion of ASPI request.

SRB_PostProc                = [in]

//this field contains the CDB as defined by target’s SCSI command set.

CDBByte[16 ]

CDBByte[0]                   = SCSI_INQUIRY

CDBByte[4] 	           = 32;

//the sense area is filled with sense data on a check condition

SenseArea[SENSE_LEN+2] = [out]

A)The INQUIRY command is typically used by the initiator after a reset to determine the device types for system configuration.

B) The INQUIRY data may change as the target executes its initialization sequence.The target may contain a minimum command set in its non-volatile memory  and may load its final firmware from the device when it becomes ready.After it has loaded the firmware,it may support more options and return different supported options information in the INQUIRY data.

C) The target should have the ability to execute the INQUIRY command even when a device error occurs that prohibits normal command execution.

ASPI COMMANDS

Synchronous SRBs

Assynchronous SRBs

Calls to SendASPI32Command with synchronous  SRBs

will not return until execution of SRB is complete.

Calls to SendASPI32Command with

asynchronous SRBs may return control

to the caller before the submitted SRB

has completed execution.

Remember while waiting for SRB completion,it is always safe to submit additional SRBs to ASPI for execution.

SCSI Request blocks(SRB) are asynchronous for SC_EXEC_SCSI_CMD

SEND DIAGNOSTIC

This command requests the target to perform diagnostic operations on itself, on the logical unit, or on both.The only mandatory implementation of this command is the self-test feature with the parameter list length of zero.