PMON - Performance Monitoring Utility V2.2

Users Guide

10/15/96

Information in this document is provided solely to enable use of Intel products. Intel assumes no liability whatsoever, including infringement of any patent or copyright, for sale and use of Intel products except as provided in Intel's Terms and Conditions of Sale for such products.

Intel Corporation makes no warranty for the use of its products and assumes no responsibility for any errors which may appear in this document nor does it make a commitment to update the information contained herein.

Intel retains the right to make changes to these specifications at any time, without notice.

Contact your local Intel sales office or your distributor to obtain the latest specifications before placing your product order.

MDS is an ordering code only and is not used as a product name or trademark of Intel Corporation.

Intel Corporation and Intel's FASTPATH are not affiliated with Kinetics, a division of Excelan, Inc. or its FASTPATH trademark or products.

*Other brands and names are the property of their respective owners.

Additional copies of this document or other Intel literature may be obtained from:

Intel Corporation
Literature Sales
P.O. Box 7641
Mt. Prospect, IL 60056-7641

or call 1-800-879-4683

© INTEL CORPORATION  1995	CG-041493

1. Objective:

2. System Requirements:

3. PMON Installation:

4. Main operation modes:

4.1 User interface

4.2 Command line interface

4.3 Programming interface

5. PMON Features

5.1 Current Files and version numbers:

5.2 The VxD

5.3 The DLL

5.4 The Utility Main features

5.4.1 Event Selection

5.4.2 Start/Stop

5.4.3 Additional Settings

5.5 Command Line Operation

5.5.1 Command Line Parameters

5.5.2 Command Line Examples

6. PMON Data Files

6.1 INI files / Log files:

6.1.1 PMON.INI

6.1.2 EVENTS.INI

6.1.3 Log File

6.1.4 Trace File

7. The PMON APIs:

8. Considerations

9. The Events

# Objective:

PMON is a performance monitoring tool for Intel processors that retrieves information about a sequence of code that is running under Windows* 95. This information may then be used for various purposes, including performance tuning, performance validation, code coverage, and system tuning.

# System Requirements:

- Windows* 95

- Intel Pentiumâ processor, Pentium Pro processor, or Pentium processor with MMXä technology.

# PMON Installation:

Create a new directory (e.g. : C:\PMON) and unzip the PMON files into it.  The executables are:

PM32_APP.EXE

PM32_DLL.DLL

PMON.VXD

The zipped file may also include an additional file :

EVENTS.INI

Note:  If this file is not included, it will be created by the Pmon Utility.

# Main operation modes:

Figure 1 illustrates the basic components of PMON.  These are described in detail below.

Figure 1

## User interface

In this mode, referred to hereafter as “Pmon Utility”, you operate the utility interactively while running the application you want to test.  Refer to Figure 2 for the features described below.

Counters Displays

Drop-Down

List Boxes

Notification Window

Start Button

Figure 2

First, select the events you want to measure through the two drop-down list boxes, and then start the counters by pushing the “Start” button.  The “Start” button will now display “Stop.”  You will see the counters updated every second.  When you hit the “Stop” button, the summary of this measurement will be displayed in the notification window.  You can then copy these messages to the Windows* clipboard or to a file.

There are several options for starting/stopping the counters, and for logging the results into a file.  For details, see the explanations later in this guide.  Also, the source code of this utility is supplied with this package and may be used for reference or modified as needed.

## Command line interface

In this mode you activate the PMON utility along with some command line parameters. The command line parameters allow you to activate the utility, select the events, start and stop the counters, and log the results to a file.  Command line implementation can be done by running it from a Windows 95 DOS* box, by creating and editing the properties of the utility’s desktop icon or by using a test tool utility such as Microsoft* Visual Test.

Note: Each time you call this utility it loads again and executes.

## Programming interface

In this mode, referred to hereafter as “Pmon API”, you interface with the DLL APIs from your code or test tool utility. Loading, initializing the DLL, starting/stopping and retrieving the information is done by the user from the application code.  This is most accurate and non-intrusive mode, however it requires some programming and you must have the source code of the application under test.  This mode can also be used with a test tool utility.

# PMON Features

Table 1 lists the features of PMON, which are described in detail below.

| Feature | User / Command line interface | Programming/API interface |
| --- | --- | --- |
| Event selection | Yes | Yes |
| Start/Stop counting | Yes | Yes |
| Delayed start | Yes | Yes (User implements) |
| Accumulate start | Yes | Yes (User implements) |
| Stop counting and log | Yes | Yes |
| Install new events | Yes (events.ini) | Yes |
| Read counters from ring3 | Yes | Yes |
| Send event directly to counters control registers | No | Yes |

Table 1

## Current Files and version numbers:

PM32_APP.exe	The application 	- V2.2

PM32_DLL.dll	The DLL 		- V2.2

PMON.vxd		The VxD 		- V2.2

Warning:  All version numbers must be as listed above or Pmon will not work correctly.  You can check the DLL and VxD version numbers by looking at the first two lines in the Notification window.

The utility checks the version numbers and displays a warning in the notification window if they do not match.

## The VxD

The VxD is dynamically loaded the first time you activate the DLL and unloaded when you exit.

Through the API or via the command line parameter you have the option to specify to unload the VxD after exit.

When you use the PMON utility the VxD stays loaded by default.  This enables the VxD to keep previous activation records and report them back later to the DLL.  This way you can activate the tool from one application and stop it from another application and get the original activation event and data.

The VxD is non intrusive. It is small, 7KB and is not active if you don’t call it. No interrupt chaining is used.

## The DLL

The DLL implements a set of APIs and communicates with the VxD. It should be loaded by the application (using the PMON utility, or by loading it within the application code), and it is unloaded automatically when the last application that uses it exits.

## The Utility Main features

### Event Selection

You can select an event using the index drop-down list box or using the event drop-down list box. Choosing the first letter of the string will move the selection to the first event that starts with that letter.  This will speed up the event selection.

When you exit, the last settings are saved in the pmon.ini file in the same directory as the Pmon Utility.  These events are selected as the defaults when the Pmon Utility starts again, if the counter are not running.  If counters are running at launch time, the Pmon Utility will display the current counter names and values and these will be written to the notification window.

The variables used in event selection are (refer to Figure 3):

- Event:  The name of the event being monitored.

- Index: The index of the event as used by the Pmon Utility.

- Umask:  This drop-down list box exists only if the processor is Pentium Pro. The value here combines with the event value to select the actual event to count.

- Ring:  The privilege level of the events to be watched.

Accumulate Start

Delayed Start

Simple Start/Stop

Ring

Umask

Index

Figure 3

The files associated with event selection are:

- PMON.INI: Stores the last selected events and initializes the next instance of the Pmon Utility to these events.

- EVENTS.INI: Modify this to add events not currently programmed into the Pmon Utility.

### Start/Stop

Refer to Figure 3 for the following:

- Simple Start/Stop: Use this for immediate starting and stopping of the counters.

- Start [Delay]: For a delayed start of the counters, first click on “Configuration” from the menu bar and then set the following parameters in the Configuration dialog box (see Figure 4):

- Delay after Start:  Select the number of seconds to wait before starting the counters.

- Auto Stop After:  You can select when you want to stop counting by entering the number of seconds here, or you can click on the simple Stop button to stop the counters immediately.

- Start [Accumulate]: A Windows timer is set to the requested interval and on each tick, it logs the counters values to a table in memory. The accuracy of this timer is at the mercy of the Windows OS and the running applications at that time. This might be okay as we are logging the cycles elapsed and most of the time we are interested in the difference between consecutive entries in the table.  The results are written a file named trace.txt in the Pmon directory.

- To use the Accumulate Start, first click on “Configuration” from the menu bar and then set the following parameters in the Configuration dialog box (see Figure 4):

- Number of Samples:  Select the number of samples to take.

- Sampling Interval:  Select the length of the interval between each sample (in milliseconds).

- Message window (optional): You can enter a descriptive line here to be written to trace.txt.

- Stop message at the notification window:  Refer to Figure 3. When counters are stopped the Pmon utility displays a summary of the last run in the notification window.  The information provided is:

- The actual programmed hexadecimal values of the performance registers

- The event string and the event index that were chosen for counter 0 and counter 1

- The ring selection

- The counter values and the elapsed cycles from start to stop

Figure 4

### Additional Settings

These settings are accessed by selecting the “Setting” option on the menu bar (see Figure 5).

- Auto Stop: Check this option to have the counters always auto stop after the time specified in the Configuration dialog box (see Figure 4).

Figure 5

- Always save results to file: Check this option to always have the counter data saved to a log file.  The file will be named pmon32.log and it will be saved in the same directory as the Pmon Utility.  If the file already exists, the new data will be appended to the end of the file.

- Stay on top:  Check here to keep the Pmon Utility always on top.

- Save Selection to Clipboard:  Highlight an area in the Notification window and then click here to have the area copied to the Windows clipboard.

- Save Selection to FILE: Highlight an area in the Notification window and then click here to have the area copied to a file. The file name will be named trace.txt and it will be saved in the same directory as the Pmon Utility.  If the file already exists, the new data will be appended to the end of the file.

## Command Line Operation

### Command Line Parameters

Table 2 lists the parameters that can be used with Pmon in the command line mode of operation.

Table 2: The Utility Command Line options

| Command | Description | Example |
| --- | --- | --- |
| /start | Start the counters now | /start |
| /T0=n /T1=m | Specify the index for the event you want to activate. See Appendix A for the events and their indices. | /T0=5 /T1=20 |
| /T0CPL=x /T1CPL=y | Set the ring level at which you want to count the events. 30 - level 012 and 3 3 - only level 3 0 - only level 0 | /T0CPL=30 - counter 0 counts all relevant events if code runs in ring 0,1,2,3 /T1CPL=3 - counter 1 counts all relevant events if code runs in ring 3 |
| /batch | Activate the utility without showing the GUI/interactive mode and exits at the end. The VxD will stay loaded after exit. | /BATCH |
| /MSG=zzzyyyxxx | Write this string to the log file as a reference. | /MSG=Test20_Just_Before_Lunch |
| /STOP | Stop the counter, read results and report | /stop |
| /FILE=xyz.log | When stopped, will log the results to this file (appended to the end) with regular date and time stamp. Note, no spaces around the equal sign | /file=July23.log |
| /ACC_START | Start counting and sample the counter every period of time (/INTERVAL). Put readings into a buffer. At the end write the whole table into a file-TRACE.TXT | /ACC_START /SAMPLS=5000 INTERVAL=1000 |
| /SAMPLES= | Number of samples | see above example |
| /INTERVAL= | Sample rate [in ms] | see above example |
| /DELAY_START | Start counting after the delay. | /DELAY_START /DELAY=5 |
| /DELAY= | Delay [sec] | see above example |

Note: The command line is not case sensitive.

Table 3 summarizes the parameters that are available with each mode of operation.

Table 3: Main Command line options Summary

| Purpose | /BATCH | /ACC_START /INTERVAL= /SAMPLES= | /START | /STOP | /DELAY_START /DELAY= | /FILE= /MSG= |
| --- | --- | --- | --- | --- | --- | --- |
| Start & Exit | X |  | X |  |  |  |
| Stop & Exit | X |  |  | X |  | X |
| Start with Delay & Exit | X |  |  |  | X |  |
| Accumulate & Exit | X | X |  |  |  |  |

### Command Line Examples

The following are examples of command lines.

1.	pmon  /t0=1 /T1=12 /t0cPl=30 /T1CPL=0 /StaRt /batch

Timer 0 is selected with event #1 with level 0,1,2,3

Timer1 is selected with event #12 with level 0,1,2,

Start the timers now.

Don't show the pmon GUI, just exit at the end.

2.	pmon32 /stop /file=log.log  /msg= Test20_Just_Before_Lunch  /batch

These are the results in the log.log file:

--------- Log xxx---------

Fri Sep 20 13:04:00 1996

CPU ID: Family= 5, Model= 2, Stepping= 11, FeatureFlags=      1BF

User Message: Test20_Just_Before_Lunch

Counter 0:	1294383

Counter 1:	11030874

Elapsed cycles (TSC):	11300

Cnt0 E:	02570243	Ring:	0     	Mask0=00

Cnt1 E:	00000000	Ring:	0     	Mask1=00

Event 0[4]:	Data Read Miss

Event 1[23]:Instructions Executed in the v-pipe e.g. parallelism

--------- Log End------

# PMON Data Files

## INI files / Log files:

### PMON.INI

The pmon.ini file contains start up configuration lines, which may be edited as needed.  An example is shown in Figure 6.  If you are using the Pmon Utility GUI, pmon.ini will be updated automatically each time you close the application.

PMON.INI

[Initialization Values]

Read PMC From Level 3=ON

Event 0 Last Selection=27

Event 1 Last Selection=2

Event 0 Last Ring Selection=0

Event 1 Last Ring Selection=0

Event 0 Last Mask Selection=0x00

Event 1 Last Mask Selection=0x00

Interval [in ms]=1000

Delay [in ms]=4

Buffer Size=5

Auto Stop Time [in sec]=3

Figure 6

Parameter Explanation:

Read PMC From Level 3

The options are ON or OFF.  Default is ON. This instruction is only available on Pentium Processor with MMX technology and on Pentium Pro family.  It allows reading the counters directly from ring 3.

All the other parameters

Used by the utility to save last usage before exit and restore those entries when launched again. Note that if the counters are running on the next launch, these parameters are not used and the current counter setting are used.

### EVENTS.INI

The events.ini file contains information about additional events which are not hard-coded into the Pmon Utility. An example is shown in Figure 7 below.  A description of how to add events to Pmon through the events.ini file follows.

EVENTS.INI

[Pentium Additional Events - Counter 0]

Number Of New Events=0

[Pentium Additional Events - Counter 1]

Number Of New Events=0

[Pentium Pro Additional Events]

Number Of New Events=27

....

E_str_11=Transitions FP  to MMX

E_enc_11=0xCC

E_msk_11=0x01

E_idx_11=92

E_str_12=My favorite Event

E_enc_12=0x17

E_msk_12=0x01

E_idx_12=0

...

Figure 7

Parameter Explanation:

Note: The parameters are case sensitive!!!

There are 3 basic sections:

[Pentium Additional Events - Counter 0]

[Pentium Additional Events - Counter 1]

[Pentium Pro Additional Events]

The utility detects under which processor it runs and looks for the appropriate section.

Number Of New Events=27

This parameter specifies how many new events exist in this section. In this example 27 new events should follow this section. Each event should have four lines with a consecutive index (starting from 1) embedded into the parameter name. For example E_str_12, refers to the 12th entry.

E_str_n=

a string that represents this event. Will be used in the drop-down list box and for the data files.

E_enc_n=

a hex number (example: 0x36) which is the event’s encoding.

E_msk_n=

a hex number (example: 0xFF) which is the event’s Umask (relevant for Pentium Pro Processor Family).

E_idx_n=

A decimal number (starting from 0) for the index in the Application/DLL event data base.  It inserts the new event into this entry of the data base and overrides the previous event.

The first unoccupied entries in the data base for this version of the utility are:

Pentium processor:  60

Pentium Pro Processor: 82

The table contains 120 entries.

### Log File

Figure 8 shows an example of the log file that is generated when you ask for this option, either by setting “Always Save Results to a File” from the “Setting” menu, or by command line parameter.

Log File

--------- Log xxx---------

Fri Sep 20 13:04:00 1996

CPU ID: Family= 5, Model= 2, Stepping= 11, FeatureFlags=      1BF

User Message: Test2A

Counter 0:	1294383

Counter 1:	11030874

Elapsed cycles (TSC):	11300

Cnt0 E:	02570243	Ring:	0     	Mask0=00

Cnt1 E:	00000000	Ring:	0     	Mask1=00

Event 0[4]:	Data Read Miss

Event 1[23]:	Instructions Executed in the v-pipe e.g. parallelism

--------- Log End------

Figure 8

The file is created if it does not exist and then appended with this information.  It includes the time the data was written, along with information about the CPU.  Also included are the user message (if given) and the data from the counters.

### Trace File

Figure 9 shows an example of the trace file that is generated when you select the accumulate mode, either by clicking on the “Start[Acc.]” button, or by command line parameter.

Trace.txt

--->> Trace From: Fri Sep 20 14:08:21 1996

CPU ID:	Family	Model	Stepping	FeatureFlags

5	  2	       11	        1BF

User Message: My second attempt

Interval used:	500	[ms]

Cnt0 Ring:	0     	Mask0=00

Cnt1 Ring:	0     	Mask1=00

Event 0[32]:	Hardware Interrupts

Event 1[27]:	FLOPs

Sample#	Cnt0	Cnt1	Elapsed Cycles

00001		34	25	46157524

00002		14	44	19760606

00003		35	24	46143440

00004		14	43	19775050

00005		33	24	46153248

00006		16	43	19757254

00007		37	24	46153758

00008		14	43	19758794

00009		33	24	46143502

00010		16	43	19765716

00011		32	22	41175440

00012		15	45	19746536

00013		34	23	46160212

00014		17	44	19761174

00015		35	23	46176638

00016		15	44	19731874

00017		36	23	46156850

00018		15	44	19752132

00019		40	23	46158564

Total:		485	628	634388312

--------- End ------

Figure 9

Each entry here represents the incremental counts that occurred between two consecutive samplings. For example, sample # 2 reads “14 counter 0”.  This means that Counter 0 was incremented by 14 after sample #1 was taken.

Note:  The above format is a tab separated in the file, which can be easily imported into a spread sheet (CSV format).

# The PMON APIs:

The APIs defined in PM32_DLL.DLL are listed in Table 4 below.

Table 4: The PMON APIs

| Category | API | Description |
| --- | --- | --- |
|  | Pmon32Init | Load/connect to the VxD. |
| Init | Pmon32InitEx | Load/connect to the VxD + Option to unload VxD on exit |
|  | Pmon32Close | Close connection to DLL/VxD. Depending on how the DLL was initialized, this may or may not unload the VxD. |
|  | Pmon32InstallEvent | Install a new event into a requested entry in the DLL event data base. This overrides any existing entry. The table below shows the occupied events in the DLL data base. Pentium Pro Processor Events occupy entries from 0-81. Pentium Processor Events occupy entries from 0 - 61. It is advisable to add your events to the unoccupied entries. Both tables have 120 entries. |
|  | Pmon32DisableRDPMCRing3 Pmon32EnableRDPMCRing3 | Allows/disables reading the counters from ring3. Only available on Pentium Processors with MMX Technology or Pentium Pro Processors. |
| Operation | Pmon32Start | Program the events, start counting |
|  | Pmon32StartDirect | Send the value directly to the counter control registers. This also attaches strings to these events so that when these counters are stopped and the EventToStr routines are called, these strings will be returned. |
|  | Pmon32StartEx | Program the events, start counting. Additional support for Umask field. |
|  | Pmon32StopAndReadCounters | Stop counters, read results. |
| Status | Pmon32ReadCounters | Read counters on the fly. |
|  | Pmon32ReadTSC | Read the time stamp counter. |
|  | Pmon32Status | Running/stopped, which events were or are being counted. |
|  | Pmon32AreCountersRunning | TRUE/FALSE for counters running. |
|  | Pmon32Event0ToStr Pmon32Event1ToStr | Returns a pointer to string for a given event index. |

The formal definitions for these APIs are (as included in the file DLL_IF.H):

DWORD	(FAR *Pmon32Init)			(struct	Pmon32Version *ver);

DWORD	(FAR *Pmon32Close)  		(void);

DWORD	(FAR *Pmon32Status) 		(struct	Pmon32Reply	*Reply);

BOOL	(FAR *Pmon32AreCountersRunning)	();

DWORD	(FAR *Pmon32Start)			(BYTE Event0, BYTE Event0Ring,

BYTE Event1, BYTE Event1Ring);

DWORD	(FAR *Pmon32StopAndReadCounters)	(struct	Pmon32Reply	*Reply);

DWORD	(FAR *Pmon32ReadCounters) 		(struct	Pmon32Reply	*Reply);

DWORD	(FAR *Pmon32ReadTSC)		(struct	Pmon32Reply	*Reply);

DWORD	(FAR *Pmon32EnableRDPMCRing3)	(void);

DWORD	(FAR *Pmon32DisableRDPMCRing3)	(void);

char	*(FAR *Pmon32Event0ToStr)		(BYTE		index);

char	*(FAR *Pmon32Event1ToStr)		(BYTE		index);

DWORD	(FAR  *Pmon32StartEx)		(struct	Pmon32StartCmd	*Cmd);

DWORD	(FAR  *Pmon32StartDirect)		(DWORD E0, 	DWORD E1, char *str0,

char *str1);

DWORD	(FAR  *Pmon32InstallEvent) 	(BYTE Cnt, BYTE Index,

BYTE	Encoding, BYTE Mask, char *str);

DWORD	(FAR  *Pmon32InitEx) 		(struct	Pmon32StartCmd	*Cmd, 							struct Pmon32Version *ver);

The application communicates with the DLL using the following structure:

struct	Pmon32StartCmd{

BYTE	Event0;	// Index as defined by the table below (Section 9)

BYTE	Event1;	// Index as defined by the table below (Section 9)

BYTE	Event0Ring;	// 0 - ring 0, 3 - ring 3, 30 - ring 3 and 0

BYTE	Event1Ring;

BYTE	Event0Mask;	// Only for Pentium Pro processors. As defined by the 				// architecture

BYTE	Event1Mask;

BYTE	Flg1;		// res

BYTE	Flg2;		// res

BOOL	unload_vxd_on_exit;		// TRUE - VxD will be unloaded on exit

BOOL	Flg3;

BOOL	Flg4;

BOOL	Flg5;

DWORD	Res1;

DWORD	Res2;

DWORD	Res3;

};

struct	Pmon32Reply{

DWORD	T0_l;		// Counter/event 0 LSB

DWORD	T0_h; 		// Counter/event 0 LSB

DWORD	T1_l; 		// Counter/event 1 MSB

DWORD	T1_h; 		// Counter/event 1 MSB

DWORD	TSC_l; 		// current Time Stamp LSB

DWORD	TSC_h; 		// current Time Stamp MSB

DWORD	ElapsedTime_l;

DWORD	ElapsedTime_h;

DWORD	TSCLast_l;

DWORD	TSCLast_h;

BYTE	Event0;	// Index as known by the DLL

BYTE	Event1;

BYTE	Event0Ring;

BYTE	Event1Ring;

BYTE	Event0Mask;

BYTE	Event1Mask;

BYTE	pad1  ;		// structure alignment

BYTE	pad2  ;

DWORD	status;

DWORD	res1;

DWORD	res2;

DWORD	res3;

DWORD	res4;

DWORD	res5;

DWORD	res6;

};

struct	Pmon32Version{

char	VxDVersion[32];	// A string

char	DLLVersion[32];	// A string

int	VxDMajor;		// Represents the VxD major version number

int	VxDMinor;		// Represents the VxD minor version number

int	DLLMajor;		// Represents the DLL major version number

int	DLLMinor;		// Represents the DLL minor version number

char	res2[16];

};

# Considerations

1. This version runs only on Windows* 95.

2. One can use batch mode to set the required events and set the option of reading the counters from level 3 (This is the default if this line is omitted). Then, the application can read the results (the counters) directly from its code (ring 3).

3. Two or more instances of the PMON Utility are allowed. Notice that the second instance will refresh its event selection only once at start, but won’t be able to report on the right selection dynamically as  the changes take place by other instances. On the other hand, it will show the changes in the counter totals. Also, if the second instance is the one that initiates the stop, it will report the right event.

- The Pmon utility does not use all the options provided in the performance monitoring of the processor.  You can observe the exact programming value that the tool is using by looking at the stop message in the notification window. However the APIs allow the programmer to directly interface with the register and utilize more features.

- Notice that the Pentium Pro events are programmed with E bit set to 0. This means that all events are a duration type and not occurrence one.

- In the Pentium Pro case some events are only applicable to a specific counter. The current version of the Pmon utility does not prevent you from using it with the wrong counter.  However the description of the event notifies you with the correct counter to use.  See the Pentium Pro event table, events 55-60.

# The Events

The following are the events and their indices. These are the default programming in the application data base and in the DLL data base. It also correlates to the result indices in the log file.

Each table consumes 120 entries and the unoccupied ones are different from Pentium to Pentium Pro tables.

Note that indices below are the indices defined by the tool and they do NOT necessarily match the ones described in the processor programmer reference manual .

Pentium Processor Events for Counter 0

| Index | Event | Encoding |
| --- | --- | --- |
| 0 | Data Read | 0x00 |
| 1 | Data Write | 0x01 |
| 2 | Data Read or Data Write | 0x28 |
| 3 | Data TLB Miss | 0x02 |
| 4 | Data Read Miss | 0x03 |
| 5 | Data Write Miss | 0x04 |
| 6 | Data Read or Data Write Miss | 0x29 |
| 7 | Write (hit) to M or E state lines | 0x05 |
| 8 | Data Cache Lines Written Back | 0x06 |
| 9 | External Snoops | 0x07 |
| 10 | External Data Cache Snoop Hits | 0x08 |
| 11 | Memory Accesses in Both Pipes | 0x09 |
| 12 | Bank Conflicts | 0x0a |
| 13 | Misaligned Data Memory or I/O References | 0x0b |
| 14 | Code Read | 0x0c |
| 15 | Code TLB Miss | 0x0d |
| 16 | Code Cache Miss | 0x0e |
| 17 | Any Segment Register Loaded | 0x0f |
| 18 | Branches | 0x12 |
| 19 | BTB Hits | 0x13 |
| 20 | Taken Branch or BTB Hit | 0x14 |
| 21 | Pipeline Flushes | 0x15 |
| 22 | Instructions Executed | 0x16 |
| 23 | Instructions Executed in the v-pipe e.g. parallelism | 0x17 |
| 24 | Locked Bus Cycle | 0x1c |
| 25 | I/O Read or Write Cycle | 0x1d |
| 26 | Non-cacheable memory reads | 0x1e |
| 27 | FLOPs | 0x22 |
| 28 | Breakpoint match on DR0 Register | 0x23 |
| 29 | Breakpoint match on DR1 Register | 0x24 |
| 30 | Breakpoint match on DR2 Register | 0x25 |
| 31 | Breakpoint match on DR3 Register | 0x26 |
| 32 | Hardware Interrupts | 0x27 |
| 33 | Clocks while a bus cycle is in progress (bus util.) | 0x18 |
| 34 | Number of clocks stalled due to full write buffers | 0x19 |
| 35 | Pipeline stalled waiting for data memory read | 0x1a |
| 36 | Stall on write to an E or M state line | 0x1b |
| 37 | Pipeline stalled due to addr generation interlock | 0x1f |
| 38 | Bus ownership latency | 0x2a |
| 39 | MMX instructions executed in U-pipe | 0x2b |
| 40 | Number of L1 M-state line sharing | 0x2c |
| 41 | EMMS instructions executed | 0x2d |
| 42 | Bus utilization due to processor activity | 0x2e |
| 43 | Saturated MMX instructions executed | 0x2f |
| 44 | Cycles not in HLT state | 0x30 |

Pentium Processor Events for Counter 0 (continued)

| Index | Event | Encoding |
| --- | --- | --- |
| 45 | MMX data memory reads | 0x31 |
| 46 | Floating point stalls | 0x32 |
| 47 | D1 starving but instruction FIFO empty | 0x33 |
| 48 | MMX data memory writes | 0x34 |
| 49 | Pipeline flushes due to wrong branch prediction | 0x35 |
| 50 | MMX misaligned data memory reference | 0x36 |
| 51 | Returns miss predictions | 0x37 |
| 52 | MMX Multiply interlock | 0x38 |
| 53 | Returns executed | 0x39 |
| 54 | BTB bogus entry detected | 0x3a |
| 55 | MMX writes backed - pipe stalled | 0x3b |
| 56 | Reserved | 0x3c |
| 57 | Reserved | 0x3d |
| 58 | Reserved | 0x3e |
| 59 | NULL event | 0x3f |
| 60 | Reserved | 0xFF |
| 61 | Reserved | 0xFF |
| .... |  |  |
| 118 | Reserved | 0xFF |
| 119 | Reserved | 0xFF |

Pentium Processor Events for Counter 1

| Index | Event |  | Encoding |
| --- | --- | --- | --- |
| 0 | Data Read | 0x00 |  |
| 1 | Data Write | 0x01 |  |
| 2 | Data Read or Data Write | 0x28 |  |
| 3 | Data TLB Miss | 0x02 |  |
| 4 | Data Read Miss | 0x03 |  |
| 5 | Data Write Miss | 0x04 |  |
| 6 | Data Read or Data Write Miss | 0x29 |  |
| 7 | Write (hit) to M or E state lines | 0x05 |  |
| 8 | Data Cache Lines Written Back | 0x06 |  |
| 9 | External Snoops | 0x07 |  |
| 10 | External Data Cache Snoop Hits | 0x08 |  |
| 11 | Memory Accesses in Both Pipes | 0x09 |  |
| 12 | Bank Conflicts | 0x0a |  |
| 13 | Misaligned Data Memory or I/O References | 0x0b |  |
| 14 | Code Read | 0x0c |  |
| 15 | Code TLB Miss | 0x0d |  |
| 16 | Code Cache Miss | 0x0e |  |
| 17 | Any Segment Register Loaded | 0x0f |  |
| 18 | Branches | 0x12 |  |
| 19 | BTB Hits | 0x13 |  |
| 20 | Taken Branch or BTB Hit | 0x14 |  |
| 21 | Pipeline Flushes | 0x15 |  |
| 22 | Instructions Executed | 0x16 |  |
| 23 | Instructions Executed in the v-pipe e.g. parallelism | 0x17 |  |
| 24 | Locked Bus Cycle | 0x1c |  |
| 25 | I/O Read or Write Cycle | 0x1d |  |
| 26 | Non-cacheable memory reads | 0x1e |  |
| 27 | FLOPs | 0x22 |  |
| 28 | Breakpoint match on DR0 Register | 0x23 |  |

Pentium Processor Events for Counter 1 (continued)

| Index | Event |  | Encoding |
| --- | --- | --- | --- |
| 29 | Breakpoint match on DR1 Register | 0x24 |  |
| 30 | Breakpoint match on DR2 Register | 0x25 |  |
| 31 | Breakpoint match on DR3 Register | 0x26 |  |
| 32 | Hardware Interrupts | 0x27 |  |
| 33 | Clocks while a bus cycle is in progress (bus util.) | 0x18 |  |
| 34 | Number of clocks stalled due to full write buffers | 0x19 |  |
| 35 | Pipeline stalled waiting for data memory read | 0x1a |  |
| 36 | Stall on write to an E or M state line | 0x1b |  |
| 37 | Pipeline stalled due to addr generation interlock | 0x1f |  |
| 38 | Bus ownership transfers | 0x2a |  |
| 39 | MMX instructions executed in V-pipe | 0x2b |  |
| 40 | Number of L1-line sharing | 0x2c |  |
| 41 | MMX ---> FP transitions | 0x2d |  |
| 42 | Writes to non-cacheable memory | 0x2e |  |
| 43 | Saturated MMX operations executed | 0x2f |  |
| 44 | Ex stage stalled due to a D-TLB miss | 0x30 |  |
| 45 | MMX data read misses | 0x31 |  |
| 46 | Taken branches | 0x32 |  |
| 47 | D1 starving but only 1 inst in inst FIFO | 0x33 |  |
| 48 | MMX data write misses | 0x34 |  |
| 49 | Pipeline flushes-wrong branch prediction in WB-Stage | 0x35 |  |
| 50 | MMX wait memory read - pipe stalled | 0x36 |  |
| 51 | Returns predicted (correctly & uncorrectly) | 0x37 |  |
| 52 | MMX store stall due to previous operation | 0x38 |  |
| 53 | RSB overflows | 0x39 |  |
| 54 | BTB miss-prediction or a not-taken branch | 0x3a |  |
| 55 | Stall on MMX write to E or M line | 0x3b |  |
| 56 | Reserved | 0x3c |  |
| 57 | Reserved | 0x3d |  |
| ... |  |  |  |
| 118 | Reserved | 0xFF |  |
| 119 | Reserved | 0xFF |  |

Pentium Pro Processor Events

| Index | Encoding | Umask | Event |
| --- | --- | --- | --- |
| 0 | 0x43 | 0x00 | DCU - Data Mem Refs |
| 1 | 0x45 | 0x00 | DCU - Lines In |
| 2 | 0x46 | 0x00 | DCU - M Lines In |
| 3 | 0x47 | 0x00 | DCU - M Lines Out |
| 4 | 0x48 | 0x00 | DCU - Miss Outstanding |
| 5 | 0x80 | 0x00 | IFU - iFetch |
| 6 | 0x81 | 0x00 | IFU - iFetch Miss |
| 7 | 0x85 | 0x00 | IFU - ITLB Miss |
| 8 | 0x86 | 0x00 | IFU - Mem Stall |
| 9 | 0x87 | 0x00 | IFU - Inst Lrngth Decoder Stall |
|  |  |  | L2 Cache |
| 10 | 0x28 | 0x0F | L2 Instructions Fetches [MESI] |
| 11 | 0x29 | 0x0F | L2 Data Loads [MESI] |
| 12 | 0x2A | 0x0F | L2 Data Stores [MESI] |
| 13 | 0x24 | 0x00 | Lines Allocated in L2 |
| 14 | 0x26 | 0x00 | Lines Removed from L2 |
| 15 | 0x25 | 0x00 | Modified Lines ALLOCATED in L2 |
| 16 | 0x27 | 0x00 | Modified Lines REMOVED from L2 |

Pentium Pro Processor Events (continued)

| Index | Encoding | Umask | Event |
| --- | --- | --- | --- |
| 17 | 0x2E | 0x0F | L2 Requests [MESI] |
| 18 | 0x21 | 0x00 | L2 Address Strobes |
| 19 | 0x22 | 0x00 | Cycles Data Bus was BUSY |
| 20 | 0x23 | 0x00 | Cycles Data Bus was BUSY in READ |
|  |  |  | External Bus Logic (EBL)2 |
| 21 | 0x62 | 0x00 | DRDY is asserted [Self/Processor] |
| 22 | 0x62 | 0x20 | DRDY is asserted [Any/any agent] |
| 23 | 0x63 | 0x00 | LOCK is asserted [Self] |
| 24 | 0x63 | 0x20 | LOCK is asserted [Any ] |
| 25 | 0x60 | 0x00 | BUS requests outstanding |
| 26 | 0x65 | 0x00 | Burst Read transactions [Self] |
| 27 | 0x65 | 0x20 | Burst Read transactions [Any ] |
| 28 | 0x66 | 0x00 | Read for Ownership Transactions [Self] |
| 29 | 0x66 | 0x20 | Read for Ownership Transactions [Any ] |
| 30 | 0x67 | 0x00 | Write Back Transactions [Self] |
| 31 | 0x67 | 0x20 | Write Back Transactions [Any ] |
| 32 | 0x68 | 0x00 | Instruction FETCH Transactions [Self] |
| 33 | 0x68 | 0x20 | Instruction FETCH Transactions [Any ] |
| 34 | 0x69 | 0x00 | BUS Invalidate Transactions [Self] |
| 35 | 0x69 | 0x20 | BUS Invalidate Transactions [Any ] |
| 36 | 0x6A | 0x00 | BUS Partial Writes [Self] |
| 37 | 0x6A | 0x20 | BUS Partial Writes [Any ] |
| 38 | 0x6B | 0x00 | BUS Partial Transactions [Self] |
| 39 | 0x6B | 0x20 | BUS Partial Transactions [Any ] |
| 40 | 0x6C | 0x00 | BUS Trans IO [Self] |
| 41 | 0x6C | 0x20 | BUS Trans IO [Any ] |
| 42 | 0x6D | 0x00 | BUS DEFFERED Transactions [Self] |
| 43 | 0x6D | 0x20 | BUS DEFFERED Transactions [Any ] |
| 44 | 0x6E | 0x00 | BUS BURST Transactions [Self] |
| 45 | 0x6E | 0x20 | BUS BURST Transactions [Any ] |
| 46 | 0x70 | 0x00 | BUS- ALL transactions [Self] |
| 47 | 0x70 | 0x20 | BUS- ALL transactions [Any ] |
| 48 | 0x6F | 0x00 | BUS- Memory Transactions [Self] |
| 49 | 0x6F | 0x20 | BUS- Memory Transactions [Any ] |
| 50 | 0x64 | 0x00 | BUS- cycles processor RECEIVING DATA |
| 51 | 0x61 | 0x00 | BUS- Cycles processor driving BNR pin |
| 52 | 0x7A | 0x00 | BUS- Cycles processor driving HIT pin |
| 53 | 0x7B | 0x00 | BUS- Cycles processor driving HITM pin |
| 54 | 0x7E | 0x00 | BUS is SNOOP Stalled |
|  |  |  | Floating Point Unit |
| 55 | 0xC1 | 0x00 | FP Ops RETIRED [Counter 0 Only] |
| 56 | 0x10 | 0x00 | FP Ops EXECUTED [Counter 0 Only] |
| 57 | 0x11 | 0x00 | FP Assist [Counter 1 Only] |
| 58 | 0x12 | 0x00 | FP MUL [Counter 1 Only] |
| 59 | 0x13 | 0x00 | FP DIV [Counter 1 Only] |
| 60 | 0x14 | 0x00 | FP Cycles Divider is Busy [Counter 0 Only] |
|  |  |  | Memory Ordering |
| 61 | 0x03 | 0x00 | Mem Ord - LD Blocks |
| 62 | 0x04 | 0x00 | Mem Ord - SB Drains |
| 63 | 0x05 | 0x00 | Mem Ord - Misalign Mem Refs |
|  |  |  | Instruction Decoding and Retirement |
| 64 | 0xC0 | 0x00 | INST - Instructions Retired |
| 65 | 0xC2 | 0x00 | INST - uOps Retired |
| 66 | 0xD0 | 0x00 | INST - Instructions decoded |
|  |  |  | Interrupts |
| 67 | 0xC8 | 0x00 | INT - Hardware INTERRUPTS received |
| 68 | 0xC6 | 0x00 | INT - Cycles INterrupts are DISABLED |

Pentium Pro Processor Events (continued)

| Index | Encoding | Umask | Event |
| --- | --- | --- | --- |
| 69 | 0xC7 | 0x00 | INT - Cycles Ints are Pending But disabled |
|  |  |  | Branches |
| 70 | 0xC4 | 0x00 | Branch Insts Retired |
| 71 | 0xC5 | 0x00 | BR Miss PRED Retired |
| 72 | 0xC9 | 0x00 | BR Taken Retired |
| 73 | 0xCA | 0x00 | BR Miss PRED Taken Retired |
| 74 | 0xE0 | 0x00 | BR Insts Decoded |
| 75 | 0xE2 | 0x00 | BRs That miss the BTB |
| 76 | 0xE4 | 0x00 | BR Bogus |
| 77 | 0xE6 | 0x00 | BACLEAR is asserted |
|  |  |  | Stalls |
| 78 | 0xA2 | 0x00 | STALLS - Resource Stalls |
| 79 | 0xD2 | 0x00 | STALLS - Partial RAT Stalls |
|  |  |  | Segment Register Loads |
| 80 | 0x06 | 0x00 | Segment Reg Loads |
|  |  |  | Clocks |
| 81 | 0x79 | 0x00 | CPU CLK UnHalted |
| 82 | 0xFF | 0xFF | Reserved |
| 83 | 0xFF | 0xFF | Reserved |
| 84 | 0xFF | 0xFF | Reserved |
| 85 | 0xFF | 0xFF | Reserved |
| 86 | 0xFF | 0xFF | Reserved |
| 87 | 0xFF | 0xFF | Reserved |
| 88 | 0xFF | 0xFF | Reserved |
| 89 | 0xFF | 0xFF | Reserved |
| 90 | 0xFF | 0xFF | Reserved |
| 91 | 0xFF | 0xFF | Reserved |
| 92 | 0xFF | 0xFF | Reserved |
| ... |  |  |  |
| 118 | 0xFF | 0xFF | Reserved |
| 119 | 0xFF | 0xFF | Reserved |