#### 7.6.1.1	GCC-Conference-Time-Remaining

The GCC-Conference-Time-Remaining request primitive may be used by the Node Controller to announce to all nodes that a certain amount of time is remaining in a timed conference. It may also be issued indicating the time remaining for a specific node, rather than for all nodes. This request is intended to be issued by the Convener, but may be issued by other nodes as well. Typically, this primitive would be issued once, near the end of a timed conference, to indicate that the conference is almost over. Table 7-47 shows the parameters and types of this primitive. Figure 7-38 shows the sequence of events when using this primitive.

Recommendation T.124     (08/95)

TABLE  7-47/T.124

GCC-Conference-Time-Remaining – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Time Remaining | M | M() |  |
| Node ID | O | O() |  |
| Source Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Time Remaining: Indication of time remaining in conference in one second increments.

Node ID: Optional parameter which, if present, indicates that the indicated time-remaining only applies to the specific node listed. If not included, the time-remaining applies to all nodes in the conferences.

Source Node ID: Node ID of the node which issued the request.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-38/T.124...[D41] = 8 CM

#### 7.6.1.2	GCC-Conference-Time-Inquire

The GCC-Conference-Time-Inquire request primitive may be used by any node to find out from the Convener how much time is remaining in the conference. Receipt of the indication form of this primitive by the Convener (if the Convener supports this primitive) results in the time remaining to be broadcast to all nodes in the conference using the GCC-Conference-Time-Remaining primitive. Table 7-48 shows the parameters and types of this primitive. Figure 7-39 shows the sequence of events when using this primitive.

TABLE  7-48/T.124

GCC-Conference-Time-Inquire – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Conference-Wide vs. Node-Specific Time Flag | M | M() |  |
| Requesting Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Conference-Wide vs. Node-Specific Time Flag: This flag indicates if the request is to find out the time remaining for the entire conference, or, if different, the time remaining for the requesting node. If the Convener only considers a single conference-wide end time, it may ignore this flag.

Requesting Node ID: Node ID of the requesting node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-39/T.124...[D42] = 8 CM

#### 7.6.1.3	GCC-Conference-Extend

The GCC-Conference-Extend request primitive may be used by the Node Controller at a node to request from the Convener that more time be added to a timed conference. Receipt of the indication form of this primitive by the Convener (if the Convener supports this primitive) results in a broadcast of the new time remaining in the conference to all nodes using the GCC-Conference-Time-Remaining primitive (even if the time was not actually extended). There is no requirement that the actual amount of time the convener adds to the time-remaining equal the requested time. Table 7-49 shows the parameters and types of this primitive. Figure 7-40 shows the sequence of events when using this primitive.

TABLE  7-49/T.124

GCC-Conference-Extend – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Time Requested | M | M() | M() |
| Conference-Wide vs. Node-Specific Time Flag | M | M() |  |
| Requesting Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Time Requested: In the request and indication, this parameter indicates the desired amount of time to extend the conference in one second increments.

Conference-Wide vs. Node-Specific Time Flag: This flag indicates if the request is to extend the time remaining for the entire conference, or the time remaining for the requesting node. If the Convener only considers a single conference-wide end time, it may ignore this flag.

Requesting Node ID: Node ID of the requesting node.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference, not conductor.

FIGURE 7-40/T.124...[D43] = 8 CM

#### 7.6.1.4	GCC-Conference-Assistance

The GCC-Conference-Assistance primitive provides a simple means to request some form of assistance from a conference operator. Issuing a GCC-Conference-Assistance request results in a GCC-Conference-Assistance indication to be broadcast to the Node Controller at all nodes in the specified conference which support this primitive. The intended response to this primitive is unspecified and outside the scope of this Recommendation. Table 7-50 shows the parameters and types of this primitive. Figure 7-41 shows the sequence of events when using this primitive.

TABLE  7-50/T.124

GCC-Conference-Assistance – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| User Data | O | O() |  |
| Source Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

User Data: Unspecified user data.

Source Node ID: The Node ID of the source of the assistance request.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-41/T.124...[D44] = 8 CM

#### 7.6.1.5	GCC-Text-Message

The GCC-Text-Message primitive provides a simple means to communicate unspecified text messages. Issuing a GCC-Text-Message request results in a GCC-Text-Message indication to be either broadcast to the Node Controller at all nodes in the specified conference which support this primitive, or to be sent to a single node. The intended response to this primitive is to display the text message to the conference participants by some means unspecified by this Recommendation. Table 7-51 shows the parameters and types of this primitive. Figure 7-42 shows the sequence of events when using this primitive.

TABLE  7-51/T.124

GCC-Text-Message – Types of primitives and their parameters

| Parameter | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Conference ID | M | M | M(RQ) |
| Text Message | M | M() |  |
| Destination Node ID | O |  |  |
| Source Node ID |  | M |  |
| Result |  |  | M |

Conference ID: Identifier of the conference to which the primitive refers.

Text Message: Unicode text message.

Destination Node ID: The Node ID of a single node to receive the text message. If no node is specified, the message is broadcast to all nodes in the specified conference.

Source Node ID: The Node ID of the source of the text message.

Result: An indication of whether the request was accepted or rejected, and if rejected, the reason why. It contains one of a list of possible results: successful, invalid conference.

FIGURE 7-42/T.124...[D45] = 8 CM

# 8	GCC Protocol Specification

## 8.1	General operation

A GCC Provider in any node is the MCS Control application, communicating with MCS via the Control MCSAP. On initialization, the GCC Provider shall establish its communication link with MCS through a Control MCSAP through local means.

In all cases, a Conference is established by creating an MCS Domain. A Conference has a one-to-one correspondence with a single MCS Domain. The actual creation of an MCS domain is done through local means. Conferences are created or joined by use of the MCS-Connect-Provider primitives.

For all request primitives which refer to a particular conference (via the Conference ID), the local GCC Provider to which the primitive is issued shall determine if the specified conference is one to which the node is currently joined. If so, the GCC Provider proceeds as described below for each primitive. If not, the request is rejected and the corresponding confirm primitive is issued specifying invalid-conference as the reason for rejection. Any response primitive issued which refers to an invalid conference is ignored by the GCC Provider (or the error is handled by some unspecified local means).

GCC Providers communicate with each other via GCC Protocol Data Units (GCCPDUs). The GCCPDUs are transmitted either via the two MCS data service primitives (MCS-Send-Data or MCS-Uniform-Send-Data; see Table 9-4) or via MCS-Connect-Provider primitives for GCCPDUs used during connection set-up.

All GCCPDUs are categorized into either request, response, or indication classes. Request PDUs are defined to be those that require a corresponding response PDU in return. Indication PDUs are those that do not require a response (or in some cases, where the response is provided indirectly). For request PDUs which do not correspond to mandatory functionality, a generic response PDU is provided (FunctionNotSupportedResponse), which allows the GCC Provider receiving a request to respond to the requesting node without needing to have knowledge of the format of the specific response PDU. To allow the requester to know what this PDU is in response to, the entire request PDU is included within the FunctionNotSupportedResponse PDU. The FunctionNotSupportedResponse PDU shall be sent at the same priority level as that of the received request PDU.

NOTE – This terminology (request, response, and indication) does not have a one-to-one correspondence to the definition of request, indication, response, and confirm used in the definition of the primitives. The terminology has been chosen to relate to the primary purpose of the PDU types with respect to the functions that they will perform. For this reason, the term confirm was not needed to describe PDUs — response PDUs are those which are sourced from a response primitive and result in a confirm primitive.

The GCC protocol includes support for non-standard extensions. On receipt of a nonStandardRequest PDU, a GCC Provider which does not understand the request, shall issue a FunctionNotSupportedResponse PDU in return. nonStandardResponse and nonStandardIndication PDUs may be ignored by a GCC Provider.

## 8.2	Conference establishment and termination

### 8.2.1	Conference creation

On receipt of the primitive GCC-Conference-Create request, a GCC Provider shall issue an MCS-Connect-Provider request primitive with the parameters shown in Table 8-1. The local GCC Provider shall allocate the Conference ID, which shall be used as the local MCS Domain Selector associated with the created conference. The Conference ID is included as the Calling Domain Selector. It is also maintained by the local GCC Provider as the means identifying this conference in future primitives.

If the combination of the Conference Name and Conference Name Modifier parameters are identical to those of a conference to which the local node is already joined (either the numerical or text forms of the name), the request is instead immediately rejected by issuing a GCC-Conference-Create response with conference-name-already-exists as the result. Otherwise, the GCC Provider shall retain the Conference Name and Conference Name Modifier (in addition to including the Conference Name in the ConferenceCreateRequest PDU) to be used in the procedures for responding to a conference query, conference join, or in initiating a conference invite.

The User Data parameter of the MCS-Connect-Provider request contains a structure which includes an Object Identifier identifying the contained PDU as adhering to this Recommendation followed by the PDU itself. The details of this structure are defined in 9.6. The contents of this PDU are shown in Table 8-2.

TABLE  8-1/T.124

MCS-Connect-Provider request parameters for ConferenceCreateRequest PDU

| Parameter | Contents |
| --- | --- |
| Calling Address | From request primitive |
| Calling Domain Selector | Conference ID as chosen by the GCC Provider |
| Called Address | From request primitive |
| Called Domain Selector | NULL |
| Upward/Downward Flag | Up |
| Domain Parameters | From request primitive |
| Quality of Service | From request primitive |
| User Data | T.124 Object Identifier |
|  | ConferenceCreateRequest PDU (See Table 8-2) |

TABLE  8-2/T.124

ConferenceCreateRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Conference Name | Request | Indication and Destination GCC Provider |
| Convener Password (optional) | Request | Indication |
| Password (optional) | Request | Indication |
| Locked Conference Flag | Request | Indication and Destination GCC Provider |
| Listed Conference Flag | Request | Indication and Destination GCC Provider |
| Conductible Conference Flag | Request | Indication and Destination GCC Provider |
| Termination Method | Request | Indication and Destination GCC Provider |
| Conductor Privilege List (optional) | Request | Indication and Destination GCC Provider |
| Conducted-Mode Conference Privilege List (optional) | Request | Indication and Destination GCC Provider |
| Non-Conducted-Mode Conference Privilege List (optional) | Request | Indication and Destination GCC Provider |
| Conference Description (optional) | Request | Indication and Destination GCC Provider |
| Caller Identifier (optional) | Request | Indication |
| User Data (optional) | Request | Indication |

On receipt of the MCS-Connect-Provider indication that includes a T.124 Object Identifier and the ConferenceCreateRequest PDU, a GCC Provider shall generate a GCC-Conference-Create indication primitive with the parameters as specified in the included ConferenceCreateRequest PDU. It shall issue this primitive to the Control GCCSAP. The GCC Provider shall also allocate a Conference ID, a locally unique string, which shall also be included in this primitive. If the conference is successfully created, the GCC Provider shall use this Conference ID as the means of identifying this conference in future primitives. If GCC does not have the resources necessary to create a new conference, it may generate the negative response automatically without generating the GCC-Conference-Create indication. Otherwise, on receipt of a successful GCC-Conference-Create response from the Control GCCSAP, the GCC Provider (now the Top GCC Provider for this conference) shall issue an MCS-Attach-User request. On receipt of the MCS-Attach-User confirm which contains the allocated Node ID, the GCC Provider shall then join the corresponding Node ID Channel by issuing an MCS-Channel-Join request. The GCC Provider shall also join the GCC-Broadcast-Channel by issuing an MCS-Channel-Join request (successive such requests may be issued prior to receiving the previous confirm). It may be necessary, prior to attaching, to locally indicate to the MCS Provider that a new domain has been created. Any exchange necessary to do this is considered a local matter not covered by this Recommendation.

If the GCC-Conference-Create response includes a Conference Name Modifier parameter, the GCC Provider (now the Top GCC Provider) shall retain this name modifier for later use in handling the conference query, conference join, and conference invite procedures.

The GCC Provider shall generate an MCS-Connect-Provider response which includes a result which is either success, or user-rejected depending on whether or not the Result parameter in the GCC-Conference-Create response primitive indicated success or failure. The User Data parameter includes the T.124 Object Identifier as well as the ConferenceCreateResponse PDU. The contents of the connect provider primitive are shown in Table 8-3. In the case of successful conference creation, the GCC Provider at the node receiving the MCS-Connect-Provider indication shall become the Top GCC Provider for the conference.

TABLE  8-3/T.124

MCS-Connect-Provider response parameters for ConferenceCreateResponse PDU

| Parameter | Contents |
| --- | --- |
| Domain Parameters | From response primitive |
| Quality of Service | From response primitive |
| Result | As specified in Rec. T.122 |
| User Data | T.124 Object Identifier |
|  | ConferenceCreateResponse PDU (See Table 8-4) |

The ConferenceCreateResponse PDU is shown in Table 8-4. The Node ID parameter, which is the User ID assigned by MCS in response to the MCS-Attach-User request issued by the GCC Provider, shall be supplied by the GCC Provider sourcing this PDU. The Tag parameter is assigned by the source GCC Provider to be locally unique. It is used to identify the returned UserIDIndication PDU. The Result parameter includes GCC-specific failure information sourced directly from the Result parameter in the GCC-Conference-Create response primitive. If the Result parameter is anything except successful, the Result parameter in the MCS-Connect-Provider response is set to user-rejected.

TABLE  8-4/T.124

ConferenceCreateResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node ID | Top GCC Provider | Destination GCC Provider |
| Tag | Top GCC Provider | Destination GCC Provider |
| Result | Response | Confirm |
| User Data (optional) | Response | Confirm |

On receipt of the ConferenceCreateResponse PDU, if the PDU indicated a successful result, a GCC Provider shall first issue an MCS-Attach-User request. On receipt of the MCS-Attach-User confirm which contains the allocated Node ID, the GCC Provider shall then join the Node ID Channel by issuing an MCS-Channel-Join request. The GCC Provider shall also join the GCC-Broadcast-Channel by issuing an MCS-Channel-Join request. The GCC Provider may also join the GCC-Convener-Channel (if it supports any of the functions which require use of this channel) by issuing an MCS-Channel-Join request. Once the GCC Provider has received an MCS-Channel-Join confirm from each of the channel join requests (successive requests may be issued prior to receiving the previous confirm), it shall send a UserIDIndication PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying as the Channel ID the Node ID of the Top GCC Provider as contained in the received ConferenceCreateResponse PDU, specifying Top data priority, and including the PDU in the Data field. The content of the UserIDIndication PDU is shown in Table 8-5. The Tag parameter is filled in with the value of the corresponding parameter received in the ConferenceCreateResponse PDU.

TABLE  8-5/T.124

UserIDIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Tag | Source GCC Provider | Destination GCC Provider |

The GCC Provider shall then generate a GCC-Conference-Create confirm primitive and issue it to the Control GCCSAP. This primitive shall include the Conference Name from the original request primitive, the Modified Conference Name (if any) and Result parameters from the received PDU, as well as the locally allocated Conference ID. If the received PDU had indicated an unsuccessful result, or if the GCC Provider receives an MCS-Disconnect-Provider indication for this connection prior to having issued a successful GCC-Conference-Create confirm, the GCC-Conference-Create confirm primitive is issued immediately, indicating an unsuccessful result, without issuing the attach-user or channel-join requests, and without sending the UserIDIndication PDU. The Result parameter in the PDU as well as the Result parameter reported in the MCS-Connect-Provider confirm (or the Reason parameter of the MCS-Disconnect-Provider) is used to generate the result reported in the GCC-Conference-Create confirm primitive. If the Result parameter of the MCS-Connect-Provider confirm is user-rejected, the Result parameter in the PDU is used to determine the reported result. Otherwise, the Result parameter in the MCS-Connect-Provider is used directly.

On receipt of a UserIDIndication, the node at which the conference was created shall compare the Tag parameter to its list of outstanding Tags. If the Tag matches one of these, it shall save the User ID of the source node (extracted from the MCS-Send-Data indication) in its data-base of Node IDs of nodes which are directly below it in the connection hierarchy. If the Tag does not match any outstanding Tags, the PDU shall be ignored.

If the newly created Top GCC Provider for this conference receives an MCS-Disconnect-Provider indication for the connection being established at any time during the process of creating the conference (this includes the time that the GCC-Conference-Create indication has been issued to the Control GCCSAP until the time that the UserIDIndication has been received), it shall issue a GCC-Conference-Terminate indication to the Control GCCSAP indicating the requested normal termination as the reason if user-initiated was the reason provided in the MCS-Disconnect-Provider indication, and error termination otherwise. It shall then stop further processing for this connection establishment procedure. If the MCS-Disconnect-Provider indication was received prior to issuing the GCC-Conference-Create indication to the Control GCCSAP, the GCC Provider shall stop further processing for this connection establishment procedure and take no further action.

The sequence of events for successful conference creation is shown in Figure 8-1.

NOTE – If a conference is created by local initiation rather than through a request from another node, the GCC Provider at that node shall perform the sequence of events performed by the Top GCC Provider shown in Figure 8-1 beginning from the MCS-Attach-User request. That is, it shall attach to MCS by issuing an MCS-Attach-User request and wait for the confirm, then it shall join its Node ID channel, and the GCC-Broadcast-Channel. It may optionally join the GCC-Convener-Channel since this node is also the Convener in this case. In this case, if another node joins the conference at a later time indicating itself to be the convener (having issued the correct Convener Password), the Top GCC Provider may choose to accept the connection, relinquishing convener privileges to the new node. Note also that it is assumed that in this case, MCS Domain Parameters are set at the time of creation rather than at the time the first connection is established.

FIGURE 8-1/T.124...[D46] = PAGE PLEINE

### 8.2.2	Querying conferences

On receipt of a GCC-Conference-Query request primitive, a GCC Provider shall issue an MCS-Connect-Provider request primitive with the parameters shown in Table 8-6.

TABLE  8-6/T.124

MCS-Connect-Provider request parameters for ConferenceQueryRequest PDU

| Parameter | Contents |
| --- | --- |
| Calling Address | From request primitive |
| Calling Domain Selector | NULL |
| Called Address | From request primitive |
| Called Domain Selector | NULL |
| Upward/Downward Flag | Up |
| Domain Parameters | Default Domain Parameters |
| Quality of Service | Default Quality of Service Parameters |
| User Data | T.124 Object Identifier |
|  | ConferenceQueryRequest PDU (See Table 8-7) |

The User Data parameter of the MCS-Connect-Provider request contains a structure which includes an Object Identifier identifying the contained PDU as adhering to this Recommendation followed by the PDU itself. The details of this structure are defined in 9.6. The contents of this PDU are shown in Table 8-7.

TABLE  8-7/T.124

ConferenceQueryRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node Type | Request | Indication |
| Asymmetry Indicator (conditional) | Request | Indication |
| User Data (optional) | Request | Indication |

On receipt of the MCS-Connect-Provider indication that includes a T.124 Object Identifier and the ConferenceQueryRequest PDU, a GCC Provider shall generate a GCC-Conference-Query indication primitive with the parameters as specified in the ConferenceQueryRequest PDU. It shall issue this primitive to the Control GCCSAP. On receipt of a GCC-Conference-Query response from the Control GCCSAP, the GCC Provider shall send an MCS-Connect-Provider response which is rejected (i.e. no conference is set up), but includes the ConferenceQueryResponse PDU in the User Data field. The parameters of the MCS-Connect-Provider response are shown in Table 8-8.

TABLE  8-8/T.124

MCS-Connect-Provider response parameters for ConferenceQueryResponse PDU

| Parameter | Contents |
| --- | --- |
| Domain Parameters | Default Domain Parameters |
| Quality of Service | Default Quality of Service Parameters |
| Result | User-rejected |
| User Data | T.124 Object Identifier |
|  | ConferenceQueryResponse PDU (See Table 8-9) |

The ConferenceQueryResponse PDU is shown in Table 8-9. The Conference Descriptor List contains an entry for each listed conference to which the queried node is currently joined. The Conference Descriptor List is generated by the GCC Provider sourcing the ConferenceQueryResponse PDU. The descriptor is filled in with the Conference Name of each conference to which the GCC Provider is joined, the locally maintained Conference Name Modifier for each conference, if any, the Conference Description, if any, the Locked/Unlocked flag, the Password In The Clear Required indicator, and the Network Address field, if one is available. The Network Address field, if one exists, is sourced from the Local Network Address parameter of the GCC-Conference-Create request, the GCC-Conference-Create response, the GCC-Conference-Join request, or the GCC-Conference-Invite response, depending on how the conference was joined at this node. The Result parameter shall indicate success if the query request is able to be fulfilled (even if the Conference List is empty), or user-rejected if so indicated in the response primitive.

If the GCC Provider processing the MCS-Connect-Provider indication receives an MCS-Disconnect-Provider indication for the same connection at any time during the process, the GCC Provider shall stop further processing for this procedure and take no further action.

TABLE  8-9/T.124

ConferenceQueryResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node Type | Response | Confirm |
| Asymmetry Indicator (conditional) | Response | Confirm |
| Conference Descriptor List | Source GCC Provider | Confirm |
| Result | Response | Confirm |
| User Data (optional) | Response | Confirm |

On receipt of an MCS-Connect-Provider response which includes the T.124 Object Identifier and a ConferenceQueryResponse PDU, a GCC Provider shall generate a GCC-Conference-Query confirm primitive and issue it to the Control GCCSAP. The content of the confirm primitive shall be obtained from the parameters of the ConferenceQueryResponse PDU. The Result parameter in the primitive, in particular, is obtained strictly from the Result parameter in the received PDU. The Result parameter of the MCS-Connect-Provider confirm is ignored since it would be set to user-rejected even in the case of a successful operation.

If the received PDU had indicated an unsuccessful result, or if the GCC Provider receives an MCS-Disconnect-Provider indication for this connection prior to having issued a successful GCC-Conference-Query confirm, the GCC-Conference-Query confirm primitive is issued immediately, indicating an unsuccessful result. The Result parameter in the PDU as well as the Result parameter reported in the MCS-Connect-Provider confirm (or the Reason parameter of the MCS-Disconnect-Provider) is used to generate the result reported in the GCC-Conference-Query confirm primitive. If the Result parameter of the MCS-Connect-Provider confirm is user-rejected, the Result parameter in the PDU is used to determine the reported result. Otherwise, the Result parameter in the MCS-Connect-Provider is used directly.

### 8.2.3	Joining a conference

On receipt of the primitive GCC-Conference-Join request, a GCC Provider shall issue an MCS-Connect-Provider request primitive with the parameters shown in Table 8-10. The local GCC Provider allocates the Conference ID, which shall be used as the local MCS Domain Selector associated with the created conference. The Conference ID is sent as part of the MCS-Connect-Provider request as the Calling Domain Selector.

TABLE  8-10/T.124

MCS-Connect-Provider request parameters for ConferenceJoinRequest PDU

| Parameter | Contents |
| --- | --- |
| Calling Address | From request primitive |
| Calling Domain Selector | Conference ID as chosen by the GCC Provider |
| Called Address | From request primitive |
| Called Domain Selector | NULL |
| Upward/Downward Flag | Up |
| Domain Parameters | From request primitive |
| Quality of Service | From request primitive |
| User Data | T.124 Object Identifier |
|  | ConferenceJoinRequest PDU (See Table 8-11) |

The User Data parameter of the MCS-Connect-Provider request contains a structure which includes an Object Identifier identifying the contained PDU as adhering to this Recommendation followed by the PDU itself. The details of this structure are defined in 9.6. The contents of this PDU are shown in Table 8-11. From the joining node, the Tag parameter shall not be included. The Conference Name and Conference Name Modifier are sourced from the request primitive — the Conference Name Modifier parameter in the PDU is sourced from the Called Node Conference Name Modifier parameter of the primitive. The Calling Node Conference Name Modifier (also from the request primitive) is maintained at the local GCC Provider as the identifiers of the conference to be used in the procedures for response to a conference join request and conference query request.

On receipt of the MCS-Connect-Provider indication that includes a T.124 Object Identifier and the ConferenceJoinRequest PDU, the action of the GCC Provider depends on whether or not it is the Top GCC Provider of the conference specified by the Conference Name and Conference Name Modifier (if any). The GCC Provider determines which conference is to be joined by comparing the Conference Name and Conference Name Modifier to the list of conferences to which this node is joined. The Conference Name received in the ConferenceJoinRequest PDU may contain either the numerical or text forms of the Conference Name. The GCC Provider shall attempt to match the indicated name with the corresponding portion of the names of existing conferences. For a conference to be considered matched, both the Conference Name and Conference Name Modifier must match those of a current conference. If no Conference Name Modifier is given in the request, the GCC Provider shall match it only with an existing conference which also has no Conference Name Modifier.

TABLE  8-11/T.124

ConferenceJoinRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Conference Name (conditional) | Request | GCC Provider receiving MCS-Connect-Provider indication |
| Conference Name Modifier (optional) | Request | GCC Provider receiving MCS-Connect-Provider indication |
| Tag (conditional) | GCC Provider receiving MCS-Connect-Provider indication | Top GCC Provider |
| Password (optional) | Request | Indication |
| Convener Password (optional) | Request | Indication |
| Caller Identifier (optional) | Request | Indication |
| User Data (optional) | Request | Indication |

If the GCC Provider which received the MCS-Connect-Provider indication is not the Top GCC Provider for the conference to which the requesting node wishes to join, and if the node is joined to the specified conference, the GCC Provider shall forward the ConferenceJoinRequest PDU to the Top-GCC-Provider of the specified conference by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying Top data priority, and including the PDU in the Data field. In this case, it shall assign a locally unique identifier and include it in the Tag parameter of the PDU. This number is used to identify the corresponding response PDU when it is returned from the Top GCC Provider. It may also leave out the Conference Name and Conference Name Modifier parameters from the received PDU as they are not needed by the Top GCC Provider.

When the Top-GCC-Provider receives this PDU, if the conference is not locked, it shall generate a GCC-Conference-Join indication primitive with the parameters as specified in the ConferenceJoinRequest PDU, as well as the Conference ID for the corresponding conference. It shall issue this primitive to the Control GCCSAP. Note that the Conference Name and Conference Name Modifier parameters are used only by the node receiving the MCS-Connect-Provider indication, and not by the Top GCC Provider. If the conference had been locked, the Top GCC Provider shall instead send a ConferenceJoinResponse PDU to the originator of the MCS-Send-Data containing the ConferenceJoinRequest PDU (not the original requester) by issuing an MCS-Send-Data request specifying the User ID of that node as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The PDU shall indicate locked-conference as the result code. Otherwise, on receipt of the GCC-Conference-Join response, the GCC Provider shall send a ConferenceJoinResponse PDU to the originator of the MCS-Send-Data containing the ConferenceJoinRequest PDU (not the original requester) by issuing an MCS-Send-Data request specifying the User ID of that node as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The contents of the ConferenceJoinResponse PDU are shown in Table 8-13. The Node ID is not included in this portion of the response, but the Top Node ID parameter, the Tag, and the parameters associated with the Conference Profile are included, as well as the Result parameter. The Tag shall have the same value as the corresponding parameter in the received ConferenceJoinRequest PDU.

On receipt of the MCS-Send-Data indication containing this PDU, if the Tag parameter matches that of its locally stored list of outstanding join requests (which allows it to identify the connection over which to send the MCS-Connect-Provider response), the GCC Provider shall generate an MCS-Connect-Provider response which includes a result which is either success, or user-rejected depending on whether or not the Result parameter in the received PDU indicated success or failure. The User Data parameter includes the T.124 Object Identifier as well as the ConferenceJoinResponse PDU. The Node ID parameter of the ConferenceJoinResponse PDU is filled in at this time indicating the Node ID of the node directly connected to the joining node. The contents of the connect provider primitive are shown in Table 8-12.

If the receiver of the original MCS-Connect-Provider indication is the Top-GCC-Provider, if the conference is not locked, it shall instead generate a GCC-Conference-Join indication primitive with the parameters as specified in the ConferenceJoinRequest PDU, as well as the Conference ID for the designated conference. It shall issue this primitive to the Control GCCSAP. If the conference had been locked, the Top GCC Provider shall instead generate an MCS-Connect-Provider response which indicates in the included ConferenceJoinResponse PDU locked-conference as the result code. The PDU shall indicate locked-conference as the result code. Otherwise, on receipt of the GCC-Conference-Join response, the GCC Provider generates an MCS-Connect-Provider response which includes the ConferenceJoinResponse PDU with a result which is either success, or user-rejected depending on whether or not the Result parameter in the GCC-Conference-Join response primitive indicated success or failure.

If the receiver of the original MCS-Connect-Provider indication is not currently joined to any conference (or if it does not support the GCC-Conference-Join indication primitive at all) it may reject the join request by immediately issuing an MCS-Connect-Provider response containing user-rejected as the result code, and including a ConferenceJoinResponse with invalid-conference respectively as the reason code in the PDU.

If the receiver of the original MCS-Connect-Provider indication is not joined to the conference specified in the Conference Name and Conference Name Modifier parameter of the ConferenceJoinRequest PDU, it shall reject the request by issuing an MCS-Connect-Provider response with the reason of no-such-domain.

TABLE  8-12/T.124

MCS-Connect-Provider response parameters for ConferenceJoinResponse PDU

| Parameter | Contents |
| --- | --- |
| Domain Parameters | From GCC Provider as previously saved |
| Quality of Service | From GCC Provider as previously saved |
| Result | As specified in Rec. T.122 |
| User Data | T.124 Object Identifier |
|  | ConferenceJoinResponse PDU (See Table 8-13) |

The ConferenceJoinResponse PDU is shown in Table 8-13. The Node ID parameter, which is the User ID assigned by MCS in response to the MCS-Attach-User request issued by the GCC Provider, shall be supplied by the GCC Provider at the node directly connected to the joining node. The Tag parameter is filled in by the GCC Provider at the node directly connected to the joining node with a locally unique value. It is used to identify the returned UserIDIndication PDU. The value of this parameter may be set to the value of the same parameter received from the Top GCC Provider since this parameter was allocated by this node originally as sent in the ConferenceJoinRequest PDU. If this is done, it requires that Tags used for either purpose are locally unique. The other parameters in this PDU are filled out by the Top GCC Provider. This includes the Top Node ID, which is the Node ID of the Top GCC Provider, as well as the parameters associated with the Conference Profile. Also filled in by the Top GCC Provider is the Conference Name Alias. This is conditionally included depending on whether the Conference Name included both numeric and text forms. If so, the Conference Name Alias is whichever form of the Conference Name was not included in the ConferenceJoinRequest PDU. The Result parameter includes GCC-specific failure information if the Result parameter in the MCS-Connect-Provider message is set to user-rejected. This information is from the Result parameter in the GCC-Conference-Create response PDU.

TABLE  8-13/T.124

ConferenceJoinResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node ID (conditional) | GCC Provider of node directly connected to joining node | Destination GCC Provider |
| Top Node ID | Top GCC Provider | Destination GCC Provider |
| Tag | Top GCC Provider | Destination GCC Provider |
| Conference Name Alias (conditional) | Top GCC Provider | Confirm and Destination GCC Provider |
| Password In The Clear Required Flag | Top GCC Provider | Confirm and Destination GCC Provider |
| Locked Conference Flag | Top GCC Provider | Confirm and Destination GCC Provider |
| Listed Conference Flag | Top GCC Provider | Confirm and Destination GCC Provider |
| Conductible Conference Flag | Top GCC Provider | Confirm and Destination GCC Provider |
| Termination Method | Top GCC Provider | Confirm and Destination GCC Provider |
| Conductor Privilege List (optional) | Top GCC Provider | Confirm and Destination GCC Provider |
| Conducted-Mode Conference Privilege List (optional) | Top GCC Provider | Confirm and Destination GCC Provider |
| Non-Conducted-Mode Conference Privilege List (optional) | Top GCC Provider | Confirm and Destination GCC Provider |
| Conference Description (optional) | Top GCC Provider | Confirm and Destination GCC Provider |
| Password (optional) | Request | Indication |
| Result | Response | Confirm |
| User Data (optional) | Response | Confirm |

On receipt of the ConferenceJoinResponse PDU, if the PDU indicated a successful result, a GCC Provider shall first issue an MCS-Attach-User request. On receipt of the MCS-Attach-User confirm which contains the allocated Node ID, the GCC Provider shall then join the corresponding Node ID Channel by issuing an MCS-Channel-Join request. The GCC Provider shall also join the GCC-Broadcast-Channel by issuing an MCS-Channel-Join request. If the original GCC-Conference-Join request had specified the Convener Password, indicating that the node was to regain its role as the Conference Convener, the GCC Provider may also join the GCC-Convener-Channel (if it supports any of the functions which require use of this channel) by issuing an MCS-Channel-Join request. Once the GCC Provider has received an MCS-Channel-Join confirm from each of the channel join requests (successive requests may be issued prior to receiving the previous confirm), it shall send a UserIDIndication PDU to the GCC Provider of the directly connected node by issuing an MCS-Send-Data request specifying the Node ID of the directly connected node as contained in the received ConferenceJoinResponse PDU, specifying Top data priority, and including the PDU in the Data field. The content of the UserIDIndication PDU is shown in Table 8-5. The Tag parameter is filled in with the value of the corresponding parameter received in the ConferenceJoinResponse PDU.

The GCC Provider shall then generate a GCC-Conference-Join confirm primitive and issue it to the Control GCCSAP. This primitive shall include the Result parameters from the received PDU as well as the locally allocated Conference ID. If the received PDU had indicated an unsuccessful result, or if the GCC Provider receives an MCS-Disconnect-Provider indication for this connection prior to having issued a successful GCC-Conference-Join confirm, the GCC-Conference-Join confirm primitive is issued immediately, indicating an unsuccessful result, without issuing the attach-user or channel-join requests, or sending the UserIDIndication PDU. The Result parameter in the PDU as well as the Result parameter reported in the MCS-Connect-Provider confirm (or the Reason parameter of the MCS-Disconnect-Provider) is used to generate the result reported in the GCC-Conference-Join confirm primitive. If the Result parameter of the MCS-Connect-Provider confirm is user-rejected, the Result parameter in the PDU is used to determine the reported result. Otherwise, the Result parameter in the MCS-Connect-Provider is used directly.

The Conference Name Alias received in the ConferenceJoinResponse PDU, if any, is appended with the Conference Name included in the ConferenceJoinRequest PDU to form the full Conference Name returned in the GCC-Conference-Join confirm. This full conference name (which includes the numerical form as well as text form of the Conference Name, if any) shall be maintained at the local GCC Provider as the identifiers of this conference to be used in the procedures for response to a conference join request, conference query request, and in initiation of a conference invite.

On receipt of a UserIDIndication, the node directly connected to the joining node shall compare the Tag parameter to its list of nodes in the conference from which it is expecting this PDU. If the Tag matches one of these, it shall save the User ID of the source node (extracted from the MCS-Send-Data indication) in its data-base of Node IDs of nodes which are directly below it in the connection hierarchy. If the Tag does not match any outstanding Tags, the PDU shall be ignored.

If the Top GCC Provider receives a UserIDIndication as a result of a successful join operation in which the Convener Password parameter had been included in the ConferenceJoinRequest PDU, the Top GCC Provider shall retain the Node ID of the joining node as indicated in the UserIDIndication as the valid conference convener. It shall use this to verify later requests which only the convener may perform.

If the GCC Provider at the node directly connected to the joining node receives an MCS-Disconnect-Provider indication for the connection being established at any time during the joining process, the GCC Provider shall stop further processing for this procedure and take no further action.

The sequence of events for successfully joining a conference from a node directly connected to the Top GCC Provider is shown in Figure 8-2. The case that the joining node is not directly connected to the Top GCC Provider is shown in Figure 8-3.

### 8.2.4	Inviting a node to a conference

On receipt of a GCC-Conference-Invite request primitive, a GCC Provider shall issue an MCS-Connect-Provider request primitive with the parameters shown in Table 8-14.

The User Data parameter of the MCS-Connect-Provider request contains a structure which includes an Object Identifier identifying the contained PDU as adhering to this Recommendation followed by the PDU itself. The details of this structure are defined in 9.6. The contents of this PDU are shown in Table 8-15. The Conference Name is the name of the conference specified by the Conference ID in the request primitive as stored in the local Conference Profile. The Node ID parameter, which is the User ID assigned by MCS in response to the MCS-Attach-User request issued by the GCC Provider, shall be supplied by the source GCC Provider. The Top Node ID is the Node ID of the Top GCC Provider, previously saved by the inviting GCC Provider at the inviting node. The Tag parameter is assigned by the source GCC Provider to be locally unique. It is used to identify the returned UserIDIndication PDU.

On receipt of the MCS-Connect-Provider indication that includes a T.124 Object Identifier and the ConferenceInviteRequest PDU, a GCC Provider shall generate a GCC-Conference-Invite indication primitive and issue it to the Control GCCSAP. The Conference ID in this primitive shall be assigned locally by the GCC Provider and shall be used as the local MCS Domain Selector. If GCC does not have the resources necessary to join a conference, it may generate the negative response automatically without generating the GCC-Conference-Invite indication. Otherwise, on receipt of the GCC-Conference-Invite response, the GCC Provider shall generate an MCS-Connect-Provider response which includes a result which is either success, or user-rejected depending on whether or not the Result parameter in the GCC-Conference-Invite response primitive indicated success or failure (and shall include the result in the ConferenceInviteResponse PDU). The User Data parameter includes the T.124 Object Identifier as well as the ConferenceInviteResponse PDU. The contents of the connect provider primitive are shown in Table 8-16.

FIGURE 8-2/T.124...[D47] = 20 CM

FIGURE 8-3/T.124...[D48] = 20 CM

TABLE  8-14/T.124

MCS-Connect-Provider request parameters for ConferenceInviteRequest PDU

| Parameter | Contents |
| --- | --- |
| Calling Address | From request primitive |
| Calling Domain Selector | Conference ID from request primitive |
| Called Address | From request primitive |
| Called Domain Selector | NULL |
| Upward/Downward Flag | Down |
| Domain Parameters | From GCC Provider as previously saved |
| Quality of Service | From GCC Provider as previously saved |
| User Data | T.124 Object Identifier |
|  | ConferenceInviteRequest PDU (See Table 8-15) |

TABLE  8-15/T.124

ConferenceInviteRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Conference Name | Source GCC Provider | Destination GCC Provider and Indication |
| Node ID | Source GCC Provider | Destination GCC Provider |
| Top Node ID | Source GCC Provider | Destination GCC Provider |
| Tag | Source GCC Provider | Destination GCC Provider |
| Password In The Clear Required Flag | Source GCC Provider | Indication and Destination GCC Provider |
| Locked Conference Flag | Source GCC Provider | Indication and Destination GCC Provider |
| Listed Conference Flag | Source GCC Provider | Indication and Destination GCC Provider |
| Conductible Conference Flag | Source GCC Provider | Indication and Destination GCC Provider |
| Termination Method | Source GCC Provider | Indication and Destination GCC Provider |
| Conductor Privilege List (optional) | Source GCC Provider | Indication and Destination GCC Provider |
| Conducted-Mode Conference Privilege List (optional) | Source GCC Provider | Indication and Destination GCC Provider |
| Non-Conducted-Mode Conference Privilege List (optional) | Source GCC Provider | Indication and Destination GCC Provider |
| Conference Description (optional) | Source GCC Provider | Indication and Destination GCC Provider |
| Caller Identifier (optional) | Request | Indication |
| User Data (optional) | Request | Indication |

If the successful response includes the Conference Name Modifier parameter, the GCC Provider shall maintain this as well as the Conference Name parameter from the received ConferenceInviteRequest PDU (which includes both the numerical form as well as the text form of the Conference Name, if any) together as the local identifier of the conference to be used in the procedure for responding to a conference join request, a conference query request, or in initiating a conference invite request. If no Conference Name Modifier is present, only the Conference Name is maintained for this purpose.

After sending the MCS-Connect-Provider response, if the invitation was acknowledged as successful, the GCC Provider shall issue an MCS-Attach-User request. On receipt of the MCS-Attach-User confirm which contains the allocated Node ID, the GCC Provider shall then join the corresponding Node ID Channel by issuing an MCS-Channel-Join request. The GCC Provider shall also join the GCC-Broadcast-Channel by issuing MCS-Channel-Join requests. Once the GCC Provider has received an MCS-Channel-Join confirm from each of the channel join requests (successive requests may be issued prior to receiving the previous confirm), it shall send a UserIDIndication PDU to the GCC Provider of the directly connected node by issuing an MCS-Send-Data request specifying the Node ID of the directly connected node as contained in the received ConferenceInviteRequest PDU, specifying Top data priority, and including the PDU in the Data field. The content of the UserIDIndication PDU is shown in Table 8-5. The Tag parameter is filled in with the value of the corresponding parameter received in the ConferenceInviteRequest PDU.

If the GCC Provider at the invited node receives an MCS-Disconnect-Provider indication for the connection being established at any time during the process of being joined to the conference (this includes the time that the GCC-Conference-Invite indication has been issued to the Control GCCSAP until the time that the UserIDIndication has been transmitted), it shall issue a GCC-Conference-Terminate indication to the Control GCCSAP indicating the requested normal termination as the reason if user-initiated was the reason provided in the MCS-Disconnect-Provider indication, and error termination otherwise. It shall then stop further processing for this connection establishment procedure. If the MCS-Disconnect-Provider indication was received prior to issuing the GCC-Conference-Invite indication to the Control GCCSAP, the GCC Provider shall stop further processing for this connection establishment procedure and take no further action.

TABLE  8-16/T.124

MCS-Connect-Provider response parameters for ConferenceInviteResponse PDU

| Parameter | Contents |
| --- | --- |
| Domain Parameters | From response primitive |
| Quality of Service | From response primitive |
| Result | As specified in Rec. T.122 |
| User Data | T.124 Object Identifier |
|  | ConferenceInviteResponse PDU (See Table 8-17) |

The ConferenceInviteResponse PDU is shown in Table 8-17.

TABLE  8-17/T.124

ConferenceInviteResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Result | Response | Confirm |
| User Data (optional) | Response | Confirm |

On receipt of the ConferenceInviteResponse PDU, if the result parameter of the MCS-Connect-Provider was successful a GCC Provider shall record the User Data parameter to fill in to the GCC-Conference-Invite confirm primitive once it is generated on receipt of the pending UserIDIndication. If the result parameter was unsuccessful, or if the GCC Provider receives an MCS-Disconnect-Provider indication for this connection prior to having issued a successful GCC-Conference-Invite confirm, the GCC Provider shall immediately generate a GCC-Conference-Invite confirm primitive and issue it to the Control GCCSAP. The Result parameter in the PDU as well as the Result parameter reported in the MCS-Connect-Provider confirm (or the Reason parameter of the MCS-Disconnect-Provider) is used to generate the result reported in the GCC-Conference-Invite confirm primitive. If the Result parameter of the MCS-Connect-Provider confirm is user-rejected, the Result parameter in the PDU is used to determine the reported result. Otherwise, the Result parameter in the MCS-Connect-Provider is used directly.

On receipt of a UserIDIndication, the node directly connected to the invited node shall compare the Tag parameter to its list of nodes in the conference from which it is expecting this PDU. If the Tag matches one of these, it shall save the User ID of the source node (extracted from the MCS-Send-Data indication) in its data-base of Node IDs of nodes which are directly below it in the connection hierarchy. If the Tag does not match any outstanding Tags, the PDU shall be ignored.

If the Tag was from an outstanding conference invite, on receipt of the UserIDIndication, the GCC Provider shall generate a GCC-Conference-Invite confirm primitive and issue it to the Control GCCSAP with a successful result parameter.

The sequence of events for successfully inviting a node to a conference is shown in Figure 8-4.

### 8.2.5	Requesting to add a node to a conference

On receipt of a GCC-Conference-Add request primitive, a GCC Provider shall send a ConferenceAddRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The contents of the ConferenceAddRequest PDU are shown in Table 8-18. The contents are filled in from the parameters passed in the GCC-Conference-Add request primitive. The Tag parameter is assigned by the requesting GCC Provider to be locally unique. This number is used to identify the corresponding response PDU when it is returned.

On receipt of a ConferenceAddRequest PDU, the Top GCC Provider shall first check whether the requesting node had the necessary privilege to add a node to the conference as indicated in the Conference Profile. If not, it shall reject the request by sending a ConferenceAddResponse PDU to the requesting node indicating invalid-requester as the result. The content of the ConferenceAddResponse PDU is shown in Table 8-19. If the requesting node had sufficient privilege, the Top GCC Provider shall then examine whether the Adding MCU parameter is present. If not present, or if the Adding MCU identifier is equal to the Node ID of the Top GCC Provider, the Top GCC Provider, if it supports the GCC-Conference-Add function, shall generate a GCC-Conference-Add indication primitive and issue it to the Control GCCSAP. The contents of the primitive are filled in from the contents of the received PDU. If the node does not support this function, it may reject the request by issuing a FunctionNotSupportedResponse PDU including the received ConferenceAddRequest in the response.

If the optional Adding MCU parameter is present and set to a value other than that of the Node ID of the Top GCC Provider, the Top GCC Provider shall send a ConferenceAddRequest PDU to the Adding MCU by issuing an MCS-Send-Data request specifying the Node ID of the Adding MCU as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the PDU is the same as that of the received request PDU. On receipt of a ConferenceAddRequest PDU, a node which is not the Top GCC Provider, if it supports the GCC-Conference-Add function, shall first check that it has been received from the Top GCC Provider by examining the User ID from the received MCS-Send-Data indication. If it has been received from the Top GCC Provider, it shall generate a GCC-Conference-Add indication primitive and issue it to the Control GCCSAP. The contents of the primitive are filled in from the contents of the received PDU. If the User ID of the received PDU does not match the Node ID of the Top GCC Provider, the received PDU is ignored and no further action is taken. If the node does not support this function, it may reject the request by sending to the Top GCC Provider a FunctionNotSupportedResponse PDU including the received ConferenceAddRequest in the response.

FIGURE 8-4/T.124...[D49] = 19 CM

On receipt of a GCC-Conference-Add response primitive, a GCC Provider shall send a ConferenceAddResponse PDU to the requesting node by issuing an MCS-Send-Data request specifying the Node ID of the requesting node, specifying High data priority, and including the PDU in the Data field. In the case that the request had been routed through the Top GCC Provider to another node, the response is issued to the original requesting node as indicated by the Requesting Node parameter of the request PDU. The contents of the ConferenceAddResponse PDU are shown in Table 8-19. The contents are filled in from the parameters passed in the GCC-Conference-Add response primitive. The Tag shall have the same value as the corresponding parameter in the received ConferenceAddRequest PDU.

On receipt of a ConferenceAddResponse PDU, a GCC Provider shall generate a GCC-Conference-Add confirm primitive and issue it to the Control GCCSAP. The contents of the primitive are obtained from the parameters of the received PDU.

TABLE  8-18/T.124

ConferenceAddRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Network Address | Request | Indication |
| Requesting Node | Requesting GCC Provider | Indication |
| Tag | Requesting GCC Provider | Destination GCC Provider |
| Adding MCU (optional) | Request | Top GCC Provider |
| User Data (optional) | Request | Indication |

TABLE  8-19/T.124

ConferenceAddResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Tag | Source GCC Provider | Destination GCC Provider |
| Result | Response | Confirm |
| User Data (optional) | Response | Confirm |

### 8.2.6	Locking a Conference

On receipt of a GCC-Conference-Lock request primitive, a GCC Provider shall send a ConferenceLockRequest by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceLockRequest PDU is shown in Table 8-20. There are no parameters in this PDU.

TABLE  8-20/T.124

ConferenceLockRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| -- No parameters -- |  |  |

On receipt of a ConferenceLockRequest PDU, if the Top GCC Provider supports the conference lock capability, it shall first determine if the requesting node has the privilege necessary to lock the conference based on the lock-unlock-privileges defined when the conference was created. If so, the Top GCC Provider shall generate a GCC-Conference-Lock indication primitive and issue it to the Control GCCSAP. The Source Node specified in the primitive shall be obtained from the Sender User ID in the MCS-Send-Data indication. On receipt of a GCC-Conference-Lock response, the GCC Provider shall send a ConferenceLockResponse PDU by issuing an MCS-Send-Data request specifying the Source Node indicated in the response as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceLockResponse PDU is shown in Table 8-21. The Result parameter is generated from the result returned in the GCC-Conference-Lock response.

If the requesting node did not have the proper privilege to support this operation, the request is immediately rejected without generating a GCC-Conference-Lock indication. This is done by generating a ConferenceLockResponse PDU specifying invalid-requester as the Result.

The Top GCC Provider shall preserve the order of received ConferenceLockRequest PDUs and ConferenceUnlockRequest PDUs and their corresponding indication primitives issued to the Control GCCSAP,
as well as between the response primitives from the Control GCCSAP and the ConferenceLockResponse and ConferenceUnlockResponse PDUs transmitted.

At the requesting node, order shall also be preserved between the request primitives and the corresponding
transmitted ConferenceLockRequest PDUs and ConferenceUnlockRequest PDUs as well as between the received ConferenceLockResponse and ConferenceUnlockResponse PDUs and the corresponding confirm primitives.

If the Top GCC Provider does not support the conference lock capability, on receipt of a ConferenceLockRequest PDU, it shall immediately generate a FunctionNotSupportedResponse PDU including the received ConferenceLockRequest PDU in the response.

TABLE  8-21/T.124

ConferenceLockResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Result | Response | Confirm |

On receipt of a ConferenceLockResponse PDU, a GCC Provider shall generate a GCC-Conference-Lock confirm primitive and issue it to the Control GCCSAP. The result parameter in the confirm primitive is obtained from the Result field in the PDU.

If the Top GCC Provider sends a ConferenceLockResponse which indicates a successful result, it shall also generate a ConferenceLockIndication PDU and send it to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceLockIndication PDU is shown in Table 8-22. This PDU contains no parameters.

TABLE  8-22/T.124

ConferenceLockIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| -- No parameters -- |  |  |

On receipt of a ConferenceLockIndication PDU, a GCC Provider which supports the optional lock indication may generate a GCC-Conference-Lock-Report indication and issue it to the Control GCCSAP. Before it does so, it shall examine the User ID as indicated in the received MCS-Uniform-Send-Data indication and compare it to the Node ID of the Top GCC Provider. The GCC-Conference-Lock-Report primitive may only be generated if the received User ID matches the Node ID of the Top GCC Provider. Otherwise, the received PDU is ignored, and no further action is taken.

If the Top GCC Provider becomes aware of a new node entering a conference by its presence in the Conference Roster and the conference is currently locked and if the possibility exists that the conference may have been unlocked when the node joined (or was invited to) the conference, the Top GCC Provider shall generate a ConferenceLockIndication PDU and send it to the new node by issuing MCS-Send-Data request specifying the Node ID of that node as the Channel ID, specifying High data priority, and including the PDU in the Data field. Alternatively, it may send this PDU to all nodes by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. Although the flag indicating whether the conference is locked or unlocked is included in the information provided to the newly joined node as part of the connection establishment process, this ensures that if the conference changed its lock state since the connection had been established, that the node is properly notified of the change. If there is no possibility that the lock state has changed since the node was joined (or was invited) into the conference (e.g. if the lock state has not changed since the creation of the conference), the Top GCC Provider need not send this PDU.

### 8.2.7	Unlocking a conference

On receipt of a GCC-Conference-Unlock request primitive, a GCC Provider shall send a ConferenceUnlockRequest by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceUnlockRequest PDU is shown in Table 8-23. There are no parameters in this PDU.

TABLE  8-23/T.124

ConferenceUnlockRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| -- No parameters -- |  |  |

On receipt of a ConferenceUnlockRequest PDU, if the Top GCC Provider supports the conference unlock capability, it shall first determine if the requesting node has the privilege necessary to unlock the conference based on the lock-unlock-privileges defined when the conference was created. If so, the Top GCC Provider shall generate a GCC-Conference-Unlock indication primitive and issue it to the Control GCCSAP. The Source Node specified in the primitive shall be obtained from the Sender User ID in the MCS-Send-Data indication. On receipt of a GCC-Conference-Unlock response, the GCC Provider shall send a ConferenceUnlockResponse PDU by issuing an MCS-Send-Data request specifying the Source Node indicated in the response as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceUnlockResponse PDU is shown in Table 8-24. The Result parameter is generated from the result returned in the GCC-Conference-Unlock response.

If the requesting node did not have the proper privilege to support this operation, the request is immediately rejected without generating a GCC-Conference-Unlock indication. This is done by generating a ConferenceUnlockResponse PDU specifying invalid-requester as the Result.

The Top GCC Provider shall preserve the order of received ConferenceLockRequest PDUs and ConferenceUnlockRequest PDUs and their corresponding indication primitives issued to the Control GCCSAP, as well as between the response primitives from the Control GCCSAP and the ConferenceLockResponse and ConferenceUnlockResponse PDUs transmitted.

At the requesting node, order shall also be preserved between the request primitives and the corresponding
transmitted ConferenceLockRequest PDUs and ConferenceUnlockRequest PDUs as well as between the received ConferenceLockResponse and ConferenceUnlockResponse PDUs and the corresponding confirm primitives.

If the Top GCC Provider does not support the conference unlock capability, on receipt of a ConferenceUnlockRequest PDU, it shall immediately generate a FunctionNotSupportedResponse PDU including the received ConferenceUnlockRequest PDU in the response.

TABLE  8-24/T.124

ConferenceUnlockResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Result | Response | Confirm |

On receipt of a ConferenceUnlockResponse PDU, a GCC Provider shall generate a GCC-Conference-Unlock confirm primitive and issue it to the Control GCCSAP. The result parameter in the confirm primitive is obtained from the Result field in the PDU.

If the Top GCC Provider sends a ConferenceUnlockResponse which indicates a successful result, it shall also generate a ConferenceUnlockIndication PDU and send it to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceUnlockIndication PDU is shown in Table 8-25. This PDU contains no parameters.

TABLE  8-25/T.124

ConferenceUnlockIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| -- No parameters -- |  |  |

On receipt of a ConferenceUnlockIndication PDU, a GCC Provider which supports the optional unlock indication may generate a GCC-Conference-Lock-Report indication and issue it to the Control GCCSAP. Before it does so, it shall examine the User ID as indicated in the received MCS-Uniform-Send-Data indication and compare it to the Node ID of the Top GCC Provider. The GCC-Conference-Lock-Report primitive may only be generated if the received User ID matches the Node ID of the Top GCC Provider. Otherwise, the received PDU is ignored, and no further action is taken.

If the Top GCC Provider becomes aware of a new node entering a conference by its presence in the Conference Roster and the conference is currently unlocked and if the possibility exists that the conference may have been locked when the node joined (or was invited to) the conference, the Top GCC Provider shall generate a ConferenceUnlockIndication PDU and send it to the new node by issuing MCS-Send-Data request specifying the Node ID of that node as the Channel ID, specifying High data priority, and including the PDU in the Data field. Alternatively, it may send this PDU to all nodes by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. Although the flag indicating whether the conference is locked or unlocked is included in the information provided to the newly joined node as part of the connection establishment process, this ensures that if the conference changed its lock state since the connection had been established, that the node is properly notified of the change. If there is no possibility that the lock state has changed since the node was joined (or was invited) into the conference (e.g. if the lock state has not changed since the creation of the conference), the Top GCC Provider need not send this PDU.

### 8.2.8	Disconnecting from a conference

On receipt of a GCC-Conference-Disconnect request, a GCC Provider shall first attempt to eject the nodes directly below it in the connection hierarchy, if any. It shall do this by sending, for each such node, a ConferenceEjectUserIndication PDU with the Node To Eject parameter set to the Node ID of the particular subordinate node and specifying higher-node-disconnected as the reason. This is done by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the data field. The content of the ConferenceEjectUserIndication PDU is shown in Table 8-31. The GCC Provider shall then wait until it has received MCS-Disconnect-Provider indications from each subordinate connection. Following this, it shall disconnect from the conference by issuing first an MCS-Detach-User request followed by an MCS-Disconnect-Provider request directed at the upward connection (the only remaining connection). If, for some reason, the GCC Provider has not received MCS-Disconnect-Provider indications from each of the lower nodes within a reasonable period of time (determined locally), the GCC Provider may proceed to disconnect those connections itself by issuing MCS-Disconnect-Provider requests directed at each remaining lower connection, followed by an MCS-Detach-User request and an MCS-Disconnect-Provider request directed at the upward connection as in the normal case. In either case, or if there had been no subordinate nodes, the GCC Provider shall then generate a GCC-Conference-Disconnect confirm primitive and issue it to the Control GCCSAP. The GCC Provider shall then remove all data-base information associated with this conference.

On receipt of an MCS-Detach-User indication, each GCC Provider in the conference shall examine the User ID indicated in the indication and compare it to its list of Node IDs in its local copy of the Conference Roster. If the User ID corresponds to a Node ID, different from its own Node ID, the GCC Provider shall generate a GCC-Conference-Disconnect indication and issue it to the Control GCCSAP. The Disconnecting Node parameter in the indication shall correspond to the User ID in the received indication. If the reason code in the received indication is user-initiated, the reason code in the GCC-Conference-Disconnect indication shall be either user-initiated or ejected node, depending on whether or not a ConferenceEjectUserIndication PDU was received earlier containing in its Node To Eject field the same User ID as the MCS-Detach-User indication. Otherwise, the reason is indicated as unknown.

### 8.2.9	Terminating a conference

On receipt of a GCC-Conference-Terminate request, a GCC Provider shall send a ConferenceTerminateRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTerminateRequest PDU is shown in Table 8-26. The reason code is obtained from the corresponding parameter in the request primitive.

TABLE  8-26/T.124

ConferenceTerminateRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Reason | Request | Top GCC Provider |

On receipt of a ConferenceTerminateRequest PDU, the Top GCC Provider shall first determine if the requesting node has the privilege necessary to terminate the conference based on the terminate-privileges defined when the conference was created. If not, the request is rejected and a ConferenceTerminateResponse is sent back to the requester by issuing an MCS-Send-Data request specifying the Node ID Channel of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTerminateResponse PDU is shown in Table 8-27. In this case, the result parameter is set to indicate invalid-requester as the reason for rejection.

TABLE  8-27/T.124

ConferenceTerminateResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Result | Top GCC Provider | Confirm |

If the requester did have the proper privilege to terminate the conference, then a ConferenceTerminateResponse shall be sent back to the requester indicating a successful result. In addition, a ConferenceTerminateIndication is sent to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTerminateIndication PDU is shown in Table 8-28. The reason code is obtained from the reason code in the request PDU.

TABLE  8-28/T.124

ConferenceTerminateIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Reason | Top GCC Provider | Indication |

On receipt of a ConferenceTerminateResponse PDU, a GCC Provider shall generate a GCC-Conference-Terminate confirm primitive and issue it to the Control GCCSAP. The result indicated in the primitive is obtained directly from the Result parameter in the PDU.

On receipt of a ConferenceTerminateIndication PDU sent by the Top GCC Provider, a GCC Provider shall first wait until it has received MCS-Disconnect-Provider indications from each connection directly below it in the connection hierarchy. Following this, it shall disconnect from the conference by issuing an MCS-Disconnect-Provider request directed at the upward connection (the only remaining connection). If, for some reason, the GCC Provider has not received MCS-Disconnect-Provider indications from each of the lower nodes within a reasonable period of time (determined locally), the GCC Provider may proceed to disconnect those connections itself by issuing MCS-Disconnect-Provider requests directed at each remaining lower connection, followed by an MCS-Disconnect-Provider request directed at the upward connection as in the normal case. In either case, the GCC Provider shall then generate a GCC-Conference-Terminate indication primitive and issue it to the Control GCCSAP. The reason indicated in the primitive is obtained directly from the Reason parameter in the PDU.

If a GCC Provider receives an MCS-Disconnect-Provider indication from the local MCS provider corresponding to its upward MCS connection, this is an indication that the MCS connection has been terminated due to an abnormal condition within MCS, the GCC Provider shall generate a GCC-Conference-Terminate indication primitive and issue it to the Control GCCSAP. The reason code shall indicate that this is an error termination.

If the Top GCC Provider receives a disconnect indication (either via PDU or MCS-Detach-User indication) which results in no nodes listed in the Conference Roster with the exception of the local node, the provider shall check the Conference Profile to determine if the conference is manually or automatically terminating. If it was manually terminating, no further action is taken. If it was automatically terminating, the GCC Provider shall indicate to the local node controller that the conference has been terminated by generating a GCC-Conference-Terminate indication and issuing it to the Control GCCSAP. The reason code shall indicate that there are no more nodes joined to an automatically terminating conference. The GCC Provider shall then remove all data-base information associated with this conference.

### 8.2.10	Ejecting a node from a conference

On receipt of a GCC-Conference-Eject-User request primitive, a GCC Provider shall first compare the Node To Eject parameter to the Node IDs of the nodes immediately below it in the connection hierarchy, if any. If the Node To Eject is a node other than a directly subordinate node, the GCC Provider shall send a ConferenceEjectUserRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The content of the ConferenceEjectUserRequest PDU is shown in Table 8-29. The contents of the PDU are obtained from the request primitive.

If the Node To Eject is a node directly below the local node in the connection hierarchy, the GCC Provider shall instead send a ConferenceEjectUserIndication PDU to all nodes specifying the Node ID of the node to be ejected in the PDU and the reason as indicated in the request primitive. It shall do this by issuing an MCS-Uniform-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the data field. The content of the ConferenceEjectUserIndication is shown in Table 8-31. The GCC Provider may then wait until it has received an MCS-Disconnect-Provider indication from the connection corresponding to the ejected node. If, for some reason, the GCC Provider has not received MCS-Disconnect-Provider indications from the ejected node within a reasonable period of time (determined locally), the GCC Provider may proceed to disconnect those connections itself by issuing an MCS-Disconnect-Provider request directed at the ejected node. The GCC Provider shall then generate a GCC-Conference-Eject-User confirm PDU indicating a successful result.

TABLE  8-29/T.124

ConferenceEjectUserRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node To Eject | Request | Top GCC Provider |
| Reason | Request | Top GCC Provider |

On receipt of a ConferenceEjectUserRequest PDU, the Top GCC Provider shall first determine if the requesting node has the privilege necessary to eject a user based on the eject-user-privileges defined when the conference was created. If not, the request is rejected and a ConferenceEjectUserResponse is sent back to the requester by issuing an MCS-Send-Data request specifying the Node ID Channel of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceEjectUserResponse PDU is shown in Table 8-30. In this case, the result parameter is set to indicate invalid-requester as the reason for rejection.

If the requester did have the proper privilege to eject a user, then a ConferenceEjectUserIndication primitive is broadcast to all nodes by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The content of the ConferenceEjectUserIndication PDU is shown in Table 8-31. The Reason parameter in this PDU is obtained from the request PDU. If the node to be ejected is valid, a response is sent back to the requester indicating a successful result. If it is not possible to eject the requested node, a response is sent which includes a negative result.

TABLE  8-30/T.124

ConferenceEjectUserResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node To Eject | Top GCC Provider | Confirm |
| Result | Top GCC Provider | Confirm |

On receipt of a ConferenceEjectUserResponse PDU, a GCC Provider shall generate a GCC-Conference-Eject-User confirm primitive and issue it to the Control GCCSAP. The contents of the confirm primitive are obtained from the ConferenceEjectUserResponse PDU.

TABLE  8-31/T.124

ConferenceEjectUserIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node To Eject | Top GCC Provider | Confirm |
| Reason | Top GCC Provider | Indication |

On receipt of a ConferenceEjectUserIndication PDU, a GCC Provider shall compare the Node To Eject parameter to its own Node ID. If they are the same, it shall then compare the User ID as indicated in the MCS-Send-Data indication to the Node ID of the Top GCC Provider and to that of the node directly above it in the connection hierarchy. If the source Node ID is the same as either of these, it shall immediately disconnect from the conference by first attempting to eject the nodes directly below it in the connection hierarchy, if any. It shall do this by sending, for each such node, a ConferenceEjectUserIndication PDU with the Node To Eject parameter set to the Node ID of the particular subordinate node and specifying higher-node-ejected as the reason. This is done by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the data field. The GCC Provider shall then wait until it has received MCS-Disconnect-Provider indications from each subordinate connection. Following this, it shall disconnect from the conference by issuing first an MCS-Detach-User request followed by an MCS-Disconnect-Provider request directed at the upward connection (the only remaining connection). If, for some reason, the GCC Provider has not received MCS-Disconnect-Provider indications from each of the lower nodes within a reasonable period of time (determined locally), the GCC Provider may proceed to disconnect those connections itself by issuing MCS-Disconnect-Provider requests directed at each remaining lower connection, followed by first an MCS-Detach-User request followed by an MCS-Disconnect-Provider request directed at the upward connection as in the normal case. In either case, or if there had been no subordinate nodes, the GCC Provider shall then generate a GCC-Conference-Eject-User indication primitive and issue it to the Control GCCSAP. If the PDU is received with a User ID not matching the Node ID of the Top GCC Provider or the node directly above in the connection hierarchy, the PDU shall be ignored with no further action taken.

If the receiving GCC Provider is an MCU which is directly connected above the node to be ejected in the connection hierarchy, it may optionally disconnect the node to be ejected from the conference by issuing an MCS-Disconnect-Provider request for the corresponding MCS connection. Prior to taking such action, it shall verify that the User ID indicated in the received PDU is the same as the Node ID of the Top GCC Provider.

On receipt of a ConferenceEjectUserIndication PDU sent either by the Top GCC Provider or by the Node ID of the node listed in the conference roster as above the node to be ejected, all nodes except the node to be ejected shall make a note of this event and consult it later if an MCS-Detach-User indication should arrive for the node to be ejected. At that time, as specified in 8.2.8, a node shall generate a GCC-Conference-Disconnect indication and issue it to the Control GCCSAP. The reason indicated shall be ejected-node.

### 8.2.11	Transferring nodes between conferences

On receipt of a GCC-Conference-Transfer request primitive, a GCC Provider shall send a ConferenceTransferRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTransferRequest PDU is shown if Table 8-32. All of the parameters in this PDU are obtained directly from the request primitive.

TABLE  8-32/T.124

ConferenceTransferRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Conference Name | Request | Top GCC Provider |
| Conference Name Modifier (optional) | Request | Top GCC Provider |
| Network Address (optional) | Request | Top GCC Provider |
| Transferring Nodes (optional) | Request | Top GCC Provider |
| Password (optional) | Request | Top GCC Provider |

On receipt of a ConferenceTransferRequest PDU, the Top GCC Provider shall first determine if the requesting node has the privilege necessary to request a transfer based on the transfer-privileges defined when the conference was created. If not, the request is rejected and a ConferenceTransferResponse is sent back to the requester by issuing an MCS-Send-Data request specifying the Node ID Channel of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTransferResponse PDU is shown in Table 8-33. In this case, the result parameter is set to indicate invalid-requester as the reason for rejection.

If the requester did have the proper privilege to request a transfer, then a the Top GCC Provider shall send a ConferenceTransferResponse PDU back to the requester as described above, but with the result parameter indicating success. It shall then broadcast a ConferenceTransferIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTransferIndication PDU is shown in Table 8-34. The parameters in this PDU are obtained from the received ConferenceTransferRequest PDU.

TABLE  8-33/T.124

ConferenceTransferResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Conference Name | Top GCC Provider | Confirm |
| Conference Name Modifier (optional) | Top GCC Provider | Confirm |
| Transferring Nodes (optional) | Top GCC Provider | Confirm |
| Result | Top GCC Provider | Confirm |

On receipt of a ConferenceTransferResponse PDU, a GCC Provider shall generate a GCC-Conference-Transfer confirm primitive and issue it to the Control GCCSAP. The contents of the confirm primitive are obtained from the ConferenceTransferResponse PDU.

TABLE  8-34/T.124

ConferenceTransferIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Conference Name | Top GCC Provider | Indication |
| Conference Name Modifier (optional) | Top GCC Provider | Indication |
| Network Address (optional) | Top GCC Provider | Indication |
| Transferring Nodes (optional) | Top GCC Provider | Indication |
| Password (optional) | Top GCC Provider | Indication |

On receipt of a ConferenceTransferIndication PDU, a GCC Provider which supports the GCC-Conference-Transfer indication primitive shall check the list of destination nodes. If the local Node ID is found on the list of destination nodes, or if the list of destination nodes is NULL, the GCC Provider shall then check that the User ID indicated in the MCS-Uniform-Send-Data indication matches the Node ID of the Top GCC Provider. If they match, it shall generate a GCC-Conference-Transfer indication primitive and issue it to the Control GCCSAP. If the GCC Provider does not support the GCC-Conference-Transfer indication primitive, if the local node is not on the list of destination nodes, or if the received PDU is not from the Top GCC Provider, the PDU is ignored and no further action is taken.

## 8.3	The conference and application rosters

### 8.3.1	Overview

Both the Conference and Application Rosters are communicated among nodes using the same set of PDUs. A single PDU, the RosterUpdateIndication PDU, is associated with all aspects of this exchange. This PDU is used to send complete or partial roster information to other nodes in the conference.

When each node (other than the top node) first announces its roster information upon joining a conference, it is done by sending a RosterUpdateIndication to the node directly above it in the hierarchy. Subsequent updates to any portion of either the Conference or Application Roster information are announced by re-issuing a RosterUpdateIndication containing the new information.

When a node updates its portion of the roster (or announces it for the first time), that information is propagated from node to node up the connection hierarchy until it reaches the Top GCC Provider which is responsible for forming the full Conference and Application rosters and distributing them to all nodes in the conference.

As the roster information propagates up the connection hierarchy, each intermediate node (MCU) is responsible for forming a subset of the full Conference and Application Rosters. That subset includes that node as well as all nodes below it in the connection hierarchy. That is, the Conference Roster includes the Node Record for all such nodes, the Application Roster entries for each Application Protocol Session includes the Application Records for each such node with that Peer Application Protocol Entity enrolled, and the Application Capabilities List for each Application Protocol Session includes the collapsed capabilities information for all such nodes. On receipt of a RosterUpdateIndication from a node below, an intermediate node makes the appropriate changes to its subset of the rosters, then passes this information up to the next higher node by issuing a RosterUpdateIndication.

If the update from a lower node included a change which caused the Application Capabilities Lists for at least one Application Protocol Session to have to be re-computed, the new roster information for the roster subset can only be generated if the individual Application Capabilities Lists for each node directly below in the connection hierarchy is known (to allow the collapse rules to be re-applied with the new updated information). The GCC Provider at each node shall maintain this information locally.

When the roster updates have reached the Top GCC Provider, it shall then broadcast the new roster information to all nodes in the conference. This is done by broadcasting a RosterUpdateIndication PDU to all nodes.

Roster information sent using the RosterUpdateIndication PDU may be sent in one of three ways. The roster may be sent as refresh of the full Conference and Application Rosters, replacing all existing roster entries. Alternatively, portions of the roster may be sent whereby the Conference Roster, and/or portions of the Application Roster associated one or more Session Keys may be refreshed. In this case, all entries in the transmitted portion of the roster are replaced, but portions that are not sent are left unchanged. Finally, changes to portions of the roster may be sent as updates whereby only those particular elements of the roster (e.g. individual Application Records) which have been added, modified, or removed are sent, and all other entries are left unchanged. When the Top GCC Provider broadcasts new roster information only the first two methods are used. When the Top GCC Provider broadcasts new roster information which has changed due to at least one node having joined the conference, it shall broadcast this information only as a full refresh since the new node or nodes have no prior roster information. When propagating roster information up the connection hierarchy, any of these three methods may be used. The method chosen depends on the scope of the information changed, and is ideally chosen on the basis of minimizing the size of the PDU in order to minimize transmission time.

NOTE – The described mechanism requires that all GCC Providers in a conference store the subset of the Conference and Application Rosters for their own node as well as nodes connected directly below them in the connection hierarchy. In addition, various functions require that the GCC Providers also store the full Conference and Application Rosters as broadcast from the Top GCC Provider. If for some reason a GCC Provider fails to retain some or all of this information, the only recourse is to disconnect from the conference and, if possible, to rejoin at a later time.

### 8.3.2	Nodes entering a conference

When a node is joined to a conference, either via conference creation, joining, or invitation, a GCC Provider shall issue a GCC-Application-Permission-To-Enroll indication to the GCCSAP for all Application Protocol Entities which have locally indicated their presence to the GCC Provider. In the case that a GCC Provider becomes aware of an additional Application Protocol Entity while it is already joined to a conference, the GCC Provider shall issue a GCC-Application-Permission-To-Enroll indication to the corresponding GCCSAP indicating the existence of this conference.

Before taking any further action, the GCC Provider shall wait until it has received a GCC-Conference-Announce-Presence request from the Control GCCSAP as well as a GCC-Application-Enroll request from all Application Protocol Entities which have been sent GCC-Application-Permission-To-Enroll indications. Some of these enroll requests may indicate that that Application Protocol Entity does not intend to enroll (the enroll/un-enroll flag is set to un-enroll). For each received GCC-Application-Enroll request with the enroll flag set, the information provided in the primitive for that Application Protocol Entity is added to the Local Application Roster.

The GCC Provider shall also assign a locally allocated Entity ID to each enrolled Application Protocol Entity. This shall be used by the GCC Provider as an identifier of each corresponding Application Protocol Entity in the RosterUpdateIndication PDU. The assigned Application Protocol Entity ID is also included in the GCC-Application-Enroll confirm primitive returned to each enrolling Application Protocol Entity along with the Node ID of the local node. The Entity ID is a 16-bit integer value which is to be unique among all Application Protocol Entities enrolled at a node. If an Application Protocol Entity un-enrolls, the value of its Entity ID shall not be re-used unless all other values not assigned to an Application Protocol Entity at the time of the un-enroll have been assigned.

The GCC Provider shall examine the Conducting Operation Capable Flag of each enrolled Application Protocol Entity with the Active/Inactive flag set to Active (inactive Application Protocol Entities are assumed not to be capable of conducting operation). If there is more than one such enrolled Application Protocol Entity with this flag set, the GCC Provider shall choose one of them for which to set this flag in the Application Roster information which is to be sent in the RosterUpdateIndication PDU. The rule for which of these to choose is a local matter not specified in this Recommendation. A typical rule may be to choose the first such Application Protocol Entity that enrolls.

Upon receiving the GCC-Conference-Announce-Presence request from the Control GCCSAP and a GCC-Application-Enroll request from all Application Protocol Entities, a GCC Provider which is not the Top GCC Provider shall send a RosterUpdateIndication to the GCC Provider directly above it in the connection hierarchy. This is done by issuing an MCS-Send-Data request specifying the Node ID of the destination node as the Channel ID, specifying High data priority, and including the PDU in the Data field. The Node ID of the node directly above in the connection hierarchy is determined at the time of connection to the conference from either the ConferenceCreateResponse, ConferenceJoinResponse, or ConferenceInviteRequest PDUs, depending on how the conference was joined. The value of the Node ID of the above node shall also be included in the conference roster portion of the PDU as the Superior Node parameter. The content of the RosterUpdateIndication PDU is shown in Table 8-35. In this case, the roster is sent as a full refresh, including the local Node Record as well as the Application Records for all enrolled Application Protocol Entities and the Application Capabilities Lists for all Application Protocol Sessions corresponding to enrolled Application Protocol Entities. If there had been more than one Application Protocol Entity locally enrolled for a given Application Protocol Session, the GCC Provider shall perform a collapse of the Application Capabilities List among the Peer Application Protocol Entities to produce the Application Capabilities List which is to be included in the RosterUpdateIndication PDU for that Application Protocol Session. This collapse shall be done by the procedure described in 8.3.8. The action taken by a node receiving a RosterUpdateIndication is described in 8.3.5.

A node which is the Top GCC Provider having just entered a conference, and received a GCC-Conference-Announce-Presence request shall include the information contained in its Conference Roster data-base, but may wait until it has received any RosterUpdateIndication PDUs from at least one other node in the conference before transmitting any PDUs. On receiving such indication, it shall update its Conference and Application Roster data-base and then broadcast the full Conference and Application Rosters by sending a RosterUpdateIndication to all nodes in the conference. This is done by issuing an MCS-Uniform-Send-Data request, specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. In the case of new nodes entering the conference, the RosterUpdateIndication is sent as a full refresh, including the Node Records from all nodes joined to the conference, as well as for each Application Protocol Session in the conference, a collapsed Application Capabilities List, and a list of Peer Application Protocol Entities which are part of that session along with the Application Record for each. The rules for generating the full Conference and Application Rosters, including the collapsed Application Capabilities List are described in 8.3.5

TABLE  8-35/T.124

RosterUpdateIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Full Refresh Flag | Source GCC Provider | Destination GCC Provider |
| Conference Information (optional) | Source GCC Provider from information in Request or received PDU | Destination GCC Provider |
| Application Information (optional) | Source GCC Provider from information in Request or received PDU | Destination GCC Provider |

### 8.3.3	Enrolling Application Protocol Entities

On receipt of a GCC-Application-Enroll request primitive, a GCC Provider shall first determine if there is already an existing entry in the Local Application Roster for the specified conference associated with the GCCSAP of the requester. If not, and if the Enroll/un-enroll flag in the request is set to Enroll, a new entry is created in the Local Application Roster containing the information specified in the request primitive. In this case the GCC Provider shall also assign an Entity ID to the newly enrolled Application Protocol Entity as described in 8.3.2. If the Enroll/un-enroll flag had been set to Un-enroll, the request is confirmed immediately by generating a GCC-Application-Enroll confirm and issuing it to the GCCSAP of the requester. In this case, no further action is taken.

If the entry in the Local Application Roster already exists and the Enroll/un-enroll flag is set to Enroll, the contents of the existing entry are modified to reflect the new values specified. If the Enroll/un-enroll flag had been set to Un-enroll, the GCC Provider shall remove the corresponding entry from the Local Application Roster.

If the GCC Provider is not the Top GCC Provider for the specified conference, the GCC Provider shall then send its new Local Application Roster to the node directly above it in the connection hierarchy, using a RosterUpdateIndication PDU. It shall do this by issuing an MCS-Send-Data request specifying the Node ID of the higher node as the Channel ID, specifying High data priority, and including the selected PDU in the Data field. The Node ID of the GCC Provider directly above in the connection hierarchy is determined at the time of connection to the conference from either the ConferenceCreateResponse, ConferenceJoinResponse, or ConferenceInviteRequest PDUs, depending on how the conference was joined. The content of the RosterUpdateIndication PDU is shown in Table 8-35.

If the enroll had occurred prior to the initial transmission of a RosterUpdateIndication PDU, then the new or altered Record is included in the initial roster information, sent as a full refresh as described in 8.3.2. If an Application Protocol Entity un-enrolled prior to the initial transmission, its record entry is removed from the roster and never included in any transmitted PDU.

If the enroll occurred after the initial transmission of a RosterUpdateIndication PDU, the new, modified, or removed Record is treated as an update to the existing roster. In this case, the GCC Provider may either choose to re-send its portion of the Conference and Application Rosters in their entirety, or it may send the information as an update – only sending information regarding the record that has changed. It is preferred that the latter method be used in order to minimize the size of the PDU, and therefore, its transmission time. If more than one change has been made (more than one GCC-Application-Enroll request had been received) since the last update, these changes may be concatenated into a single PDU. For a given Application Protocol Session, if only the Application Record information has changed since the last instance of that Application Protocol Session, the Application Capabilities List need not be included in the RosterUpdateIndication PDU, only the modified Application Record. For a given Application Protocol Session, the Application Capabilities List requires updating if its contents from an already enrolled Application Protocol Entity had been changed, or if a newly enrolled Application Protocol Entity is part of the Application Protocol Session for which Peer Application Protocol Entities had already been enrolled at the local node or at nodes below the local node in the connection hierarchy, or if a node is un-enrolled in the case that there are other Peer Application Protocol Entities which remain enrolled at the local node or at nodes below the local node in the connection hierarchy. In the case where there is more than one Peer Application Protocol Entity either after an enroll, or before an un-enroll, the GCC Provider shall recreate the collapsed Application Capabilities List for that Application Protocol Session prior to including it in the RosterUpdateIndication PDU. This collapse shall be done by the procedure described in 8.3.8.

Before sending the RosterUpdateIndication PDU, if the Active/Inactive flag for the enrolling Application Protocol Entity is set to Active, the GCC Provider shall examine the Conducting Operation Capable Flag. If this flag is set, the GCC Provider shall ensure that only one Peer Application Protocol Entity from this node for each Application Protocol Session is included in the Application Roster with this flag set. If no such Peer Application Protocol Entity has been chosen indicated in the Application Roster so far, the newly enrolling Application Protocol Entity may be included in the transmitted RosterUpdateIndication PDU with this flag set. If there has already been a Peer Application Protocol Entity at this node which has been included in the Application Roster with this flag set, the GCC Provider may either include the newly enrolling Application Protocol Entity in the RosterUpdateIndication PDU without this flag set, or it may include the newly enrolling Application Protocol Entity with this flag set, and the previous designated conducting-capable Application Protocol Entity with this flag now set to FALSE. The rule for choosing which Peer Application Protocol Entity to include in the Application Roster with this flag set is a local matter beyond the scope of this Recommendation. If the Active/Inactive flag for the enrolling Application Protocol Entity is set to Inactive, the Conducting Operation Capable Flag indicated in the request primitive shall be ignored, and the corresponding field in the updated roster shall be set to FALSE.

In the case the Top GCC Provider, the successful change of an entry in the Local Application Roster results in a direct modification to the Conference Application Roster and the associated procedure for notification of all other nodes in the conference of the new Conference Application Roster as described in 8.3.5.

In all of the successful cases, the GCC Provider shall generate a GCC-Application-Enroll confirm primitive indicating a successful result and issue it to the GCCSAP of the requester.

### 8.3.4	Updating a conference roster entry

If a GCC-Conference-Announce-Presence request is received after having already transmitted an initial roster, the new Node Record contained in this request shall be treated as an update to the Conference Roster. As in the case of an updated Application Record, the GCC Provider shall then send its new Node Record to the node directly above it in the connection hierarchy, using a RosterUpdateIndication PDU. It shall do this by issuing an MCS-Send-Data request specifying Node ID of higher node as the Channel ID, specifying High data priority, and including the selected PDU in the Data field. The content of the RosterUpdateIndication PDU is shown in Table 8-35.

As in the case of the enroll, the GCC Provider may either choose to re-send its portion of the Conference and Application Rosters in their entirety, or it may send only the new Node Record as an update. It is preferred that the latter method be used in order to minimize the size of the PDU, and therefore, its transmission time.

If a GCC provider receives an MCS-Detach-User indication for which the User ID is that of a locally enrolled Application Protocol Entity, it shall update the roster to remove that Application Protocol Entity, and it shall issue a GCC-Permission-To-Enroll indication primitive to the GCCSAP corresponding to that Application Protocol Entity.

### 8.3.5	Propagation of roster updates to the Top GCC Provider

On receipt of a RosterUpdateIndication PDU from a node directly below in the connection hierarchy, a GCC Provider shall first update its subset of the Conference and Application Rosters. Each node in the conference shall maintain that subset of the Conference and Application Rosters which corresponds to that node as well as all nodes below it in the connection hierarchy. For each Application Protocol Entity which has been enrolled at any of these nodes, a list of Application Records including the Node ID of the node to which that record corresponds, and a partially collapsed Application Capabilities List are maintained. This is the Application Capabilities List which corresponds to the outcome of the set of rules to be applied to the Application Capabilities Lists of the Nodes directly below this node in the connection hierarchy (which are themselves partially collapsed).

The procedure that the GCC Provider shall use to update its roster subset depends on the update method used in the received RosterUpdateIndication. If the update was indicated to be a full refresh (as indicated by the full-refresh flag in the PDU), the following procedure is used. First, all Node Records and Application Records corresponding to the node from which the PDU was received, or nodes which were previously known to be below that node in the connection hierarchy are removed from the local subset of the Conference and Application Rosters. All Conference and Application Records listed in the PDU are then added to the roster. The entire Application Capabilities List, corresponding to the requesting node below, which may have been stored at this node is removed. The Application Capabilities List specified in the PDU replaces it. For all Application Protocol Entities either currently known to be enrolled at or below the current node, as well as any new Application Protocol Entities indicated in the newly received PDU (including ones not specified in this PDU), a new collapsed Application Capabilities List shall be computed from each of the Application Capabilities Lists from the nodes directly below. This collapse shall be done by the procedure described in 8.3.8.

In the case that the RosterUpdateIndication did not indicate a full refresh, a slightly different procedure is followed. First, if the Node Record information was indicated as changed, if the change was done as a refresh of the list of Node Records, all Node Records corresponding to the node from which the PDU was received, or nodes which were previously known to be below that node in the connection hierarchy are removed from the local subset of the Conference Roster. All Node Records listed in the PDU are then added to the roster. If the change was done as a series of updates to Node Records, any records indicated as added are added to the roster subset, any indicated as replaced are used to replace the existing record, and any indicated for removal are removed. An attempt to change a record which already exists, or to modify or remove a record which does not exist is ignored. In any case, for any nodes which were previously in the roster subset and have been removed, all Application Records corresponding to those nodes are also removed (regardless of whether the corresponding update information was included in the Application Roster portion of the PDU). If any Application Record information was indicated as changed, for each Application Protocol Session, a similar procedure is followed. For each Application Protocol Session, if the change was done as a refresh, all Application Records corresponding to the node from which the PDU was received, or nodes which were previously known to be below that node in the connection hierarchy are removed from the local subset of the Application Roster. All Application Records listed in the PDU are then added to the roster. It is possible that for a given Application Protocol Session, it is indicated that there are no nodes which have that Application Protocol Entity enrolled. If the change was done as a series of updates to Application Records, any records indicated as added are added to the roster subset, any indicated as replaced are used to replace the existing record, and any indicated for removal are removed. An attempt to change a record which already exists, or to modify or remove a record which does not exist is ignored. For Application Capabilities List, the same procedure is followed as described for the full-refresh case. In this case, however, the new Application Capabilities Lists are only re-computed for sets of Peer Application Protocol Entities which have indicated a change.

In either of the above cases, the updated information for the Conference Roster and for each Application Protocol Session in the Application Roster includes an instance number. If the conference information has changed due to the received update, the instance number for the conference information shall be incremented by one modulo 216. Similarly, for each Application Protocol Session for which any information was modified (either Application Records or the Application Capabilities List), this instance number shall be incremented by one modulo 216. If multiple changes are made to be forwarded to other nodes as a single update, these changes may be counted as a single increment to the instance number. The instance numbers are maintained locally and corresponds to its local subset of the roster. That is, that node plus the nodes located below it in the connection hierarchy. In the case of the Top GCC Provider, the instance numbers apply to the Conference and Application Rosters which are broadcast to all nodes and reported in the GCC-Application-Roster-Report primitive. The conference information also includes a flag indicating whether nodes have been added and/or removed since the last instance. Similarly, for each Application Protocol Session, a flag indicates whether nodes have been added and/or removed since the last instance of the information for that set of Application Protocol Entities. In this case, nodes being added or removed may indicate that an Application Protocol Entity has been enrolled or un-enrolled at a node, respectively, not necessarily that the entire node has been added or removed from the conference.

Once the Conference and/or Application Roster information has been re-computed, a GCC Provider which is not the Top GCC Provider shall generate a RosterUpdateIndication PDU which shall then be sent to the GCC Provider directly above it in the connection hierarchy. The format of the information contained in this PDU may be either a full refresh, a refresh of some sets of Peer Application Protocol Entities and/or the conference information, or as individual updates. The choice of format to send the update information is left to the GCC Provider. It is preferred that a choice which minimizes the size of the PDU, and therefore minimizes transmission time, be used.

### 8.3.6	Distribution of the conference and application rosters

When the Top GCC Provider has received a RosterUpdateIndication, it shall modify the Conference and Application Roster information which it maintains (which, in this case, are the full Conference and Application Rosters rather than a subset) in a manner identical to that described for an intermediate MCU updating its subset of the roster. Once the complete roster has been updated, the Top GCC Provider shall broadcast the updated roster information to all nodes by sending a RosterUpdateIndication PDU. This is done by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel-ID, specifying High data priority, and including the PDU in the Data field.

If the case that the Conference Roster has been modified to include new nodes, the RosterUpdateIndication shall be broadcast as a full refresh. That is, the Conference and Application Rosters shall be transmitted in full to all nodes. In the case that a modification had been made to the roster which did not involve the addition of new nodes, the Top GCC Provider may choose to send the update either as a full refresh, or as a refresh to the list of Node Records, and/or the list of Application Records for some or all sets of Peer Application Protocol Entities, and/or the Application Capabilities List for some or all Application Protocol Sessions.

On receipt of a RosterUpdateIndication on the GCC-Broadcast-Channel, each GCC Provider shall generate a GCC-Conference-Roster-Report indication and issue it to the Control GCCSAP if the received PDU indicated any change to the Conference Roster. If the received PDU had indicated a change to some or all of the Application Roster, the GCC Provider shall generate a series of GCC-Application-Roster-Report indications and issue them to the GCCSAP associated with each enrolled Application Protocol Entity corresponding to an Application Protocol Session for which an Application Roster update has been received in the PDU. It may also issue GCC-Application-Roster-Report indications to other GCCSAPs, although the need to do so is considered a local matter beyond the scope of this Recommendation. Only that portion of the Conference Application Roster associated with the Session Key for that Application Protocol Entity must be included in the corresponding primitive. The GCC Provider may choose to include portions of the roster corresponding to other Session Keys, although the need to do this is considered a local matter beyond the scope of this Recommendation. In the case of an Application Protocol Entity enrolled inactively with no Session ID, the GCC Provider shall issue a GCC-Application-Roster-Report indication to the corresponding GCCSAP for portions of the roster corresponding to any Application Protocol Session with the same base Application Protocol as the Application Protocol Entity. The GCC Provider shall also issue a GCC-Application-Roster-Report indication to the Control GCCSAP, including Application Roster updates for all sets of Peer Application Protocol Entities which have been indicated as changed by the received PDU.

### 8.3.7	Nodes leaving a conference

On receiving an MCS-Detach-User indication, MCU nodes which have nodes below them in the connection hierarchy shall check the User ID indicated and determine if it corresponds to the Node ID of a node directly below it in the connection hierarchy. If so, it shall all remove all entries corresponding to this node, as well as any nodes known to be connected below that node, from its subset of the Conference and Application Rosters. It shall then re-compute the Application Capabilities Lists for all sets of Peer Application Protocol Entities in the manner described in 8.3.8. Once the roster subset has been fully updated to reflect the leaving node, that GCC Provider shall follow the procedure described in 8.3.5 to propagate this update to the other nodes in the conference.

At a node which has disconnected from a conference (via either disconnection, termination of the conference, or ejection from the conference), the GCC Provider shall generate a GCC-Permission-To-Enroll indication which revokes permission to enroll in the corresponding conference. It shall issue this to all non-Control GCCSAPs.

### 8.3.8	Collapsing application capabilities lists

Each GCC Provider is responsible for forming, for each Application Protocol Session, the collapsed Application Capability List corresponding to the nodes below it in the connection hierarchy, as well as any Peer Application Protocol Entities enrolled at the local node. Since the nodes directly below in the connection hierarchy, the local GCC Provider need only be aware of the collapsed Application Capability Lists from those nodes, rather than the Application Capability Lists from all lower nodes. Given as input, these collapsed Application Capabilities Lists as well as the Application Capabilities Lists from Peer Application Protocol Entities at the local node from the Local Application Roster, the GCC Provider shall produce a collapsed Application Capabilities List by the following procedure:

•	For each capability item in each list, determine the class of the capability.

•	For any class, the value of the count parameter for the entry in the collapsed Application Capabilities List shall be set to the sum of the counts indicated in the corresponding entry of each of the Application Capabilities Lists in the input set.

•	For the Unsigned-minimum and Unsigned-maximum classes, the new entry should also include the minimum or maximum value, respectively, of the values given for the corresponding entry of each of the Application Capabilities Lists in the input set.

### 8.3.9	Application and conference roster inquiry

On receipt of a GCC-Application-Roster-Inquire request primitive, a GCC Provider which supports this primitive may respond by generating a GCC-Application-Roster-Inquire confirm primitive and issuing it to the GCCSAP of the requester. The content of the confirm primitive is generated from the locally maintained Conference Application Roster data-base and shall include only those entries in the roster for which the Session Key specified in the request exactly matches the Session Key of the entry over the length of the key given in the request.

On receipt of a GCC-Conference-Roster-Inquire request primitive, a GCC Provider which supports this primitive shall respond by generating a GCC-Conference-Roster-Inquire confirm primitive and issuing it to the GCCSAP of the requester. The content of the confirm primitive is generated from the local Conference Roster data-base accounting for all nodes known to be in the conference.

### 8.3.10	An Example of a roster update

Figure 8-5 shows an example of a GCC-Application-Enroll request issued during a conference causing an update of the Conference Application Roster. In this example, the node is in the third layer of the connection hierarchy. It issues the update indication to the next higher node which then formats and sends an update indication to the next higher node, which in this example, is the top node in the hierarchy. The top node assembles the full Application Roster and broadcasts the portions associated with the updated Application Protocol Session to all nodes in the conference, resulting in GCC-Application-Report primitives being issued at all nodes to the Node Controller, and if present, the Peer Application Protocol Entities.

FIGURE 8-5/T.124...[D50] = 12 CM

### 8.3.11	Remotely invoking an Application Protocol Entity

On receipt of a GCC-Application-Invoke request primitive via either the Control GCCSAP or an ordinary GCCSAP, a GCC Provider which supports this primitive shall broadcast an ApplicationInvokeIndication PDU to all nodes in the specified conference. This is done by issuing an MCS-Send-Data request or MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. Alternatively, the GCC Provider may issue one or more MCS-Send-Data request specifying the Node ID of one of the listed destination nodes as the Channel ID, also specifying High data priority, and including the PDU in the Data field. In this case, it may leave the Destination Node List parameter in the PDU empty. The content of the ApplicationInvokeIndication PDU is shown if Table 8-36. The GCC Provider shall then generate a GCC-Application-Invoke confirm primitive indicating if the requested operation was successful and issue it to the GCCSAP of the requester.

TABLE  8-36/T.124

ApplicationInvokeIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Application Protocol Entity List | Request | Indication |
| Destination Node List or NULL | Request | Destination GCC Providers |

On receipt of an ApplicationInvokeIndication PDU, a GCC Provider which supports the GCC-Application-Invoke primitive shall first determine if the local node is on the list of destination nodes. If the list is either NULL, or if the local Node ID is present in the list, the GCC Provider may generate a GCC-Application-Invoke indication primitive and issue it to the Control GCCSAP. The Invoking Node parameter is filled in from the Sender User ID field in the received MCS-Send-Data indication or MCS-Uniform-Send-Data indication. If the local node is not on the list of destination nodes (if listed explicitly), no further action is taken.

## 8.4	The Application Registry

The Application Registry is an active data-base which resides at the Top GCC Provider for a conference. Any MCU node shall support the full set of registry services which operate on this data-base, while a terminal node may choose to support only those registry services required by Application Protocols to be supported within that terminal.

When a conference is created, the registry data-base, located at the Top GCC Provider, shall be initialized to a state in which all registry entries are empty and no registry entries are designated to be monitored. This shall be done prior to the time that the Top GCC Provider joins its Node ID Channel.

During operation of the registry within a conference, each non-empty registry entry contains the following information:

•	The Registry Key which identifies the entry.

•	The type of information contained – either a Channel ID, a Token ID, or a Parameter.

•	The monitoring state – either monitoring enabled, or monitoring disabled.

•	The value of the entry – the actual Channel ID, Token ID, or Parameter.

•	The owner of the entry – either none, or the Node ID and Entity ID corresponding to the owning Application Protocol Entity.

•	For Parameter type entries only, the modification rights of the entry – either Owner, Session, or Public.

Processing by the Top GCC Provider of multiple registry requests from a single node shall be performed strictly in the order that the requests were received. The sequence of response PDUs to a requesting node shall preserve the order of the received requests from that node.

At the requesting node, order shall also be preserved between the request primitives and the corresponding transmitted PDUs as well as between the received PDUs and the corresponding confirm primitives.

### 8.4.1	Registering a channel

On receipt of a GCC-Registry-Register-Channel request primitive, a GCC Provider shall send a RegistryRegisterChannelRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryRegisterChannelRequest PDU is shown in Table 8-37.

Recommendation T.124     (08/95)

TABLE  8-37/T.124

RegistryRegisterChannelRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Source GCC Provider | Top GCC Provider |
| Key | Request | Top GCC Provider |
| Channel ID | Request | Top GCC Provider |

On receipt of the RegistryRegisterChannelRequest PDU, the Top GCC Provider shall first verify that the Sender User ID in the received MCS-Send-Data indication and the Entity ID included in the PDU correspond exactly to the Node ID and Entity ID of an Application Protocol Entity currently in the Application Roster. If so, it shall attempt to register the channel by creating an appropriate registry entry. First, it checks whether the registry entry corresponding to the specified Key already exists in the registry data-base. If the registry entry does not exist, the Top GCC Provider creates an entry for this Key and includes in the data-base for this entry the Node ID as indicated by the Sender User ID in the received MCS-Send-Data indication along with the Entity ID included in the PDU as the entry owner, the entry type being a Channel ID, and the value being the Channel ID specified in the PDU.

The Top GCC Provider then indicates that the channel has been properly registered by sending a RegistryResponse PDU to the requesting node by issuing an MCS-Send-Data request specifying the Node ID of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryResponse PDU is shown in Table 8-38. In the case of a successful action, the Result parameter is specified as successful. The Modification Rights field is not filled in (this field is for Parameter type entries only).

If the registry entry already existed, if the requester was invalid due to not appearing in the Application Roster, or if the registry entry could not be created due to a limitation in the available resources, the registry is not modified and a RegistryResponse PDU is returned as above, but with a negative Result indicating the reason for the failure. In this case, the value of the entry prior to attempting to modify it is returned as the Registry Item in the RegistryResponse PDU.

TABLE  8-38/T.124

RegistryResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Top GCC Provider | Destination GCC Provider |
| Primitive type (register-channel, assign-token, set-parameter, retrieve-entry, delete-entry, monitor-entry) | Top GCC Provider | Destination GCC Provider |
| Key | Top GCC Provider | Confirm |
| Registry Item | Top GCC Provider | Confirm |
| Owner | Top GCC Provider | Confirm |
| Modification Rights (optional) | Top GCC Provider | Confirm |
| Result | Top GCC Provider | Confirm |

In the case of a successful modification of an existing registry entry, the Top GCC Provider checks to determine if the registry entry had been set to be monitored. If not, no further action is taken by the Top GCC Provider. If the entry had been set to be monitored, the Top GCC Provider sends a RegistryMonitorEntryIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryMonitorEntryIndication PDU is shown in Table 8-44.

On receipt of a RegistryResponse PDU of this type, a GCC Provider shall generate a GCC-Registry-Register-Channel confirm primitive indicating whether or not the request was successful as indicated in the Result parameter of the RegistryResponse PDU and issue it on the GCCSAP of the Application Protocol Entity indicated by the Entity ID. If the GCC Provider knows of no currently enrolled Application Protocol Entity with the corresponding Entity ID, the indication is ignored and no further action is taken.

### 8.4.2	Assigning a token

On receipt of a GCC-Registry-Assign-Token request primitive, a GCC Provider shall send a RegistryAssignTokenRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryAssignTokenRequest PDU is shown in Table 8-39.

TABLE  8-39/T.124

RegistryAssignTokenRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Source GCC Provider | Top GCC Provider |
| Key | Request | Top GCC Provider |

On receipt of the RegistryAssignTokenRequest PDU, the Top GCC Provider shall first verify that the Sender User ID in the received MCS-Send-Data indication and the Entity ID included in the PDU correspond exactly to the Node ID and Entity ID of an Application Protocol Entity currently in the Application Roster. If so, it shall attempt to assign a token by allocating a Token ID and creating an appropriate registry entry. First, it checks whether the registry entry corresponding to the specified Key already exists in the registry data-base. If the registry entry does not exist, the Top GCC Provider first allocates a new Token ID from the space of dynamic Token IDs (16 384 through 65 535). The Top GCC Provider then creates an entry for this Key and includes in the data-base for this entry the Node ID as indicated by the Sender User ID in the received MCS-Send-Data indication along with the Entity ID included in the PDU as the entry owner, the entry type being a Token ID, and the value being the allocated Token ID.

The Top GCC Provider then indicates that the token has been properly allocated, and returns the value of the Token ID to the requester, by sending a RegistryResponse PDU to the requesting node by issuing an MCS-Send-Data request specifying the Node ID of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryResponse PDU is shown in Table 8-38. In the case of a successful action, the Result parameter is specified as successful. The Modification Rights field is not filled in (this field is for Parameter type entries only).

If the registry entry already existed, if the requester was invalid due to not appearing in the Application Roster, if the registry entry could not be created due to a limitation in the available resources, or if there are no more available dynamic Token IDs, the registry is not modified and a RegistryResponse PDU is returned as above, but with a negative Result indicating the reason for the failure. In this case, the value of the entry prior to attempting to modify it is returned as the Registry Item in the RegistryResponse PDU.

In the case of a successful modification of an existing registry entry, the Top GCC Provider checks to determine if the registry entry had been set to be monitored. If not, no further action is taken by the Top GCC Provider. If the entry had been set to be monitored, the Top GCC Provider sends a RegistryMonitorEntryIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryMonitorEntryIndication PDU is shown in Table 8-44.

On receipt of a RegistryResponse PDU of this type, a GCC Provider shall generate a GCC-Registry-Assign-Token confirm primitive indicating whether or not the request was successful as indicated in the Result parameter of the RegistryResponse PDU, and if successful, the value of the allocated Token ID and issue it on the GCCSAP of the Application Protocol Entity indicated by the Entity ID. If the GCC Provider knows of no currently enrolled Application Protocol Entity with the corresponding Entity ID, the indication is ignored and no further action is taken.

### 8.4.3	Setting a parameter

On receipt of a GCC-Registry-Set-Parameter request primitive, a GCC Provider shall send a RegistrySetParameterRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistrySetParameterRequest PDU is shown in Table 8-40.

TABLE  8-40/T.124

RegistrySetParameterRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Source GCC Provider | Top GCC Provider |
| Key | Request | Top GCC Provider |
| Parameter | Request | Top GCC Provider |
| Modification Rights (optional) | Request | Top GCC Provider |

On receipt of the RegistrySetParameterRequest PDU, the Top GCC Provider shall first verify that the Sender User ID in the received MCS-Send-Data indication and the Entity ID included in the PDU correspond exactly to the Node ID and Entity ID of an Application Protocol Entity currently in the Application Roster. If so, it shall attempt to set a parameter in an existing registry entry or create a new entry with the specified parameter. First, it checks whether the registry entry corresponding to the specified Key already exists in the registry data-base, and if so, determines the type of the entry, the owner, and the current modification rights. If the registry entry does not exist, the Top GCC Provider then creates an entry for this Key and includes in the data-base for this entry the Node ID as indicated by the Sender User ID in the received MCS-Send-Data indication along with the Entity ID included in the PDU as the entry owner, the entry type being a Parameter, and the value being the Parameter value specified in the PDU. If the registry entry already existed, and if the entry was of the Parameter type, the GCC Provider shall first check if the requester has the right to modify this entry. If the owner has modification rights, the value of the registry entry is modified to reflect the type as a Parameter and the value as the Parameter value specified in the PDU. If the entry was not owned prior to being set, the entry is modified to indicate the new owner.

The determination of whether the owner has modification rights is by the following rules. If the current Modification Rights attribute of the entry is set to Owner, the requester, as indicated by the Entity ID and the Node ID from the Sender User ID field of the received MCS-Send-Data indication, must be the owner or the entry must currently be un-owned for the request to succeed.  If the current Modification Rights attribute of the entry is set to Session, the requester must be part of the same Application Protocol Session for this request to succeed. This is determined by finding the entry in the current Application Roster for the requester and examining the Session Key. The Session Key must be identical to that of the owner. Again, if the entry is not owned, this restriction does not apply. If the current Modification Rights attribute of the entry is set to Public, there are no restrictions on the requester for the request to succeed.

When a Parameter entry is first created, the state of Modification Rights for the entry is determined. If the Modification Rights parameter was included in the request PDU which resulted in the creation of the entry, the Modification Type is set to the indicated value. If not, the value Public is assumed for Modification Rights. On requests to set the parameter, if the Modification Rights entry is not present, the Modification Rights state is not changed. If it is present, the GCC Provider shall change the Modification Rights state to the state indicated in the PDU only if the requester is the current owner of the entry, or if the entry is currently un-owned. Otherwise, the Modification Rights state is not changed. In the latter case, the set-parameter operation shall still proceed as normal, and the result shall not be effected.

In either of the above cases, the Top GCC Provider then indicates that the parameter has been properly set by sending a RegistryResponse PDU to the requesting node by issuing an MCS-Send-Data request specifying the Node ID of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryResponse PDU is shown in Table 8-38. In the case of a successful action, the Result parameter is specified as successful. The Modification Rights parameter shall be included in the response PDU.

If the registry entry could not be created due to a limitation in the available resources, if the entry already existed but the requester did not have modification rights, if the requester was invalid due to not appearing in the Application Roster, or if the entry was of the Parameter type, the registry is not modified and a RegistryResponse PDU is returned as above, but with a negative Result indicating the reason for the failure. In this case, the value of the entry prior to attempting to modify it is returned as the Registry Item in the RegistryResponse PDU. In the case of insufficient modification rights, the result parameter shall be: belongs to other.

In the case of a successful modification of an existing registry entry, the Top GCC Provider checks to determine if the registry entry had been set to be monitored. If not, no further action is taken by the Top GCC Provider. If the entry had been set to be monitored, the Top GCC Provider sends a RegistryMonitorEntryIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryMonitorEntryIndication PDU is shown in Table 8-44.

On receipt of a RegistryResponse PDU of this type, a GCC Provider shall generate a GCC-Registry-Set-Parameter confirm primitive indicating whether or not the request was successful as indicated in the Result parameter of the RegistryResponse PDU and issue it on the GCCSAP of the Application Protocol Entity indicated by the Entity ID. If the GCC Provider knows of no currently enrolled Application Protocol Entity with the corresponding Entity ID, the indication is ignored and no further action is taken.

### 8.4.4	Retrieving an entry

On receipt of a GCC-Registry-Retrieve-Entry request primitive, a GCC Provider shall send a RegistryRetrieveEntryRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryRetrieveEntryRequest PDU is shown in Table 8-41.

TABLE  8-41/T.124

RegistryRetrieveEntryRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Source GCC Provider | Top GCC Provider |
| Key | Request | Top GCC Provider |

On receipt of the RegistryRetrieveEntryRequest PDU, the Top GCC Provider shall examine the contents of the registry entry specified by the Key. The registry entry may be in one of four possible states: empty, containing a Channel ID, a Token ID, or a Parameter. For any of these cases, the Top GCC Provider returns the state of the entry to the requester by sending a RegistryResponse PDU to the requesting node by issuing an MCS-Send-Data request specifying the Node ID of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryResponse PDU is shown in Table 8-38. The Registry Item parameter contains the state of the entry, and in the case of a non-empty state, the value that the entry currently contains. The Result parameter is specified as successful if the entry is non-empty, and entry-not-found if the entry is empty. If the entry is indicated to be of the Parameter type, the Modification Rights parameter shall be included in the response PDU.

On receipt of a RegistryResponse PDU of this type, a GCC Provider shall generate a GCC-Registry-Retrieve-Entry confirm primitive indicating the registry item included in the response PDU and issue it on the GCCSAP of the Application Protocol Entity indicated by the Entity ID. If the GCC Provider knows of no currently enrolled Application Protocol Entity with the corresponding Entity ID, the indication is ignored and no further action is taken.

### 8.4.5	Deleting an entry

On receipt of a GCC-Registry-Delete-Entry request primitive, a GCC Provider shall send a RegistryDeleteEntryRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryDeleteEntryRequest PDU is shown in Table 8-42.

TABLE  8-42/T.124

RegistryDeleteEntryRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Source GCC Provider | Top GCC Provider |
| Key | Request | Top GCC Provider |

On receipt of the RegistryDeleteEntryRequest PDU, the Top GCC Provider shall attempt to delete the designated registry entry. First, it checks whether the registry entry corresponding to the specified Key exists in the registry data-base, and if so, determines the owner. If the registry entry exists and is owned by the requester (both the Node ID and Entity ID of the owner and those of the requester are identical) or if the entry is not currently owned, the Top GCC Provider deletes the contents of the entry setting the state to empty and not monitored.

In the registry entry is successfully deleted, or if the entry already did not exist, the Top GCC Provider then indicates that the entry has been properly deleted by sending a RegistryResponse PDU to the requesting node by issuing an MCS-Send-Data request specifying the Node ID of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryResponse PDU is shown in Table 8-38. In the case of a successful action, the Result parameter is specified as successful, and the registry item is specified to be empty.

If the registry entry already existed but was owned by another node, the registry is not modified and a RegistryResponse PDU is returned as above, but with a negative Result indicating the reason for the failure. In this case, the registry item is set to the value of the entry prior to the deletion attempt.

In the case of a successful deletion of an existing registry entry, the Top GCC Provider checks to determine if the registry entry had been set to be monitored (prior to deletion). If not, no further action is taken by the Top GCC Provider. If the entry had been set to be monitored, the Top GCC Provider sends a RegistryMonitorEntryIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryMonitorEntryIndication PDU is shown in Table 8-44.

In the case of a successful deletion of a Token type registry entry, the Top GCC Provider may de-allocate the corresponding token. That is, the Top GCC Provider may later re-use the Token ID which had been included in the deleted entry in response to a RegistryAssignTokenRequest.

On receipt of a RegistryResponse PDU of this type, a GCC Provider shall generate a GCC-Registry-Delete-Entry confirm primitive indicating whether or not the request was successful as indicated in the Result parameter of the RegistryResponse PDU and issue it on the GCCSAP of the Application Protocol Entity indicated by the Entity ID. If the GCC Provider knows of no currently enrolled Application Protocol Entity with the corresponding Entity ID, the indication is ignored and no further action is taken.

### 8.4.6	Monitoring an entry

On receipt of a GCC-Registry-Monitor request primitive with the Enable/Disable flag set to Enable, a GCC Provider shall send a RegistryMonitorEntryRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryAssignTokenRequest PDU is shown in Table 8-43. A local record that the requesting Application Protocol Entity had enabled monitoring this entry is made by the GCC Provider to determine whether to generate GCC-Registry-Monitor indications on later receipt of RegistryMonitorEntryIndication PDU corresponding to this entry.

On receipt of a GCC-Registry-Monitor request primitive with the Enable/Disable flag set to Disable, a local record that the requesting Application Protocol Entity had disabled monitoring this entry is made by the GCC Provider to determine whether to generate GCC-Registry-Monitor indications on later receipt of RegistryMonitorEntryIndication PDU corresponding to this entry.

TABLE  8-43/T.124

RegistryMonitorEntryRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Source GCC Provider | Top GCC Provider |
| Key | Request | Top GCC Provider |

On receipt of the RegistryMonitorEntryRequest PDU, the Top GCC Provider shall attempt to change the monitoring state of the registry entry. First, it checks whether the registry entry corresponding to the specified Key already exists in the registry data-base. If the registry entry exists, the monitoring state for the specified entry is set to enabled.

NOTE – Once monitoring is enabled, monitor indications will continue to be broadcast whenever changes are made to the entry for the duration of the conference, or until the entry is deleted.

Once the appropriate action has been taken, the Top GCC Provider sends a RegistryResponse PDU to the requesting node by issuing an MCS-Send-Data request specifying the Node ID of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryResponse PDU is shown in Table 8-38. In the case of a successful action, the Result parameter is specified as successful. If the requested entry did not exist, the Result parameter is specified as entry-not-found. If the entry is indicated to be of the Parameter type, the Modification Rights parameter shall be included in the response PDU.

If the registry entry did not exist, the registry is not modified and a RegistryResponse PDU is returned as above, but with a negative Result indicating the reason for the failure.

On receipt of a RegistryResponse PDU of this type, a GCC Provider shall generate a GCC-Registry-Monitor confirm primitive indicating whether or not the request was successful as indicated in the Result parameter of the RegistryResponse PDU and issue it on the GCCSAP of the Application Protocol Entity indicated by the Entity ID. If the GCC Provider knows of no currently enrolled Application Protocol Entity with the corresponding Entity ID, the indication is ignored and no further action is taken.

While any registry entry is set to be monitored, any change in the content of the registry entry, including setting a parameter, deletion of the entry, change in ownership, or change in the Modification Rights state, causes the Top GCC Provider to send a RegistryMonitorEntryIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryMonitorEntryIndication PDU is shown in Table 8-44.

TABLE  8-44/T.124

RegistryMonitorEntryIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Key | Top GCC Provider | Indication |
| Registry Item | Top GCC Provider | Indication |
| Owner | Top GCC Provider | Confirm |
| Modification Rights (optional) | Top GCC Provider | Confirm |

On receipt of a RegistryMonitorEntryIndication PDU, a GCC Provider for a node which supports the registry monitoring function shall generate a GCC-Registry-Monitor indication primitive and send this primitive to any GCCSAP which had previously issued a GCC-Registry-Monitor request specifying the same Key as is indicated in the indication PDU with the Enable/Disable flag set to Enable (without having more recently received one with this flag set to Disable). If there have not been any requesters for the particular Key at this node, the indication PDU is ignored by the GCC Provider. It is a function of each local GCC Provider to keep a local data-base of requesting GCCSAPs which is updated whenever a GCC-Registry-Monitor request is received.

NOTE – As a local matter, a particular GCC Provider implementation may choose not to keep track of which Application Protocol Entities have enabled or disabled monitoring for each entry and instead issue GCC-Registry-Monitor indications to the GCCSAPs corresponding to all enrolled Application Protocol Entities when a RegistryMonitorEntryIndication PDU is received.

### 8.4.7	Allocation of unique handles

On receipt of a GCC-Registry-Allocate-Handle request primitive, a GCC Provider shall send a RegistryAllocateHandleRequest PDU to the Top GCC Provider by issuing an MCS-Send-Data request specifying the Node ID Channel of the Top GCC Provider as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryAllocateHandleRequest PDU is shown in Table 8-45.

TABLE  8-45/T.124

RegistryAllocateHandleRequest GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Source GCC Provider | Top GCC Provider |
| Number of Handles | Request | Top GCC Provider |

On receipt of the RegistryAllocateHandleRequest PDU, the Top GCC Provider shall generate a unique set of handles of the number requested. This shall be done by incrementing a 32-bit state variable after each allocation modulo 232, using the incremented value for allocation of the next requested handle. If the full set of possible handles has been allocated, the request is rejected indicating that no handles are available. If a sufficient number of handles are available, the handles are returned to the requester by specifying the first allocated handle along with the number of allocated handles. The Top GCC Provider returns the result by sending a RegistryAllocateHandleResponse PDU to the requester. This is done by issuing an MCS-Send-Data request specifying the Node ID of the requester as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the RegistryAllocateHandleResponse PDU is shown in Table 8-46. If the result was successful, the registry item parameter contains the list of handles.

TABLE  8-46/T.124

RegistryAllocateHandleResponse GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Entity ID | Top GCC Provider | Destination GCC Provider |
| Number of Handles | Top GCC Provider | Confirm |
| First Handle | Top GCC Provider | Confirm |
| Result | Top GCC Provider | Confirm |

On receipt of a RegistryAllocateHandleResponse indication, a GCC Provider shall generate a GCC-Registry-Allocate-Handle confirm primitive and issue it to the GCCSAP of the Application Protocol Entity indicated by the Entity ID. If the GCC Provider knows of no currently enrolled Application Protocol Entity with the corresponding Entity ID, the indication is ignored and no further action is taken.

### 8.4.8	Changes in ownership and registry cleanup

Whenever the Application Roster is updated, the Top GCC Provider shall determine if any Application Protocol Entities were removed from the roster. If so, it shall examine the entire registry data-base to determine if the disconnecting Application Protocol Entities were the owner of any registry entries. If so, these entries are modified to indicate that there is no current owner.

If the result of a change in the Application Roster indicates that all Application Protocol Entities in a session have become un-enrolled, the Top GCC Provider shall examine all registry entries to determine if the Session Key portion of the Registry Key matches the Session Key of the former session. For entries that do correspond to the removed session, the Top GCC Provider shall delete the registry entry. If that registry entry corresponded to a Token type entry, the Top GCC Provider may deallocate the Token to allow for its later reuse.

## 8.5	Conference Conductorship

Conference conductorship is controlled through the use of the Conference-Conductorship-Token. When this token is free the conference is in Non-Conducted Mode. When this token is grabbed the conference is in Conducted Mode, and the grabber of the token is the Conference Conductor.

Upon acquiring the Conference-Conductorship-Token, the Conference Conductor (unless it is the Top GCC Provider itself) shall demonstrate possession of the token to the Top GCC Provider by offering to give it away, invoking MCS-Token-Give request and specifying the Top GCC Provider as recipient. The Top GCC Provider shall respond negatively to the resulting MCS-Token-Give indication, leaving the token in possession of the Conference Conductor. If the token is offered again later by the current Conference Conductor, the Top GCC Provider shall interpret it as part of a GCC-Conductor-Give operation and need not refuse automatically.

### 8.5.1	Grabbing conductorship

On receipt of a GCC-Conductor-Assign request primitive, a GCC Provider shall first examine the Conference Profile to determine if the conference is conductible or not. If not, it shall generate a GCC-Conductor-Assign confirm primitive with an unsuccessful result indicating that the conference is non-conductible. If the conference is conductible, the GCC Provider shall then examine its local data-base to determine if it is currently the conference conductor. If so, it shall generate a GCC-Conductor-Assign confirm primitive with a successful result and issue it to the Control GCCSAP. If not, it shall attempt to grab the conductor token by issuing the MCS primitive MCS-Token-Grab request, specifying the Conference-Conductorship-Token as the token ID to be grabbed.

On receipt of the MCS-Token-Grab confirm primitive, the GCC Provider shall examine the Result parameter. If the Result is successful, the GCC Provider shall then issue the MCS primitive MCS-Token-Give request, specifying the Conference-Conductorship-Token as the token ID to be offered and the Node ID of the Top GCC Provider as the User ID to receive the token. On receipt of the MCS-Token-Give confirm primitive, the GCC Provider shall examine the Result parameter. If the Result is unsuccessful, the GCC Provider shall locally generate the GCC-Conductor-Assign confirm primitive indicating a successful result and issue it to the Control GCCSAP.

On receipt of an MCS-Token-Give indication specifying the Conference-Conductorship-Token as the token ID offered, from a donor User ID that the Top GCC Provider does not recognize as the current Conference Conductor, the Top GCC Provider shall update its records to recognize the donor as Conference Conductor. It shall also send a ConductorAssignIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The content of the ConductorAssignIndication PDU is shown in Table 8-47.

If the Result parameter of the MCS-Token-Grab confirm indicates an unsuccessful result or the Result parameter of the MCS-Token-Give confirm indicates an successful result, the GCC Provider shall generate a GCC-Conductor-Assign confirm primitive indicating a negative result which reflects the unsatisfactory result and issue it to the Control GCCSAP.

TABLE  8-47/T.124

ConductorAssignIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Conducting Node | Top GCC Provider | Destination GCC Provider |

### 8.5.2	Releasing conductorship

On receipt of a GCC-Conductor-Release request primitive, a GCC Provider shall first examine its local data-base to determine if it is currently the conference conductor. If not, it shall generate a GCC-Conductor-Release confirm primitive indicating a negative result and issue it to the Control GCCSAP. If it is currently the conductor, it shall first send a ConductorReleaseIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The content of the ConductorReleaseIndication PDU is shown in Table 8-48 (there are no parameters in this PDU). It shall then release the conductor token by issuing the MCS primitive MCS-Token-Release request, specifying the Conference-Conductorship-Token as the token ID to be released.

On receipt of the MCS-Token-Release confirm primitive, the GCC Provider shall locally generate the GCC-Conductor-Release confirm primitive indicating a successful result and issue it to the Control GCCSAP.

TABLE  8-48/T.124

ConductorReleaseIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| -- No parameters -- |  |  |

### 8.5.3	Conductor assignment and release indications

A node which supports Application Protocols that are specified to behave differently in Conducted mode and Non-conducted mode shall respond to receipt of ConductorAssignIndication and ConductorReleaseIndication PDUs. A node which does not support such Application Protocols may choose to ignore these indications.

On receipt of a ConductorAssignIndication PDU as part of an MCS-Uniform-Send-Data indication, a GCC Provider shall examine the Conference Profile to determine if the conference is conductible or not. If not, it shall ignore this PDU. If the conference is conductible it may generate a GCC-Conductor-Assign indication primitive and issue it to the Control GCCSAP as well as the GCCSAP of all enrolled Application Protocol Entities. It shall first determine if the Sender User ID field of the MCS-Uniform-Send-Data indication indicates that the PDU was transmitted by the Top GCC Provider. If not, the PDU shall be ignored, and no indication primitive shall be generated. If the PDU was transmitted by the Top GCC Provider, the Node ID parameter of the primitive is set to the value indicated by the Conducting Node parameter in the received PDU. For a node which supports conductorship primitives, the GCC Provider shall also store in its local data-base the fact that the conference is now in conducted mode, as well as the Node ID of the conductor.

On receipt of a ConductorReleaseIndication PDU as part of an MCS-Uniform-Send-Data indication, a GCC Provider may generate a GCC-Conductor-Release indication primitive and issue it to the Control GCCSAP as well as to the GCCSAP of all enrolled Application Protocol Entities. It shall first determine if the Sender User ID field of the MCS-Uniform-Send-Data indication indicates that the PDU was transmitted by the Top GCC Provider or by the node that it currently records as the conductor. If not, the PDU shall be ignored, and no indication primitive shall be generated. For a node which supports conductorship primitives, the GCC Provider shall also store in its local data-base the fact that the conference is no longer in conducted mode. The GCC Provider shall also set its locally stored permission flag to indicate that no conducted-mode permissions have been granted.

A GCC Provider shall preserve the order of received ConductorAssignIndication and ConductorReleaseIndication PDUs in generation of the corresponding indication primitives.

The GCC Provider in a node which supports the conductorship primitives shall issue a GCC-Conductor-Assign or GCC-Conductor-Release indication to the GCCSAP associated with an Application Protocol Entity which newly enrolls after a conference has already been established. The current conductorship state as known by the GCC Provider determines which of these two primitives to issue.

### 8.5.4	Asking to be given conductorship

On receipt of a GCC-Conductor-Please request primitive, a GCC Provider shall first examine its local data-base to determine if the conference is in conducted mode. If not, it shall generate a GCC-Conductor-Please confirm with a negative result indicating that the conference is not in conducted mode and issue it to the Control GCCSAP. If in conducted mode, the GCC Provider issues an MCS-Token-Please request specifying the Conference-Conductorship-Token as the token requested. It also shall generate a GCC-Conductor-Please confirm indicating a successful result and issue it to the Control GCCSAP.

On receipt of an MCS-Token-Please indication specifying the Conference-Conductorship-Token as the Token ID, a GCC Provider shall generate a GCC-Conductor-Please indication primitive indicating the Node ID of the requesting node obtained from the User ID parameter received in the MCS-Token-Please indication and issue it to the Control GCCSAP.

### 8.5.5	Giving conductorship

On receipt of a GCC-Conductor-Give request primitive, a GCC Provider shall first examine its local data-base to determine if it is the current conference conductor. If not, it shall generate a GCC-Conductor-Give confirm with a negative result indicating that the node is not currently the conductor and issue it to the Control GCCSAP. Otherwise, the GCC Provider shall attempt to give conductorship to the specified node by issuing an MCS-Token-Give request specifying the Conference-Conductorship-Token as the token ID, and the recipient node as specified in the GCC-Conductor-Give request.

On receipt of the MCS-Token-Give confirm primitive, the GCC Provider shall examine the Result parameter. If the Result is successful, the GCC Provider shall locally generate the GCC-Conductor-Give confirm primitive indicating a successful result and issue it to the Control GCCSAP. If the Result parameter of the MCS-Token-Give confirm indicates an unsuccessful result, the GCC Provider shall generate a GCC-Conductor-Give confirm primitive indicating a negative result which reflects the result indicated by the MCS-Token-Grab confirm and issue it to the Control GCCSAP.

On receipt of the MCS-Token-Give indication primitive specifying the Conference-Conductorship-Token, a GCC Provider other than the Top GCC Provider shall generate a GCC-Conductor-Give indication primitive and issue it to the Control GCCSAP. On receipt of the GCC-Conductor-Give response, the GCC Provider shall examine the result parameter. If the result is successful, the GCC Provider shall issue the MCS-Token-Give response indicating that the token was accepted.

If the GCC-Conductor-Give response indicates that the token is not to be accepted, the GCC Provider shall issue the MCS-Token-Give response indicating that the token was user-rejected.

If a node which does not support conductorship primitives receives an MCS-Token-Give indication, the GCC Provider shall respond with an MCS-Token-Give response indicating a result of user-rejected.

On receipt of an MCS-Token-Give indication primitive specifying the Conference-Conductorship-Token from a donor User ID that the Top GCC Provider does not record as the current conductor, the Top GCC Provider shall respond as specified in 8.5.1, automatically rejecting the token. If it already recognizes the donor as conductor; however, the Top GCC Provider shall generate a GCC-Conductor-Give indication primitive as specified above, like any other GCC Provider.

### 8.5.6	Getting conductorship status

On receipt of a GCC-Conductor-Inquire request, a GCC Provider shall first examine its own local data-base to determine if the conference is currently in conducted mode. If not, the GCC Provider shall generate a GCC-Conductor-Inquire confirm which indicates that the conference is not in conducted mode and issue it to the Control GCCSAP. Otherwise, it shall generate a confirm indicating that the conference is in conducted mode, and shall specify the Node ID of the conducting node as stored in its local data-base as the current conductor. In this case, if a node has just entered a conference and has not yet received a ConductorAssignIndication PDU from the conductor (and does not yet have the conductor’s Node ID in its local data-base), the GCC Provider shall wait until this PDU is received before generating the GCC-Conductor-Inquire confirm.

If the conference is in conducted mode, the GCC Provider shall determine the setting of the Permission Flag to be included in the GCC-Conductor-Inquire confirm by determining whether the local node is the current conductor (in which case permission is assumed to be granted), or, if not, whether the Node ID of the local node had been listed in the Permission List of most recent ConductorPermissionGrantIndication PDU which had been received since the last transition from non-conducted to conducted mode. If the local node does appear in this list, this flag is set to TRUE. If not, or if there had not been a ConductorPermissionGrantIndication PDU received since the most recent transition from non-conducted to conducted mode, then this flag is set to FALSE.

### 8.5.7	Conductorship announcement when new nodes enter a conference

If the Top GCC Provider receives an update to the Conference Roster with the flag set indicating that new nodes are present in the conference and the conference is currently in conducted mode, the Top GCC Provider shall send a ConductorAssignIndication PDU to all nodes in the conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The content of the ConductorAssignIndication PDU is shown in Table 8-47. The Conducting Node parameter indicates the Node ID of the current conductor.

If the Top GCC Provider receives an update to the Conference Roster with the flag set indicating that new nodes are present in the conference and the conference is not currently in conducted mode, the Top GCC Provider shall send a ConductorReleaseIndication PDU to all nodes by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying Top data priority, and including the PDU in the Data field. The content of the ConductorReleaseIndication PDU is shown in Table 8-48.

If the Top GCC Provider receives a ConductorReleaseIndication from another node, which signals a change from the current conductorship state, prior to receiving its own transmitted ConductorAssignIndication, the Top GCC Provider shall re-issue a ConductorReleaseIndication to reflect the new state.

A node which has just become joined to a conference shall delay processing of any of the conductor related request primitives until receiving either a ConductorAssignIndication or a ConductorReleaseIndication. This is because all conductor related request primitives require that the local node check its local data-base of conductorship status which is not in harmony with the actual status of the conference until one of these PDUs is received.

### 8.5.8	Unexpected disconnection of the conductor

If the conductor of a conference unexpectedly disconnects from a conference, it may not have an opportunity to send a ConductorReleaseIndication PDU to all nodes indicating that the conference is now in non-conducted mode. The GCC Provider at each node (if it supports the conductorship primitives) shall monitor received MCS-Detach-User indications and compare the User ID in the indication to the Node ID of the node that it currently believes to be the conductor. If they match, the GCC Provider shall store in its local data-base that the conference is no longer in conducted mode, and the GCC Provider shall generate a GCC-Conductor-Release indication and issue it to the Control GCCSAP as well as to the GCCSAP of all enrolled Application Protocol Entities. The GCC Provider shall also set its locally stored permission flag to indicate that no conducted-mode permissions have been granted.

### 8.5.9	Asking to be given conducted-mode permission

On receipt of a GCC-Conductor-Permission-Ask request primitive via the Control GCCSAP, a GCC Provider which supports this primitive shall first determine if the specified conference is in conducted mode. If not it shall generate a GCC-Conductor-Permission-Ask confirm primitive and issue it to the Control GCCSAP indicating not-in-conducted-mode as the result. If in conducted mode, it shall send a ConductorPermissionAskIndication PDU to the conductor of the specified conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the data field. The content of the ConductorPermissionAskIndication PDU is shown in Table 8-49. When successfully completed, the GCC Provider shall generate a GCC-Conductor-Permission-Ask confirm primitive and issue it to the Control GCCSAP indicating success as the result.

TABLE  8-49/T.124

ConductorPermissionAskIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Grant/Release Flag | Request | Indication |

On receipt of a ConductorPermissionAskIndication PDU, a GCC Provider shall determine if it is currently the conference conductor. If not, it shall ignore the PDU and take no further action. If so, if the GCC Provider supports the corresponding primitive, it shall generate a GCC-Conductor-Permission-Ask indication primitive and issue it to the Control GCCSAP. The Node ID of the Requester parameter shall be filled in with the Sender User ID parameter from the received MCS-Uniform-Send-Data indication. A GCC Provider shall preserve the order of received ConductorPermissionAskIndication PDUs in generating the corresponding indication primitives.

### 8.5.10	Granting conducted-mode permission

On receipt of a GCC-Conductor-Permission-Grant request via the Control GCCSAP, a GCC Provider which supports this primitive shall first determine if it is currently the conductor of the specified conference. If not, it shall generate a GCC-Conductor-Permission-Grant confirm indicating not-conductor as the reason for rejection, and issue it to the Control GCCSAP. It shall then take no further action. If the node is the current conference conductor, the GCC Provider shall broadcast a ConductorPermissionGrantIndication PDU to all nodes in the conference. See Table 8-50. It shall do this by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying Top data priority, and including the PDU in the data field. The content of the ConductorAssignIndication PDU is shown in Table 8-47. The permission and waiting list parameters are filled in with the list of Node IDs provided in the corresponding parameters of the request primitive. The order of both lists is preserved. When successfully completed, the GCC Provider shall generate a GCC-Conductor-Permission-Grant confirm primitive and issue it to the Control GCCSAP indicating success as the result.

TABLE  8-50/T.124

ConductorPermissionGrantIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Permission List | Request | Indication |
| Waiting List (optional) | Request | Indication |

On receipt of a ConductorPermissionGrantIndication PDU, a GCC Provider which supports the corresponding primitive shall first determine if the conference is currently in conducted mode. If not, it shall ignore the received PDU and take no further action. If the conference is in conducted mode, it shall determine if this PDU was received from the node which is the current conference conductor. If not, it shall also ignore this PDU and take no further action. If the source is the current conductor, it shall generate a GCC-Conductor-Permission-Grant indication primitive and issue it to the Control GCCSAP as well as to the GCCSAPs of all enrolled Application Protocol Entities. The Permission and Waiting Lists in the indication primitive are filled in directly from the corresponding parameters of the PDU, preserving the order of these lists. The GCC Provider shall examine the Permission List parameter and determine if the local node is included in this list. If so, it shall set the Permission Flag parameter of the indication primitive to TRUE. Otherwise, it shall set this flag to FALSE. A GCC Provider shall preserve the order of received ConductorPermissionGrantIndication PDUs in generating the corresponding indication primitives.

## 8.6	Miscellaneous functions

### 8.6.1	Timed conferences

On receipt of a GCC-Conference-Time-Remaining request primitive, a GCC Provider shall send a ConferenceTimeRemainingIndication PDU to all nodes in the specified conference by issuing an MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTimeRemainingIndication PDU is shown in Table 8-51. The value of the Time Remaining parameter is obtained from the contents of the request primitive. The GCC Provider shall also generate a GCC-Conference-Time-Remaining confirm primitive indicating a successful result, and issue it to the Control GCCSAP.

TABLE  8-51/T.124

ConferenceTimeRemainingIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Time Remaining | Request | Indication |
| Node Identifier (optional) | Request | Indication |

On receipt of a ConferenceTimeRemainingIndication PDU, a GCC Provider may optionally generate a GCC-Conference-Time-Remaining indication primitive and issue it to the Control GCCSAP. The Time Remaining parameter is obtained from the contents of the PDU. The Source Node parameter is filled in from the Sender User ID in the received MCS-Uniform-Send-Data indication.

On receipt of a GCC-Conference-Time-Inquire request primitive, a GCC Provider shall send a ConferenceTimeInquireIndication PDU to the Conference Convener by issuing an MCS-Send-Data request specifying the GCC-Convener-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTimeInquireIndication PDU is shown in Table 8-52. The GCC Provider shall then generate a GCC-Conference-Time-Inquire confirm primitive and issue it to the Control GCCSAP.

TABLE  8-52/T.124

ConferenceTimeInquireIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Node-Specific Time Flag | Request | Indication |

On receipt of a ConferenceTimeInquireIndication PDU, the GCC Provider (of the Conference Convener) may optionally generate a GCC-Conference-Time-Inquire indication primitive and issue it to the Control GCCSAP. The Node ID of the requester, as indicated in the Source Node parameter of the MCS-Send-Data indication, is used as the Node ID of the Requesting Node parameter in the GCC-Conference-Time-Inquire indication primitive.

On receipt of a GCC-Conference-Extend request primitive, a GCC Provider shall send a ConferenceTimeExtendIndication PDU to the Conference Convener by issuing an MCS-Send-Data request specifying the GCC-Convener-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceTimeExtendIndication PDU is shown in Table 8-53. The GCC Provider shall then generate a GCC-Conference-Extend confirm primitive and issue it to the Control GCCSAP.

TABLE  8-53/T.124

ConferenceTimeExtendIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Time to Extend | Request | Indication |
| Node-Specific Time Flag | Request | Indication |

On receipt of a ConferenceTimeExtendIndication PDU, the GCC Provider (of the Conference Convener) may optionally generate a GCC-Conference-Extend indication primitive and issue it to the Control GCCSAP. The Node ID of the requester, as indicated in the Source Node parameter of the MCS-Send-Data indication, is used as the Node ID of the Requesting Node parameter in the GCC-Conference-Extend indication primitive.

### 8.6.2	Requesting conference assistance

On receipt of a GCC-Conference-Assistance request primitive, a GCC Provider shall send a ConferenceAssistanceIndication PDU to the all nodes in the specified conference by issuing an MCS-Send-Data or MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the ConferenceAssistanceIndication PDU is shown in Table 8-54. The GCC Provider shall then generate a GCC-Conference-Assistance confirm primitive indicating whether the operation was successful.

TABLE  8-54/T.124

ConferenceAssistanceIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| User Data (optional) | Request | Indication |

On receipt of the ConferenceAssistanceIndication PDU, a node which supports this function may generate a GCC-Conference-Assistance indication primitive and issue it to the Control GCCSAP. The Source Node parameter is obtained from the Sender User ID in the received MCS-Uniform-Send-Data indication.

### 8.6.3	Broadcasting a text message

On receipt of a GCC-Text-Message request primitive, if no Destination Node parameter was included in the request, a GCC Provider shall send a TextMessageIndication PDU to the all nodes in the specified conference by issuing either an MCS-Send-Data request or MCS-Uniform-Send-Data request specifying the GCC-Broadcast-Channel as the Channel ID, specifying High data priority, and including the PDU in the Data field. If a Destination Node was indicated, the GCC Provider shall send the same PDU to that node by issuing an MCU-Send-Data request specifying the Node ID of the requested node as the Channel ID, specifying High data priority, and including the PDU in the Data field. The content of the TextMessageIndication PDU is shown in Table 8-55. The GCC Provider shall then generate a GCC-Text-Message confirm primitive indicating whether the operation was successful.

TABLE  8-55/T.124

TextMessageIndication GCCPDU

| Content | Source | Sink |
| --- | --- | --- |
| Message | Request | Indication |

On receipt of the TextMessageIndication PDU, a node which supports this function may generate a GCC-Text-Message indication primitive and issue it to the Control GCCSAP. The Source Node parameter is obtained from the Sender User ID in the received MCS-Uniform-Send-Data indication.

## 8.7	GCCPDU definitions

The structure of GCCPDUs is specified as follows using the notation ASN.1 of Recommendation X.680. All GCCPDUs shall be encoded for transmission by applying the Packed Encoding Rules of Recommendation X.691 using the Basic Aligned variant.

NOTE – The use of Automatic Tags in the GCC protocol definition implies that the order of SEQUENCE and CHOICE structures contained within this definition effects the actual encoded values.

GCC-PROTOCOL DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

--  Export all symbols

--  Part 1: Elements of which messages are composed

ChannelID			::= INTEGER (1..65535)

StaticChannelID		::= INTEGER (1..1000)	--  Those assigned by specifications

DynamicChannelID		::= INTEGER (1001..65535)	--  Those created and deleted by MCS

UserID				::= DynamicChannelID

TokenID				::= INTEGER (1..65535)

StaticTokenID			::= INTEGER (1..16383)	--  Those assigned by specifications

DynamicTokenID		::= INTEGER (16384..65535)	--  Those assigned by the registry

Time				::= INTEGER (-2147483648..2147483647)	--  Time in seconds

Handle				::= INTEGER (0..4294967295)	--  32-bit value

H221NonStandardIdentifier ::= OCTET STRING (SIZE (4..255))

--  First four octets shall be country code and
					--  Manufacturer code, assigned as specified in
					--  Annex A/H.221 for NS-cap and NS-comm

Key ::= CHOICE			--  Identifier of a standard or non-standard object

{

object				OBJECT IDENTIFIER,

h221NonStandard		H221NonStandardIdentifier

}

NonStandardParameter ::= SEQUENCE

{

key		Key,

data	OCTET STRING

}

TextString ::= BMPString (SIZE (0..255))	--  Basic Multilingual Plane of ISO/IEC 10646-1 (Unicode)

simpleTextFirstCharacter UniversalString ::= {0, 0, 0, 0}

simpleTextLastCharacter UniversalString ::= {0, 0, 0, 255}

SimpleTextString ::= BMPString (SIZE (0..255)) (FROM (simpleTextFirstCharacter..simpleTextLastCharacter))

SimpleNumericString ::= NumericString (SIZE (1..255)) (FROM (“0123456789”))

DialingString ::= NumericString (SIZE (1..16)) (FROM (“0123456789”))

SubAddressString ::= NumericString (SIZE (1..40)) (FROM (“0123456789”))

ExtraDialingString ::= TextString (SIZE (1..255)) (FROM (“0123456789#*,”))

UserData ::= SET OF SEQUENCE

{

key			Key,

value		OCTET STRING OPTIONAL

}

Password ::= SEQUENCE

{

numeric		SimpleNumericString,

text		SimpleTextString OPTIONAL,

...

}

PasswordSelector ::= CHOICE

{

numeric		SimpleNumericString,

text		SimpleTextString,

...

}

ChallengeResponseItem ::= CHOICE

{

passwordString		PasswordSelector,

responseData			UserData,

...

}

ChallengeResponseAlgorithm ::= CHOICE

{

passwordInTheClear	NULL,

nonStandardAlgorithm	NonStandardParameter,

...

}

ChallengeItem ::= SEQUENCE

{

responseAlgorithm		ChallengeResponseAlgorithm,

challengeData		UserData,

...

}

ChallengeRequest ::= SEQUENCE

{

challengeTag			INTEGER,

challengeSet			SET OF ChallengeItem,	--  Set of algorithms offered for response

...

}

ChallengeResponse ::= SEQUENCE

{

challengeTag			INTEGER,

responseAlgorithm		ChallengeResponseAlgorithm,

--  Specific algorithm selected from the set of
					--  items presented in the ChallengeRequest

responseItem			ChallengeResponseItem,

...

}

PasswordChallengeRequestResponse ::= CHOICE

{

passwordInTheClear	PasswordSelector,

challengeRequestResponse	SEQUENCE

{

challengeRequest	ChallengeRequest OPTIONAL,

challengeResponse	ChallengeResponse OPTIONAL,

...

},

...

}

ConferenceName ::= SEQUENCE

{

numeric		SimpleNumericString,

text		SimpleTextString OPTIONAL,

...

}

ConferenceNameSelector ::= CHOICE

{

numeric		SimpleNumericString,

text		SimpleTextString,

...

}

ConferenceNameModifier ::= SimpleNumericString

Privilege  ::= ENUMERATED

{

terminate	(0),

ejectUser	(1),

add		(2),

lockUnlock	(3),

transfer		(4),

...

}

TerminationMethod ::= ENUMERATED

{

automatic	(0),

manual		(1),

...

}

NetworkAddress ::= SEQUENCE (SIZE (1..64)) OF CHOICE	--  Listed in order of use

{

aggregatedChannel	SEQUENCE

{

transferModes		SEQUENCE	--  One or more

{

speech		BOOLEAN,

voice-band	BOOLEAN,

digital-56k	BOOLEAN,

digital-64k	BOOLEAN,

digital-128k	BOOLEAN,

digital-192k	BOOLEAN,

digital-256k	BOOLEAN,

digital-320k	BOOLEAN,

digital-384k	BOOLEAN,

digital-512k	BOOLEAN,

digital-768k	BOOLEAN,

digital-1152k	BOOLEAN,

digital-1472k	BOOLEAN,

digital-1536k	BOOLEAN,

digital-1920k	BOOLEAN,

packet-mode	BOOLEAN,

frame-mode	BOOLEAN,

atm			BOOLEAN,

...

},

internationalNumber	DialingString,

subAddress	SubAddressString OPTIONAL,

extraDialing	ExtraDialingString OPTIONAL,

highLayerCompatibility	SEQUENCE

{

telephony3kHz	BOOLEAN,

telephony7kHz	BOOLEAN,

videotelephony	BOOLEAN,

videoconference	BOOLEAN,

audiographic	BOOLEAN,

audiovisual	BOOLEAN,

multimedia	BOOLEAN,

...

} OPTIONAL,

...

},

transportConnection	SEQUENCE

{

nsapAddress		OCTET STRING (SIZE (1..20)),

transportSelector	OCTET STRING OPTIONAL

},

nonStandard		NonStandardParameter,

...

}

NodeType ::= ENUMERATED

{

terminal				(0),

multiportTerminal		(1),

mcu				(2),

...

}

NodeProperties ::= SEQUENCE

{

managementDevice	BOOLEAN,	--  Is the node a device such as a reservation system

peripheralDevice		BOOLEAN,	--  Is the node a peripheral to a primary node

...

}

AsymmetryIndicator ::= CHOICE

{

callingNode		NULL,

calledNode		NULL,

unknown		INTEGER (0..4294967295)	--  Uniformly distributed 32-bit random number

}

AlternativeNodeID ::= CHOICE

{

h243NodeID			OCTET STRING (SIZE (2)),

...

}

ConferenceDescriptor ::= SEQUENCE

{

conferenceName		ConferenceName,

conferenceNameModifier	ConferenceNameModifier OPTIONAL,

conferenceDescription	TextString OPTIONAL,

lockedConference		BOOLEAN,

passwordInTheClearRequired	BOOLEAN,

networkAddress		NetworkAddress OPTIONAL,

...

}

NodeRecord ::= SEQUENCE

{

superiorNode			UserID OPTIONAL,	--  Not present only for the Top GCC Provider

nodeType			NodeType,

nodeProperties		NodeProperties,

nodeName			TextString OPTIONAL,

participantsList		SEQUENCE OF TextString OPTIONAL,

siteInformation		TextString OPTIONAL,

networkAddress		NetworkAddress OPTIONAL,

alternativeNodeID		AlternativeNodeID OPTIONAL,

userData			UserData OPTIONAL,

...

}

SessionKey ::= SEQUENCE

{

applicationProtocolKey	Key,

sessionID			ChannelID OPTIONAL

}

ChannelType ::= ENUMERATED

{

static				(0),

dynamicMulticast		(1),

dynamicPrivate		(2),

dynamicUserId		(3)

}

ApplicationRecord ::= SEQUENCE

{

applicationActive		BOOLEAN,	--  Active/Inactive flag

conductingOperationCapable	BOOLEAN,	--  Maximum one per node per session

startupChannel		ChannelType OPTIONAL,

applicationUserID		UserID OPTIONAL,  --  User ID assigned to the Application Protocol Entity

nonCollapsingCapabilities	SET OF SEQUENCE

{

capabilityID		CapabilityID,

applicationData	OCTET STRING OPTIONAL

} OPTIONAL,

...

}

CapabilityID ::= CHOICE

{

standard			INTEGER (0..65535),	--  Assigned by Application Protocol specifications

nonStandard			Key

}

CapabilityClass ::= CHOICE

{

logical				NULL,

unsignedMin			INTEGER (0..MAX),	--  Capability value

unsignedMax			INTEGER (0..MAX),	--  Capability value

...

}

EntityID ::= INTEGER (0..65535)

ApplicationInvokeSpecifier ::= SEQUENCE

{

sessionKey			SessionKey,

expectedCapabilitySet	SET OF SEQUENCE

{

capabilityID	CapabilityID,

capabilityClass	CapabilityClass,

...

} OPTIONAL,

startupChannel		ChannelType OPTIONAL,

mandatoryFlag		BOOLEAN,	--  TRUE indicates required Application Protocol Entity

...

}

RegistryKey ::= SEQUENCE

{

sessionKey			SessionKey,

resourceID			OCTET STRING (SIZE (0..64))

}

RegistryItem	 ::= CHOICE

{

channelID		DynamicChannelID,

tokenID			DynamicTokenID,

parameter		OCTET STRING (SIZE (0..64)),

vacant			NULL,

...

}

RegistryEntryOwner ::= CHOICE

{

owned			SEQUENCE

{

nodeID			UserID,	--  Node ID of the owning node

entityID			EntityID	--  Entity ID of the owning

},					--  Application Protocol Entity

notOwned		NULL	--  There is no current owner

}

RegistryModificationRights ::= ENUMERATED

{

owner			(0),

session			(1),

public			(2)

}

--  Part 2: PDU Messages

UserIDIndication ::= SEQUENCE

{

tag						INTEGER,

...

}

ConferenceCreateRequest ::= SEQUENCE

{					--  MCS-Connect-Provider request user data

conferenceName		ConferenceName,

convenerPassword		Password OPTIONAL,

password			Password OPTIONAL,

lockedConference		BOOLEAN,

listedConference		BOOLEAN,

conductibleConference	BOOLEAN,

terminationMethod	TerminationMethod,

conductorPrivileges	SET OF Privilege OPTIONAL,

conductedPrivileges	SET OF Privilege OPTIONAL,

nonConductedPrivileges	SET OF Privilege OPTIONAL,

conferenceDescription	TextString OPTIONAL,

callerIdentifier		TextString OPTIONAL,

userData			UserData OPTIONAL,

...

}

ConferenceCreateResponse ::= SEQUENCE

{					--  MCS-Connect-Provider response user data

nodeID				UserID,	--  Node ID of the sending node

tag					INTEGER,

result				ENUMERATED

{

success			(0),

userRejected		(1),

resourcesNotAvailable	(2),

rejectedForSymmetryBreaking	(3),

lockedConferenceNotSupported	(4),

...

},

userData			UserData OPTIONAL,

...

}

ConferenceQueryRequest ::= SEQUENCE

{					--  MCS-Connect-Provider request user data

nodeType			NodeType,

asymmetryIndicator	AsymmetryIndicator OPTIONAL,

userData			UserData OPTIONAL,

...

}

ConferenceQueryResponse ::= SEQUENCE

{					--  MCS-Connect-Provider response user data

nodeType			NodeType,

asymmetryIndicator	AsymmetryIndicator OPTIONAL,

conferenceList		SET OF ConferenceDescriptor,

result				ENUMERATED

{

success			(0),

userRejected		(1),

...

},

userData			UserData OPTIONAL,

...

}

ConferenceJoinRequest ::= SEQUENCE

{					--  MCS-Connect-Provider request user data as well as
					--  MCS-Send-Data on Node ID Channel of Top GCC sent
					--  by the receiver of the MCS-Connect-Provider

conferenceName		ConferenceNameSelector OPTIONAL,

--  Required when part of MCS-Connect-Provider

conferenceNameModifier	ConferenceNameModifier OPTIONAL,

tag					INTEGER OPTIONAL,  --  Filled in when sent on Node ID Channel of Top GCC

password			PasswordChallengeRequestResponse OPTIONAL,

convenerPassword		PasswordSelector OPTIONAL,

callerIdentifier		TextString OPTIONAL,

userData			UserData OPTIONAL,

...

}

ConferenceJoinResponse ::= SEQUENCE

{					--  MCS-Connect-Provider response user data as well as

--  MCS-Send-Data on Node ID Channel of
					--  the receiver of the MCS-Connect-Provider

nodeID				UserID OPTIONAL,	--  Node ID of directly connected node only

topNodeID			UserID, 	--  Node ID of Top GCC Provider

tag					INTEGER,

conferenceNameAlias	ConferenceNameSelector OPTIONAL,

passwordInTheClearRequired	BOOLEAN,

lockedConference		BOOLEAN,

listedConference		BOOLEAN,

conductibleConference	BOOLEAN,

terminationMethod	TerminationMethod,

conductorPrivileges	SET OF Privilege OPTIONAL,	 --  No privilege shall be listed more than once

conductedPrivileges	SET OF Privilege OPTIONAL,	 --  No privilege shall be listed more than once

nonConductedPrivileges	SET OF Privilege OPTIONAL,	 --  No privilege shall be listed more than once

conferenceDescription	TextString OPTIONAL,

password			PasswordChallengeRequestResponse OPTIONAL,

result				ENUMERATED

{

success		(0),

userRejected	(1),

invalidConference	(2),

invalidPassword	(3),

invalidConvenerPassword	(4),

challengeResponseRequired	(5),

invalidChallengeResponse	(6),

...

},

userData			UserData OPTIONAL,

...

}

ConferenceInviteRequest ::= SEQUENCE

{					--  MCS-Connect-Provider request user data

conferenceName		ConferenceName,

nodeID				UserID,	--  Node ID of the sending node

topNodeID			UserID,	--  Node ID of Top GCC Provider

tag					INTEGER,

passwordInTheClearRequired	BOOLEAN,

lockedConference		BOOLEAN,

listedConference		BOOLEAN,

conductibleConference	BOOLEAN,

terminationMethod	TerminationMethod,

conductorPrivileges	SET OF Privilege OPTIONAL,	 --  No privilege shall be listed more than once

conductedPrivileges	SET OF Privilege OPTIONAL,	 --  No privilege shall be listed more than once

nonConductedPrivileges	SET OF Privilege OPTIONAL,	 --  No privilege shall be listed more than once

conferenceDescription	TextString OPTIONAL,

callerIdentifier		TextString OPTIONAL,

userData			UserData OPTIONAL,

...

}

ConferenceInviteResponse ::= SEQUENCE

{					--  MCS-Connect-Provider response user data

result				ENUMERATED

{

success			(0),

userRejected		(1),

...

},

userData			UserData OPTIONAL,

...

}

ConferenceAddRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC or
					--  Node ID Channel of Adding MCU if specified

networkAddress		NetworkAddress,

requestingNode		UserID,

tag					INTEGER,

addingMCU			UserID OPTIONAL,

userData			UserData OPTIONAL,

...

}

ConferenceAddResponse ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

tag					INTEGER,

result				ENUMERATED

{

success			(0),

invalidRequester	(1),

invalidNetworkType	(2),

invalidNetworkAddress	(3),

addedNodeBusy	(4),

networkBusy		(5),

noPortsAvailable	(6),

connectionUnsuccessful	(7),

...

},

userData			UserData OPTIONAL,

...

}

ConferenceLockRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

--  No parameters

...

}

ConferenceLockResponse ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

result		ENUMERATED

{

success			(0),

invalidRequester	(1),

alreadyLocked		(2),

...

},

...

}

ConferenceLockIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel
					--  or MCS-Send-Data on Node ID Channel

--  No parameters

...

}

ConferenceUnlockRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

--  No parameters

...

}

ConferenceUnlockResponse ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

result		ENUMERATED

{

success			(0),

invalidRequester	(1),

alreadyunlocked	(2),

...

},

...

}

ConferenceUnlockIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel
					--  or MCS-Send-Data on Node ID Channel

--  No parameters

...

}

ConferenceTerminateRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

reason		ENUMERATED

{

userInitiated		(0),

timedConferenceTermination	(1),

...

},

...

}

ConferenceTerminateResponse ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

result		ENUMERATED

{

success			(0),

invalidRequester	(1),

...

},

...

}

ConferenceTerminateIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

reason		ENUMERATED

{

userInitiated			(0),

timedConferenceTermination	(1),

...

},

...

}

ConferenceEjectUserRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

nodeToEject		UserID,	--  Node ID of the node to eject

reason			ENUMERATED

{

userInitiated		(0),

...

},

...

}

ConferenceEjectUserResponse ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

nodeToEject		UserID,	--  Node ID of the node to eject

result			ENUMERATED

{

success			(0),

invalidRequester	(1),

invalidNode		(2),

...

},

...

}

ConferenceEjectUserIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

nodeToEject		UserID,	--  Node ID of the node to eject

reason			ENUMERATED

{

userInitiated		(0),

higherNodeDisconnected	(1),

higherNodeEjected	(2),

...

},

...

}

ConferenceTransferRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

conferenceName		ConferenceNameSelector,	  --  Name of conference to transfer to

conferenceNameModifier	ConferenceNameModifier OPTIONAL,

networkAddress		NetworkAddress OPTIONAL,

transferringNodes		SET (SIZE (1..65536)) OF UserID OPTIONAL,

password			PasswordSelector OPTIONAL,

...

}

ConferenceTransferResponse	 ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

conferenceName		ConferenceNameSelector,  	--  Name of conference to transfer to

conferenceNameModifier	ConferenceNameModifier OPTIONAL,

transferringNodes		SET (SIZE (1..65536)) OF UserID OPTIONAL,

result				ENUMERATED

{

success			(0),

invalidRequester	(1),

...

},

...

}

ConferenceTransferIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

conferenceName		ConferenceNameSelector,	  --  Name of conference to transfer to

conferenceNameModifier	ConferenceNameModifier OPTIONAL,

networkAddress		NetworkAddress OPTIONAL,

transferringNodes		SET (SIZE (1..65536)) OF UserID OPTIONAL,

--  List of Node IDs,

--  not present if destined for all nodes

password			PasswordSelector OPTIONAL,

...

}

RosterUpdateIndication ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel or

--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

fullRefresh			BOOLEAN,	--  Conference Roster and all

--  ApplicationProtocol Sessions refreshed

nodeInformation		SEQUENCE

{

nodeRecordList	CHOICE

{

noChange		NULL,

refresh		SET (SIZE (1..65536)) OF SEQUENCE

--  One for each node in the conference;
					--  no node shall be listed more than once

{

nodeID	UserID,	--  Node ID of the node

nodeRecord	NodeRecord

},

update		SET (SIZE (1..65536)) OF SEQUENCE

--  One for each node changing its node record;
					--  no node shall be listed more than once

{

nodeID	UserID,	--  Node ID of the node

nodeUpdate	CHOICE

{

addRecord	NodeRecord,

replaceRecord	NodeRecord,

removeRecord	NULL,

...

}

},

...

},

rosterInstanceNumber	INTEGER (0..65535),

nodesAdded		BOOLEAN,	--  Nodes have been added since last instance

nodesRemoved		BOOLEAN,	--  Nodes have been removed since last instance

...

} ,

applicationInformation	SET (SIZE (0..65535)) OF SEQUENCE

--  One for each Application Protocol Session;
					--  all Application Protocol Sessions if full refresh;
					--  no Application Protocol Session shall be
					--  listed more than once

{

sessionKey		SessionKey,

applicationRecordList	CHOICE

{

noChange		NULL,

refresh		SET (SIZE (0..65535)) OF SEQUENCE

--  One for each node with the
					--  Application Protocol Session enrolled;
					--  no node shall be listed more than once

{

nodeID	UserID,	--  Node ID of node

entityID	EntityID,	--  ID for this Application Protocol Entity at this node

applicationRecord	ApplicationRecord

},

update		SET (SIZE (1..65536)) OF SEQUENCE

--  One for each node modifying its Application Record;
					--  no node shall be listed more than once

{

nodeID	UserID,	--  Node ID of node

entityID	EntityID,	--  ID for this Application Protocol Entity at this node

applicationUpdate	CHOICE

{

addRecord	ApplicationRecord,

replaceRecord	ApplicationRecord,

removeRecord	NULL,

...

}

},

...

},

applicationCapabilitiesList	CHOICE

{

noChange		NULL,

refresh		SET OF SEQUENCE

{

capabilityID	CapabilityID,

capabilityClass	CapabilityClass,

numberOfEntities	INTEGER (1..65536),

--  Number of Application Protocol Entities
							--  which issued the capability

...

},

...

},

rosterInstanceNumber	INTEGER (0..65535),

peerEntitiesAdded	BOOLEAN,		--  Peer Entities have been added since last instance

peerEntitiesRemoved	BOOLEAN,		--  Peer Entities have been removed since last instance

...

},

...

}

ApplicationInvokeIndication ::= SEQUENCE

{					--  MCS-Send-Data or MCS-Uniform-Send-Data
					--  on GCC-Broadcast-Channel or Node ID Channel

applicationProtocolEntityList	SET (SIZE (1..65536)) OF ApplicationInvokeSpecifier,

destinationNodes		SET (SIZE (1..65536)) OF UserID OPTIONAL,

--  List of Node IDs,
					--  not present if destined for all nodes

...

}

RegistryRegisterChannelRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

entityID				EntityID,

key					RegistryKey,

channelID			DynamicChannelID,

...

}

RegistryAssignTokenRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

entityID				EntityID,

key					RegistryKey,

...

}

RegistrySetParameterRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

entityID				EntityID,

key					RegistryKey,

parameter			OCTET STRING (SIZE (0..64)),

modificationRights		RegistryModificationRights OPTIONAL,

...

}

RegistryRetrieveEntryRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

entityID				EntityID,

key					RegistryKey,

...

}

RegistryDeleteEntryRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

entityID				EntityID,

key					RegistryKey,

...

}

RegistryMonitorEntryRequest ::= SEQUENCE

{						--  MCS-Send-Data on Node ID Channel of Top GCC

entityID				EntityID,

key					RegistryKey,

...

}

RegistryMonitorEntryIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

key					RegistryKey,

item				RegistryItem,	--  Contents: channel, token, parameter, or empty

owner				RegistryEntryOwner,

modificationRights		RegistryModificationRights OPTIONAL,

...

}

RegistryAllocateHandleRequest ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of Top GCC

entityID				EntityID,

numberOfHandles		INTEGER (1..1024),

...

}

RegistryAllocateHandleResponse ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

entityID				EntityID,

numberOfHandles		INTEGER (1..1024),

firstHandle			Handle,

result				ENUMERATED

{

successful			(0),

noHandlesAvailable	(1),

...

},

...

}

RegistryResponse ::= SEQUENCE

{					--  MCS-Send-Data on Node ID Channel of requester

entityID				EntityID,	--  Entity ID of the requesting Application Protocol Entity

primitiveType		ENUMERATED

{

registerChannel	(0),

assignToken		(1),

setParameter		(2),

retrieveEntry		(3),

deleteEntry		(4),

monitorEntry		(5),

...

},

key					RegistryKey,	--  Data-base index

item				RegistryItem,	--  Contents: channel, token, parameter, or vacant

owner				RegistryEntryOwner,

modificationRights		RegistryModificationRights OPTIONAL,

result				ENUMERATED

{

successful			(0),

belongsToOther	(1),

tooManyEntries	(2),

inconsistentType	(3),

entryNotFound	(4),

entryAlreadyExists	(5),

invalidRequester	(6),

...

},

...

}

ConductorAssignIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

conductingNode		UserID,

...

}

ConductorReleaseIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

--  No parameters

...

}

ConductorPermissionAskIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

grantFlag			BOOLEAN,	--  TRUE to request permission grant, FALSE to release

...

}

ConductorPermissionGrantIndication ::= SEQUENCE

{					--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

permissionList		SEQUENCE (SIZE (0..65535)) OF UserID,

--  Node ID of nodes granted permission

waitingList			SEQUENCE (SIZE (1..65536)) OF UserID OPTIONAL,

--  Node ID of nodes waiting for permission

...

}

ConferenceTimeRemainingIndication ::= SEQUENCE

{					--  MCS-Send-Data on GCC-Broadcast-Channel

timeRemaining		Time,

nodeID				UserID OPTIONAL,

...

}

ConferenceTimeInquireIndication ::= SEQUENCE

{					--  MCS-Send-Data on GCC-Convener-Channel

nodeSpecificTimeFlag	BOOLEAN,	--  FALSE for conference-wide, TRUE for node-specific

...

}

ConferenceTimeExtendIndication ::= SEQUENCE

{					--  MCS-Send-Data on GCC-Convener-Channel

timeToExtend		Time,

nodeSpecificTimeFlag	BOOLEAN,	--  FALSE for conference-wide, TRUE for node-specific

...

}

ConferenceAssistanceIndication ::= SEQUENCE

{						--  MCS-Uniform-Send-Data on GCC-Broadcast-Channel

userData		UserData OPTIONAL,

...

}

TextMessageIndication ::= SEQUENCE

{					--  MCS-Send-Data or MCS-Uniform-Send-Data

message			TextString,	--  on GCC-Broadcast-Channel or Node ID Channel

...

}

FunctionNotSupportedResponse ::= SEQUENCE

{

request			RequestPDU

}

NonStandardPDU ::= SEQUENCE

{

data			NonStandardParameter,

...

}

--  Part 3: Messages sent as MCS-Connect-Provider user data

ConnectData ::= SEQUENCE

{

t124Identifier		Key,	--  This shall be set to the value {itu recommendation t 124 version(0) 1}

connectPDU		OCTET STRING

}

ConnectGCCPDU ::= CHOICE

{

conferenceCreateRequest	ConferenceCreateRequest,

conferenceCreateResponse	ConferenceCreateResponse,

conferenceQueryRequest	ConferenceQueryRequest,

conferenceQueryResponse	ConferenceQueryResponse,

conferenceJoinRequest	ConferenceJoinRequest,

conferenceJoinResponse	ConferenceJoinResponse,

conferenceInviteRequest	ConferenceInviteRequest,

conferenceInviteResponse	ConferenceInviteResponse,

...

}

--  Part 4: Messages sent using MCS-Send-Data or MCS-Uniform-Send-Data

GCCPDU ::= CHOICE

{

request				RequestPDU,

response				ResponsePDU,

indication			IndicationPDU

}

RequestPDU ::= CHOICE

{

conferenceJoinRequest	ConferenceJoinRequest,

conferenceAddRequest	ConferenceAddRequest,

conferenceLockRequest	ConferenceLockRequest,

conferenceUnlockRequest	ConferenceUnlockRequest,

conferenceTerminateRequest	ConferenceTerminateRequest,

conferenceEjectUserRequest	ConferenceEjectUserRequest,

conferenceTransferRequest	ConferenceTransferRequest,

registryRegisterChannelRequest	RegistryRegisterChannelRequest,

registryAssignTokenRequest	RegistryAssignTokenRequest,

registrySetParameterRequest	RegistrySetParameterRequest,

registryRetrieveEntryRequest	RegistryRetrieveEntryRequest,

registryDeleteEntryRequest	RegistryDeleteEntryRequest,

registryMonitorEntryRequest	RegistryMonitorEntryRequest,

registryAllocateHandleRequest	RegistryAllocateHandleRequest,

nonStandardRequest	NonStandardPDU,

...

}

ResponsePDU ::= CHOICE

{

conferenceJoinResponse	ConferenceJoinResponse,

conferenceAddResponse	ConferenceAddResponse,

conferenceLockResponse	ConferenceLockResponse,

conferenceUnlockResponse	ConferenceUnlockResponse,

conferenceTerminateResponse	ConferenceTerminateResponse,

conferenceEjectUserResponse	ConferenceEjectUserResponse,

conferenceTransferResponse	ConferenceTransferResponse,

registryResponse		RegistryResponse,

registryAllocateHandleResponse	RegistryAllocateHandleResponse,

functionNotSupportedResponse	FunctionNotSupportedResponse,

nonStandardResponse	NonStandardPDU,

...

}

IndicationPDU ::= CHOICE

{

userIDIndication		UserIDIndication,

conferenceLockIndication	ConferenceLockIndication,

conferenceUnlockIndication	ConferenceUnlockIndication,

conferenceTerminateIndication	ConferenceTerminateIndication,

conferenceEjectUserIndication	ConferenceEjectUserIndication,

conferenceTransferIndication	ConferenceTransferIndication,

rosterUpdateIndication	RosterUpdateIndication,

applicationInvokeIndication	ApplicationInvokeIndication,

registryMonitorEntryIndication	RegistryMonitorEntryIndication,

conductorAssignIndication	ConductorAssignIndication,

conductorReleaseIndication	ConductorReleaseIndication,

conductorPermissionAskIndication	ConductorPermissionAskIndication,

conductorPermissionGrantIndication	ConductorPermissionGrantIndication,

conferenceTimeRemainingIndication	ConferenceTimeRemainingIndication,

conferenceTimeInquireIndication	ConferenceTimeInquireIndication,

conferenceTimeExtendIndication	ConferenceTimeExtendIndication,

conferenceAssistanceIndication	ConferenceAssistanceIndication,

textMessageIndication	TextMessageIndication,

nonStandardIndication	NonStandardPDU,

...

}

END

# 9	Use of the Multipoint Communication Service

All GCC communication shall be through the Multipoint Communication Service (MCS) as specified in Recommendation T.122. This clause details how GCC makes use of MCS services, channel allocation, token allocation and data priorities.

## 9.1	MCS services

GCC assumes the MCS services indicated in Table 9-1. All primitives and parameters marked with an “M” are used by mandatory components of GCC. Items marked with an “O” are used only by optional portions of GCC.

TABLE  9-1/T.124

MCS services used by GCC

| Primitives | Use | Parameters | Use |
| --- | --- | --- | --- |
| MCS-Connect-Provider request MCS-Connect-Provider indication MCS-Connect-Provider response MCS-Connect-Provider confirm | M M M M | Calling Address Calling Domain Selector Called Address Called Domain Selector Upward/Downward Flag Domain Parameters Quality of Service Result User Data | O M O – M M M M M |
| MCS-Disconnect-Provider request MCS-Disconnect-Provider indication | M M | Reason | M |
| MCS-Attach-User request MCS-Attach-User confirm | M M | Domain Selector Result User ID | M M M |
| MCS-Detach-User request MCS-Detach-User indication | M M | Reason User ID | M M |
| MCS-Channel-Join request MCS-Channel-Join confirm | M M | Channel to Join Result | M M |
| MCS-Channel-Leave request MCS-Channel-Leave indication | – M | Channel to Leave Reason | – – |
| MCS-Channel-Convene request MCS-Channel-Convene confirm | – – | Result Channel | – – |
| MCS-Channel-Disband request MCS-Channel-Disband indication | – – | Channel Reason | – – |
| MCS-Channel-Admit request MCS-Channel-Admit indication | – – | Channel Manager User ID List of User IDs | – – – |
| MCS-Channel-Expel request MCS-Channel-Expel indication | – – | Channel List of User IDs Reason | – – – |
| MCS-Send-Data request MCS-Send-Data indication | M | Priority Channel ID Sender User ID Data | M M M M |
| MCS-Uniform-Send-Data request MCS-Uniform-Send-Data indication | M M | Priority Channel ID Sender User ID Data | M M M M |
| MCS-Token-Grab request MCS-Token-Grab confirm | O O | Token ID Result | O O |
| MCS-Token-Inhibit request MCS-Token-Inhibit confirm | O O | Token ID Result | O O |

TABLE  9-1/T.124 (end)

MCS services used by GCC

| Primitives | Use | Parameters | Use |
| --- | --- | --- | --- |
| MCS-Token-Give request MCS-Token-Give indication MCS-Token-Give response MCS-Token-Give confirm | O M M O | User ID Giving User ID to Receive Token ID Result | M O M M |
| MCS-Token-Please request MCS-Token-Please indication | O O | User ID Requesting Token ID | O O |
| MCS-Token-Release request MCS-Token-Release confirm | O O | Token ID Result | O O |
| MCS-Token-Test request MCS-Token-Test confirm | O O | Token ID Token Status | O O |

## 9.2	Channel allocation

GCC reserves two static channels for its exclusive use. One channel, GCC-CHANNEL-0 is joined by all GCC Providers in a conference. Another channel, GCC-CHANNEL-1, is joined only by the Conference Convener. Each GCC Provider also joins the Node ID Channel allocated to it by MCS. Table 9-2 shows the channel usage by GCC.

TABLE  9-2/T.124

GCC channel usage

| Channel ID | Type | Mnemonic | Description |
| --- | --- | --- | --- |
| GCC-CHANNEL-0 | Static | GCC-Broadcast-Channel | For communication from any GCC Provider in a conference to all GCC Providers. |
| GCC-CHANNEL-1 | Static | GCC-Convener-Channel | For communication from any GCC Provider in a conference to the Conference Convener. |
| – | Dynamic | Node ID Channel | For communication from any GCC Provider in a conference to the GCC Provider at a particular node. Each node is identified by its Node ID. |

## 9.3	Token allocation

GCC reserves a single token for its exclusive use. This token, GCC-TOKEN-0, is used as the Conference Conductorship Token. GCC also reserves tokens 16 384 through 65 535 for use as dynamic tokens which GCC allocates for use by Application Protocol Entities. Table 9-3 shows the token usage by GCC.

TABLE  9-3/T.124

GCC token usage

9.4	Use of MCS data transmission services

| Token ID | Mnemonic | Description |
| --- | --- | --- |
| GCC-TOKEN-0 | Conference-Conductorship-Token | Grabbed by a GCC Provider to become the Conference Conductor. |
| 16 384 through 65 535 | Dynamic Tokens | Allocated by GCC for use by Application Protocol Entities using GCC-Registry-Assign-Token primitive. |

Table 9-4 summarizes the use of MCS data transmission services (MCS-Send-Data and MCS-Uniform-Send-Data) for each GCCPDU. Listed for each GCCPDU are the type of MCS data service used, the Channel ID, and requested data priority.

TABLE  9-4/T.124

Use of MCS send data for GCCPDUs

| GCCPDU | Channel | Send data type | Priority |
| --- | --- | --- | --- |
| UserIDIndication | Node ID Channel of directly connected node | MCS-Send-Data | Top |
| ConferenceCreateRequest | – | – | – |
| ConferenceCreateResponse | – | – | – |
| ConferenceQueryRequest | – | – | – |
| ConferenceQueryResponse | – | – | – |
| ConferenceJoinRequest | Node ID Channel of Top GCC | MCS-Send-Data | Top |
| ConferenceJoinResponse | Node ID Channel of requester | MCS-Send-Data | Top |
| ConferenceInviteRequest | – | – | – |
| ConferenceInviteResponse | – | – | – |
| ConferenceAddRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
|  | Node ID Channel of adding MCU | MCS-Send-Data | High |
| ConferenceAddResponse | Node ID Channel of requester | MCS-Send-Data | High |
| ConferenceLockRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| ConferenceLockResponse | Node ID Channel of requester | MCS-Send-Data | High |
| ConferenceLockIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
|  | Node ID Channel | MCS-Send-Data | High |
| ConferenceUnlockRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| ConferenceUnlockResponse | Node ID Channel of requester | MCS-Send-Data | High |
| ConferenceUnlockIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
|  | Node ID Channel | MCS-Send-Data | High |
| ConferenceTerminateRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| ConferenceTerminateResponse | Node ID Channel of requester | MCS-Send-Data | High |
| ConferenceTerminateIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
| ConferenceEjectUserRequest | Node ID Channel of Top GCC | MCS-Send-Data | Top |
| ConferenceEjectUserResponse | Node ID Channel of requester | MCS-Send-Data | High |
| ConferenceEjectUserIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | Top |
| ConferenceTransferRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |

TABLE  9-4/T.124 (end)

Use of MCS send data for GCCPDUs

| GCCPDU | Channel | Send data type | Priority |
| --- | --- | --- | --- |
| ConferenceTransferResponse | Node ID Channel of requester | MCS-Send-Data | High |
| ConferenceTransferIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
| RosterUpdateIndication | Node ID Channel of recipient | MCS-Send-Data | High |
|  | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
| ApplicationInvokeIndication | GCC-Broadcast-Channel | MCS-Send-Data or MCS-Uniform-Send-Data | High |
|  | Node ID Channel of recipient | MCS-Send-Data | High |
| RegistryRegisterChannelRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| RegistryAssignTokenRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| RegistrySetParameterRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| RegistryRetrieveEntryRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| RegistryDeleteEntryRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| RegistryMonitorEntryRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| RegistryMonitorEntryIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
| RegistryAllocateHandleRequest | Node ID Channel of Top GCC | MCS-Send-Data | High |
| RegistryAllocateHandleResponse | Node ID Channel of requester | MCS-Send-Data | High |
| RegistryResponse | Node ID Channel of requester | MCS-Send-Data | High |
| ConductorAssignIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | Top |
| ConductorReleaseIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | Top |
| ConductorPermissionAskIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
| ConductorPermissionGrantIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | Top |
| ConferenceTimeRemainingIndication | GCC-Broadcast-Channel | MCS-Uniform-Send-Data | High |
| ConferenceTimeInquireIndication | GCC-Convener-Channel | MCS-Send-Data | High |
| ConferenceTimeExtendIndication | GCC-Convener-Channel | MCS-Send-Data | High |
| ConferenceAssistanceIndication | GCC-Broadcast-Channel | MCS-Send-Data or MCS-Uniform-Send-Data | High |
| TextMessageIndication | GCC-Broadcast-Channel | MCS-Send-Data or MCS-Uniform-Send-Data | High |
|  | Node ID Channel of recipient | MCS-Send-Data | High |
| FunctionNotSupportedResponse | Node ID Channel of requester | MCS-Send-Data | Same as request |
| NonStandardPDU | Not defined | Not defined | Not defined |

## 9.5	Encoding of PDUs in MCS primitives

All PDUs defined in this Recommendation are encoded and placed in the data field of one of several possible MCS primitives. These are either MCS-Connect-Provider, MCS-Send-Data, or MCS-Uniform-Send-Data. In any of these cases, the bit string that results from the ASN.1 encoding is placed in the OCTET STRING used by MCS in the order such that for each octet, the leading bit is placed in the most significant bit position, and the trailing bit is placed in the least significant bit position.

In the case of MCS-Connect-Provider, the PDU itself is not placed in the MCS data parameter directly, but is contained within an enclosing structure defined in 9.6 In this case, the description above applies to the outer structure rather than the PDU itself.

## 9.6	Format of User Data parameter of MCS-Connect-Provider

The User Data parameter of the MCS-Connect-Provider and MCS-Disconnect-Provider primitives as used by this Recommendation is of a format which provides unique identification of the MCS Controller (as defined in Recommendation T.122). The MCS Controller may either be a standard type (such as one that adheres to this Recommendation), or may be a non-standard type. To distinguish these, the User Data consists of an ASN.1 abstract type which includes an identifier of the type of MCS Controller followed by the PDU itself. The identifier field is of type Key which is a choice of either an Object Identifier, or an H221NonStandardIdentifier. The overall abstract type as well as its components is defined in 8.7. The overall structure is encoded using Packed Encoding Rules (Aligned variant) as defined in Recommendation X.691.

When the MCS Controller adheres to this Recommendation, this header shall be set to the Object Identifier choice with a value {itu recommendation t 124 version(0) 1}.

Within this structure, the PDU is not directly encoded but rather is included in a data field which is of type OCTET STRING. The PDU is separately encoded, also using Packed Encoding Rules (Aligned variant) as with all other PDUs defined in this Recommendation. The resulting encoded bit string is placed into the OCTET STRING in the order such that for each octet, the leading bit is placed in the most significant bit position, and the trailing bit is placed in the least significant bit position. The PDU and the enclosing structure are encoded separately to allow for future Recommendations or non-standard MCS Controllers to use different encoding of the PDUs while providing a uniquely encoded identifier of the MCS Controller type (always to be encoded using Packed Encoding Rules).

## 9.7	Interpretation of the MCS Domain Selector

In using the MCS-Connect-Provider primitive, this Recommendation makes the following assumptions in its use of the Domain Selector.

The Calling Domain Selector always accurately reflects the local Domain Selector which is equal to the Conference ID (which is also used by Application Protocols in making an MCS attachment).

The Called Domain Selector is never used by this Recommendation. Instead it is assumed that the MCS provider passes MCS-Connect-Provider indications to the Control MCSAP regardless of their contents. It is the role of the GCC Provider to determine if the connection should be established, and if so, which domain the connection should be associated with. This is done slightly differently for the four T.124 primitives which generate MCS-Connect-Provider requests:

•	GCC-Conference-Create:	In this case, there is no domain already in existence prior to reception of the MCS-Connect-Provider indication to which the connection could be attached. On receipt of a valid MCS-Connect-Provider indication, a GCC Provider must establish a new MCS domain and indicate to MCS that this is the domain which the new connection is to be associated with. The Domain Selector is chosen at that time by the GCC Provider as the local Conference ID. The means to perform both of these actions are local matters not specified in Recommendation T.122.

•	GCC-Conference-Query:	In this case, no connection is established since the MCS-Connect-Provider response indicates that the request was user-rejected. As a result, it is not necessary to associate the request to any MCS domain.

•	GCC-Conference-Join:	In this case, a domain does already exist at the receiving node. If the Join request is accepted by the GCC Provider, the GCC Provider must indicate to the MCS provider which domain the connection is to be associated with. The Conference Name (and Conference Name Modifier) are used by the GCC Provider to determine which domain to indicate. The Domain Selector is the local Conference ID of that conference. The means to indicate the domain to the MCS provider is a local matter not specified in Recommendation T.122.

•	GCC-Conference-Invite:	In this case, as in the Create case, the receiving node does not already have a domain established. The GCC Provider, if the Invite is accepted, chooses a Conference ID which is to be used as the Domain Selector. The MCS provider must be told of the establishment of this domain at this node, and that the particular connection is to be associated with this domain. As in the case of the Create, the means to perform both of these actions are local matters not specified in Recommendation T.122.

Annex A

Static channel and token ID assignments

(This annex forms an integral part of this Recommendation)

## A.1.	Static channel ID assignments

Table A.1 lists the numerical assignment of static channel IDs for the static channels allocated for use by this Recommendation. The numerical assignment of static channel IDs is intended to be centralized in Recommendation T.120, but is included here until Recommendation T.120 is completed.

TABLE  A.1/T.124

Static channel ID assignments

| Symbolic name | Channel ID |
| --- | --- |
| GCC-CHANNEL-0 | 1 |
| GCC-CHANNEL-1 | 2 |

## A.2.	Static token ID assignments

Table A.2 lists the numerical assignment of static token IDs for the static tokens allocated for use by this Recommendation. The numerical assignment of static token IDs is intended to be centralized in Recommendation T.120, but is included here until Recommendation T.120 is completed.

TABLE  A.2/T.124

Static token ID assignments

| Symbolic name | Token ID |
| --- | --- |
| GCC-TOKEN-0 | 1 |

Annex B

Object Identifier assignments

(This annex forms an integral part of this Recommendation)

Table B.1 lists the assignment of Object Identifiers defined for use by this Recommendation.

TABLE  B.1/T.124

| Object Identifier value | Description |  |
| --- | --- | --- |
| {itu recommendation t 124 version(0) 1} |  | This Object Identifier is used to indicate the version of this Recommendation in use as the MCS Controller. At this time there is a single standardized version defined. |