|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	T.125

TELECOMMUNICATION	(04/94)
STANDARDIZATION  SECTOR
OF  ITU

TELEMATIC  SERVICES

TERMINAL  EQUIPMENTS  AND  PROTOCOLS
FOR  TELEMATIC  SERVICES

MULTIPOINT  COMMUNICATION  SERVICE
PROTOCOL  SPECIFICATION

ITU-T  Recommendation  T.125

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis..

The World Telecommunication Standardization Conference (WTSC), which meets every four years, etablishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation T.125 was prepared by ITU-T Study Group 8 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 7th of April 1994.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1994

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation T.125     (04/94)

Page

1	Scope		1

2	References		1

3	Definitions		2

4	Abbreviations		3

5	Overview of the MCS protocol		3

5.1	Model of the MCS layer		3

5.2	Services provided by the MCS layer		4

5.3	Services assumed from the transport layer		4

5.4	Functions of the MCS layer		4

5.5	Hierarchical processing		7

5.6	Domain parameters		9

6	Use of the transport service		9

6.1	Model of the transport service		9

6.2	Use of multiple connections		10

6.3	Transport connection release		11

7	Structure of MCSPDUs		11

8	Encoding of MCSPDUs		21

9	Routing of MCSPDUs		21

9.1	Connect MCSPDUs		21

9.2	Domain MCSPDUs		22

10	Meaning of MCSPDUs		25

10.1	Connect-Initial		25

10.2	Connect-Response		26

10.3	Connect-Additional		27

10.4	Connect-Result		27

10.5	PDin		28

10.6	EDrq		28

10.7	MCrq		29

10.8	MCcf		30

10.9	PCin		30

10.10	MTrq		31

10.11	MTcf		32

10.12	PTin		33

10.13	DPum		33

10.14	RJum		33

10.15	AUrq		34

10.16	AUcf		34

10.17	DUrq		35

10.18	DUin		35

10.19	CJrq		36

10.20	CJcf		37

10.21	CLrq		37

10.22	CCrq		38

10.23	CCcf		38

10.24	CDrq		39

10.25	CDin		39

10.26	CArq		39

10.27	CAin		40

Page

10.28	CErq		40

10.29	CEin		41

10.30	SDrq		41

10.31	SDin		42

10.32	USrq		43

10.33	USin		43

10.34	TGrq		44

10.35	TGcf		44

10.36	TIrq		45

10.37	TIcf		45

10.38	TVrq		46

10.39	TVin		46

10.40	TVrs		47

10.41	TVcf		48

10.42	TPrq		48

10.43	TPin		48

10.44	TRrq		49

10.45	TRcf		49

10.46	TTrq		50

10.47	TTcf		50

11	MCS provider information base		50

11.1	Hierarchical replication		50

11.2	Channel information		52

11.3	Token information		52

12	Elements of procedure		54

12.1	MCSPDU sequencing		54

12.2	Input flow control		55

12.3	Throughput enforcement		56

12.4	Domain configuration		57

12.5	Domain merger		57

12.6	Domain disconnection		59

12.7	Channel id allocation		60

12.8	Token status		61

13	Reference implementation		61

Appendix IVI –  Alternative encodings of an MCSPDU		62

I.1	Send Data Request		62

I.2	Basic Encoding Rules (BER)		62

I.3	Packed Encoding Rules (PER)		63

Appendix IIV –  SDL decomposition of an MCS provider		64

Appendix IIII –  SDL specification of the Control process		80

Appendix IVI –  SDL specification of the Domain process		97

Appendix VII – SDL specification of the Endpoint process		142

Appendix VII – SDL specification of the Attachment process		146

Appendix VII – Characteristics of the reference implementation		156

VII.1	SDL decomposition		156

VII.2	Service definitions		157

VII.3	Portals onto a domain		157

VII.4	Alignment of MCSPDUs		158

VII.5	Method of using SDL		158

VII.6	Remarks on the Domain process		159

# SUMMARY

This Recommendation defines a protocol operating through the hierarchy of a multipont communication domain. It specifies the format of protocol messages and procedures governing their exchange over a set of transport connections. The purpose of the protocol is to implement the Multipoint Communication Service defined by ITU-T Recommendation T.122.

Recommendation T.125

Recommendation T.125     (04/94)

Recommendation T.125     (04/94)

MULTIPOINT  COMMUNICATION  SERVICE
PROTOCOL  SPECIFICATION

(Geneva, 1994)

# 1	Scope

This Recommendation specifies:

a)	procedures for a single protocol for the transfer of data and control information from one MCS provider to a peer MCS provider;

b)	the structure and encoding of the MCS protocol data units used for the transfer of data and control information.

The procedures are defined in terms of:

a)	the interactions between peer MCS providers through the exchange of MCS protocol data units;

b)	the interactions between an MCS provider and MCS users through the exchange of MCS primitives;

c)	the interactions between an MCS provider and a transport service provider through the exchange of transport service primitives;

These procedures are applicable to instances of multipeer communication among systems that support MCS and wish to interconnect in an open systems environment.

# 2	References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation T.122 (1993), Multipoint Communication Service for audiographics and audio visual conferencing service definition.

–	ITU-T Recommendation T.123 (1993), Protocol stacks for audiographic and audiovisual teleconference applications.

–	CCITT Recommendation X.200 (1988), Reference model of Open Systems Interconnection for CCITT applications.

–	CCITT Recommendation X.214 (1988), Transport service definition for Open Systems Interconnection for CCITT applications.

–	CCITT Recommendation X.208 (1988), Specification of Abstract Syntax Notation One (ASN.1).

–	CCITT Recommendation X.209 (1988), Specification of Basic Encoding Rules for Abstract Syntax Notation One (ASN.1).

–	Draft ITU-T Recommendation X.691 | ISO/IEC DIS 8825-2, Information Technology – Open Systems Interconnection – ASN.1 encoding rules – Specification of Packed Encoding Rules (PER).

# 3	Definitions

NOTE – These definitions make use of the abbreviations defined in clause 4.

This Recommendation is based on the concepts developed in CCITT Recommendation X.200 and makes use of the following terms defined therein:

a)	flow control;

b)	reassembling;

c)	recombining;

d)	segmenting;

e)	sequencing;

f)	splitting;

g)	transfer syntax;

h)	transport connection;

i)	transport connection endpoint identifier;

j)	transport service;

k)	transport service access point;

l)	transport service access point address;

m)	transport service data unit.

This Recommendation is also based on the concepts developed in ITU-T Recommendation T.122 and makes use of the following terms defined therein:

a)	Control MCSAP;

b)	MCS attachment;

c)	MCS channel;

d)	MCS connection;

e)	MCS domain;

f)	MCS domain selector;

g)	MCS private channel;

h)	MCS private channel manager;

i)	MCS provider;

j)	MCS service access point;

k)	MCS user;

l)	MCS user id;

m)	Top MCS provider.

For the purposes of this Recommendation, the following definitions apply.

3.1	MCS service data unit: An amount of MCS user data whose identity is preserved during 
the transfer from transmitter to receivers. Specifically, the content of one MCS-SEND-DATA request or one 
MCS-UNIFORM-SEND-DATA request.

3.2	MCS interface data unit: The unit of information transferred across an MCSAP between an MCS user and an MCS provider in a single interaction. Each MCS interface data unit contains interface control information and may also contain all or part of an MCS service data unit.

3.3	MCS protocol data unit: A unit of information exchanged in the MCS protocol, consisting of control information transferred between MCS providers to coordinate their joint operation and possibly data transferred on behalf of MCS users to whom they are providing service.

3.4	MCS data transfer priority: One of four levels: top, high, medium, low. The value is communicated unchanged from transmitter to receivers. Depending on an MCS domain parameter for the number of distinct data transfer priorities implemented, two or more lowest priorities may receive the same quality of service.

3.5	valid MCSPDU: An MCSPDU whose structure and encoding complies with this Recommendation.

3.6	invalid MCSPDU: An MCSPDU that is not a valid MCSPDU.

3.7	protocol error: Use of an MCSPDU in a manner inconsistent with the procedures of this Recommendation.

3.8	connect MCSPDU: Any one of Connect-Initial, Connect-Response, Connect-Additional, Connect-Result.

3.9	domain MCSPDU: Any MCSPDU that is not a connect MCSPDU.

3.10	data MCSPDU: Any one of SDrq, SDin, USrq, USin.

3.11	control MCSPDU: Any domain MCSPDU that is not a data MCSPDU.

3.12	initial TC: The first transport connection of an MCS connection, used to exchange control MCSPDUs and data MCSPDUs of top priority.

3.13	additional TC: A subsequent transport connection belonging to an MCS connection, used to exchange data MCSPDUs of lesser priority.

3.14	subtree of an MCS provider: In the context of an MCS domain, this consists of the MCS provider itself and its MCS attachments plus all MCS providers hierarchically subordinate to it and their MCS attachments.

3.15	height of an MCS provider: In the context of an MCS domain, this is one more than the maximum height of all hierarchically subordinate MCS providers. An MCS provider without subordinates has height one.

# 4	Abbreviations

For the purposes of this Recommendation, the following abbreviations apply.

MCS	Multipoint Communication Service

MCSAP	MCS service access point

MCSPDU	MCS protocol data unit

TC	Transport connection

TS	Transport service

TSAP	Transport service access point

TSDU	Transport service data unit

# 5	Overview of the MCS protocolXE "Overview of the MCS protocol"§

## 5.1	Model of the MCS layerXE "Model of the MCS layer"§

An MCS provider communicates with MCS users through an MCSAP by means of the MCS primitives defined in ITU-T Recommendation T.122. These primitives can be the cause or result of MCSPDU exchanges between peer MCS providers using an MCS connection, or they can be the cause or result of actions taken within a single MCS provider. MCSPDU exchanges occur between MCS providers that host the same MCS domain.

An MCS provider can have multiple peers, each reached directly by a different MCS connection or indirectly through a peer MCS provider. An MCS connection is composed of one or more transport connections, depending on the number of data transfer priorities implemented in an MCS domain. Protocol exchanges are effected using the services of the transport layer through a pair of TSAPs.

This model of the MCS layer is illustrated in Figure 5-1.

FIGURE  5-1/T.125.....[D01]  =  7.5 cm

## 5.2	Services provided by the MCS layerXE "Services provided by the MCS layer"§

The MCS protocol supports the services defined in ITU-T Recommendation T.122. Information is transferred to and from an MCS user using the MCS primitives listed in Table 5-1.

## 5.3	Services assumed from the transport layer

The MCS protocol assumes the use of a subset of the connection-oriented transport service defined in CCITT Recommendation X.214. Information is transferred to and from a TS provider by the primitives listed in Table 5-2.

## 5.4	Functions of the MCS layerXE "Functions of the MCS layer"§

Table 5-1 identifies the functional units of MCS and the MCSPDUs associated with each MCS primitive. MCSPDUs are defined in clause 7. The relationship between primitives and MCSPDUs can be as simple as cause and effect, in either direction. For example, MCS-ATTACH-USER request generates AUrq, and AUcf generates MCS-ATTACH-USER confirm. Other cases may be more complicated. The completion of MCS-CONNECT-PROVIDER, for example, requires the exchange of additional MCSPDUs as side effects of the four-phase primitive. And any one of five MCSPDUs can cause an MCS-DETACH-USER indication, any of four an MCS-CHANNEL-EXPEL indication.

### 5.4.1	Domain management

The MCS layer maintains the integrity of the MCS connections comprising an MCS domain. An MCS connection is oriented, with one end hierarchically superior to the other. There is a single MCS provider at the top of each domain.

Establishing an MCS connection merges two domains into one. The MCS layer ensures that one top provider remains. It resolves any conflicts of unique identity or exclusive ownership that may arise.

Disconnecting an MCS connection splits a domain into two portions. The portion containing the top provider survives. The bottom portion eradicates itself.

The MCS layer uniquely identifies users attached to a domain. Users may become aware of each other through their interactions via MCS primitives. The MCS layer notifies all users of a domain when one of them detaches. The MCS layer recovers any resources of a detached user.

TABLE  5-1/T.125

MCS primitives

| Functional Unit | Primitives | Associated MCSPDUs |
| --- | --- | --- |
| Domain Management | - MCS-CONNECT-PROVIDER request MCS-CONNECT-PROVIDER indication MCS-CONNECT-PROVIDER response MCS-CONNECT-PROVIDER confirm (side effects) | Connect-Initial Connect-Initial Connect-Response Connect-Response Connect-Additional Connect-Result PDin EDrq MCrq MCcf PCin MTrq MTcf PTin |
|  | - MCS-DISCONNECT-PROVIDER request MCS-DISCONNECT-PROVIDER indication | DPum DPum RJum |
|  | - MCS-ATTACH-USER request MCS-ATTACH-USER confirm | AUrq AUcf |
|  | - MCS-DETACH-USER request MCS-DETACH-USER indication | DUrq DUin MCcf PCin MTcf PTin |
| Channel Management | - MCS-CHANNEL-JOIN request MCS-CHANNEL-JOIN confirm | CJrq CJcf |
|  | - MCS-CHANNEL-LEAVE request MCS-CHANNEL-LEAVE indication | CLrq MCcf PCin |
|  | - MCS-CHANNEL-CONVENE request MCS-CHANNEL-CONVENE confirm | CCrq CCcf |
|  | - MCS-CHANNEL-DISBAND request MCS-CHANNEL-DISBAND indication | CDrq MCcf PCin |
|  | - MCS-CHANNEL-ADMIT request MCS-CHANNEL-ADMIT indication | CArq CAin |
|  | - MCS-CHANNEL-EXPEL request MCS-CHANNEL-EXPEL indication | CErq CEin CDin MCcf PCin |
| Data Transfer | - MCS-SEND-DATA request MCS-SEND-DATA indication | SDrq SDin |
|  | - MCS-UNIFORM-SEND-DATA request MCS-UNIFORM-SEND-DATA indication | USrq USin |

TABLE  5-1/T.125 (end)

MCS primitives

| Functional Unit | Primitives | Associated MCSPDUs |
| --- | --- | --- |
| Token Management | - MCS-TOKEN-GRAB request MCS-TOKEN-GRAB confirm | TGrq TGcf |
|  | - MCS-TOKEN-INHIBIT request MCS-TOKEN-INHIBIT confirm | TIrq TIcf |
|  | - MCS-TOKEN-GIVE request MCS-TOKEN-GIVE indication MCS-TOKEN-GIVE response MCS-TOKEN-GIVE confirm | TVrq TVin TVrs TVcf |
|  | - MCS-TOKEN-PLEASE request MCS-TOKEN-PLEASE indication | TPrq TPin |
|  | - MCS-TOKEN-RELEASE request MCS-TOKEN-RELEASE confirm | TRrq TRcf |
|  | - MCS-TOKEN-TEST request MCS-TOKEN-TEST confirm | TTrq TTcf |

### 5.4.2	Channel management

The MCS layer records which parts of an MCS domain contain one or more users joined to a given channel, so that it can optimize the transfer of data to destinations that wish to receive it.

The MCS layer treats user ids as single-member channels, which only the designated user is allowed to join. On request, it can create private channels to which only admitted users are allowed access or assign public channels to which no other users are currently joined.

### 5.4.3	Data transfer

The MCS layer maintains a sequenced flow of data to the users who have joined a channel. A channel becomes, in effect, a multicast distribution list, with a range somewhere between zero destinations and a complete broadcast.

By default, the MCS layer routes data to each receiver over the shortest path of MCS connections. Optionally, it routes specified MCS service data units through the top MCS provider, thereby guaranteeing their uniform receipt at all receivers, which may include the transmitter too.

The MCS layer recognizes one or more priorities of data transfer and extends them preferential processing. Through segmentation, it allows MCS service data units of unlimited size.

The MCS layer regulates the global flow of data within a domain. The inability of a receiver to accept data at the rate it is offered eventually creates back-pressure that causes transmitters to be blocked. A user may be detached involuntarily if it fails to maintain a minimum receiving rate.

The MCS layer guarantees error-free receipt of transmitted data, as long as the source and destination users remain attached and the destination user remains joined to the channel. However, higher priority data takes precedence, and a surfeit of it may indefinitely delay the delivery of lower priority data.

TABLE  5-2/T.125

Transport service primitives

| Primitives | Use | Parameters | Use |
| --- | --- | --- | --- |
| T-CONNECT request T-CONNECT indication | X X | Called address Calling address Expedited data option Quality of service TS user-data | X X – X – |
| T-CONNECT response T-CONNECT confirm | X X | Responding address Expedited data option Quality of service TS user-data | – – X – |
| T-DATA request T-DATA indication | X X | TS user-data | X |
| T-EXPEDITED-DATA request T-EXPEDITED-DATA indication | – – | TS user-data | – |
| T-DISCONNECT request | X | TS user-data | – |
| T-DISCONNECT indication | X | Reason TS user-data | – – |
| X The MCS protocol assumes that this feature is always available. – The MCS protocol does not use this feature. |  |  |  |

### 5.4.4	Token management

The MCS layer implements token operations at the top MCS provider, thereby ensuring consistency and exclusion.

## 5.5	Hierarchical processing

Hierarchical processing in an MCS domain is illustrated in Figure 5-2.

The nodes in the figure represent MCS providers, and the labelled arrows represent MCSPDUs. This example focuses on a period of time after the domain has been established through connections between MCS providers and the use of data transfer is beginning to expand. At step 1 provider D requests on behalf of a user to join a channel over which data will be distributed, and at step 2 the request is confirmed as successful. At step 3 a user attached to provider A sends data, and the corresponding SDrq begins to flow upward. Assuming that only attachments at providers A, C, and D are joined to the channel over which the data is being sent, the request MCSPDU is reflected downward at steps 6 and 7 as SDin. Provider E, aware that no other subordinate needs to receive the data, simply forwards SDrq upward at step 4. Provider F forwards SDrq upward at step 5 but also reflects it downward at step 6, knowing that provider C has expressed interest in the channel.

MCS providers are not especially concerned with their height in the hierarchy, except for their role in maintaining an overall limit on the height of the domain and in the broad sense that they are either the top provider or they are not. The top MCS provider has no upward connection. All  others have exactly one.

An MCS provider records information about channels and tokens used in its subtree of an MCS domain.

FIGURE  5-2/T.125.....[D02]  =  9.5 cm

An MCS provider records channels that are joined by users within the subtree and, for each such channel, where the joins originate, that is, from which attachments and from which downward MCS connections. It records user ids that are assigned in the subtree and where they originate. It records private channels that have either a manager or an admitted user in the subtree, and it records the associated user ids.

An MCS provider records tokens that are grabbed or inhibited by users within the subtree, and it records the associated user ids.

An MCS provider inspects requests arising from its subtree to verify that the initiating user id is legitimately assigned to the originating attachment or downward MCS connection. This creates rings of protection around the top MCS provider and limits how much disruption a malicious participant can cause in an otherwise cooperative domain.

In general terms, to which there are several exceptions, the operation of the MCS layer can be described as follows.

a)	An MCS primitive request invoked at an MCS attachment generates an MCSPDU at the corresponding MCS provider and dispatches it upward towards the top MCS provider. There, where full information about the MCS domain is held, the MCSPDU is acted on.

b)	A confirm MCSPDU may be generated at the top MCS provider to return results to the requesting attachment. MCS providers that pass it along update their records according to the impact of the operation on their subtree. A confirm is routed to the initiating user id by consulting local records at each successive downward hop.

c)	An indication MCSPDU may be generated instead to inform other attachments about the action taken. Indications may be replicated and sent downward on several connections that lead to affected users. MCS providers may also update their records with the impact of an operation as part of processing an indication.

The preceding description is an overview intended to create a conceptual framework. Full details are specified in later clauses concerning the information recorded in an MCS provider and how specific MCSPDUs are processed.

Among exceptions are the following: Some requests, notably CJrq and CLrq, may stop rising at a level short of the top provider, and some indications, notably SDin, may be generated at a level below the top provider. One MCSPDU, TVrs, belongs to the response category. And some MCSPDUs may be generated by an MCS provider as a continuation of processing unlike MCSPDUs, such as CLrq following DUin.

## 5.6	Domain parameters

The MCS providers hosting a single MCS domain allocate resources and execute procedures according to the following parameters. The values of these parameters are identical throughout a domain.

a)	Maximum number of MCS channels that may be in use simultaneously. This includes channels that are joined by any user, user ids that have been assigned, and private channels that have been created.

b)	Maximum number of user ids that may be assigned simultaneously. This is a sublimit within the constraint of the preceding parameter.

c)	Maximum number of token ids that may be grabbed or inhibited simultaneously.

d)	Number of data transfer priorities implemented. This equals the number of TCs in an MCS connection. An MCS user may still send and receive data with priorities outside the limit. However, such priorities may be treated the same as the lowest priority that is implemented.

e)	Enforced throughput. Although global flow control limits data transfer within a domain to the rate of the slowest receiver, receivers must not be allowed to run arbitrarily slowly. Otherwise, one party in a conference may obstruct all others. This parameter instructs MCS providers to enforce a minimum receiving rate at each MCS attachment and over each downward MCS connection. Violators run the risk of being involuntarily detached or disconnected, respectively.

f)	Maximum height. This constrains the height of all MCS providers, in particular the top provider.

g)	Maximum size of domain MCSPDUs. Global flow control is based on buffering domain MCSPDUs within an MCS provider (but not connect MCSPDUs). For simplicity, fixed-size buffers are assumed. An MCS provider shall not generate larger MCSPDUs. This constrains the amount of information that can be packed into a single control MCSPDU and suggests where unlimited user data should be segmented in data MCSPDUs.

h)	Protocol version. This takes one of two values defining different encodings for domain MCSPDUs.

NOTE – A given instance of an MCS provider may operate with local resource constraints that are also parameterized. These may include the amount of memory available for buffering MCSPDUs awaiting transport, the maximum number of MCS attachments, and the maximum number of MCS connections to other providers. Such parameters are local matters and are not communicated across an MCS domain.

# 6	Use of the transport service

## 6.1	Model of the transport serviceXE "Model of the transport service"§

This description paraphrases relevant parts of CCITT Recommendation X.214 assuming that no use is made of expedited data.

The transport service offers these features to a TS user:

a)	Means to establish a TC with another TS user for the purpose of exchanging TSDUs. More than one TC may exist between the same pair of TS users.

b)	Associated with each TC at its time of establishment, the opportunity to request, negotiate, and have agreed by the TS provider a certain Quality of Service as specified by parameters representing characteristics such as throughput, transit delay, residual error rate, and priority.

c)	Means of transferring TSDUs on a TC. The transfer of TSDUs, which consist of an integral number of octets, is transparent, in that the boundaries of TSDUs and the contents of TSDUs are preserved unchanged by the TS provider.

d)	Means by which a receiving TS user may control the rate at which the sending TS user may send data.

e)	The unconditional and therefore possibly destructive release of a TC.

The operation of a TC is modelled in an abstract way by a pair of queues linking two TSAPs. There is one queue for each direction of information flow. Each TC is modelled by a separate pair of queues.

The queue model is used to express the flow control feature. A queue has a limited capacity, but this capacity is not necessarily either fixed or determinable. Connect, TSDU, and disconnect objects are entered and removed from a queue as the result of interactions at the two TSAPs. The ability of a TS user to add objects to a queue is determined by the behaviour of the TS user removing objects from that queue and the state of the queue. The only objects that can be placed in a queue by the TS provider are disconnect objects. Objects are added to a queue subject to control by the TS provider. Objects are normally removed from the queue subject to control by the receiving TS user. Objects are normally removed in the same order that they were added. The only exception to normal removal is that an object may be deleted by the TS provider if, and only if, the following one is a disconnect object.

A TC endpoint identification mechanism must be provided locally if the TS user and the TS provider need to distinguish between several TCs at a TSAP. All primitives must then make use of this identification mechanism to identify the TC to which they apply. This implicit identification is not shown as a parameter of the TS primitives and must not be confused with the address parameters of T-CONNECT.

## 6.2	Use of multiple connectionsXE "Use of multiple connections"§

An MCS connection consists of one or more TCs between the same pair of MCS providers. The first TC established is called the initial TC; those established subsequently are called additional TCs. All the TCs that belong to one MCS connection are established by the same MCS provider, in reaction to an MCS-CONNECT-PROVIDER request. This request contains address parameters that are the calling and called TSAP addresses. These are used unmodified in the T-CONNECT requests that result.

The number of TCs per MCS connection is uniform throughout an MCS domain. This domain parameter equals the number of data transfer priority levels implemented. Separate TCs are required because each is a vehicle for flow control. Blockages in lower priority data should not result in back pressure against higher priority data. To be fully implemented, lower and higher priority data must be carried on different TCs.

The quality of service requested for a TC may vary depending on the data priority for which it is established. These quality of service targets need not be uniform across an MCS domain.

Aspects of quality of service that are of interest include maximum or average throughput and transit delay. High priority data may favour low transit delay for real-time response but may not require high throughput. Low priority data, on the other hand, may favour high throughput for bulk transfers but may not require low transit delay.

TC priority is another aspect of quality of service, although it does not align precisely with the concept of MCS data transfer priority. It specifies the relative order in which TCs are to have their quality of service degraded, if necessary. High TC priority may be requested along with other characteristics, like low transit delay, to ensure that MCS priority data receives the preferential treatment it deserves.

Connect MCSPDUs occur only as the first TSDU carried in either direction of a TC. Connect-Initial and Connect-Response traverse the initial TC of an MCS connection. Connect-Additional and Connect-Result traverse additional TCs, if any.

A calling MCS provider, issuing T-CONNECT requests, controls through its own actions which TCs are part of the same MCS connection and what data transfer priorities they represent.

A called MCS provider, receiving T-CONNECT indications, must in general accept a TC and read its first TSDU before learning its significance. Connect-Initial identifies an incoming TC as the beginning of a new MCS connection. Connect-Additional identifies an incoming TC as part of an MCS connection in progress.

Connect-Additional contains a value assigned by the called MCS provider and conveyed to the calling MCS provider in a Connect-Response over the initial TC that designates the additional TC as belonging to the same MCS connection. Connect-Additional also states explicitly the data priority that the TC represents.

Connect MCSPDUs are exchanged immediately following TC establishment. Once an MCS connection has been confirmed, it becomes part of a hierarchical MCS domain. Thereafter the MCS connection conveys domain MCSPDUs.

With the exception of data MCSPDUs, domain MCSPDUs traverse the initial TC of an MCS connection. Data MCSPDUs traverse the TC that corresponds to their data priority. If the priority specified is beyond the number implemented in an MCS domain, a data MCSPDU traverses the TC of the lowest priority that is implemented.

If only one priority is implemented in an MCS domain, its MCS connections each consist of a single TC, no use is made of Connect-Additional or Connect-Result, and all MCSPDUs travel in sequence between providers.

## 6.3	Transport connection releaseXE "Transport connection release"§

A TS provider adds reliability to end-to-end connections by executing enough protocol to compensate for any weakness in underlying network services. An MCS provider does not duplicate this functionality. It does not attempt further automatic recovery in the event of a transport failure.

Unrecoverable errors are announced through a T-DISCONNECT indication. If any of the TCs belonging to an MCS connection is disconnected, the others are immediately disconnected too. Unless this was requested by the user, an MCS-DISCONNECT-PROVIDER indication is generated, and the reason is given as provider-initiated.

An MCS-DISCONNECT-PROVIDER request, on the other hand, should appear as an indication at the other side with the reason given as user-requested. Despite assurances in X.214, the simplest class of transport protocol does not allow passing user data in T-DISCONNECT. Hence, the disconnect reason code is transferred in an explicit MCSPDU. This MCSPDU compels an MCS provider, upon receipt, to disconnect the MCS connection that conveyed it.

# 7	Structure of MCSPDUsXE "Structure of MCSPDUs"§

The structure of MCSPDUs is specified using the notation ASN.1 of CCITT Recommendation X.208. The use and significance of these MCSPDUs is further described in clauses 9 and 10.

MCS-PROTOCOL DEFINITIONS  ::=

BEGIN

--  Part 1: Fundamental MCS types

ChannelId  ::=  INTEGER (0..65535)	--  range is 16 bits

StaticChannelId  ::=  ChannelId (1..1000)	--  those known permanently

DynamicChannelId  ::=  ChannelId (1001..65535)	--  those created and deleted

UserId  ::=  DynamicChannelId	--  created by Attach-User
--  deleted by Detach-User

PrivateChannelId  ::=  DynamicChannelId	--  created by Channel-Convene
--  deleted by Channel-Disband

AssignedChannelId  ::=  DynamicChannelId	--  created by Channel-Join zero
--  deleted by last Channel-Leave

TokenId  ::=  INTEGER (1..65535)	--  all are known permanently

TokenStatus  ::=  ENUMERATED
{

notInUse	(0),
selfGrabbed	(1),
otherGrabbed	(2),
selfInhibited	(3),
otherInhibited	(4),
selfRecipient	(5),
selfGiving	(6),
otherGiving	(7)

}

DataPriority  ::=  ENUMERATED
{

top	(0),
high	(1),
medium	(2),
low	(3)

}

Segmentation  ::=  BIT STRING
{

begin	(0),
end	(1)

} (SIZE (2))

DomainParameters  ::=  SEQUENCE
{

maxChannelIds	INTEGER (0..MAX),
--  a limit on channel ids in use,
--  static + user id + private + assigned

maxUserIds	INTEGER (0..MAX),
--  a sublimit on user id channels alone

maxTokenIds	INTEGER (0..MAX),
--  a limit on token ids in use
--  grabbed + inhibited + giving + ungivable + given

numPriorities	INTEGER (0..MAX),
--  the number of TCs in an MCS connection

minThroughput	INTEGER (0..MAX),
--  the enforced number of octets per second

maxHeight	INTEGER  (0..MAX),
--  a limit on the height of a provider

maxMCSPDUsize	INTEGER  (0..MAX),
--  an octet limit on domain MCSPDUs

protocolVersion	INTEGER (0..MAX)

}

--  Part 2: Connect provider

Connect-Initial  ::=  [APPLICATION 101] IMPLICIT SEQUENCE
{

callingDomainSelector	OCTET STRING,
calledDomainSelector	OCTET STRING,
upwardFlag	BOOLEAN,
	--  TRUE if called provider is higher

targetParameters	DomainParameters,

minimumParameters	DomainParameters,

maximumParameters	DomainParameters,

userData	OCTET STRING

}

Connect-Response  ::=  [APPLICATION 102] IMPLICIT SEQUENCE
{

result	Result,

calledConnectId	INTEGER (0..MAX),
--  assigned by the called provider
--  to identify additional TCs of 
--  the same MCS connection

domainParameters	DomainParameters,

userData	OCTET STRING

}

Connect-Additional  ::=  [APPLICATION 103] IMPLICIT SEQUENCE
{

calledConnectId	INTEGER (0..MAX),

dataPriority	DataPriority

}

Connect-Result  ::=  [APPLICATION 104] IMPLICIT SEQUENCE
{

result	Result

}

--  Part 3: Merge domain

PDin  ::=  [APPLICATION 0] IMPLICIT SEQUENCE  --  plumb domain indication
{

heightLimit	INTEGER (0..MAX)
--  a restriction on the MCSPDU receiver

}

EDrq  ::=  [APPLICATION 1] IMPLICIT SEQUENCE  --  erect domain request
{

subHeight	INTEGER (0..MAX),
--  height in domain of the MCSPDU transmitter

subInterval	INTEGER (0..MAX)
--  its throughput enforcement interval in milliseconds

}

ChannelAttributes  ::=  CHOICE
{

static	[0]	IMPLICIT SEQUENCE
{

channelId		StaticChannelId
--  joined is implicitly TRUE

},

userId	[1]	IMPLICIT SEQUENCE
{

joined		BOOLEAN,

userId		UserId
--  TRUE if user is joined to its user id

},

private	[2]	IMPLICIT SEQUENCE
{

joined		BOOLEAN,
--  TRUE if channel id is joined below

channelId		PrivateChannelId,

manager		UserId,

admitted		SET OF UserId
--  may span multiple MCrq

},

assigned	[3]	IMPLICIT SEQUENCE
{

channelId		AssignedChannelId
--  joined is implicitly TRUE

}

}

MCrq  ::=  [APPLICATION 2] IMPLICIT SEQUENCE  --  merge channels request
{

mergeChannels	SET OF ChannelAttributes,

purgeChannelIds	SET OF ChannelId

}

MCcf  ::=  [APPLICATION 3] IMPLICIT SEQUENCE  --  merge channels confirm
{

mergeChannels	SET OF ChannelAttributes,

purgeChannelIds	SET OF ChannelId

}

PCin  ::=  [APPLICATION 4] IMPLICIT SEQUENCE  --  purge channels indication
{

detachUserIds	SET OF UserId,
--  purge user id channels

purgeChannelIds	SET OF ChannelId
--  purge other channels

}

TokenAttributes  ::=  CHOICE
{

grabbed	[0]	IMPLICIT SEQUENCE
{

tokenId		TokenId,

grabber		UserId

},

inhibited	[1]	IMPLICIT SEQUENCE
{

tokenId		TokenId,

inhibitors		SET OF UserId
--  may span multiple MTrq

},

giving	[2]	IMPLICIT SEQUENCE
{

tokenId		TokenId,

grabber		UserId,

recipient		UserId

},

ungivable	[3]	IMPLICIT SEQUENCE
{

tokenId		TokenId,

grabber		UserId
--  recipient has since detached

},

given	[4]	IMPLICIT SEQUENCE
{

tokenId		TokenId,

recipient		UserId
--  grabber released or detached

}

}

MTrq  ::=  [APPLICATION 5] IMPLICIT SEQUENCE  --  merge tokens request
{

mergeTokens	SET OF TokenAttributes,

purgeTokenIds	SET OF TokenId

}

MTcf  ::=  [APPLICATION 6] IMPLICIT SEQUENCE  --  merge tokens indication
{

mergeTokens	SET OF TokenAttributes,

purgeTokenIds	SET OF TokenId

}

PTin  ::=  [APPLICATION 7] IMPLICIT SEQUENCE  --  purge tokens indication
{

purgeTokenIds	SET OF TokenId

}

--  Part 4: Disconnect provider

DPum  ::=  [APPLICATION 8] IMPLICIT SEQUENCE  --  disconnect provider ultimatum
{

reason	Reason

}

RJum  ::=  [APPLICATION 9] IMPLICIT SEQUENCE  --  reject MCSPDU ultimatum
{

diagnostic	Diagnostic,

initialOctets	OCTET STRING

}

--  Part 5: Attach/Detach user

AUrq  ::=  [APPLICATION 10] IMPLICIT SEQUENCE  --  attach user request
{

}

AUcf  ::=  [APPLICATION 11] IMPLICIT SEQUENCE  --  attach user confirm
{

result	Result,

initiator	UserId OPTIONAL

}

DUrq  ::=  [APPLICATION 12] IMPLICIT SEQUENCE  --  detach user request
{

reason	Reason,

userIds	SET OF UserId

}

DUin  ::=  [APPLICATION 13] IMPLICIT SEQUENCE  --  detach user indication
{

reason	Reason,

userIds	SET OF UserId

}

--  Part 6: Channel management

CJrq  ::=  [APPLICATION 14] IMPLICIT SEQUENCE  --  channel join request
{

initiator	UserId,

channelId	ChannelId
--  may be zero

}

CJcf  ::=  [APPLICATION 15] IMPLICIT SEQUENCE  --  channel join confirm
{

result	Result,

initiator	UserId,

requested	ChannelId,
--  may be zero

channelId	ChannelId OPTIONAL

}

CLrq  ::=  [APPLICATION 16] IMPLICIT SEQUENCE  --  channel leave request
{

channelIds	SET OF ChannelId

}

CCrq  ::=  [APPLICATION 17] IMPLICIT SEQUENCE  --  channel convene request
{

initiator	UserId

}

CCcf  ::=  [APPLICATION 18] IMPLICIT SEQUENCE  --  channel convene confirm
{

result	Result,

initiator	UserId,

channelId	PrivateChannelId OPTIONAL

}

CDrq  ::=  [APPLICATION 19] IMPLICIT SEQUENCE  --  channel disband request
{

initiator	UserId,

channelId	PrivateChannelId

}

CDin  ::=  [APPLICATION 20] IMPLICIT SEQUENCE  --  channel disband indication
{

channelId	PrivateChannelId

}

CArq  ::=  [APPLICATION 21] IMPLICIT SEQUENCE  --  channel admit request
{

initiator	UserId,

channelId	PrivateChannelId,

userIds	SET OF UserId

}

CAin  ::=  [APPLICATION 22] IMPLICIT SEQUENCE  --  channel admit indication
{

initiator	UserId,

channelId	PrivateChannelId,

userIds	SET OF UserId

}

CErq  ::=  [APPLICATION 23] IMPLICIT SEQUENCE  --  channel expel request
{

initiator	UserId,

channelId	PrivateChannelId,

userIds	SET OF UserId

}

CEin  ::=  [APPLICATION 24] IMPLICIT SEQUENCE  --  channel expel indication
{

channelId	PrivateChannelId,

userIds	SET OF UserId

}

--  Part 7: Data transfer

SDrq  ::=  [APPLICATION 25] IMPLICIT SEQUENCE  --  send data request
{

initiator	UserId,

channelId	ChannelId,

dataPriority	DataPriority,

segmentation	Segmentation,

userData	OCTET STRING

}

SDin  ::=  [APPLICATION 26] IMPLICIT SEQUENCE  --  send data indication
{

initiator	UserId,

channelId	ChannelId,

dataPriority	DataPriority,

segmentation	Segmentation,

userData	OCTET STRING

}

USrq  ::=  [APPLICATION 27] IMPLICIT SEQUENCE  --  uniform send data request
{

initiator	UserId,

channelId	ChannelId,

dataPriority	DataPriority,

segmentation	Segmentation,

userData	OCTET STRING

}

USin  ::=  [APPLICATION 28] IMPLICIT SEQUENCE  --  uniform send data indication
{

initiator	UserId,

channelId	ChannelId,

dataPriority	DataPriority,

segmentation	Segmentation,

userData	OCTET STRING

}

--  Part 8: Token management

TGrq  ::=  [APPLICATION 29] IMPLICIT SEQUENCE  --  token grab request
{

initiator	UserId,

tokenId	TokenId

}

TGcf  ::=  [APPLICATION 30] IMPLICIT SEQUENCE  --  token grab confirm
{

result	Result,

initiator	UserId,

tokenId	TokenId,

tokenStatus	TokenStatus

}

TIrq  ::=  [APPLICATION 31] IMPLICIT SEQUENCE  --  token inhibit request
{

initiator	UserId,

tokenId	TokenId

}

TIcf  ::=  [APPLICATION 32] IMPLICIT SEQUENCE  --  token inhibit confirm
{

result	Result,

initiator	UserId,

tokenId	TokenId,

tokenStatus	TokenStatus

}

TVrq  ::=  [APPLICATION 33] IMPLICIT SEQUENCE  --  token give request
{

initiator	UserId,

tokenId	TokenId,

recipient	UserId

}

TVin  ::=  [APPLICATION 34] IMPLICIT SEQUENCE  --  token give indication
{

initiator	UserId,

tokenId	TokenId,

recipient	UserId

}

TVrs  ::=  [APPLICATION 35] IMPLICIT SEQUENCE  --  token give response
{

result	Result,

recipient	UserId,

tokenId	TokenId

}

TVcf  ::=  [APPLICATION 36] IMPLICIT SEQUENCE  --  token give confirm
{

result	Result,

initiator	UserId,

tokenId	TokenId,

tokenStatus	TokenStatus

}

TPrq  ::=  [APPLICATION 37] IMPLICIT SEQUENCE  --  token please request
{

initiator	UserId,

tokenId	TokenId

}

TPin  ::=  [APPLICATION 38] IMPLICIT SEQUENCE  --  token please indication
{

initiator	UserId,

tokenId	TokenId

}

TRrq  ::=  [APPLICATION 39] IMPLICIT SEQUENCE  --  token release request
{

initiator	UserId,

tokenId	TokenId

}

TRcf  ::=  [APPLICATION 40] IMPLICIT SEQUENCE  --  token release confirm
{

result	Result,

initiator	UserId,

tokenId	TokenId,

tokenStatus	TokenStatus

}

TTrq  ::=  [APPLICATION 41] IMPLICIT SEQUENCE  --  token test request
{

initiator	UserId,

tokenId	TokenId

}

TTcf  ::=  [APPLICATION 42] IMPLICIT SEQUENCE  --  token test confirm
{

initiator	UserId,

tokenId	TokenId,

tokenStatus	TokenStatus

}

--  Part 9: Status codes

Reason  ::=  ENUMERATED	 --  in DPum, DUrq, DUin
{

rn-domain-disconnected	(0),
rn-provider-initiated	(1),
rn-token-purged	(2),
rn-user-requested	(3),
rn-channel-purged	(4)

}

Result  ::=  ENUMERATED	 --  in Connect, response, confirm
{

rt-successful	(0),
rt-domain-merging	(1),
rt-domain-not-hierarchical	(2),
rt-no-such-channel	(3),
rt-no-such-domain	(4),
rt-no-such-user	(5),
rt-not-admitted	(6),
rt-other-user-id	(7),
rt-parameters-unacceptable	(8),
rt-token-not-available	(9),
rt-token-not-possessed	(10),
rt-too-many-channels	(11),
rt-too-many-tokens	(12),
rt-too-many-users	(13),
rt-unspecified-failure	(14),
rt-user-rejected	(15)

}

Diagnostic  ::=  ENUMERATED	 --  in RJum
{

dc-inconsistent-merge	(0),
dc-forbidden-PDU-downward	(1),
dc-forbidden-PDU-upward	(2),
dc-invalid-BER-encoding	(3),
dc-invalid-PER-encoding	(4),
dc-misrouted-user	(5),
dc-unrequested-confirm	(6),
dc-wrong-transport-priority	(7),
dc-channel-id-conflict	(8),
dc-token-id-conflict	(9),

dc-not-user-id-channel	(10),
dc-too-many-channels	(11),
dc-too-many-tokens	(12),
dc-too-many-users	(13)

}

--  Part 10: MCSPDU repertoire

ConnectMCSPDU  ::=  CHOICE
{

connect-initial	Connect-Initial,

connect-response	Connect-Response,

connect-additional	Connect-Additional,

connect-result	Connect-Result

}

DomainMCSPDU  ::=  CHOICE
{

pdin	PDin,
	edrq	EDrq,
	mcrq	MCrq,
	mccf	MCcf,
	pcin	PCin,
	mtrq	MTrq,
	mtcf	MTcf,
	ptin	PTin,
	dpum	DPum,
	rjum	RJum,
	aurq	AUrq,
	aucf	AUcf,
	durq	DUrq,
	duin	DUin,
	cjrq	CJrq,
	cjcf	CJcf,
	clrq	CLrq,
	ccrq	CCrq,
	cccf	CCcf,
	cdrq	CDrq,
	cdin	CDin,
	carq	CArq,
	cain	CAin,
	cerq	CErq,
	cein	CEin,
	sdrq	SDrq,
	sdin	SDin,
	usrq	USrq,
	usin	USin,
	tgrq	TGrq,
	tgcf	TGcf,
	tirq	TIrq,
	ticf	TIcf,
	tvrq	TVrq,
	tvin	TVin,
	tvrs	TVrs,
	tvcf	TVcf,
	tprq	TPrq,
	tpin	TPin,
	trrq	TRrq,
	trcf	TRcf,
	ttrq	TTrq,
	ttcf	TTcf

}

END

# 8	Encoding of MCSPDUsXE "Encoding of MCSPDUs"§

Each MCSPDU is transported as one TSDU across a TC belonging to an MCS connection. Connect MCSPDUs are unlimited in size. Domain MCSPDUs are limited in size by a parameter of the MCS domain.

A standard ASN.1 data value encoding is used to transfer MCSPDUs between peer MCS providers. Two versions of this protocol are defined, differing only in the specification of encoding rules:

–	Version 1 – Uses the Basic Encoding Rules of CCITT Recommendation X.209 for all MCSPDUs.

–	Version 2 – Uses the Basic Encoding Rules for connect MCSPDUs and the Packed Encoding Rules of ISO/IEC 8825-2 for all subsequent domain MCSPDUs. Specifically, the ALIGNED variant of BASIC-PER shall be applied to the ASN.1 type DomainMCSPDU. The bit string produced shall be conveyed as an integral number of octets. The leading bit of this string shall coincide with the most significant bit of the first octet.

Appendix I provides an example of a data transfer MCSPDU under the alternative encodings of Version 1 and 2.

Negotiation of the protocol version involves the exchange of a Connect-Initial and a Connect-Response MCSPDU over the initial TC. These two MCSPDUs are always encoded by the Basic Encoding Rules. So are any Connect-Additional and Connect-Result MCSPDUs that follow, prior to the onset of domain MCSPDUs. Domain MCSPDUs begin with the second TSDU transmitted over a TC.

Version 2 of this protocol shall not be used until the Packed Encoding Rules have been adopted as part of an ITU-T Recommendation or ISO/IEC International Standard.

NOTES

1	The Packed Encoding Rules yield more compact MCSPDU headers.

2	Both BER and PER are self-delimiting, in the sense that they contain enough information to locate the end of each encoded MCSPDU. It might be argued that this makes the use of TSDUs unnecessary and that this protocol could be implemented over non-standard transport services that convey octet streams without preserving TSDU boundaries. However, such an approach is more vulnerable to implementation errors. If the boundary between MCSPDUs were ever lost, recovery would be difficult.

# 9	Routing of MCSPDUsXE "Routing of MCSPDUs"§

## 9.1	Connect MCSPDUsXE "Connect MCSPDUs"§

Figure 9-1 specifies the exchange of connect MCSPDUs.

With the receipt of a Connect-Response, the calling MCS provider learns the negotiated value for the number of data transfer priorities implemented in the domain. For illustration, connect MCSPDUs on additional TCs are shown obeying the strict sequence 2, 3, 4 at the called MCS provider. In reality, transport connections may not be established in the same order that they are requested. Indeed, a Connect-Additional may arrive in a different order than it was sent, causing a Connect-Result to be returned out of sequence too. Or the latter, even if sent in sequence, may still be reordered in transit. A calling MCS provider need not wait for all additional TCs to be established before sending the first Connect-Additional. The called MCS provider need not wait for a full set of Connect-Additional MCSPDUs to arrive before returning the first Connect-Result. Receipt of a full set of successful results at the calling MCS provider, in whatever order, generates a successful MCS-CONNECT-PROVIDER confirm.

An unsuccessful Connect-Response or Connect-Result or a T-DISCONNECT indication at some intermediate point shall cause all TCs belonging to the MCS connection so far to be disconnected and shall generate an unsuccessful MCS-CONNECT-PROVIDER confirm.

An MCS-CONNECT-PROVIDER request specifies which of two MCS providers is higher than the other. This hierarchical relationship determines the subsequent routing of domain MCSPDUs, and the distinction between calling and called MCS provider is thereafter irrelevant. For example, a next step is for the MCS layer to merge the resources of two previously independent domains. MCrq and MTrq are generated by the lower MCS provider and are transmitted across the new MCS connection to the higher MCS provider. The direction in which these MCSPDUs are sent could be either calling-to-called or called-to-calling, depending on how the upward flag was set.

FIGURE  9-1/T.125.....[D03]  =  15 cm

## 9.2	Domain MCSPDUsXE "Domain MCSPDUs"§

Table 9-1 specifies the routing of domain MCSPDUs.

If an MCS provider generates or forwards an MCSPDU of category request, it travels over the unique MCS connection upward. EDrq, CJrq, and CLrq may be consumed at some intermediate MCS provider. Other requests rise to be acted on by the top MCS provider, unless the content of a request is determined to be invalid, in which case its MCSPDU may be ignored, without confirmation.

TABLE  9-1/T.125

Routing of domain MCSPDUs

| Category | MCSPDUs | TC | Direction |
| --- | --- | --- | --- |
| Request | EDrq MCrq MTrq AUrq DUrq CJrq CLrq CCrq CDrq CArq CErq TGrq TIrq TVrq TPrq TRrq TTrq | I | Up |
|  | SDrq USrq | A |  |
| Indication | PDin PCin PTin DUin CDin CAin CEin TVin TPin | I | Down |
|  | SDin USin | A |  |
| Response | TVrs | I | Up |
| Confirm | MCcf MTcf AUcf CJcf CCcf TGcf TIcf TVcf TRcf TTCF | I | Down |
| Ultimatum | DPum RJum | I | Up or Down |
| I The MCSPDU traverses the initial TC. A The MCSPDU may traverse an additional TC, depending on its data transfer priority. Up The MCSPDU travels toward the top MCS provider. Down The MCSPDU travels away from the top MCS provider. |  |  |  |

If an MCS provider generates or forwards an MCSPDU of category indication, copies of it, possibly amended in content, travel over zero or more MCS connections downward, according to the following rules:

a)	PDin is sent on all MCS connections downward. The height limit it contains is decremented by one. An MCS provider receiving this MCSPDU with a height limit of zero shall disconnect.

b)	PCin is sent on all MCS connections downward. The set of user ids forwarded is unchanged, so that all detached users will be announced to those that remain. The set of other channel ids forwarded may be restricted to those in use in a subtree. At a former top provider that is still merging into an upper domain, both user ids and other channel ids are restricted to those whose acceptance into the upper domain has been confirmed.

c)	PTin is sent on all MCS connections downward. The set of token ids forwarded may be restricted to those in use in a subtree. At a former top provider that is still merging into an upper domain, token ids are restricted to those whose acceptance into the upper domain has been confirmed.

d)	DUin is sent on all MCS connections downward. The set of user ids forwarded is unchanged, so that all detached users will be announced to those that remain. At a former top provider that is still merging into an upper domain, user ids are restricted to those whose acceptance into the upper domain has been confirmed.

e)	CDin is sent on all MCS connections downward that contain in their subtree the manager of the private channel or any admitted user.

f)	CAin and CEin are sent on all MCS connections downward that contain one or more of the affected users in their subtree. The set of user ids forwarded may be restricted to those residing in a subtree.

g)	TVin is sent on a single MCS connection downward that contains the designated recipient in its subtree.

h)	TPin is sent on all MCS connections downward that contain in their subtree a user who has grabbed, inhibited, or is being given the token.

i)	SDin and USin are sent on all MCS connections downward by which the specified channel is joined, except that when SDin is generated, it is not sent back on the connection by which SDrq arrived.

Indications PCin, PTin, DUin, CAin, and CEin need not be forwarded if the sets of ids they contain are empty.

If an MCS provider generates or forwards an MCSPDU of category response, it travels over the unique MCS connection upward. It rises to be acted on by the top MCS provider, unless its content is determined to be invalid.

If an MCS provider generates or forwards an MCSPDU of category confirm, it travels over a single MCS connection downward, according to the following rules.

a)	MCcf retraces, in the opposite direction, the path of the earliest MCrq that has not yet been answered by a confirm. This requires each MCS provider to maintain a first-in first-out queue of pending requests.

b)	MTcf retraces, in the opposite direction, the path of the earliest MTrq that has not yet been answered by a confirm. This requires each MCS provider to maintain a first-in first-out queue of pending requests.

c)	AUcf retraces, in the opposite direction, the path of an earlier AUrq that has not yet been answered by a confirm. It is not critical which AUrq, if more than one is pending, but to be fair each MCS provider should maintain a first-in first-out queue. Upon sending AUcf, an MCS provider shall record to which subtree the user id it contains is thereby being assigned.

d)	Other MCSPDUs of category confirm contain an initiator user id that was previously assigned through the action of AUcf as just explained. These MCSPDUs are sent on the MCS connection downward that leads to the subtree where the user id was assigned. Continuing in this way, they eventually return to the provider that hosts the requesting MCS attachment.

Confirms are generated in the course of processing like requests. All but CJcf are generated by the top MCS provider.

If an MCS provider generates an MCSPDU of category ultimatum, it travels over a single MCS connection, either upward or downward. DPum commands the receiving MCS provider to disconnect the MCS connection that conveys it. RJum rejects an erroneous MCSPDU with a diagnostic code and invites the MCS provider that transmitted it to disconnect. Ultimatums are not forwarded.

# 10	Meaning of MCSPDUsXE "Meaning of MCSPDUs"§

Tables 10-1 through 10-47 reiterate the contents of individual MCSPDUs as defined in clause 8.

## 10.1	Connect-InitialXE "Connect-Initial"§

Connect-Initial is generated by an MCS-CONNECT-PROVIDER request. It is sent as the first TSDU over the initial TC of a new MCS connection. At the receiver it generates an MCS-CONNECT-PROVIDER indication.

TABLE  10-1/T.125

Connect-Initial MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Calling Domain Selector | Request | Indication |
| Called Domain Selector | Request | Indication |
| Upward Flag | Request | Indication |
| Target Domain Parameters | Request | Indication |
| Minimum Domain Parameters | Request | Indication |
| Maximum Domain Parameters | Request | Indication |
| User Data | Request | Indication |

Calling transport address and called transport address are additional parameters of the MCS-CONNECT-PROVIDER request and indication. They become parameters of T-CONNECT and are not passed explicitly in any MCSPDU. The same pair of transport addresses shall be used to request all TCs belonging to the same MCS connection.

Transport quality of service is an additional parameter of the MCS-CONNECT-PROVIDER request but not of the indication. Quality of service can vary from one TC to another, and the quality available is only disclosed in the process of establishing a given TC. Since the number of additional TCs needed is not known until MCS-CONNECT-PROVIDER has negotiated the domain parameter for number of data priorities implemented, this primitive cannot at the same time fully negotiate their transport quality of service. A called MCS provider shall therefore accept incoming TCs automatically at the offered quality of service, so long as this meets any minimum optionally specified by the calling MCS provider and indicated through each individual T-CONNECT.

The interpretation of domain selector values is a local matter for each MCS provider. These are octet strings that have the characteristics of an address. Acceptable values may be determined through the process of configuring an MCS provider. More than one value may select the same domain. An unspecified domain selector is an octet string of length zero. This may be resolved, through local convention, to some explicit value.

The upward flag specifies the direction of a new MCS connection: true if the called provider is to be considered higher than the calling provider and false otherwise. An MCS provider plays a role in the hierarchy of a domain based on the direction of MCS connections in which it participates. No provider shall allow two connections to higher providers. A provider without a connection to a higher provider shall act as top MCS provider.

The target domain parameters of Connect-Initial individually shall lie between the minimum and maximum values specified. An MCS provider shall revise the requested domain parameters to reflect limits of its implementation or to impose values already agreed among the existing members of a domain. It may increase the minimums and decrease the maximums. It shall change targets only to keep them within the interval. An MCS provider should be prepared to honour any response that falls within the range of values it proposes.

User data is an arbitrary octet string. It may have length zero.

An MCS provider automatically accepts each incoming TC to the limit of its capacity. User data in T-CONNECT is unused. The first TSDU received in data transfer, being either a Connect-Initial or a Connect-Additional MCSPDU, determines the nature of the TC. If the content is unacceptable, a called MCS provider may disconnect the TC immediately. The preferred reaction is to return a Connect-Response or Connect-Result, as the case may be, explaining why the MCS connection failed. The calling MCS provider shall then disconnect.

## 10.2	Connect-ResponseXE "Connect-Response"§

Connect-Response is generated by an MCS-CONNECT-PROVIDER response. It is the first TSDU sent in reverse over the initial TC of a new MCS connection. It conveys the acceptance of an MCS connection to the calling MCS provider, which then proceeds to establish any additional TCs required.

TABLE  10-2/T.125

Connect-Response MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Response | Confirm |
| Domain Parameters | Response | Confirm |
| Called Connect Id | Called provider | Calling provider |
| User Data | Response | Confirm |

If the result is successful, this MCSPDU fixes the domain parameters in effect. Among these is the number of MCS data transfer priorities implemented, equal to the number of TCs in an MCS connection. If this exceeds one, additional TCs shall be created and bound to the MCS connection through the exchange of Connect-Additional and Connect-Result MCSPDUs.

The called connect id serves as the means for associating additional incoming TCs at the called MCS provider with this initial TC. Its value is chosen for this purpose alone. It shall uniquely identify one MCS connection in progress at the called provider. This id has no lasting significance following the completion of MCS-CONNECT-PROVIDER.

Most of the parameters of MCS-CONNECT-PROVIDER confirm are conveyed in Connect-Response. If the result is unsuccessful or no additional TCs are needed, confirm is generated immediately. Otherwise, it is deferred until the results are known for binding additional TCs to the MCS connection.

## 10.3	Connect-AdditionalXE "Connect-Additional"§

Connect-Additional is generated following receipt of Connect-Response. It is sent as the first TSDU over an additional TC of a new MCS connection.

Data priority takes on the values high, medium, and low in sequence, up to the number of additional TCs required.

TABLE  10-3/T.125

Connect-Additional MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Called Connect Id | Calling provider | Called provider |
| Data Priority | Calling provider | Called provider |

## 10.4	Connect-ResultXE "Connect-Result"§

Connect-Result is generated following receipt of Connect-Additional. It is the first TSDU sent in reverse over an additional TC of a new MCS connection.

TABLE  10-4/T.125

Connect-Result MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Called provider | Confirm |

If any result is unsuccessful, MCS-CONNECT-PROVIDER confirm shall be generated immediately. All TCs associated with the MCS connection shall be disconnected and any MCSPDUs they convey shall be ignored.

Otherwise, successful results shall be awaited for each additional TC. These may return out of sequence. When all have been collected, a successful MCS-CONNECT-PROVIDER confirm shall be generated.

After MCS-CONNECT-PROVIDER confirm, domain MCSPDUs may flow across an MCS connection. Each MCS connection belongs to a single domain. In configurations where an MCS provider hosts more than one domain, the MCS connection that carries MCSPDUs determines which domain they apply to. The descriptions of domain MCSPDUs that occupy the remainder of this clause are set in the context of a single domain.

## 10.5	PDinXE "PDin"§

PDin is generated following the successful completion of MCS-CONNECT-PROVIDER. It plumbs the hierarchy of MCS providers below a new MCS connection to ensure that no cycle has been created. PDin is also generated by the top MCS provider to enforce the maximum height of a domain.

TABLE  10-5/T.125

PDin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Height Limit | Former top or top | Subordinates |

PDin is generated at the lower end of a new MCS connection, by the provider that has ceased to be top of a domain. Its content is initialized to the domain parameter for the maximum height of the domain. PDin is transmitted over all MCS connections downward.

As needed, PDin is generated in the same way at the top MCS provider.

Wherever PDin is received, the height limit it contains is inspected. If greater than zero, the limit is decremented by one and PDin is forwarded over all MCS connections downward. A value of zero, on the other hand, means that the receiver lies too far from the top provider. It shall react by disconnecting the MCS connection upward. This deletes an entire subtree and helps to repair the height of the domain.

In the presence of a cycle, the height limit of PDin must decrease until it reaches zero. The provider that detects this will break the cycle, at the expense of deleting all providers in the cycle and their subordinates from the domain.

NOTE – An MCS provider, with purely local knowledge of MCS connections, cannot prevent the creation of cycles. It can ensure that there is at most one connection upward at all times, but it cannot ensure that the upward connection does not loop back to some provider below. When a cycle is created, the immediate cause is a faulty upward connection from the top MCS provider. Controller applications, which specify the MCS connections to be created, must strive to avoid such errors.

## 10.6	EDrqXE "EDrq"§

EDrq is generated following the successful completion of MCS-CONNECT-PROVIDER. It communicates upward changes in the height of providers and their throughput enforcement intervals. EDrq is generated by an MCS provider whenever its height or interval changes.

The height of an MCS provider may change when an MCS connection is added or dropped or when a subordinate provider reports a change through EDrq. Its monitoring interval to enforce the minimum throughput specified as a domain parameter may change by adapting to the intervals reported by subordinates or for other reasons. If either value changes, an MCS provider shall transmit EDrq to its immediate superior.

TABLE  10-6/T.125

EDrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Height in Domain | Subordinate | Higher provider |
| Throughput Enforcement Interval | Subordinate | Higher provider |

## 10.7	MCrqXE "MCrq"§

MCrq is generated following the successful completion of MCS-CONNECT-PROVIDER. It communicates upward the attributes of channels held by a former top provider so that they may be incorporated into the merged domain.

TABLE  10-7/T.125

MCrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Merge Channels | Former top | Top provider |
| Purge Channel Ids | Intermediates | Top provider |

MCrq may be filled with the attributes of multiple channels, up to the domain limit on MCSPDU size. As detailed in the ASN.1 definitions of clause 7, each of the four kinds of channel in use (static, user id, private, assigned) has its relevant set of attributes. These are held in the information base of the top MCS provider and are partially replicated into the subtrees where a channel is used. When two domains are merged, through the action of MCS-CONNECT-PROVIDER, channels that are in use in the lower domain must either be incorporated into the information base of the upper domain or be purged from the lower domain. This decision rests with the top provider of the merged domain.

Each channel shall be considered individually. If the domain limits on channels in use allow it and the channel id has not already been put to some conflicting use, the upper domain shall expand to include it. The use of a static channel id is never a conflict. The use of a private channel id in the lower domain is not a conflict if it is also used as a private channel id in the upper domain and has the same user id as manager. All other combinations of simultaneous use are disallowed, requiring the channel id to be purged from the lower domain.

If a private channel has a large set of admitted users, its attributes may not fit into a single MCrq and shall be sent upward in multiple MCSPDUs. However, the second and succeeding requests to merge the same private channel shall be delayed until an MCcf has been received in reply to the first. Only then is it known if domain limits have allowed the channel to be put into use in the upper domain. If the first request fails, it shall not be repeated with a remaining subset of admitted users.

Each MCrq elicits an MCcf reply from the top MCS provider, in the same sequence. An MCcf contains nothing that explicitly identifies the preceding MCrq. Replies shall be routed solely by the order in which these MCSPDUs are received. MCS providers above the former top shall make a record of each unanswered MCrq and whence it arrived, so that the corresponding MCcf may be returned via the same MCS connection.

Intermediate MCS providers shall validate the user ids proclaimed in private channel attributes, to ensure that they are legitimately assigned to the subtree where the MCrq originates. Invalid user ids shall be deleted. If the manager of a private channel is deleted, all of the channel attributes shall be deleted from the merge request and the channel id alone shall be included in the set to be purged. Except for this validation of user ids, intermediate MCS providers shall not modify the contents of an MCrq.

A former top provider shall await individual confirmation that all user ids and all token ids have been incorporated into a merged domain or purged before it begins to submit static, assigned, or private channel attributes for merger.

## 10.8	MCcfXE "MCcf"§

MCcf replies to a preceding MCrq. It reflects the same set of channel ids and a subset of the attributes. Channel attributes not incorporated into the merged domain are reported as channel ids to be purged.

Accepted channel ids are reflected with the attributes that were entered into the information base of the top MCS provider. Intermediate providers shall update their information base to conform.

TABLE  10-8/T.125

MCcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Merge Channels | Top provider | Intermediates |
| Purge Channel Ids | Top provider | Former top |

Channels to be purged from the lower domain are listed by id only. If the same channel ids are used in the upper domain, they are left undisturbed. Intermediate providers shall forward purged channel ids without acting on them.

MCS providers shall route MCcf to the source of the antecedent MCrq, using the knowledge that there is a one-to-one reply. MCcf returns to the former top provider that generated MCrq. There the merged channels may be ignored, as they have remained in the information base pending a reply. Purged channel ids shall be deleted as they are for PCin.

Intermediate MCS providers shall confirm that user ids proclaimed in private channel attributes are assigned to the subtree to which MCcf is routed. If a private channel manager has been detached and reassigned elsewhere in the time since the antecedent MCrq was validated, an intermediate provider shall generate a CDrq making the private channel a casualty of domain merger and shall move the channel id into the purged set. If any admitted users have been reassigned elsewhere, it shall exclude them from the channel.

## 10.9	PCinXE "PCin"§

PCin is generated at a former top provider following receipt of MCcf. It is broadcast downward and purges the use of specified channel ids from subordinate providers.

TABLE  10-9/T.125

PCin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Detach User Ids | Former top | Subordinates |
| Purge channel Ids | Former top | Subordinates |

Depending on the current use of a channel id, the effect of purging it is: MCS-DETACH-USER indication to all users if a user id channel; MCS-CHANNEL-LEAVE indication to the joined users if a static or assigned channel id; MCS-CHANNEL-DISBAND indication to the manager and MCS-CHANNEL-EXPEL indication to the admitted users if a private channel id.

A former top provider, knowing the use of all channels in its lower domain, can generate the proper indications from channel ids alone. Its subordinates, however, may have only partial knowledge. They must be told which channel ids represent detached users, for which an indication is always generated, and which represent other kinds of channels, for which an indication is generated only if the channel is in use at the subordinate provider. Therefore, PCin divides the channel ids to be purged into these two categories.

The purging of a user id through PCin shall have the same consequences as deletion through DUin, except that, since the receiver of PCin is no longer top provider, it need not generate CDin or TVcf as a side effect.

NOTE – A provider may receive in PCin static and assigned channel ids to which it is not joined or private channel ids for which its attachments are neither managers nor admitted users. Records of use maintained in the information base allow a provider to suppress primitive indications for such channel ids.

## 10.10	MTrqXE "MTrq"§

MTrq is generated following the successful completion of MCS-CONNECT-PROVIDER. It communicates upward the attributes of tokens held by a former top provider so that they may be incorporated into the merged domain.

TABLE  10-10/T.125

MTrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Merge Tokens | Former top | Top provider |
| Purge Token Ids | Intermediates | Top provider |

MTrq may be filled with the attributes of multiple tokens, up to the domain limit on MCSPDU size. As detailed in the ASN.1 definitions of clause 7, each state of a token in use (grabbed, inhibited, giving, ungivable, given) has its relevant set of attributes. These are held in the information base of the top MCS provider and are partially replicated into the subtrees where a token is used. When two domains are merged, through the action of MCS-CONNECT-PROVIDER, tokens that are in use in the lower domain must either be incorporated into the information base of the upper domain or be purged from the lower domain. This decision rests with the top provider of the merged domain.

Each token shall be considered individually. If the domain limits on tokens in use allow it and the token id has not already been put to some conflicting use, the upper domain shall expand to include it. The inhibiting of a token id in the lower domain is not a conflict if it is also inhibited in the upper domain. All other combinations of simultaneous use are disallowed, requiring the token id to be purged from the lower domain.

If a token has a large set of inhibiting users, its attributes may not fit into a single MTrq and shall be sent upward in multiple MCSPDUs. However, the second and succeeding requests for the same inhibited token shall be delayed until an MTcf has been received in reply to the first. Only then is it known if domain limits have allowed the token to be put into use in the upper domain. If the first request fails, it shall not be repeated with a remaining subset of inhibitors.

Each MTrq elicits an MTcf reply from the top MCS provider, in the same sequence. An MTcf contains nothing that explicitly identifies the preceding MTrq. Replies shall be routed solely by the order in which these MCSPDUs are received. MCS providers above the former top shall make a record of each unanswered MTrq and whence it arrived, so that the corresponding MTcf may be returned via the same MCS connection.

Intermediate MCS providers shall validate the user ids proclaimed in token attributes, to ensure that they are legitimately assigned to the subtree where the MTrq originates. Invalid user ids shall be deleted. A token being given shall remain grabbed if either the grabber or the recipient, but not both, is deleted. If a token becomes released through this deletion of user ids, all of its attributes shall be deleted from the merge request and the token id alone shall be included in the set to be purged. An inhibited token shall remain inhibited in MTrq even if all inhibitors are deleted, leaving an empty set in the attributes, because inhibitors may survive from other MCSPDUs. Except for this validation of user ids, intermediate MCS providers shall not modify the contents of an MTrq.

A former top provider shall await individual confirmation that all user ids have been incorporated into a merged domain or purged before it begins to submit token attributes for merger.

## 10.11	MTcfXE "MTcf"§

MTcf replies to a preceding MTrq. It reflects the same set of token ids and a subset of the attributes. Token attributes not incorporated into the merged domain are reported as token ids to be purged.

TABLE  10-11/T.125

MTcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Merge Tokens | Top provider | Intermediates |
| Purge Token Ids | Top provider | Former top |

Accepted token ids are reflected with the attributes that have been entered into the information base at the top MCS provider. Intermediate providers shall update their information base to conform.

Tokens to be purged from the lower domain are listed by id only. If the same token ids are used in the upper domain, they are left undisturbed. Intermediate providers shall forward purged token ids without acting on them.

MCS providers shall route MTcf to the source of the antecedent MTrq, using the knowledge that there is a one-to-one reply. MTcf returns to the former top provider that generated MTrq. There the merged tokens may be ignored, as they have remained in the information base pending a reply. Purged token ids shall be deleted as they are for PTin.

Intermediate MCS providers shall confirm that the user ids proclaimed in token attributes are assigned to the subtree to which MTcf is routed. If any user ids have been detached and reassigned elsewhere in the time since the antecedent MTrq was validated, an intermediate provider shall generate for them a DUrq with reason code channel purged, making them casualties of domain merger. If a non-inhibited token id becomes released through this deletion of user ids, it shall be moved into the purged set.

## 10.12	PTinXE "PTin"§

PTin is generated at a former top provider following receipt of MTcf. It is broadcast downward and purges the use of specified token ids from subordinate providers.

TABLE  10-12/T.125

PTin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Purge Token Ids | Former top | Subordinates |

The effect of purging a token is severe: MCS-DETACH-USER indication to any user who has grabbed, inhibited, or is being given one of the token ids. A provider shall implement this by generating DUrq on behalf of the affected users with reason token purged.

NOTE – It is anticipated that Recommendation T.122 may be revised in the future to allow MCS-TOKEN-RELEASE indication in this situation. This would allow the affected user to remain attached even though its right to use the token is withdrawn.

## 10.13	DPumXE "DPum"§

DPum is generated by an MCS-DISCONNECT-PROVIDER request. In turn, it generates an MCS-DISCONNECT-PROVIDER indication at the other end of an MCS connection. DPum compels the receiver to disconnect the MCS connection that conveyed it.

DPum may also be generated by an MCS provider when it detects an error condition like the existence of a cycle in the domain hierarchy. In such cases, the reason is other than user-requested.

TABLE  10-13/T.125

DPum MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Reason | Requesting provider | Indication |

## 10.14	RJumXE "RJum"§

RJum is generated when an MCS provider receives an invalid MCSPDU or detects an MCS protocol error. It invites the peer provider at the other end of an MCS connection to disconnect, since recovery is uncertain from a situation that should not occur.

RJum diagnoses the error and returns an initial portion of the offending TSDU, typically as many octets as will fit in the maximum size MCSPDU. The receiving provider has the option to disconnect or to persevere.

TABLE  10-14/T.125

RJum MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Diagnostic | Rejecting provider | Rejected provider |
| Initial Octets | Rejecting provider | Rejected provider |

## 10.15	AUrqXE "AUrq"§

AUrq is generated by an MCS-ATTACH-USER request. It rises to the top MCS provider, which returns an AUcf reply. If the domain limit on number of user ids allows, a new user id is generated.

TABLE  10-15/T.125

AUrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| (None) | – | – |

AUrq contains no information other than its MCSPDU type. The domain to which the user attaches is determined by the MCS connection conveying the MCSPDU. The only initial characteristic of the user id generated is its uniqueness.

An MCS provider shall make a record of each unanswered AUrq received and by which MCS connection it arrived, so that a replying AUcf can be routed back to the same source. To distribute replies fairly, each provider should maintain a first-in, first-out queue for this purpose.

## 10.16	AUcfXE "AUcf"§

AUcf is generated at the top MCS provider upon receipt of AUrq. Routed back to the requesting provider, it generates an MCS-ATTACH-USER confirm.

TABLE  10-16/T.125

AUcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Top provider | Confirm |
| Initiator (optional) | Top provider | Confirm |

AUcf contain a user id if and only if the result is successful. Providers that receive a successful AUcf shall enter the user id into their information base.

MCS providers shall route AUcf to the source of an antecedent AUrq, using the knowledge that there is a one-to-one reply. A provider that transmits AUcf shall note to which downward MCS connection the new user id is thereby assigned, so that it may validate the user id when it arises later in other requests.

## 10.17	DUrqXE "DUrq"§

DUrq is generated by an MCS-DETACH-USER request. If valid, it rises to the top MCS provider, which deletes the user from its information base and broadcasts DUin to advise other providers of the change.

TABLE  10-17/T.125

DUrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Reason | Requesting provider | Top provider |
| User Ids | Requesting provider | Top provider |

An MCS-DETACH-USER request generates a DUrq containing reason user-requested and a single user id.

DUrq shall also be generated by an MCS provider when a downward MCS connection is disconnected. At that point, all users in the affected subtree are lost and shall be reported as detached with reason domain disconnected. If user id assignments are pending, a later reply, either MCcf or AUcf, may then be left with no route back to its source in the disconnected subtree. A provider confronted with this shall also generate DUrq to delete the unassignable user ids.

Providers that receive DUrq shall validate the user ids it contains to ensure that they are legitimately assigned to the subtree of origin. Invalid user ids shall be deleted. If no user ids remain, a DUrq shall be ignored.

The user ids contained in DUrq shall not be deleted from the information base until a provider receives DUin. This maintains consistency with the top MCS provider.

NOTE – If more than one data priority is implemented in an MCS domain, DUin may arrive at a given provider before data sent earlier but at a low priority by the same user. This protocol does not prevent data being delivered to an attachment even after it was reported through DUin that the sender had detached.

## 10.18	DUinXE "DUin"§

DUin is generated at the top MCS provider upon receipt of DUrq. It is broadcast downward to all other providers and generates MCS-DETACH-USER indications at all attachments.

At a surviving attachment, DUin generates one MCS-DETACH-USER indication for each user id it contains. It does not matter whether the notified user was previously aware of the existence of a detached user.

Upon receipt of a DUin containing its own user id, an MCS attachment ceases to exist. Any channels than become unjoined as a result of a user detaching, except for the user id channel itself, shall be left via CLrq.

TABLE  10-18/T.125

DUin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Reason | Top provider | Indication |
| User Ids | Top provider | Indication |

Recommendation T.125     (04-94)

Providers that receive DUin shall delete the specified user ids from their information base. Private channels managed by a detached user shall be deleted if there are no other admitted users. If other users remain, deletion of the manager shall cause the top provider to multicast a CDin towards them. Any tokens grabbed, being given to, or inhibited by a detached user shall have their state adjusted accordingly. The deletion of an intended token recipient shall cause the top provider to generate an unsuccessful TVcf towards the donor, unless it has released the token or itself detached.

## 10.19	CJrqXE "CJrq"§

CJrq is generated by an MCS-CHANNEL-JOIN request. If valid, it rises until it reaches an MCS provider with enough information to generate a CJcf reply. This may be the top MCS provider.

TABLE  10-19/T.125

CJrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting Provider | Higher provider |
| Channel Id | Request | Higher provider |

The user id of the initiating MCS attachment is supplied by the MCS provider that receives the primitive request. Providers that receive CJrq subsequently shall validate the user id to ensure that it is legitimately assigned to the subtree of origin. If the user id is invalid, the MCSPDU shall be ignored.

NOTE – This allows for the possibility that CJrq may be racing upward against a purge of the initiating user id flowing down. A provider that receives PCin first might receive a CJrq soon thereafter that contains an invalid user id. This is a normal occurrence and is not cause for rejecting the MCSPDU.

CJrq may rise to an MCS provider that has the requested channel id in its information base. Any such provider, being consistent with the top MCS provider, will agree whether the request should succeed. If the request should fail, the provider shall generate an unsuccessful CJcf. If it should succeed and the provider is already joined to the same channel, the provider shall generate a successful CJcf. In these two cases, MCS-CHANNEL-JOIN completes without necessarily visiting the top MCS provider. Otherwise, if the request should succeed but the channel is not yet joined, a provider shall forward CJrq upward.

If CJrq rises to the top MCS provider, the channel id requested may be zero, which is in no information base because it is an invalid id. If the domain limit on the number of channels in use allows, a new assigned channel id shall be generated and returned in a successful CJcf. If the channel id requested is in the static range and the domain limit on the number of channels in use allows, the channel id shall be entered into the information base and shall likewise be returned in a successful CJcf.

Otherwise, the request will succeed only if the channel id is already in the information base of the top MCS provider. A user id channel can only be joined by the same user. A private channel id can be joined only by users previously admitted by its manager. An assigned channel id can be joined by any user.

## 10.20	CJcfXE "CJcf"§

CJcf is generated at a higher MCS provider upon receipt of CJrq. Routed back to the requesting provider, it generates an MCS-CHANNEL-JOIN confirm.

TABLE  10-20/T.125

CJcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Higher provider | Confirm |
| Initiator | Higher provider | MCSPDU routing |
| Requested | Higher provider | Confirm |
| Channel Id (optional) | Higher provider | Confirm |

CJcf contains a joined channel id if and only if the result is successful.

The channel id requested is the same as in CJrq. This helps the initiating attachment relate MCS-CHANNEL-JOIN confirm to an antecedent request. Since CJrq need not rise to the top provider, confirms may occur out of order.

If the result is successful, CJcf joins the receiving MCS provider to the specified channel. Thereafter, higher providers shall route to it any data that users send over the channel. A provider shall remain joined to a channel as long as any of its attachments or subordinate providers does. To leave the channel, a provider shall generate CLrq.

Providers that receive a successful CJcf shall enter the channel id into their information base. If not already there, the channel id shall be given type static or assigned, depending on its range.

CJcf shall be forwarded in the direction of the initiating user id. If the user id is unreachable because an MCS connection no longer exists, the provider shall decide whether it has reason to remain joined to the channel. If not, it shall generate CLrq.

## 10.21	CLrqXE "CLrq"§

CLrq is generated by an MCS provider to remove itself from a set of channels. The motivation may be an MCS-CHANNEL-LEAVE request from the last attachment joined to a channel. CLrq continues to rise if higher providers, as a consequence, also lose their reason for being joined.

Providers that receive CLrq shall stop routing to the MCS connection that conveyed it any data that users send over the specified channels. When the last attachment or subordinate provider leaves a channel, an MCS provider shall generate a corresponding CLrq.

TABLE  10-21/T.125

CLrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Channel Ids | Requesting provider | Higher provider |

## 10.22	CCrqXE "CCrq"§

CCrq is generated by an MCS-CHANNEL-CONVENE request. If valid, it rises to the top MCS provider, which returns a CCcf reply. If the domain limit on number of channel ids allows, a new private channel id is generated.

CCrq contains the initiating user id, which shall be validated as explained for CJrq.

The requester becomes manager of the private channel. Initially the channel is unjoined and its manager is the only admitted user.

TABLE  10-22/T.125

CCrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Higher provider |

## 10.23	CCcfXE "CCcf"§

CCcf is generated at the top MCS provider upon receipt of CCrq. Routed back to the requesting provider, it generates an MCS-CHANNEL-CONVENE confirm.

TABLE  10-23/T.125

CCcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Top provider | Confirm |
| Initiator | Top provider | MSCPDU routing |
| Channel Id (optional) | Top provider | Confirm |

CCcf contains a private channel id if and only if the result is successful.

Providers that receive a successful CCcf shall enter the channel id into their information base as a private channel with the initiating user id as its manager.

CCcf shall be forwarded in the direction of the initiating user id. If the user id is unreachable because an MCS connection no longer exists, no special actions need be taken, as a DUin must arrive later to report that the initiator has detached. Since the initiator is its manager, this will delete the channel id from the information base.

## 10.24	CDrqXE "CDrq"§

CDrq is generated by an MCS-CHANNEL-DISBAND request. If valid, it rises to the top MCS provider, which deletes the private channel id and generates CDin.

TABLE  10-24/T.125

CDrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Channel Id | Request | Top provider |

CDrq may also be generated by an MCS provider on its own initiative to disband a channel.

CDrq contains the initiating user id, which shall be validated to ensure that it is legitimately assigned to the subtree of origin. If the initiator does not equal the manager of the private channel, as recorded in the information base, the MCSPDU shall be ignored.

## 10.25	CDinXE "CDin"§

CDin is generated at the top MCS provider upon receipt of CDrq. It is multicast downward to providers that contain the manager or an admitted user in their subtree. It generates MCS-CHANNEL-EXPEL indications to admitted users with reason channel disbanded and, if provider initiated, an MCS-CHANNEL-DISBAND indication to the manager.

TABLE  10-25/T.125

CDin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Channel Id | Top provider | Indication |

CDin shall also be generated by the top MCS provider when the manager of a private channel is detached.

Providers that receive CDin shall delete the channel from their information base.

## 10.26	CArqXE "CArq"§

CArq is generated by an MCS-CHANNEL-ADMIT request. If valid, it rises to the top MCS provider, which admits the specified users to the private channel and multicasts CAin to advise providers in whose subtree they reside.

CArq contains the initiating user id, which shall be validated as explained for CDrq.

The other user ids of CArq, representing users to be admitted, shall be validated at the top MCS provider, which alone knows the entire user population. Those that are invalid shall be omitted from the resulting CAin.

TABLE  10-26/T.125

CArq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Channel Id | Request | Top provider |
| User Ids | Request | Top provider |

The user ids contained in CArq shall not be admitted to the private channel until a provider receives CAin. This maintains consistency with the top MCS provider.

## 10.27	CAinXE "CAin"§

CAin is generated at the top MCS provider upon receipt of CArq. It is multicast downward to providers that contain a newly admitted user in their subtree. It generates MCS-CHANNEL-ADMIT indications at the affected attachments.

Providers that receive CAin shall ordinarily update the channel in their information base, admitting the specified users that reside in their subtree. However, if a provider is the former top of a lower domain that is being merged as a result of MCS-CONNECT-PROVIDER, it may refuse the admission by generating DUrq for the affected user ids with reason channel purged.

TABLE  10-27/T.125

CAin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Top provider | Indication |
| Channel Id | Top provider | Indication |
| User Ids | Top provider | MCSPDU routing |

## 10.28	CErqXE "CErq"§

CErq is generated by an MCS-CHANNEL-EXPEL request. If valid, it rises to the top MCS provider, which expels the specified users from the private channel and multicasts CEin to advise providers in whose subtree they reside.

CErq contains the initiating user id, which shall be validated as explained for CDrq.

The other user ids of CErq, representing users to be expelled, shall be validated at the top MCS provider, which alone knows the entire set of admitted users. Those that were not admitted shall be omitted from the resulting CEin.

The user ids contained in CErq shall not be expelled from the private channel until a provider receives CEin. This maintains consistency with the top MCS provider.

TABLE  10-28/T.125

CErq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Channel Id | Request | Top provider |
| User Ids | Request | Top provider |

## 10.29	CEinXE "CEin"§

CEin is generated at the top MCS provider upon receipt of CErq. It is multicast downward to providers that contain an expelled user in their subtree. It generates MCS-CHANNEL-EXPEL indications at the affected attachments with reason user-requested.

TABLE  10-29/T.125

CEin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Channel Id | Top provider | Indication |
| User Ids | Top provider | MCSPDU routing |

Providers that receive CEin shall update the channel in their information base, deleting the specified users from the set of users admitted to the channel. If the set of users admitted to a private channel becomes empty and the manager does not reside in the subtree, the channel id shall be deleted from the information base. Otherwise, if the channel becomes unjoined as a result of expulsions, a provider shall generate a corresponding CLrq.

A provider forwarding CEin shall compute, for each destination subtree, whether it afterwards contains any attachments admitted to the private channel. If none, a provider shall conclude that the corresponding subordinate provider is no longer joined to the private channel and shall update its information base to this effect immediately, without waiting for CLrq.

## 10.30	SDrqXE "SDrq"§

SDrq is generated by an MCS-SEND-DATA request. If valid, it rises toward the top MCS provider. Along the way, providers may generate from it an SDin with identical contents and multicast this downward.

SDrq contains the initiating user id, which shall be validated as explained for CJrq.

If the channel id is listed in the information base of the receiving MCS provider as a private channel and the initiator of SDrq is not an admitted user, the MCSPDU shall be ignored.

The initial or additional TC that conveys SDrq shall match its data priority, taking into account the number of priorities implemented in the domain. MCSPDUs arriving over an MCS connection by the wrong TC shall be rejected.

TABLE  10-30/T.125

SDrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Higher provider |
| Channel Id | Request | Higher provider |
| Data Priority | Request | Higher provider |
| Segmentation | Requesting provider | Higher provider |
| User Data | Request | Higher provider |

The segmentation flags begin and end shall be set by a provider to show the relationship of user data in the SDrq to the boundaries of an MCS service data unit. Providers have freedom to fragment and reassemble MCSPDUs that are part of the same MCS service data unit, so long as this does not disturb the integrity of user data. However, there should be little advantage in such manipulation, as the maximum size of an MCSPDU is constant throughout a domain.

A provider shall generate from SDrq an SDin with the same content and shall transmit it to all providers that are joined to the specified channel, excepting the subordinate provider that transmitted SDrq upward. Unless the channel is listed in the provider’s information base as a user id residing in its subtree, it shall also forward SDrq upward.

## 10.31	SDinXE "SDin"§

SDin is generated at a higher MCS provider upon receipt of SDrq. It is multicast downward and generates MCS-SEND-DATA indications at all attachments that are joined to the channel.

TABLE  10-31/T.125

SDin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Higher provider | Indication |
| Channel Id | Higher provider | Indication |
| Data Priority | Higher provider | Indication |
| Segmentation | Higher provider | Indicating provider |
| User Data | Higher provider | Indication |

The initial or additional TC that conveys SDin shall match its data priority, taking into account the number of priorities implemented in a domain. MCSPDUs arriving over an MCS connection by the wrong TC shall be rejected.

The segmentation flags begin and end permit user data to be reassembled into a complete MCS service data unit. These flags shall be interpreted in the context of SDin MCSPDUs arriving from the same user over the same channel and at the same priority. A stream of fragments to be reassembled may be interleaved with other MCSPDUs and data from other users over other channels at other priorities.

It is a matter of local implementation how service data units are indicated to attached MCS users. One possibility is to deliver each MCSPDU as a separate interface data unit, with segmentation flags included. Alternative approaches that may seek to reassemble within the receiving provider should make some provision for large service data units and should reflect to the user the relative order in which service data units begin to arrive.

Providers that receive SDin shall forward it to all subordinates that are joined to the channel.

## 10.32	USrqXE "USrq"§

USrq is generated by an MCS-UNIFORM-SEND-DATA request. If valid, it rises to the top MCS provider, which generates from it a USin with identical contents and multicasts this downward.

TABLE  10-32/T.125

USrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Channel Id | Request | Top provider |
| Data Priority | Request | Top provider |
| Segmentation | Requesting provider | Top provider |
| User Data | Request | Top provider |

USrq contains the initiating user id, which shall be validated as explained for CJrq.

If the channel id is listed in the information base of the receiving MCS provider as a private channel and the initiator of USrq is not an admitted user, the MCSPDU shall be ignored.

The initial or additional TC that conveys USrq shall match its data priority, taking into account the number of priorities implemented in the domain. MCSPDUs arriving over an MCS connection by the wrong TC shall be rejected.

The segmentation flags begin and end shall be set by a provider to show the relationship of user data in the USrq to the boundaries of an MCS service data unit. Providers have freedom to fragment and reassemble MCSPDUs that are part of the same MCS service data unit, so long as this does not disturb the integrity of user data. However, there should be little advantage in such manipulation, as the maximum size of an MCSPDU is constant throughout a domain.

The top MCS provider shall generate from USrq a USin with the same content.

## 10.33	USinXE "USin"§

USin is generated at the top MCS provider upon receipt of USrq. It is multicast downward and generates MCS-UNIFORM-SEND-DATA indications at all attachments that are joined to the channel.

TABLE  10-33/T.125

USin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Top provider | Indication |
| Channel Id | Top provider | Indication |
| Data Priority | Top provider | Indication |
| Segmentation | Top provider | Indicating provider |
| User Data | Top provider | Indication |

The initial or additional TC that conveys USin shall match its data priority, taking into account the number of priorities implemented in the domain. MCSPDUs arriving over an MCS connection by the wrong TC shall be rejected.

The segmentation flags begin and end permit user data to be reassembled into a complete MCS service data unit. These flags shall be interpreted in the context of USin MCSPDUs arriving from the same user over the same channel and at the same priority. A stream of fragments to be reassembled may be interleaved with other MCSPDUs and data from other users over other channels at other priorities.

It is a matter of local implementation how service data units are indicated to attached MCS users.

Providers that receive USin shall forward it to all subordinates that are joined to the channel.

## 10.34	TGrqXE "TGrq"§

TGrq is generated by an MCS-TOKEN-GRAB request. If valid, it rises to the top MCS provider, which returns a TGcf reply.

TABLE  10-34/T.125

TGrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Token Id | Request | Top provider |

TGrq contains the initiating user id, which shall be validated as explained for CJrq.

If the token is free and the domain limit on the number of tokens in use allows, it shall become grabbed. If the token is inhibited by the requesting user only, it shall become grabbed. Otherwise, the state of the token shall not change.

## 10.35	TGcfXE "TGcf"§

TGcf is generated at the top MCS provider upon receipt of TGrq. Routed back to the requesting provider, it generates an MCS-TOKEN-GRAB confirm.

TABLE  10-35/T.125

TGcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Top provider | Confirm |
| Initiator | Top provider | MCSPDU routing |
| Token Id | Top provider | Confirm |
| Token Status | Top provider | Confirm |

The result shall be successful if the token was previously free or if the token was converted from inhibited to grabbed by the same user. Other results are too many tokens and token not available. The latter applies to a token already grabbed by the requester; this may be discerned by examining the token status.

Providers that receive TGcf shall update the token state in their information base to agree with the status returned.

TGcf shall be forwarded in the direction of the initiating user id. If the user id is unreachable because an MCS connection no longer exists, no special actions need be taken, as a DUin must arrive later to report that the initiator has detached. This will release its hold on the token id in the information base.

## 10.36	TIrqXE "TIrq"§

TIrq is generated by an MCS-TOKEN-INHIBIT request. If valid, it rises to the top MCS provider, which returns a TIcf reply.

TABLE  10-36/T.125

TIrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Token Id | Request | Top provider |

TIrq contains the initiating user id, which shall be validated as explained for CJrq.

If the token is free and the domain limit on the number of tokens in use allows, it shall become inhibited. If the token is grabbed by the requesting user, it shall be become inhibited. If the token is already inhibited, the requester shall be added to the set of inhibitors. Otherwise, the state of the token shall not change.

## 10.37	TIcfXE "TIcf"§

TIcf is generated at the top MCS provider upon receipt of TIrq. Routed back to the requesting provider, it generates an MCS-TOKEN-INHIBIT confirm.

The result shall be successful if the token was previously free or inhibited or if the token was converted from grabbed to inhibited by the same user. Other results are too many tokens and token not available.

TABLE  10-37/T.125

TIcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Top provider | Confirm |
| Initiator | Top provider | MCSPDU routing |
| Token Id | Top provider | Confirm |
| Token Status | Top provider | Confirm |

Providers that receive TIcf shall update the token state in their information base to agree with the status returned.

This MCSPDU is routed in the same way as TGcf.

## 10.38	TVrqXE "TVrq"§

TVrq is generated by an MCS-TOKEN-GIVE request. If valid, it rises to the top MCS provider, which generates either TVin or an unsuccessful TVcf.

TABLE  10-38/T.125

TVrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Token Id | Request | Top provider |
| Recipient | Request | Top provider |

TVrq contains the initiating user id, which shall be validated as explained for CJrq.

If the token is grabbed by the requester and the intended recipient exists, TVin shall be transmitted toward the recipient. Otherwise, the request shall fail, the state of the token shall be unchanged, and TVcf shall be transmitted toward the requester with the result token not possessed or no such user.

## 10.39	TVinXE "TVin"§

TVin is generated at the top MCS provider upon receipt of TVrq. Routed to the intended recipient, it generates an MCS-TOKEN-GIVE indication.

Providers that receive TVin shall ordinarily update the token id in their information base to the state of being given from initiator to recipient. However, if a provider is the former top of a lower domain that is being merged as a result of MCS-CONNECT-PROVIDER, it may refuse the offered token by generating TVrs with reason domain merging.

TABLE  10-39/T.125

TVin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Top provider | Indication |
| Token Id | Top provider | Indication |
| Recipient | Top provider | MCSPDU routing |

TVin shall be forwarded in the direction of the recipient user id. If the user id is unreachable because an MCS connection no longer exists, no special actions need be taken, as a DUin must arrive later to report that the recipient has detached. This will release its hold on the token id in the information base.

## 10.40	TVrsXE "TVrs"§

TVrs is generated by an MCS-TOKEN-GIVE response. If valid, it rises to the top MCS provider, which generates TVcf to inform the token's donor of the outcome.

TABLE  10-40/T.125

TVrs MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Response | Top provider |
| Recipien | Responding provider | Top provider |
| Token Id | Responding provider | Top provider |

A successful result shall signify the recipient’s acceptance of the offered token.

The user id of the responding MCS attachment is supplied by the MCS provider that receives the primitive response. Providers that receive TVrs subsequently shall validate the user id to ensure that it is legitimately assigned to the subtree of origin. If the user id is invalid, the MCSPDU shall be ignored.

If the token id is not listed in the provider’s information base as being given to the recipient, the MCSPDU shall be ignored. If the token id is still grabbed by the donor, its state shall be updated to grabbed by the recipient if the result is successful; otherwise it shall revert to grabbed by the donor or shall be deleted from the information base, depending on whether the donor resides in the subtree of the provider. If the token id has since been released by the donor and the result is not successful, the token shall be deleted from the provider’s information base.

If the MCSPDU is not invalid and ignored, it shall be forwarded upward. The top MCS provider shall act on TVrs as specified above. In addition, if the donor has not already released the token, the top provider shall generate TVcf containing the same result as TVrs.

## 10.41	TVcfXE "TVcf"§

TVcf is generated at the top MCS provider upon receipt of TVrs. Routed back to the requesting provider, it generates a MCS-TOKEN-GIVE confirm.

TABLE  10-41/T.125

TVcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Top provider | Confirm |
| Initiator | Top provider | MCSPDU routing |
| Token Id | Top provider | Confirm |
| Token Status | Top provider | Confirm |

TVcf shall also be generated by the top MCS provider upon receipt of TVrq if a token cannot be offered to the intended recipient. This takes the place of generating TVin. TVcf shall also be generated with result no such user if the recipient is detached before TVrs is received.

Providers that receive TVcf shall update the token state in their information base to agree with the status returned.

This MCSPDU is routed in the same way as TGcf.

## 10.42	TPrqXE "TPrq"§

TPrq is generated by an MCS-TOKEN-PLEASE request. If valid, it rises to the top MCS provider, which multicasts TPin to alert current users of the token.

TPrq contains the initiating user id, which shall be validated as explained for CJrq.

TABLE  10-42/T.125

TPrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Token Id | Request | Top provider |

## 10.43	TPinXE "TPin"§

TPin is generated at the top MCS provider upon receipt of TPrq. It is multicast downward and generates MCS-TOKEN-PLEASE indications.

Providers that receive TPin shall forward it to all subordinates that contain in their subtree a user who has grabbed, inhibited, or is being given the specified token.

TABLE  10-43/T.125

TPin MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Top provider | Indication |
| Token Id | Top provider | Indication |

## 10.44	TRrqXE "TRrq"§

TRrq is generated by an MCS-TOKEN-RELEASE request. If valid, it rises to the top MCS provider, which returns a TRcf reply.

TABLE  10-44/T.125

TRrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Token Id | Request | Top provider |

TRrq contains the initiating user id, which shall be validated as explained for CJrq.

If the token is grabbed by the requester, it shall become free. If it is inhibited, the requester shall be removed from the set of inhibitors; if this set becomes empty, the token shall become free. If the token is in the process of being given away by the requester, it shall enter a distinct intermediate state of given to the intended recipient, pending receipt of TVrs. Otherwise the state of the token shall not change.

## 10.45	TRcfXE "TRcf"§

TRcf is generated at the top MCS provider upon receipt of TRrq. Routed back to the requesting provider, it generates an MCS-TOKEN-RELEASE confirm.

TABLE  10-45/T.125

TRcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Result | Top provider | Confirm |
| Initiator | Top provider | MCSPDU routing |
| Token Id | Top provider | Confirm |
| Token Status | Top provider | Confirm |

The result shall be successful if the token was grabbed or inhibited by the requester or if the requester was in the process of giving it away. The other possible result is token not possessed.

Providers that receive TRcf shall update the token state in their information base to agree with the status returned.

This MCSPDU is routed in the same way as TGcf.

## 10.46	TTrqXE "TTrq"§

TTrq is generated by an MCS-TOKEN-TEST request. If valid, it rises to the top MCS provider, which returns a TTcf reply.

TTrq contains the initiating user id, which shall be validated as explained for CJrq.

TABLE  10-46/T.125

TTrq MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Requesting provider | Top provider |
| Token Id | Request | Top provider |

## 10.47	TTcfXE "TTcf"§

TTcf is generated at the top MCS provider upon receipt of TTrq. Routed back to the requesting provider, it generates an MCS-TOKEN-TEST confirm.

Providers that receive TTcf should find that the token state in their information base agrees with the status returned.

This MCSPDU is routed in the same way as TGcf.

TABLE  10-47/T.125

TTcf MCSPDU

| Contents | Source | Sink |
| --- | --- | --- |
| Initiator | Top provider | MCSPDU |
| Token Id | Top provider | Confirm |
| Token Status | Top provider | Confirm |

# 11	MCS provider information baseXE "MCS provider information base"§

## 11.1	Hierarchical replication

Although an MCS provider may host multiple domains, it serves each one independently. It maintains logically separate information bases for each to record the state of channel and token resources in use. The description that follows is set in the context of a single domain.

The MCS resources that need to be managed in a domain are channel ids and token ids. User ids are a subset of channel ids. Domain parameters limit how many ids of each category can be in use simultaneously. This allows a provider to compute how much memory is needed for the information base in the worst case of a fully utilized domain.

In the hierarchy of a domain, the ids in use at any given MCS provider stabilize to a subset of those in use at its immediate superior. Information about an id is recorded where it can be used to support MCS services involving that id. Recording information more widely would entail extra costs in MCSPDU traffic to keep the information up to date. Since the information recorded at a provider is consistent with that recorded at superior providers, within the limit of MCSPDU propagation delay, it may be said that the provider information base is partially replicated through the domain hierarchy.

Domain parameters become fixed and unchangeable with the establishment of the first MCS connection of a domain. A provider that lacks capacity for the maximum number of ids specified in each category may negotiate to join a domain on false pretense. It may speculate that at its low position in a hierarchy it will not be called upon to retain more than a fraction of the total information base. Such a provider may not support attachments and subordinates with the full range of MCS services they expect. Nonetheless, until its capacity is actually exceeded, such a provider may appear to be an equal member of the domain. This strategy may appeal to terminal nodes with limited aspirations.

Ids are placed into use first at the top MCS provider. They are placed into use at subordinate providers by a selective downward flow of MCSPDUs. Most are deleted from use in the same top-down manner. There are necessarily intervals during which a subordinate provider records as in use an id that its superiors do not, because the MCSPDU that deletes the id remains in transit. This is not, however, a situation that endures. Control MCSPDUs are received and processed in the order of their transmission. The consequences of processing an MCSPDU, including its creation or deletion of channel and token ids, take effect before attention shifts to the next input event.

An exception to the preceding paragraph is the deletion of static and assigned channel ids. Although placed into use by a downward flow of CJcf, these channel ids are deleted in the opposite order – from the bottom up. Specifically, they are deleted when an accumulation of MCS-CHANNEL-LEAVE requests from attachments and CLrq MCSPDUs from subordinate providers combine to leave a channel unjoined. Such transitions motivate the transmission of CLrq further upward. Thus, for these two cases, the channel ids recorded as in use are a strict subset of those in use at the superior provider. This is an accidental corollary of optimizations designed to speed channel management as a prelude to data transfer.

Channels ids are put into use by MCcf, AUcf, CJcf, CCcf, and CAin; they are deleted by MCcf, PCin, DUin, CLrq, CDin, and CEin. Token ids are put into use by MTcf, TGcf, TIcf, and TVin; they are deleted by MTcf, PTin, TRcf, TVrs and TVcf. When an id is put into use at a given provider, the MCSPDU that is the cause may be forwarded to zero, one, several, or all subordinate providers. The use of an id may grow or contract gradually as, for example, individual users are admitted to and expelled from a private channel. When an id is deleted from a given provider, the MCSPDU that effects this is forwarded to all subordinates who may still be recording the id as in use.

The use of an id is ultimately tied to actions on a channel or token by a user attached to the domain (although there may be some delay, as explained, in communicating changes through the transmission of MCSPDUs). The ids recorded stably as in use at a given MCS provider are those that are actively employed by some user in the subtree of the provider. It follows that they are a subset of those recorded stably at any superior provider.

Deleting a user id has the corollary effect of deleting channel ids and token ids of which it is the sole user in a subtree.

Criteria for considering channel ids and token ids to be in use are specified in the following subclauses.

## 11.2	Channel information

The four kinds of channel have corresponding criteria to determine whether a given attachment is considered to be using the channel id, hence whether it is to be represented in a provider’s information base:

a)	A static channel id (range 1..1000) is in use if the user has joined the channel with a successful MCS-CHANNEL-JOIN confirm and not left by MCS-CHANNEL-LEAVE request or indication.

b)	A user id channel is in use if it was assigned to the user by a successful MCS-ATTACH-USER confirm and the user has not detached by MCS-DETACH-USER request or indication.

c)	A private channel id is in use if the user has created the channel with a successful MCS-CHANNEL-CONVENE confirm or been admitted to it with MCS-CHANNEL-ADMIT indication and not expelled by MCS-CHANNEL-EXPEL indication and the channel has not been disbanded by MCS-CHANNEL-DISBAND request or indication.

d)	An assigned channel id is in use if the user has joined the channel with a successful MCS-CHANNEL-JOIN confirm and not left by MCS-CHANNEL-LEAVE request or indication.

This information shall be recorded for a channel id in use:

a)	The kind of channel it represents (static, user id, private, or assigned).

b)	By which MCS attachments and which MCS connections to subordinate providers the channel is joined.

c)	If a user id channel, the direction to it, that is, either the local MCS attachment to which the user id is assigned or the downward MCS connection to a subordinate provider in whose subtree the user resides.

d)	If a private channel id, the user id of the manager who convened it (whether or not the manager itself is in the subtree of the provider) and the set of all user ids in the subtree of the provider who have been admitted to the channel.

The information recorded for channel ids is employed as explained in clause 10 to validate request MCSPDUs and to route indication and confirm MCSPDUs.

## 11.3	Token information

The state transitions of a token id are shown in Figure 11-1.

An individual token id can be grabbed by a single user or inhibited by one or more. The action of TVin converts the state to giving along the branch of a domain hierarchy leading from the top MCS provider toward the intended recipient. This state decays to ungivable if the recipient detaches before its provider responds with TVrs. It resolves to given if instead the donor releases the token explicitly or detaches. During the giving of a token, the branch of a domain hierarchy leading from the donor intersects the branch leading toward the recipient at least at the top MCS provider. The token state changes from grabbed to giving, and possibly thereafter to ungivable or given, only along this intersection.

FIGURE 11-1/T.125...[D04] =  10 CM

The user of a token stands in relationship to it as grabber, inhibitor, recipient, or both grabber and recipient (when giving a token to itself):

a)	The user is a grabber if it has seized a token with a successful MCS-TOKEN-GRAB confirm and not released it by MCS-TOKEN-RELEASE request or successful MCS-TOKEN-GIVE confirm nor converted it with a successful MCS-TOKEN-INHIBIT confirm or if it has accepted an offered token with a successful MCS-TOKEN-GIVE response.

b)	The user is an inhibitor if it has seized a token with a successful MCS-TOKEN-INHIBIT confirm and not released it with MCS-TOKEN-RELEASE request nor converted it with a successful MCS-TOKEN-GRAB confirm.

c)	The user is a recipient if it has been offered a token by MCS-TOKEN-GIVE indication and not released it with an unsuccessful MCS-TOKEN-GIVE response.

This information shall be recorded for a token id in use:

a)	The state of the token id at the MCS provider (not necessarily identical to that at the top provider).

b)	If grabbed or ungivable, the user id of the grabber in the subtree of the provider.

c)	If giving, the user id of the grabber (whether or not the grabber is in the subtree of the provider).

d)	If giving or given, the user id of the recipient in the subtree of the provider.

e)	If inhibited, the set of all user ids in the subtree of the provider who have inhibited the token.

The information recorded for token ids in use is employed as explained in clause 10 to validate response MCSPDUs and to route indication MCSPDUs.

The state of a token id at a subordinate provider need not be identical to that at the top MCS provider. This is due to the fact that a token donor does not in general process TVin or TVrs and that a recipient does not in general process a donor’s TRcf. Figure 11-2 shows states that may arise in a complex token interaction.

FIGURE 11-2/T.125...[D05] = 12 CM

The figures focuses on one token id in the information base of providers A through H. A plausible history is that the token was given by user A, attached to provider A, to user B, attached to provider B. Before user B could respond, however, provider G connected the domain to a new top provider H and began a merger. The new domain, unable to accept user B because of a conflict in channel ids, initiated its purge through PCin. This MCSPDU is shown having completed part of its journey through providers G and D. Provider G, as a result, has adjusted its token state from giving to ungivable and merged the token, with this state, into the new domain. The new top provider H, presented with a token in such a state, has queued for transmission an unsuccessful TVcf to return the token to user A. At the time shown, the token has also finally been accepted by user B, and a TVrs is making its way up the recipient’s branch of the domain hierarchy, changing the token state at each provider from giving to grabbed by B. Two MCSPDUs now converge on provider F, and whichever arrives first determines its subsequent state there: either returned to grabbed by A or transiently grabbed by B and then not in use again after user B is detached. In either case, the state of the token will stabilize to grabbed by A when the new top provider transmits the TVcf it has pending.

# 12	Elements of procedure

## 12.1	MCSPDU sequencingXE "MCSPDU sequencing"§

Control MCSPDUs remain in sequence between any pair of MCS providers because they travel over a single TC, the initial TC of an MCS connection. MCS providers shall process received MCSPDUs and shall transmit any resulting output MCSPDUs in the same order. This applies to MCSPDUs that are simply forwarded up or down the domain

hierarchy as well as to MCSPDUs that are transformed, such as requests or responses into indications or confirms. The sequencing of MCSPDUs shall be maintained within an MCS provider if it is necessary to queue outputs for later transmission due to the back-pressure of flow control along a TC.

Data MCSPDUs of different priorities need not remain in sequence. On the contrary, the benefit of relative priorities is only achieved when higher priority data is advanced ahead of lower priority data. This means that it should be transmitted over separate TCs and queued separately within each MCS provider. If the number of data priorities implemented in a domain is less than the maximum, fewer TCs are available, but providers that elect to do so may still maintain separate queues internally. This realizes some but not all the benefits of relative priority.

An MCS provider shall maintain the sequence of data MCSPDUs transmitted at a given priority. This is a tighter restriction than that imposed by ITU-T Recommendation T.122, which guarantees only the sequencing of service data units transmitted at a given priority over the same destination channel.

Control MCSPDUs and data MCSPDUs of top priority are transmitted over the same initial TC and shall receive equal attention by an MCS provider. Data of lower priorities may lag behind. Control indications advancing ahead may arrive before lower priority data that was actually transmitted first.

## 12.2	Input flow control

An MCS provider has goals that sometimes conflict: to keep data moving briskly through a domain despite transient blockages towards some receivers, to give transmitters fair access to the available bandwidth, and to prevent any party from lagging far behind peers who are receiving the same multicast data. MCS is a reliable service that preserves the integrity of user data. Since a provider has limited capacity to store MCSPDUs when they cannot be transmitted immediately, it must have a defense of sometimes refusing further inputs. While details of the interface to transport services are a local matter, the abstract effect must be that incoming TSDUs are held in TC pipelines, intact and in sequence, to be received at some future time when flow control is lifted. As TC pipelines fill, remote MCS providers may find that they are blocked by back-pressure from transmitting further MCSPDUs and may need to invoke a similar defense.

Flow control is not signalled explicitly in the MCS protocol. This is a function of lower layers that would be wasteful to duplicate. As a result, it is difficult to sense, through the medium of an intervening TC, whether a remote provider is resisting further inputs. To meet conflicting goals as well as possible, the policies described next may be recommended.

An MCS provider may grant each incoming TC a fixed quota of buffers that it may fill with MCSPDUs before back pressure is applied. Each buffer is processed as specified in this protocol, then assigned for output to zero or more outgoing TCs. Output may occur immediately or may be delayed because a transport pipeline is full. Until it is output to the last TC, a buffer is charged against the input quota of the TC by which it arrived. After it is output to all requisite TCs, it is recycled as an increment to that quota. When a quota is exhausted, further inputs over the corresponding TC are halted. Input quotas may be set taking into account whether a TC is part of an upward or downward MCS connection and what data priority it represents.

Buffering can mitigate a disparity of rates between transmitters and receivers. A quota on inputs can prevent any one TC from monopolizing resources. It can also bound how far out of step two receivers of the same multicast data can get. The recommended scheme is not clever enough, however, to anticipate all patterns of use and may sometimes slow the rate of data transfer through a domain when there are acceptable alternatives. The invention of better flow control policies (implemented locally and requiring no additional communication of MCSPDUs) may be a means of product differentiation.

## 12.3	Throughput enforcementXE "Throughput enforcement"§

In contrast to input flow control, some support for throughput enforcement is explicit in the MCS protocol. Firstly, the rate that is enforced is a domain parameter negotiated through MCS-CONNECT-PROVIDER. Secondly, the time interval over which throughput is monitored before taking adverse action is communicated by each MCS provider to its superior through EDrq. To describe the throughput enforcement interval requires that providers share some common principles of behaviour. Still, enforcement remains a heuristic technique with room for invention.

Enforcing a minimum input rate at each receiver is an option available to controller applications, which establish the MCS connections of a domain. This option is selected through the domain parameter for enforced throughput, which is stated in octets per second. While it seems intuitive that a party should not be allowed to run arbitrarily slowly and thereby obstruct data transfer among others, there is danger in seeking to enforce throughput too strictly.

Complex patterns of multiple transmitters are a problem. The kinds of back-pressure to which an enforcement policy reacts may not result from a single abnormally slow receiver. In the first place, MCSPDUs heading downward must compete for attention with MCSPDUs that rise from below but are reflected back, notably SDin. The downward flow experienced through a peer provider may therefore attain only a fraction of the nominal bandwidth and may vary dynamically with the number of other connections and attachments to the peer. Secondly, only top priority MCSPDUs can be expected to flow continuously over an MCS connection. Lesser priorities can properly be blocked by a peer provider for long periods of time due to the intensity of traffic received from other sources at higher priorities. Finally, there can be great variation in instantaneous throughput if it is measured by how long a blocked MCSPDU must wait before being accepted onto an MCS connection. Among other things, this can depend on how many and in what order MCSPDUs from other sources are queued inside the peer provider.

Nonetheless, throughput enforcement is a valuable option in practical situations where patterns of data transfer are known to be more uniform. It shall be interpreted as requiring a minimal output of MCSPDUs to each direct MCS attachment and to each MCS connection downward over a time interval to be specified by the enforcing MCS provider. Each MCSPDU output, both control and data, shall count towards throughput as though it were the maximum size allowed by domain parameters. The output of an MCSPDU to an MCS attachment shall mean delivery of the associated primitive indication or confirm. Output to a downward MCS connection shall mean the absence of back-pressure at the transport service interface and acceptance into the corresponding TC pipeline.

Output shall be monitored as long as one or more MCSPDUs are queued, regardless of data priority, towards a given attachment or downward connection. Whenever the queues empty, monitoring shall cease, with no enforcement action taken. At the same time, no credit for good behaviour shall be recorded to offset future slowdowns. Monitoring shall resume as soon as back-pressure prevents any MCSPDU from being output and requires it to be queued instead. While one or more MCSPDUs remain queued, the number actually output shall be counted over a set interval of time.

A throughput enforcement interval shall be chosen by each MCS provider. The interval shall be long enough to allow at least one MCSPDU of maximum size to be output at the minimum throughput. An MCS provider shall advise its superior of the interval chosen and of any subsequent changes to it by transmitting EDrq upward. A provider shall act against the offending MCS attachment or downward MCS connection at the end of any interval during which the monitored throughput fails to meet expectations. It shall detach the user or disconnect the connection.

Superior providers may set their throughput enforcement interval to be longer than that of any subordinate plus some margin of reaction time. The aim is to encourage enforcement action first at the lowest provider in a position to detect a problem. When a violator is removed at the end of some lower interval, superior providers should have enough time to sense the restoration of throughput to adequate levels. If they act too quickly, they may penalize a larger subtree than necessary and disrupt innocents in the domain.

The controller applications that set domain parameters should be conservative in their demands, expecting that throughput may occasionally dip during periods of intense application stimuli. If their concern is simply to defend

against receivers that stop accepting anything at all, they may set the minimum throughput very low. Knowing the maximum MCSPDU size and enforced throughput rate, controllers can calculate the minimum interval that must elapse before any blockage is detected and overcome.

## 12.4	Domain configurationXE "Domain configuration"§

ITU-T Recommendation T.122 provides no mechanism for configuring the set of domains supported by an MCS provider. This must be considered a local matter whose standardization may be the subject of further study. This protocol assumes that an MCS provider will recognize some domain selectors as valid and others as invalid. It provides for domain selectors to be communicated as part of establishing an MCS connection.

An MCS provider participates implicitly in the negotiation of domain parameters. Whether the calling or the called side, it constrains the range of allowed parameter values according to limits for which it is configured. An MCS provider shall freeze the negotiability of domain parameters once any user has attached to a domain or once the first MCS connection has been established.

## 12.5	Domain mergerXE "Domain merger"§

Domains are merged as a consequence of MCS-CONNECT-PROVIDER. If it is convenient to arrange that one domain or the other be empty at this point, there is little complication in a merger. In the most general case, however, provision must be made for updating the information base at the remaining top provider to contain the information base of the former top provider and for resolving any conflicts that become apparent. The details of this are explained in clause 10.

To aid understanding, an example of domain merger is illustrated in the sequence of Figures 12-1 through 12-4. Here provider E represents a former top that has joined a new domain by the highlighted MCS connection to an intermediate provider F. It is irrelevant whether provider E or provider F initiated the MCS connection.

FIGURE 12-1/T.125...[D06] = 10 CM

Provider E, finding itself at the lower end of a connection upward, assumes responsibility for executing the merger. Since it is unsafe to transact other activity while the information base is in flux, provider E stops accepting inputs from its subtree. Any MCSPDUs already in transit from providers A and B or new ones generated before the merger is completed will be safeguarded and processed later. The downward flow of MCSPDUs, however, is not impeded, whether generated by provider E or forwarded from further above.

Until the merger completes, the only confirm MCSPDUs provider E will receive are MCcf and MTcf, since user requests are not allowed upward. These will confirm or purge channel and token ids already in use. Indication MCSPDUs PCin, PTin, DUin, and CDin of the upper domain may also arrive, deleting channel and token ids from the lower domain whose merger has been individually confirmed. Unconfirmed ids of the lower domain are protected against deletion, because as yet they mean something different from the same id of the upper domain. PDin must be obeyed to enforce the domain height limit. Remaining indications need not be applied by provider E while merger is in progress. Data transfers, in particular, need not flow between the formerly separate domains until merger is completed; they cannot flow until at least the channels conveying them have been sorted out. Two indications that can place new ids into use may be inconvenient to apply. To keep the information base consistent, if provider E refuses CAin or TVin, it will react as specified in clause 10.

The first actions of provider E are to send PDin downward, to ensure that the latest MCS connection has not created a cycle that would invalidate the principle that each domain hierarchy have exactly one top provider, and to send EDrq upward to report its existing height and throughput enforcement interval. Provider F, advancing thereby from height 2 to 3, passes EDrq along to the top provider, which then attains height 4.

In the second stage of domain merger, provider E sends upward as many instances of MCrq as it takes to contain the user ids in its information base. User ids that do not conflict with the upper domain are confirmed and the rest are purged in equally many instances of MCcf. Provider E generates PCin from MCcf to report any purges throughout its subtree. This stage ends when all user ids have either been explicitly confirmed or purged.

FIGURE 12-2/T.125...[D07] = 9.5 CM

Stage three resembles stage two, but concerns token ids rather than user id channels, using a parallel set of MCSPDUs. If user ids had not been merged first, portions of a later MTrq could be refused as invalid and the affected token ids would be unnecessarily purged. In the case of an inhibited token, the entire set of inhibiting users may not fit into a single MCSPDU. Provider E waits for confirmation of the first subset it sends upward before sending the inhibited token again with the remaining user ids. This protects against the first set being refused due to too many token ids in use but the remainder later being accepted, which would corrupt the information base. This stage ends when all token ids have either been explicitly confirmed or purged.

FIGURE 12-3/T.125...[D08] =  9.5  CM

Stage four involves the same MCSPDUs as stage two, but they contain different channel ids. User id channels having been taken care of, what remains are static, private, and assigned channel ids. If user ids had not been merged first, portions of a later MCrq could be refused as invalid and the affected channel ids would be unnecessarily purged. In the case of a private channel, the entire set of users admitted by the channel manager may not fit into a single MCSPDU. Provider E waits for confirmation of the first subset it sends upward before sending the private channel again with the remaining user ids. This protects against the first set being refused due to too many channel ids in use but the remainder later being accepted, which would corrupt the information base. This stage ends and domain merger completes when all remaining channel ids have either been explicitly confirmed or purged.

NOTE – Merging token ids first makes their exclusive possession more effective in suppressing data flow conflicts. Otherwise, data may leak between domains, as channel ids are confirmed, before the conflict is revealed through a token purge.

## 12.6	Domain disconnectionXE "Domain disconnection"§

When an upward MCS connection is disconnected, an MCS provider shall eradicate its subtree of the domain by detaching all its direct MCS attachments and disconnecting all its other MCS connections. The affected provider cannot in general establish a residual domain in its own subtree, because it has no record of request MCSPDUs that were sent upward for which it will never receive a matching confirm MCSPDU.

When a downward MCS connection is disconnected, an MCS provider shall generate DUrq MCSPDUs for all users residing in that portion of its subtree, giving as reason domain disconnected.

FIGURE 12-4/T.125...[D09] = 10 CM

## 12.7	Channel id allocationXE "Channel id allocation"§

Channel ids in the range of 1001 and above are allocated dynamically at the top MCS provider during the processing of primitive requests MCS-ATTACH-USER, MCS-CHANNEL-JOIN zero, and MCS-CHANNEL-CONVENE. There is no requirement that the values allocated fit any particular pattern. Rather, it is desirable that the values be randomly dispersed over the allowed range. This makes it more likely that two domains that operate independently for a substantial period of time may later be merged without conflicts in their respective allocation of channel ids. It also defends against recycling ids too quickly within a single domain as they are released from one use and then reallocated for a different use. Applications using MCS should have time to adjust to the disappearance of a channel or user id before it returns in a new incarnation.

In situations where the seamless merging of active domains is required, conflicts may be avoided by selecting channel ids from a subrange unique to each provider. Subranges may be created by dividing the dynamically allocated channel ids 1001..65535 into several bands. Furthermore, within a subrange, ids may be allocated sequentially in one direction, from top or bottom. Providers that employ subrange allocation shall still obey all aspects of this protocol, including the procedure for domain merger. They shall thereby accommodate peer providers that may lack unique subranges.

NOTE – It is a matter of local implementation how a provider chooses a subrange from which to allocate. Pre-arranged conferences may be banded by a network management system.

Token ids, unlike channel ids, are not allocated, and the dividing line of value 1001 has no significance for them. A free token id with a given value is statically available to be grabbed or inhibited at any time, subject only to a domain limit on the total number in use at once.

## 12.8	Token statusXE "Token status"§

Token status is defined formally in clause 7. It is returned as a component of token confirm MCSPDUs and is used to update the record for a token id in the information base of subordinate providers. Token status is not necessarily reported directly through a confirm primitive to the initiating user but may be reported indirectly through a result value.

When more that one token status value describes the relationship of a given user to a specified token id, the order of preference shall be as follows. Self-recipient shall be reported first, if it fits, to remind the user that they must respond to an MCS-TOKEN-GIVE indication. Next preferred is self-giving, to remind the user that they are engaged in an uncompleted operation, followed by self-grabbed or self-inhibited. Last in order are the remaining status values, reflecting the token’s current state as a result of its sole use by other parties.

An MCS provider relies on the specified preference for token status values in order to update the token state correctly in its information base.

# 13	Reference implementation

Appendices II through VII contain an implementation of an MCS provider expressed in SDL. They demonstrate that the protocol specified can be realized with reasonable effort. The provision of this example should save implementors the work of reinventing equivalent logic and should speed the introduction of compatible systems.

SDL is a formal description technique more powerful than conventional state tables and better suited to the complexity of MCS. It admits two equivalent representations, textual and graphical. The reference implementation is cast in the former, which is more concise and easier to translate to a conventional programming language. It makes extensive use of abstract data types like the powerset generator. Those unfamiliar with SDL may wish to consult these references:

–	CCITT Recommendation Z.100 (1988), Specification and Description Language (SDL).

–	Belina, Hogrefe, and Sarma: SDL with Applications from Protocol Specification, (Prentice Hall, 1991), ISBN 0-13-785890-6.

–	Belina and Hogrefe: The CCITT Specification and Description Language SDL, Computer Networks and ISDN Systems, 16 (1988/89), pages 311-341.

Appendix II begins with a figure that summarizes relationships codified in the system and block definitions that follow. Processes appearing in the figure – control, domain, endpoint, and attachment – are defined individually in subsequent Appendices III through VI. Appendix VII explains assumptions of the model and illustrates key signal flows.

The appendices have been reviewed by technical experts and are believed to constitute a correct implementation of the MCS protocol defined in this specification, but they are not normative. In case of any discrepancy, descriptions contained in the body of this specification take precedence.

The MCS protocol may also be realized by other designs, which need not be based on the reference implementation.

The reference implementation of Appendices II through VII is parameterized to expand to a fully general MCS provider. Alternative implementations that are limited to a special case may be more compact and efficient. Of particular interest is the case of an MCS provider limited to one MCS connection, a condition appropriate for a terminal node. This might be further specialized to simplify mergers by voluntarily purging any ids employed in a lower domain. The selection of important special cases and their implementation in SDL remains a topic for further study.

Appendix I
Alternative encodings of an MCSPDUXE "Alternative encodings of an MCSPDU"§

(This appendix does not form an integral part of this Recommendation.)

## I.1	Send Data Request

The relevant definitions, extracted from clause 7, are:

DomainMCSPDU	::=  CHOICE

{

. . .

sdrq		SDrq,

. . .

}

SDrq  ::=  [APPLICATION 25] IMPLICIT SEQUENCE

{

initiator				UserId,

channelId			ChannelId,

dataPriority			DataPriority,

segmentation			Segmentation,

userData				OCTET STRING

}

UserId			::=  DynamicChannelId

DynamicChannelId	::=  ChannelId (1001..65535)

ChannelId		::=  INTEGER (0..65535)

DataPriority		::=  ENUMERATED

{

top			(0),

high		(1),

medium		(2),

low			(3)

}

Segmentation	::=  BIT STRING

{

begin		(0),

end			(1)

} (SIZE (2))

A sample value of this type is:

sdrq

{

initiator				1701,

channelId			5,

dataPriority		high,

segmentation		{begin},
		userData				'4D4353'H

}

## I.2	Basic Encoding Rules (BER)

BER applies a scheme of identifier-length-contents recursively to component types:

SDrq	Length	Contents

79		13

INTEGER		Length		Contents

02			02		06  A5

INTEGER		Length		Contents

02			01		05

ENUMERATED	Length		Contents

0A			01		01

BIT STRING		Length		Contents

03			02		06  80

OCTET STRING	Length		Contents

04			03		4D  43  53

Header octets exclusive of user data: 18-24, depending on MCSPDU tag, channel id, and user data length.

## I.3	Packed Encoding Rules (PER)

PER can be decoded only if it is known in advance what ASN.1 type is contained in the encoding, since tags are not conveyed. This is one reason for defining a combined type of domain MCSPDUs. The application tags of domain MCSPDUs advance sequentially beginning from zero. This is convenient, because PER encodes values as offsets from the base of their range. With this arrangement, the value 25 identifies SDrq under both BER and PER:

CHOICE							--  6 bits  +  pad

64

INTEGER (1001..65535)				--  offset 1001

02  BC

INTEGER (0..65535)				--  offset 0

00  05

ENUMERATED + BIT STRING (SIZE (2))		--  2 bits  +  2 bits  +  pad

60

CTET STRING					--  length  +  contents

03  4D  43  53

Header octets exclusive of user data: 7-8, depending on user data length.

PER encodes an unconstrained octet string length up to 127 in one octet and up to 16 384 in two octets.

Appendix  II
SDL decomposition of an MCS providerXE "SDL decomposition of an MCS provider"§

(This appendix does not form an integral part of this Recommendation.)

SYSTEM  MCS;

SYNONYM oneSecond			Duration = 1000;		/* time is in milliseconds */

/* Type definitions */

SYNTYPE	ChannelId			=  Integer CONSTANTS 0:65535

ENDSYNTYPE;

NEWTYPE	UserId				INHERITS ChannelId

OPERATORS ALL;

ADDING

OPERATORS

UserId:		ChannelId		–> UserId;	/* type cast */

ChannelId:	UserId		–> ChannelId;	/* type cast */

AXIOMS

UserId(0) = 0;

FOR ALL c in ChannelId

(
			ChannelId(UserId(c))  =  c;

);

ENDNEWTYPE;

SYNTYPE		TokenId		=  Integer CONSTANTS 1:65535

ENDSYNTYPE;

NEWTYPE	ChannelIdSet	SetOf(ChannelId);

ENDNEWTYPE;

NEWTYPE	UserIdSet		SetOf(UserId);

ENDNEWTYPE;

NEWTYPE	TokenIdSet	SetOf(TokenId);

ENDNEWTYPE;

NEWTYPE	TokenStatus

LITERALS
	NotInUse,
	SelfGrabbed,
	OtherGrabbed,
	SelfInhibited,
	OtherInhibited,
	SelfRecipient,
	SelfGiving,
	OtherGiving;
ENDNEWTYPE;

SYNTYPE	DataPriority	=  Integer CONSTANTS 0:3

ENDSYNTYPE;

NEWTYPE	Segmentation
STRUCT
	begin				Boolean;
	end					Boolean;

ENDNEWTYPE;

NEWTYPE		DomainParameters
STRUCT
	maxChannelIds		Natural;
	maxUserIds			Natural;
	maxTokenIds			Natural;
	numPriorities			Natural;
	minThroughput		Natural;
	maxHeight			Natural;
	maxMCSPDUsize	Natural;
	protocolVersion	Natural;
ENDNEWTYPE;

SYNTYPE		DomainSelector	=  OctetString

ENDSYNTYPE;

SYNTYPE		TSAPAddress		=  OctetString

ENDSYNTYPE;

NEWTYPE		TransportQOS		/* quality of service */

STRUCT

throughput		Natural;			/*octets per second */

transitDelay		Duration;		/* one-way */

dataPriority		Natural;			/* 0 is highest */

ENDNEWTYPE;

NEWTYPE	TransportQOSByPri	Array(DataPriority, TransportQOS);

ENDNEWTYPE;

SYNTYPE	UserData		=  OctetString

ENDSYNTYPE;

SYNTYPE	TSDU			=  OctetString

ENDSYNTYPE;

SYNTYPE	Octet			=  Integer  CONSTANTS  0:255

ENDSYNTYPE;

NEWTYPE	OctetString	String(Octet, NullString);

ENDNEWTYPE;

GENERATOR	SetOf (TYPE ItemType)	/* subsets with choice operator */

/*AS*/

Powerset(ItemType);

ADDING

OPERATORS

Pick:	SetOf		–> ItemType;	/* chooses any element */

AXIOMS

Pick(Empty)  ==  ERROR!;

FOR ALL s IN SetOf

(

s /= Empty ==> Pick(s) in s;
	);
DEFAULT
	Empty;
ENDGENERATOR;

NEWTYPE	Reason

LITERALS
	RN_domain_disconnected,
	RN_provider_initiated,
	RN_token_purged,
	RN_user_requested,
	RN_channel_purged,
	RN_channel_disbanded,	/* not in MCSPDUs */
	RN_domain_not_hierarchical,	/* not in MCSPDUs */
	RN_parameters_unacceptable,	/* not in MCSPDUs */
	RN_unspecified;		/* not in MCSPDUs */
ENDNEWTYPE;

NEWTYPE	Result

LITERALS

RT_successful,
	RT_domain_merging,
	RT_domain_not_hierarchical,
	RT_no_such_channel,
	RT_no_such_domain,
	RT_no_such_user,
	RT_not_admitted,
	RT_other_user_id,
	RT_parameters_unacceptable,
	RT_token_not_available,
	RT_token_not_possessed,
	RT_too_many_channels,
	RT_too_many_tokens,
	RT_too_many_users,

RT_unspecified_failure,
	RT_user_rejected,
	RT_congested,			/* not in MCSPDUs */
	RT_domain_disconnected;	/* not in MCSPDUs */
ENDNEWTYPE;

/* The next three identifier types distinguish separate instances

of communication across the interface between an MCS provider and
	its environment.  MCSConnectionId maps to some resource within the
	Control process.  MCSAttachmentId, which equals the process id of
	an Attachment process, could be considered implicit, since it is
	the source or destination address of corresponding signals, but
	the usage is clearer when it is made an explicit signal parameter.
	The same model is assumed for TCEndpointId. */

SYNTYPE		MCSConnectionId  =  Natural

ENDSYNTYPE;

SYNTYPE		MCSAttachmentId =  PId

ENDSYNTYPE;

SYNTYPE		TCEndpointId   =  PId

ENDSYNTYPE;

/* Block decomposition */

BLOCK	MCS.provider		REFERENCED;

CHANNEL Control.MCSAP

FROM ENV TO MCS.provider WITH

(Controller.to.Control);

FROM MCS.provider TO ENV WITH

(Control.to.Controller);

ENDCHANNEL;

SIGNALLIST Controller.to.Control  =

MCS.Connect.Provider.request,

MCS.Connect.Provider.response,

MCS.Disconnect.Provider.request;

SIGNALLIST Control.to.Controller  =

MCS.Connect.Provider.indication,

MCS.Connect.Provider.confirm,

MCS.Disconnect.Provider.indication;

SIGNAL	MCS.Connect.Provider.request

(

Natural,			/* requester's label */

TSAPAddress,			/* calling */

DomainSelector,

TSAPAddress,			/* called */

DomainSelector,

Boolean,			/* upward */

DomainParameters 		/* target */

DomainParameters,		/* minimum */

DomainParameters,		/* maximum */

TransportQOSByPri,		/* target */

TransportQOSByPri,		/* minimum */

UserData

);

SIGNAL	MCS.Connect.Provider.indication

(

MCSConnectionId,		/* provider-assigned */

TSAPAddress,			/* calling */

DomainSelector,

TSAPAddress,			/* called */

DomainSelector,

Boolean,			/* upward */

DomainParameters,		/* target */

DomainParameters,		/* minimum */

DomainParameters,		/* maximum */

UserData

);

SIGNAL	MCS.Connect.Provider.response

(

MCSConnectionId,

Result,

DomainParameters,

UserData

);

SIGNAL	MCS.Connect.Provider.confirm

(

Natural,			/* requester's label */

Result,

MCSConnectionId,		/* provider-assigned */

DomainParameters,

UserData

);

SIGNAL	MCS.Disconnect.Provider.request

(

MCSConnectionId

);
SIGNAL	MCS.Disconnect.Provider.indication

(

MCSConnectionId,

Reason

);

CHANNEL MCSAPs

FROM ENV TO MCS.provider WITH

MCS.Attach.User.request,

(Users.to.Attachment);

FROM MCS.provider TO ENV WITH

(Attachment.to.Users);

ENDCHANNEL;

SIGNALLIST Users.to.Attachment  =

MCS.ready,

MCS.Detach.User.request,

MCS.Channel.Join.request,

MCS.Channel.Leave.request,

MCS.Channel.Convene.request,

MCS.Channel.Disband.request,

MCS.Channel.Admit.request,

MCS.Channel.Expel.request,

MCS.Send.Data.request,

MCS.Uniform.Send.Data.request,

MCS.Token.Grab.request,

MCS.Token.Inhibit.request,

MCS.Token.Give.request,

MCS.Token.Give.response,

MCS.Token.Please.request,

MCS.Token.Release.request,

MCS.Token.Test.request;

SIGNALLIST Attachment.to.Users  =

MCS.ready,

MCS.Attach.User.confirm,

MCS.Detach.User.indication,

MCS.Channel.Join.confirm,

MCS.Channel.Leave.indication,

MCS.Channel.Convene.confirm,

MCS.Channel.Disband.indication,

MCS.Channel.Admit.indication,

MCS.Channel.Expel.indication,

MCS.Send.Data.indication,

MCS.Uniform.Send.Data.indication,

MCS.Token.Grab.confirm,

MCS.Token.Inhibit.confirm,

MCS.Token.Give.indication,

MCS.Token.Give.confirm,

MCS.Token.Please.indication,

MCS.Token.Release.confirm,

MCS.Token.Test.confirm;

SIGNAL	MCS.ready					/* allows one MCS.[Uniform.]Send.Data */

(

MCSAttachmentId,

DataPriority

);

SIGNAL	MCS.Attach.User.request

(

Natural,			/* requester's label */

DomainSelector

);

SIGNAL	MCS.Attach.User.confirm

(

Natural,			/* requester's label */

Result,

MCSAttachmentId,		/* provider-assigned */

UserId

);

SIGNAL	MCS.Detach.User.request

(

MCSAttachmentId

);

SIGNAL	MCS.Detach.User.indication

(

MCSAttachmentId,

UserId,

Reason

);

SIGNAL	MCS.Channel.Join.request

(

MCSAttachmentId,

ChannelId

);

SIGNAL	MCS.Channel.Join.confirm

(

MCSAttachmentId,

ChannelId,			/* requested */

Result,

ChannelId

);

SIGNAL	MCS.Channel.Leave.request

(

MCSAttachmentId,

ChannelId

);

SIGNAL	MCS.Channel.Leave.indication

(

MCSAttachmentId,

ChannelId,

Reason

);

SIGNAL	MCS.Channel.Convene.request

(

MCSAttachmentId

);

SIGNAL	MCS.Channel.Convene.confirm

(

MCSAttachmentId,

Result,

ChannelId

);

SIGNAL	MCS.Channel.Disband.request

(

MCSAttachmentId,

ChannelId

);

SIGNAL	MCS.Channel.Disband.indication

(

MCSAttachmentId,

ChannelId,

Reason

);

SIGNAL	MCS.Channel.Admit.request

(

MCSAttachmentId,

ChannelId,

UserIdSet

);

SIGNAL	MCS.Channel.Admit.indication

(

MCSAttachmentId,

ChannelId,

UserId

);

SIGNAL	MCS.Channel.Expel.request

(

MCSAttachmentId,

ChannelId,

UserIdSet

);

SIGNAL	MCS.Channel.Expel.indication

(

MCSAttachmentId,

ChannelId,

Reason

);

SIGNAL	MCS.Send.Data.request

(

MCSAttachmentId,

ChannelId,

DataPriority,

Segmentation,

UserData

);

SIGNAL	MCS.Send.Data.indication

(

MCSAttachmentId,

ChannelId,

DataPriority,

UserId,

Segmentation,

UserData

);

SIGNAL	MCS.Uniform.Send.Data.request

(

MCSAttachmentId,

ChannelId,

DataPriority,

Segmentation,

UserData

);

SIGNAL	MCS.Uniform.Send.Data.indication

(

MCSAttachmentId,

ChannelId,

DataPriority,

UserId,

Segmentation,

UserData

);

SIGNAL	MCS.Token.Grab.request

(

MCSAttachmentId,

TokenId

);

SIGNAL	MCS.Token.Grab.confirm

(

MCSAttachmentId,

TokenId,

Result

);

SIGNAL	MCS.Token.Inhibit.request

(

MCSAttachmentId,

TokenId

);

SIGNAL	MCS.Token.Inhibit.confirm

(

MCSAttachmentId,

TokenId,

Result

);

SIGNAL	MCS.Token.Give.request

(

MCSAttachmentId,

TokenId,

UserId

);

SIGNAL	MCS.Token.Give.indication

(

MCSAttachmentId,

TokenId,

UserId

);

SIGNAL	MCS.Token.Give.response

(

MCSAttachmentId,

TokenId,

Result

);

SIGNAL	MCS.Token.Give.confirm

(

MCSAttachmentId,

TokenId,

Result

);

SIGNAL	MCS.Token.Please.request

(

MCSAttachmentId,

TokenId

);

SIGNAL	MCS.Token.Please.indication

(

MCSAttachmentId,

TokenId,

UserId

);

SIGNAL	MCS.Token.Release.request

(

MCSAttachmentId,

TokenId

);

SIGNAL	MCS.Token.Release.confirm

(

MCSAttachmentId,

TokenId,

Result

);

SIGNAL	MCS.Token.Test.request

(

MCSAttachmentId,

TokenId

);

SIGNAL	MCS.Token.Test.confirm

(

MCSAttachmentId,

TokenId,

TokenStatus

);

CHANNEL TSAPs

FROM MCS.provider TO ENV WITH

(Endpoint.to.Transport);

FROM ENV TO MCS.provider WITH

T.Connect.indication,

(Transport.to.Endpoint);

ENDCHANNEL;

SIGNALLIST Endpoint.to.Transport  =

T.ready,

T.Connect.request,

T.Connect.response,

T.Data.request,

T.Disconnect.request;

SIGNALLIST Transport.to.Endpoint  =

T.ready,

T.Connect.confirm,

T.Data.indication,

T.Disconnect.indication;

SIGNAL	T.ready						/* allows one T.Data */

(

TCEndpointId

);

SIGNAL	T.Connect.request

(

Natural,			/* requester's label */

TSAPAddress,			/* calling */

TSAPAddress,			/* called */

TransportQOS,			/* target */

TransportQOS			/* minimum */

);

SIGNAL	T.Connect.indication

(

TCEndpointId,			/* provider-assigned */

TSAPAddress,			/* calling */

TSAPAddress,			/* called */

TransportQOS,			/* offered */

TransportQOS			/* minimum */

);

SIGNAL	T.Connect.response

(

TCEndpointId,

TransportQOS			/* selected */

);

SIGNAL	T.Connect.confirm

(

Natural,			/* requester's label */

TCEndpointId,			/* provider-assigned */

TransportQOS			/* selected */

);

SIGNAL  T.Data.request

(

TCEndpointId,

TSDU

);

SIGNAL	T.Data.indication

(

TCEndpointId,

TSDU

);

SIGNAL	T.Disconnect.request

(

TCEndpointId

);

SIGNAL	T.Disconnect.indication

(

Natural,			/* requester's label */

TCEndpointId			/* provider-assigned */

);

ENDSYSTEM;

BLOCK	MCS.provider;

SYNONYM maxPortalIds		Natural  =  EXTERNAL;		/* an implementation limit */

/* Data type definitions */

NEWTYPE		PDUKind		/* domain MCSPDUs */

LITERALS

PDin,			/* plumb domain indication */

EDrq,			/* erect domain request */

MCrq,			/* merge channels request */

MCcf,			/* merge channels confirm */

PCin,			/* purge channels indication */

MTrq,			/* merge tokens request */

MTcf,			/* merge tokens confirm */

PTin,			/* purge tokens indication */

DPum,			/* disconnect provider ultimatum */

RJum,			/* reject MCSPDU ultimatum */

AUrq,			/* attach user request */

AUcf,			/* attach user confirm */

DUrq,			/* detach user request */

DUin,			/* detach user confirm */

CJrq,			/* channel join request */

CJcf,			/* channel join confirm */

CLrq,			/* channel leave request */

CCrq,			/* channel convene request */

CCcf,			/* channel convene confirm */

CDrq,			/* channel disband request */

CDin,			/* channel disband confirm */

CArq,			/* channel admit request */

CAin,			/* channel admit confirm */

CErq,			/* channel expel request */

CEin,			/* channel expel confirm */

SDrq,			/* send data request */

SDin,			/* send data indication */

USrq,			/* uniform send data request */

USin,			/* uniform send data indication */

TGrq,			/* token grab request */

TGcf,			/* token grab confirm */

TIrq,				/* token inhibit request */

TIcf,				/* token inhibit confirm */

TVrq,			/* token give request */

TVin,			/* token give indication */

TVrs,			/* token give response */

TVcf,			/* token give confirm */

TPrq,			/* token please request */

TPin,			/* token please indication */

TRrq,			/* token release request */

TRcf,			/* token release confirm */

TTrq,			/* token test request */

TTcf;			/* token test confirm */

ENDNEWTYPE;

NEWTYPE		PDUStruct

STRUCT

kind					PDUKind;

/* fields used depend on kind */

channelId			ChannelId;

channelIds			ChannelIdSet;

dataPriority			DataPriority;

detachUserIds		UserIdSet;

diagnostic			Diagnostic;

heightLimit			Natural;

initialOctets			OctetString;

initiator				UserId;

mergeChannels		ChannelAttributesSet;

mergeTokens			TokenAttributesSet;

purgeChannelIds	ChannelIdSet;

purgeTokenIds		TokenIdSet;

reason				Reason;

recipient				UserId;

requested			ChannelId;

result				Result;

segmentation			Segmentation;

subHeight			Natural;

subInterval			Duration;

tokenId				TokenId;

tokenStatus			TokenStatus;

userData				UserData;

userIds				UserIdSet;

ENDNEWTYPE;

NEWTYPE		ChannelKind

LITERALS

Static,			/* range 1:1000 = static: known permanently */

UserId,			/* dynamic: Attach-User / Detach-User */

Private,			/* dynamic: Channel-Convene / Channel-Disband */

Assigned;		/* dynamic: Channel-Join zero / last Channel-Leave */

ENDNEWTYPE;

NEWTYPE		ChannelAttributes

STRUCT

channelId		ChannelId;	/* the channel with these attributes */

kind				ChannelKind;	/* (Static,UserId,Private,Assigned) */

manager			UserId;		/* if (Private): the channel manager */

admitted			UserIdSet;	/* if (Private): zero or more users */

joined			Boolean;	/* if (UserId,Private): True if joined */

ENDNEWTYPE;

NEWTYPE		ChannelAttributesSet	SetOf(ChannelAttributes);

ENDNEWTYPE;

NEWTYPE		TokenKind

LITERALS

Grabbed,			/* assigned exclusively to one user */

Inhibited,		/* inhibited by one or more users */

Giving,			/* reassigning grabbed to a new user */

Ungivable,		/* the recipient has since detached */

Given;			/* donor released token or detached */

ENDNEWTYPE;

NEWTYPE		TokenAttributes

STRUCT

tokenId		TokenId;		/* the token with these attributes */

kind			TokenKind;	/* (Grabbed,Inhibited,Giving,Ungivable,Given) */

grabber		UserId;		/* if (Grabbed,Giving,Ungivable): user */

recipient		UserId;		/* if (Giving,Given): an intended user */

inhibitors	UserIdSet;	/* if (Inhibited): one or more users */

ENDNEWTYPE;

NEWTYPE		TokenAttributesSet	SetOf(TokenAttributes);

ENDNEWTYPE;

SYNTYPE		PortalId	=  Integer CONSTANTS 0:maxPortalIds

ENDSYNTYPE;

NEWTYPE		PortalIdSet	SetOf(PortalId);

ENDNEWTYPE;

NEWTYPE		PortalKind

LITERALS

Attached,	/* MCS Attachment through an MCSAP */

Downlink,	/* MCS Connection to a provider below */

Uplink;		/* MCS Connection to a provider above */

ENDNEWTYPE;

NEWTYPE		PIdByPri	Array(DataPriority, PId);

ENDNEWTYPE;

NEWTYPE		Diagnostic

LITERALS

DC_inconsistent_merge,

DC_forbidden_PDU_downward,

DC_forbidden_PDU_upward,

DC_invalid_BER_encoding,

DC_invalid_PER_encoding,

DC_misrouted_user,

DC_unrequested_confirm,

DC_wrong_transport_priority,

DC_channel_id_conflict,

DC_token_id_conflict,

DC_not_user_id_channel,

DC_too_many_channels,

DC_too_many_tokens,

DC_too_many_users,

DC_OK,						/* not in MCSPDUs */

DC_ignore,					/* not in MCSPDUs */

DC_height_limit_exceeded,		/* not in MCSPDUs */

DC_throughput_inadequate;		/* not in MCSPDUs */

ENDNEWTYPE;

/* Process decomposition */

PROCESS	Control			(1,1)	REFERENCED;	/* CO */

PROCESS	Attachment		(0,)	REFERENCED;	/* AT */

PROCESS	Domain			(0,)	REFERENCED;	/* DM */

PROCESS	Endpoint			(0,)	REFERENCED;	/* EP */

CONNECT Control.MCSAP AND Control.MCSAP.CO;

SIGNALROUTE Control.MCSAP.CO

FROM ENV TO Control WITH

(Controller.to.Control);

FROM Control TO ENV WITH

(Control.to.Controller);

CONNECT MCSAPs AND MCSAPs.CO, MCSAPs.AT;

SIGNALROUTE MCSAPs.CO

FROM ENV TO Control WITH

MCS.Attach.User.request;

FROM Control TO ENV WITH

MCS.Attach.User.confirm;

SIGNALROUTE MCSAPs.AT

FROM ENV TO Attachment WITH

(Users.to.Attachment);

FROM Attachment TO ENV WITH

(Attachment.to.Users);

CONNECT TSAPs AND TSAPs.CO, TSAPs.EP;

SIGNALROUTE TSAPs.CO

FROM ENV TO Control WITH

T.Connect.indication;

FROM Control TO ENV WITH

T.Disconnect.request;

SIGNALROUTE TSAPs.EP

FROM ENV TO Endpoint WITH

(Transport.to.Endpoint);

FROM Endpoint TO ENV WITH

(Endpoint.to.Transport);

SIGNALROUTE CO.AT

FROM Control TO Attachment WITH

Quit,

Exit;

FROM Attachment TO Control WITH

Quit;

SIGNALROUTE CO.DM

FROM Control TO Domain WITH

Open.portal,

Drop.portal,

Shut.portal;

FROM Domain TO Control WITH

Drop.portal,

Report.portal,

Shut.portal;

SIGNALROUTE CO.EP

FROM Control TO Endpoint WITH

(Connect.PDU),

Quit,

Exit;

FROM Endpoint TO Control WITH

(Connect.PDU),

RJum,

Quit;

SIGNALROUTE DM.AT

FROM Domain TO Attachment WITH

PDU.ready,

(PDU.indication),

(PDU.confirm);

FROM Attachment TO Domain WITH

PDU.ready,

(PDU.request),

(PDU.response);

SIGNALROUTE DM.EP

FROM Domain TO Endpoint WITH

PDU.ready,

(PDU.merge),

(PDU.request),

(PDU.indication),

(PDU.response),

(PDU.confirm),

(PDU.ultimatum);

FROM Endpoint TO Domain WITH

PDU.ready,

(PDU.merge),

(PDU.request),

(PDU.indication),

(PDU.response),

(PDU.confirm),

(PDU.ultimatum);

SIGNAL	Open.portal

(

PortalId,

PortalKind,

PIdByPri

);

SIGNAL	Report.portal

(

PortalId,

Diagnostic

);

SIGNAL	Drop.portal

(

PortalId,

Reason

);

SIGNAL	Shut.portal

(

PortalId

);

SIGNAL	Quit;

SIGNAL	Exit;

SIGNALLIST Connect.PDU  =

Connect.Initial,

Connect.Response,

Connect.Additional,

Connect.Result;

SIGNAL	Connect.Initial

(

DomainSelector,		/* calling */

DomainSelector,		/* called */

Boolean,			 /* upward */

DomainParameters,		 /* target */

DomainParameters,		 /* minimum */

DomainParameters,		 /* maximum */

UserData

);

SIGNAL	Connect.Response

(

Result,

Natural,

DomainParameters,

UserData

);

SIGNAL  Connect.Additional

(

Natural,

DataPriority

);

SIGNAL  Connect.Result

(

Result

);

SIGNALLIST PDU.merge  =

PDin, EDrq, MCrq, MCcf, MTrq,

MTcf;

SIGNALLIST PDU.request  =

AUrq, DUrq, CJrq, CLrq, CCrq,

CDrq, CArq, CErq, SDrq, USrq,

TGrq, TIrq, TVrq, TPrq, TRrq,

TTrq;

SIGNALLIST PDU.indication  =

PCin, PTin, DUin, CDin, CAin,

CEin, SDin, USin, TVin, TPin;

SIGNALLIST PDU.response  =

TVrs;

SIGNALLIST PDU.confirm  =

AUcf, CJcf, CCcf, TGcf, TIcf,

TVcf, TRcf, TTcf;

SIGNALLIST PDU.ultimatum  =

DPum, RJum;

SIGNAL  PDU.ready				/* allows one domain MCSPDU */

(

DataPriority

);

SIGNAL	PDin	(PDUStruct);	/* plumb domain indication */

SIGNAL	EDrq	(PDUStruct);	/* erect domain request */

SIGNAL	MCrq	(PDUStruct);	/* merge channels request */

SIGNAL	MCcf	(PDUStruct);	/* merge channels confirm */

SIGNAL	PCin	(PDUStruct);	/* purge channels indication */

SIGNAL	MTrq	(PDUStruct);	/* merge tokens request */

SIGNAL	MTcf	(PDUStruct);	/* merge tokens confirm */

SIGNAL	PTin	(PDUStruct);	/* purge tokens indication */

SIGNAL	DPum	(PDUStruct);	/* disconnect provider ultimatum */

SIGNAL	RJum	(PDUStruct);	/* reject MCSPDU ultimatum */

SIGNAL	AUrq	(PDUStruct);	/* attach user request */

SIGNAL	AUcf	(PDUStruct);	/* attach user confirm */

SIGNAL	DUrq	(PDUStruct);	/* detach user request */

SIGNAL	DUin	(PDUStruct);	/* detach user confirm */

SIGNAL	CJrq	(PDUStruct);	/* channel join request */

SIGNAL	CJcf	(PDUStruct);	/* channel join confirm */

SIGNAL	CLrq	(PDUStruct);	/* channel leave request */

SIGNAL	CCrq	(PDUStruct);	/* channel convene request */

SIGNAL	CCcf	(PDUStruct);	/* channel convene confirm */

SIGNAL	CDrq	(PDUStruct);	/* channel disband request */

SIGNAL	CDin	(PDUStruct);	/* channel disband confirm */

SIGNAL	CArq	(PDUStruct);	/* channel admit request */

SIGNAL	CAin	(PDUStruct);	/* channel admit confirm */

SIGNAL	CErq	(PDUStruct);	/* channel expel request */

SIGNAL	CEin	(PDUStruct);	/* channel expel confirm */

SIGNAL	SDrq	(PDUStruct);	/* send data request */

SIGNAL	SDin	(PDUStruct);	/* send data indication */

SIGNAL	USrq	(PDUStruct);	/* uniform send data request */

SIGNAL	USin	(PDUStruct);	/* uniform send data indication */

SIGNAL	TGrq	(PDUStruct);	/* token grab request */

SIGNAL	TGcf	(PDUStruct);	/* token grab confirm */

SIGNAL	TIrq		(PDUStruct);	/* token inhibit request */

SIGNAL	TIcf		(PDUStruct);	/* token inhibit confirm */

SIGNAL	TVrq	(PDUStruct);	/* token give request */

SIGNAL	TVin	(PDUStruct);	/* token give indication */

SIGNAL	TVrs	(PDUStruct);	/* token give response */

SIGNAL	TVcf	(PDUStruct);	/* token give confirm */

SIGNAL	TPrq	(PDUStruct);	/* token please request */

SIGNAL	TPin	(PDUStruct);	/* token please indication */

SIGNAL	TRrq	(PDUStruct);	/* token release request */

SIGNAL	TRcf	(PDUStruct);	/* token release confirm */

SIGNAL	TTrq	(PDUStruct);	/* token test request */

SIGNAL	TTcf		(PDUStruct);	/* token test confirm */

ENDBLOCK;

Appendix  III

SDL specification of the Control processxe "SDL specification of the Control process"§

(This appendix does not form an integral part of this Recommendation)

PROCESS	Control;

	/* Type definitions */

NEWTYPE		Proc
LITERALS
	Nil,

Receiving,					/*			Endpoint */

Responding,					/*			Endpoint */

Engaged,					/* Attachment Endpoint */

Quitting,						/* Attachment Endpoint */

Quit;						/* Attachment Endpoint */

ENDNEWTYPE;

NEWTYPE		ProcByPri		Array(DataPriority, Proc);

ENDNEWTYPE;

NEWTYPE		CallSide
LITERALS
	Calling,
	Called;
ENDNEWTYPE;

Recommendation T.125     (04/94)

NEWTYPE		PortalStruct
STRUCT
	mcId				MCSConnectionId;				/* equals PortalId index */
	ccId					Natural;							/* equals PortalId index */
	kind					PortalKind;						/* (Attached,Downlink,Uplink) */
	pids					PIdByPri;							/* processes comprising a portal */
	proc				ProcByPri;						/* state of each created process */
	label				Natural;							/* requester's label for confirm */
	domain				DomainSelector;					/* domain selected by the portal */
	opened				Boolean;							/* True if portal has been opened */
	notify				Boolean;							/* True to notify when portal quit */
	minParms			DomainParameters;				/* lower limit for negotiation */
	maxParms			DomainParameters;				/* upper limit for negotiation */
	parameters			DomainParameters;				/* values negotiated by portal */
	callSide				CallSide;							/* portal is calling or called */
	localTSAP			TSAPAddress;						/* local address for T.Connect */
	remoteTSAP			TSAPAddress;						/* remote address for T.Connect */
	targetQOSByPri		TransportQOSByPri;				/* desired quality of service */
	minQOSByPri		TransportQOSByPri;				/* minimum that is acceptable */
	userData				UserData;						/* of response, pending confirm */
ENDNEWTYPE;
	/* Note:  In a practical implementation, the user data stored from
	Connect.Response, awaiting establishment of additional TCs, need be
	only one transport interface data unit, not a complete TSDU.  Any
	excess can be left in the pipeline of the initial TC to be read out
	when MCS.Connect.Provider.confirm is issued. */

NEWTYPE		Portal			Array(PortalId, PortalStruct);
ENDNEWTYPE;

NEWTYPE		DomainStruct
STRUCT
	pid				PId;								/* process for the domain or Null */
	portals			Natural;							/* number of portals open to domain */
	upward			portalId;							/* unique connection upward or zero */
	minParms		DomainParameters;					/* lower limit of configuration */
	maxParms		DomainParameters;					/* upper limit of configuration */
	parameters		DomainParameters;					/* values established in domain */
ENDNEWTYPE;

NEWTYPE		Domain			Array(DomainSelector, DomainStruct);
ENDNEWTYPE;

NEWTYPE		DomainSelectorSet			SetOf(DomainSelector);
ENDNEWTYPE;

/* Data declarations */

DCL	domain			Domain,							/* resource arrays */
	portal			Portal;

/* Note: The fields of a domain or portal array element
	are undefined if the corresponding index is not in dUsed
	or pUsed respectively. */

DCL	dUsed			DomainSelectorSet,					/* indexes used */
	pUsed			PortalIdSet;

DCL	pFree			PortalIdSet;							/* indexes free */

DCL	nullParms		DomainParameters;					/* initializer */

/* Procedure decomposition */
	/*			Initialize_resources
				Identify_sender							(p, dp)
				Min_parms							(min, a, b)

Max_parms							(max, a, b)
				Test_parms							(result, x, min, max)
				MCS_Connect_Provider_request		(...)
				T_Connect_indication					(...)
				Connect_Initial							(...)
				MCS_Connect_Provider_response		(...)
				Connect_Response						(...)
				Connect_Additional						(...)
				Connect_Result							(...)
				MCS_Disconnect_Provider_request		(...)
				MCS_Attach_User_request				(...)
				Open_portal							(p)
				Drop_portal							(p, reason)
				Report_portal							(p, diagnostic)
				RJum									(pdu)
				Quit_portal							(p, reason)
				Quit
				Shut_portal							(p)
				Exit_portal							(p)	*/
																/*-------------------------------*/
PROCEDURE			Initialize_resources;								/*  Initialize_resources  */
																/*-------------------------------*/
	DCL			p				PortalId,
				ds				DomainSelector,
				dSet			DomainSelectorSet;
	START
	COMMENT	'Initialize data structures during process start-up
				before accepting the first input signal.
				Note that each SetOf automatically defaults to Empty.
				Configure one hypothetical domain as an example.
				Some limits are determined by the implementation.
				';
	TASK		nullParms!maxChannelIds := 0,
				nullParms!maxUserIds := 0,
				nullParms!maxTokenIds := 0,
				nullParms!numPriorities := 0,
				nullParms!minThroughput := 0,
				nullParms!maxHeight := 0,
				nullParms!maxMCSPDUsize := 0,
				nullParms!protocolVersion := 0,
				p := maxPortalIds;
	 1b :			/* for p = ?..1 */
	DECISION p > 0;
	 (True):		TASK		portal(p)!mcId := p,
							portal(p)!ccId := p,
							pFree := Incl(p, pFree),
							p := p - 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	TASK		ds := Mkstring(77) // Mkstring(67) // Mkstring(83),
				dUsed := Incl(ds, dUsed),
				domain(ds)!Pid := Null,
				domain(ds)!portals := 0,
				domain(ds)!upward := 0,
				dSet := dUsed;
	 2b :			/* for ds in dSet */
	DECISION dSet =	 Empty;
	 (False):		TASK		ds := Pick(dSet),
							dSet := Del(ds, dSet),
							domain(ds)!minParms!maxChannelIds := 0,
							domain(ds)!minParms!maxUserIds := 0,
							domain(ds)!minParms!maxTokenIds := 0,
							domain(ds)!minParms!numPriorities := 1,
							domain(ds)!minParms!minThroughput := 0,
							domain(ds)!minParms!maxHeight := 1,
							domain(ds)!minParms!maxMCSPDUsize := 35,

domain(ds)!minParms!protocolVersion := 1,
							domain(ds)!maxParms!maxChannelIds := 65535,
							domain(ds)!maxParms!maxUserIds := 65535,
							domain(ds)!maxParms!maxTokenIds := 65535,
							domain(ds)!maxParms!numPriorities := 4,
							domain(ds)!maxParms!minThroughput := 1000000,
							domain(ds)!maxParms!maxHeight := 1000,
							domain(ds)!maxParms!maxMCSPDUsize := 32768,
							domain(ds)!maxParms!protocolVersion := 2;
				JOIN 2b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-------------------------*/
PROCEDURE			Identify_sender;									/*  Identify_sender  */
FPAR	IN/OUT		p				PortalId,						/*-------------------------*/
	IN/OUT		dp				DataPriority;

DCL			pSet			PortalIdSet;
	START
	COMMENT	'An alternative would be to carry this
				information explicitly in SDL signals.
				';
	TASK		pSet := pUsed;
	 1b :			/* for p in pSet */
	DECISION pSet =	 Empty;
	 (False):		TASK		p := Pick(pSet),
							pSet := Del(p, pSet),
							dp := 0;
				 2b :		/* for dp = 0..3 */
				DECISION dp < 4;
				 (True):	DECISION portal(p)!pids(dp) = SENDER;
							 (True): RETURN;
							ELSE:ENDDECISION;
							TASK	dp := dp + 1;
							JOIN 2b;
				ELSE:ENDDECISION;
				JOIN 1b;
	ELSE:ENDDECISION;
	TASK		p := 0,
				dp := 0;
	RETURN;
	ENDPROCEDURE;

/*-------------------*/
PROCEDURE			Min_parms;									/*  Min_parms  */
FPAR	IN/OUT		min			DomainParameters,				/*-------------------*/
				a				DomainParameters,
				b				DomainParameters;

START
	COMMENT	'Return the minimum of two parameter sets.
				';
	TASK		min!maxChannelIds := IF a!maxChannelIds < b!maxChannelIds
								THEN a!maxChannelIds ELSE b!maxChannelIds FI,
				min!maxUserIds := IF a!maxUserIds < b!maxUserIds
								THEN a!maxUserIds ELSE b!maxUserIds FI,
				min!maxTokenIds := IF a!maxTokenIds < b!maxTokenIds
								THEN a!maxTokenIds ELSE b!maxTokenIds FI,
				min!numPriorities := IF a!numPriorities < b!numPriorities
								THEN a!numPriorities ELSE b!numPriorities FI,
				min!minThroughput := IF a!minThroughput < b!minThroughput
								THEN a!minThroughput ELSE b!minThroughput FI,
				min!maxHeight := IF a!maxHeight < b!maxHeight
								THEN a!maxHeight ELSE b!maxHeight FI,

min!maxMCSPDUsize := IF a!maxMCSPDUsize < b!maxMCSPDUsize
								THEN a!maxMCSPDUsize ELSE b!maxMCSPDUsize FI,
				min!protocolVersion := IF a!protocolVersion < b!protocolVersion
								THEN a!protocolVersion ELSE b!protocolVersion FI;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Max_parms;									/*  Max_parms  */
FPAR	IN/OUT		max			DomainParameters,				/*--------------------*/
				a				DomainParameters,
				b				DomainParameters;

START
	COMMENT	'Return the maximum of two parameter sets.
				';
	TASK		max!maxChannelIds := IF a!maxChannelIds > b!maxChannelIds
								THEN a!maxChannelIds ELSE b!maxChannelIds FI,
				max!maxUserIds := IF a!maxUserIds > b!maxUserIds
								THEN a!maxUserIds ELSE b!maxUserIds FI,
				max!maxTokenIds := IF a!maxTokenIds > b!maxTokenIds
								THEN a!maxTokenIds ELSE b!maxTokenIds FI,
				max!numPriorities := IF a!numPriorities > b!numPriorities
								THEN a!numPriorities ELSE b!numPriorities FI,
				max!minThroughput := IF a!minThroughput > b!minThroughput
								THEN a!minThroughput ELSE b!minThroughput FI,
				max!maxHeight := IF a!maxHeight > b!maxHeight
								THEN a!maxHeight ELSE b!maxHeight FI,
				max!maxMCSPDUsize := IF a!maxMCSPDUsize > b!maxMCSPDUsize
								THEN a!maxMCSPDUsize ELSE b!maxMCSPDUsize FI,
				max!protocolVersion := IF a!protocolVersion > b!protocolVersion
								THEN a!protocolVersion ELSE b!protocolVersion FI;
	RETURN;
	ENDPROCEDURE;
																/*--------------------*/
PROCEDURE			Test_parms;									/*  Test_parms  */
FPAR	IN/OUT		result			Result,							/*--------------------*/
				z				DomainParameters,
				min			DomainParameters,
				max			DomainParameters;

START
	COMMENT	'Check that the parameters lie between min and max.
				';
	DECISION	(z!maxChannelIds >= min!maxChannelIds)
		   and	(z!maxChannelIds <= max!maxChannelIds)
		   and	(z!maxUserIds >= min!maxUserIds)
		   and	(z!maxUserIds <= max!maxUserIds)
		   and	(z!maxTokenIds >= min!maxTokenIds)
		   and	(z!maxTokenIds <= max!maxTokenIds)
		   and	(z!numPriorities >= min!numPriorities)
		   and	(z!numPriorities <= max!numPriorities)
		   and	(z!minThroughput >= min!minThroughput)
		   and	(z!minThroughput <= max!minThroughput)
		   and	(z!maxHeight >= min!maxHeight)
		   and	(z!maxHeight <= max!maxHeight)
		   and	(z!maxMCSPDUsize >= min!maxMCSPDUsize)
		   and	(z!maxMCSPDUsize <= max!maxMCSPDUsize)
		   and	(z!protocolVersion >= min!protocolVersion)
		   and	(z!protocolVersion <= max!protocolVersion);
	 (True):	TASK	result := RT_successful;
	 (False):	TASK	result := RT_parameters_unacceptable;
	ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*--------------------------------------------------*/
PROCEDURE		MCS_Connect_Provider_request;				/*  MCS_Connect_Provider_request  */
FPAR			label				Natural,					/*--------------------------------------------------*/
			localTSAP			TSAPAddress,
			localDomain		DomainSelector,
			remoteTSAP		TSAPAddress,
			remoteDomain	DomainSelector,
			upward				Boolean,
			targetParms		DomainParameters,
			minParms			DomainParameters,
			maxParms			DomainParameters,
			targetQOSByPri	TransportQOSByPri,
			minQOSByPri		TransportQOSByPri,
			userData			UserData;

DCL		result				Result,
			p					PortalId,
			dp					DataPriority,
			ds					DomainSelector;
	START
	COMMENT	'Process an MCS.Connect.Provider.request input signal.
				Begin parameter negotiation and allocate a portal.
				Create an endpoint process for the initial TC and
				transmit Connect.Initial through it.
				';
	DECISION pFree = Empty;
	 (True):		TASK		result := RT_congested;
				JOIN 1f;
	ELSE:ENDDECISION;
	TASK		ds := localDomain;
	DECISION ds in dUsed;
	 (False):		TASK		result := RT_no_such_domain;
				JOIN 1f;
	ELSE:ENDDECISION;
	DECISION upward and domain(ds)!upward /= 0;
	 (True):		TASK		result := RT_domain_not_hierarchical;
				JOIN 1f;
	ELSE:ENDDECISION;
	CALL		Max_parms(minParms, minParms, domain(ds)!minParms);
	CALL		Min_parms(maxParms, maxParms, domain(ds)!maxParms);
	DECISION domain(ds)!portals > 0;
	 (True):		TASK		targetParms := domain(ds)!parameters;
	 (False):		CALL		Max_parms(targetParms, targetParms, minParms);
				CALL		Min_parms(targetParms, targetParms, maxParms);
	ENDDECISION;
	CALL		Test_parms(result, targetParms, minParms, maxParms);
	DECISION result = RT_successful;
	 (False): 1f :
				OUTPUT	MCS.Connect.Provider.confirm
							(label, result, 0, nullParms, NullString);
				RETURN;
	ELSE:ENDDECISION;
	DECISION domain(ds)!portals > 0;
	 (True):		TASK		minParms := targetParms,
							maxParms := targetParms;
	ELSE:ENDDECISION;
	CREATE		Endpoint(Null, localTSAP, remoteTSAP,
				targetQOSByPri(0), minQOSByPri(0),
				nullParms);
	OUTPUT		Connect.Initial(localDomain, remoteDomain, upward,
				targetParms, minParms, maxParms, userData)
				TO OFFSPRING;
	TASK		p := Pick(pFree),
				pFree := Del(p, pFree),
				pUsed := Incl(p, pUsed),
				portal(p)!kind := IF upward THEN Uplink ELSE Downlink FI,

portal(p)!label := label,
				portal(p)!domain := ds,
				portal(p)!opened := False,
				portal(p)!notify := True,
				portal(p)!minParms := minParms,
				portal(p)!maxParms := maxParms,
				portal(p)!parameters := nullParms,
				portal(p)!callSide := Calling,
				portal(p)!localTSAP := localTSAP,
				portal(p)!remoteTSAP := remoteTSAP,
				portal(p)!targetQOSByPri := targetQOSByPri,
				portal(p)!minQOSByPri := minQOSByPri,
				portal(p)!pids(0) := OFFSPRING,
				portal(p)!proc(0) := Receiving,
				dp := 1;
	 2b :			/* for dp = 1..3 */
	DECISION dp < 4;
	 (True):		TASK		portal(p)!pids(dp) := Null,
							portal(p)!proc(dp) := Nil,
							dp := dp + 1;
				JOIN 2b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*----------------------------------*/
PROCEDURE		T_Connect_indication;								/*  T_Connect_indication  */
FPAR			tcId					TCEndpointId,					/*----------------------------------*/
			remoteTSAP		TSAPAddress,
			localTSAP			TSAPAddress,
			offeredQOS		TransportQOS,
			minQOS			TransportQOS;

DCL		p					PortalId,
			dp					DataPriority;
	START
	COMMENT	'Process a T.Connect.indication input signal.
				Allocate a portal, in case this is an initial TC.
				Create an endpoint process to receive Connect.Initial
				or Connect.Additional.
				';
	DECISION pFree = Empty
				or offeredQOS!throughput < minQOS!throughput
				or offeredQOS!transitDelay > minQOS!transitDelay
				or offeredQOS!dataPriority > minQOS!dataPriority;
	 (True):		OUTPUT	T.Disconnect.request(tcId);
				RETURN;
	ELSE:ENDDECISION;
	CREATE		Endpoint(tcId, localTSAP, remoteTSAP,
				offeredQOS, minQOS,
				nullParms);
	TASK		p := Pick(pFree),
				pFree := Del(p, pFree),
				pUsed := Incl(p, pUsed),
				portal(p)!kind := Downlink,
				portal(p)!opened := False,
				portal(p)!notify := False,
				portal(p)!parameters := nullParms,
				portal(p)!callSide := Called,
				portal(p)!localTSAP := localTSAP,
				portal(p)!remoteTSAP := remoteTSAP,
				portal(p)!pids(0) := OFFSPRING,
				portal(p)!proc(0) := Receiving,
				dp := 1;
	 1b :			/* for dp = 1..3 */

DECISION dp < 4;
	 (True):		TASK		portal(p)!pids(dp) := Null,
							portal(p)!proc(dp) := Nil,
							dp := dp + 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*------------------------*/
PROCEDURE			Connect_Initial;									/*  Connect_Initial  */
FPAR				remoteDomain	DomainSelector,				/*------------------------*/
				localDomain	DomainSelector,
				upward		Boolean,
				targetParms	DomainParameters,
				minParms	DomainParameters,
				maxParms	DomainParameters,
				userData		UserData;

DCL			result			Result,
				p				PortalId,
				dp				DataPriority,
				ds				DomainSelector;
	START
	COMMENT	'Process a Connect.Initial input signal.
				Retain the portal and begin parameter negotiation.
				Indicate the connection to the controlling user.
				';
	CALL		Identify_sender(p, dp);
	DECISION p in pUsed and portal(p)!callSide = Called
				and dp = 0 and portal(p)!proc(0) = Receiving;
	 (False):		TASK		result := RT_unspecified_failure;
				JOIN 1f;
	ELSE:ENDDECISION;
	TASK		ds := localDomain;
	DECISION ds in dUsed;
	 (False):		TASK		result := RT_no_such_domain;
				JOIN 1f;
	ELSE:ENDDECISION;
	DECISION upward or domain(ds)!upward = 0;
	 (False):		TASK		result := RT_domain_not_hierarchical;
				JOIN 1f;
	ELSE:ENDDECISION;
	CALL		Max_parms(minParms, minParms, domain(ds)!minParms);
	CALL		Min_parms(maxParms, maxParms, domain(ds)!maxParms);
	DECISION domain(ds)!portals > 0;
	 (True):		TASK		targetParms := domain(ds)!parameters;
	 (False):		CALL		Max_parms(targetParms, targetParms, minParms);
				CALL		Min_parms(targetParms, targetParms, maxParms);
	ENDDECISION;
	CALL		Test_parms(result, targetParms, minParms, maxParms);
	DECISION result = RT_successful;
	 (False): 1f :
				OUTPUT	Connect.Response(result, 0, nullParms, NullString)
							TO SENDER;
				CALL		Quit_portal(p, RN_unspecified);
				RETURN;
	ELSE:ENDDECISION;
	DECISION domain(ds)!portals > 0;
	 (True):		TASK		minParms := targetParms,
							maxParms := targetParms;
	ELSE:ENDDECISION;
	TASK		portal(p)!kind := IF upward THEN Downlink ELSE Uplink FI,
				portal(p)!domain := ds,
				portal(p)!notify := True,
				portal(p)!minParms := minParms,

portal(p)!maxParms := maxParms,
				portal(p)!proc(0) := Responding;
	OUTPUT		MCS.Connect.Provider.indication
				(portal(p)!mcId, portal(p)!localTSAP, localDomain, portal(p)!remoteTSAP,
				remoteDomain, upward, targetParms, minParms, maxParms, userData);
	RETURN;
	ENDPROCEDURE;

/*----------------------------------------------------*/
PROCEDURE			MCS_Connect_Provider_response;		/*  MCS_Connect_Provider_response  */
FPAR				mcId			MCSConnectionId,		/*----------------------------------------------------*/
				result			Result,
				parameters	DomainParameters,
				userData		UserData;

DCL			p				PortalId;
	START
	COMMENT	'Process an MCS.Connect.Provider.response input signal.
				Check negotiated parameters and force a preference.
				Transmit Connect.Response.
				If there are no additional TCs, open the portal.
				';
	TASK		p := mcId;
	DECISION p in pUsed and portal(p)!proc(0) = Responding;
	 (False):		RETURN;
	ELSE:ENDDECISION;
	DECISION result = RT_successful;
	 (False):		TASK		result := RT_user_rejected,
							portal(p)!notify := False;
				JOIN 1f;
	ELSE:ENDDECISION;
	CALL		Test_parms(result, parameters, portal(p)!minParms, portal(p)!maxParms);
	DECISION result = RT_successful;
	 (False): 1f :
				OUTPUT	Connect.Response(result, 0, parameters, userData)
							TO portal(p)!pids(0);
				CALL		Quit_portal(p, RN_parameters_unacceptable);
				RETURN;
	ELSE:ENDDECISION;
	CALL		Max_parms(parameters, parameters, portal(p)!minParms);
	OUTPUT		Connect.Response(result, portal(p)!ccId, parameters, userData)
				TO portal(p)!pids(0);
	TASK		portal(p)!parameters := parameters,
				portal(p)!proc(0) := Engaged;
	CALL		Open_portal(p);
	RETURN;
	ENDPROCEDURE;

/*-------------------------------*/
PROCEDURE			Connect_Response;								/*  Connect_Response  */
FPAR				result			Result,							/*-------------------------------*/
				ccId			Natural,
				parameters	DomainParameters,
				userData		UserData;

DCL			p				PortalId,
				dp				DataPriority;
	START
	COMMENT	'Process a Connect.Response input signal.
				Check the negotiated parameters.
				Create endpoint processes for additional TCs and
				transmit Connect.Additional through them.
				If there are no additional TCs, open the portal.
				';
	CALL		Identify_sender(p, dp);

DECISION p in pUsed and portal(p)!callSide = Calling
				and dp = 0 and portal(p)!proc(0) = Receiving;
	 (False):		CALL		Quit_portal(p, RN_unspecified);
				RETURN;
	ELSE:ENDDECISION;
	TASK		portal(p)!parameters := parameters,
				portal(p)!userData := userData;
	DECISION result = RT_successful;
	 (False):		JOIN 1f;
	ELSE:ENDDECISION;
	CALL		Test_parms(result, parameters, portal(p)!minParms, portal(p)!maxParms);
	DECISION result = RT_successful;
	 (False): 1f :
				OUTPUT	MCS.Connect.Provider.confirm
							(portal(p)!label, result, 0, parameters, userData);
				TASK		portal(p)!notify := False;
				CALL		Quit_portal(p, RN_unspecified);
				RETURN;
	ELSE:ENDDECISION;
	TASK		portal(p)!proc(0) := Engaged,
				dp := 1;
	 2b :			/* for dp = 1..? */
	DECISION dp < parameters!numPriorities;
	 (True):		CREATE	Endpoint(Null, portal(p)!localTSAP, portal(p)!remoteTSAP,
							portal(p)!targetQOSByPri(dp), portal(p)!minQOSByPri(dp),
							parameters);
				OUTPUT	Connect.Additional(ccId, dp)
							TO OFFSPRING;
				TASK		portal(p)!pids(dp) := OFFSPRING,
							portal(p)!proc(dp) := Receiving,
							dp := dp + 1;
				JOIN 2b;
	ELSE:ENDDECISION;
	CALL		Open_portal(p);
	RETURN;
	ENDPROCEDURE;
																/*-------------------------------*/
PROCEDURE			Connect_Additional;								/*  Connect_Additional  */
FPAR				ccId			Natural,							/*-------------------------------*/
				dp				DataPriority;

DCL			p				PortalId,
				r				PortalId,
				x				DataPriority;
	START
	COMMENT	'Process a Connect.Additional input signal.
				Release the allocated portal and piggyback onto
				the preceding Connect.Initial.
				Transmit Connect.Result.
				If all TCs are established, open the portal.
				';
	CALL		Identify_sender(r, x);
	DECISION r in pUsed and portal(r)!callSide = Called
				and x = 0 and portal(r)!proc(0) = Receiving;
	 (False):		JOIN 1f;
	ELSE:ENDDECISION;
	TASK		p := ccId;
	DECISION p in pUsed and portal(p)!callSide = Called
				and dp > 0 and dp < portal(p)!parameters!numPriorities
				and portal(p)!proc(0) = Engaged and portal(p)!proc(dp) = Nil;
	 (False): 1f :
				OUTPUT	Connect.Result(RT_unspecified_failure)
							TO SENDER;
				CALL		Quit_portal(r, RN_unspecified);
				RETURN;
	ELSE:ENDDECISION;

TASK		pUsed := Del(r, pUsed),
				pFree := Incl(r, pFree);
	OUTPUT		Connect.Result(RT_successful)
				TO SENDER;
	TASK		portal(p)!pids(dp) := SENDER,
				portal(p)!proc(dp) := Engaged;
	CALL		Open_portal(p);
	RETURN;
	ENDPROCEDURE;
																/*--------------------------*/
PROCEDURE			Connect_Result;								/*  Connect_Result  */
FPAR				result			Result;							/*--------------------------*/
	DCL			p				PortalId,
				dp				DataPriority;
	START
	COMMENT	'Process a Connect.Result input signal.
				If all TCs are established, open the portal.
				';
	CALL		Identify_sender(p, dp);
	DECISION p in pUsed and portal(p)!callSide = Calling
				and dp > 0 and portal(p)!proc(dp) = Receiving;
	 (False):		CALL		Quit_portal(p, RN_unspecified);
				RETURN;
	ELSE:ENDDECISION;
	DECISION result = RT_successful;
	 (False):		OUTPUT	MCS.Connect.Provider.confirm
							(portal(p)!label, result, 0,
							portal(p)!parameters, portal(p)!userData);
				TASK		portal(p)!notify := False;
				CALL		Quit_portal(p, RN_unspecified);
				RETURN;
	ELSE:ENDDECISION;
	TASK		portal(p)!proc(dp) := Engaged;
	CALL		Open_portal(p);
	RETURN;
	ENDPROCEDURE;
														/*------------------------------------------------------*/
PROCEDURE			MCS_Disconnect_Provider_request;		/*  MCS_Disconnect_Provider_request  */
FPAR				mcId			MCSConnectionId;		/*------------------------------------------------------*/
	DCL			p				PortalId,
				ds				DomainSelector;
	START
	COMMENT	'Process an MCS.Disconnect.Provider.request input signal.
				If the portal is open, this can be done gracefully.
				';
	TASK		p := mcId;
	DECISION p in pUsed and portal(p)!notify;
	 (True):		TASK		portal(p)!notify := False,
							ds := portal(p)!domain;
				DECISION portal(p)!opened and domain(ds)!portals > 0;
				 (True):	OUTPUT Drop.portal(p, RN_user_requested) TO domain(ds)!pid;
				 (False):	CALL	Quit_portal(p, RN_unspecified);
				ENDDECISION;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
															/*------------------------------------------*/
PROCEDURE			MCS_Attach_User_request;					/*  MCS_Attach_User_request  */
FPAR				label			Natural,						/*------------------------------------------*/
				localDomain	DomainSelector;

DCL			p				PortalId,
				dp				DataPriority,
				ds				DomainSelector;
	START
	COMMENT	'Process an MCS.Attach.User.request input signal.
				Allocate a portal, expecting to create an attachment,
				and open the portal.
				';
	DECISION pFree = Empty;
	 (True):		TASK		result := RT_congested;
				JOIN 1f;
	ELSE:ENDDECISION;
	TASK		ds := localDomain;
	DECISION ds in dUsed;
	 (False):		TASK		result := RT_no_such_domain;
				 1f :
				OUTPUT	MCS.Attach.User.confirm
							(label, result, Null, 0);
				RETURN;
	ELSE:ENDDECISION;
	TASK		p := Pick(pFree),
				pFree := Del(p, pFree),
				pUsed := Incl(p, pUsed),
				portal(p)!kind := Attached,
				portal(p)!label := label,
				portal(p)!domain := ds,
				portal(p)!opened := False,
				portal(p)!notify := False,
				portal(p)!pids(0) := Null,
				portal(p)!proc(0) := Engaged,
				dp := 1;
	 2b :			/* for dp = 1..3 */
	DECISION dp < 4;
	 (True):		TASK		portal(p)!pids(dp) := Null,
							portal(p)!proc(dp) := Nil,
							dp := dp + 1;
				JOIN 2b;
	ELSE:ENDDECISION;
	CALL		Open_portal(p);
	RETURN;
	ENDPROCEDURE;

/*---------------------*/
PROCEDURE			Open_portal;									/*  Open_portal  */
FPAR				p				PortalId;						/*---------------------*/

DCL			dp				DataPriority,
				ds				DomainSelector,
				numP			Natural,
				parameters	DomainParameters;
	START
	COMMENT	'When all TCs have been established, or if this
				is a user attachment, open the portal to a domain.
				If the domain process is stopping, try again later.
				Attachments must wait until domain parameters are set.
				';
	TASK		ds := portal(p)!domain,
				dp := 0;
	DECISION portal(p)!kind = Attached;
	 (True):		TASK		parameters := domain(ds)!minParms,
							numP := 1;
	 (False):		TASK		parameters := portal(p)!parameters,
							numP := parameters!numPriorities;
	ENDDECISION;
	 1b :			/* for dp = 0..? */

DECISION dp < numP;
	 (True):		DECISION portal(p)!proc(dp) = Engaged;
				 (False):RETURN;
				ELSE:ENDDECISION;
				TASK		dp := dp + 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	DECISION domain(ds)!pid = Null;
	 (False):		DECISION domain(ds)!portals = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
	 (True):		CREATE	Domain(parameters);
				TASK		domain(ds)!pid := OFFSPRING,
							domain(ds)!portals := 0,
							domain(ds)!upward := 0,
							domain(ds)!parameters := parameters;
	ENDDECISION;
	DECISION portal(p)!kind = Attached;
	 (True):		CREATE	Attachment(portal(p)!label, domain(ds)!parameters);
				TASK		portal(p)!pids(0) := OFFSPRING;
	 (False):		DECISION domain(ds)!parameters = parameters;
				 (False):	CALL	Quit_portal(p, RN_parameters_unacceptable);
							RETURN;
				ELSE:ENDDECISION;
				DECISION portal(p)!kind = Uplink;
				 (True):	DECISION domain(ds)!upward = 0;
							 (False):	CALL	Quit_portal(p, RN_domain_not_hierarchical);
									RETURN;
							 (True):	TASK	domain(ds)!upward := p;
							ENDDECISION;
				ELSE:ENDDECISION;
				DECISION portal(p)!callSide = Called;
				 (False):	OUTPUT	MCS.Connect.Provider.confirm
									(portal(p)!label, RT_successful, portal(p)!mcId,
									portal(p)!parameters, portal(p)!userData);
				ELSE:ENDDECISION;
	ENDDECISION;
	OUTPUT		Open.portal(p, portal(p)!kind, portal(p)!pids) TO domain(ds)!pid;
	TASK		domain(ds)!portals := domain(ds)!portals + 1,
				portal(p)!opened := True;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Drop_portal;									/*  Drop_portal  */
FPAR				p				PortalId,						/*--------------------*/
				reason		Reason;

START
	COMMENT'	Process a Drop.portal input signal.
				';
	CALL		Quit_portal(p, reason);
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/
PROCEDURE			Report_portal;									/*  Report_portal  */
FPAR				p				PortalId,						 /*----------------------*/
				diagnostic	Diagnostic;

DCL			reason		Reason;
	START
	COMMENT	'Process a Report.portal input signal.
				For testing, local diagnostic could be logged.
				';
	TASK		reason := RN_unspecified;

DECISION diagnostic;
	 (DC_throughput_inadequate):
				TASK		reason := RN_provider_initiated;
	 (DC_height_limit_exceeded):
				TASK		reason := RN_domain_not_hierarchical;
	ELSE:ENDDECISION;
	CALL		Quit_portal(p, reason);
	RETURN;
	ENDPROCEDURE;

/*-----------*/
PROCEDURE			RJum;										/*  RJum  */
FPAR				pdu			PDUStruct;						/*-----------*/

DCL			p				PortalId,
				dp				DataPriority;
	START
	COMMENT	'Process an RJum input signal.
				For testing, remote pdu!diagnostic could be logged.
				';
	CALL		Identify_sender(p, dp);
	CALL		Quit_portal(p, RN_unspecified);
	RETURN;
	ENDPROCEDURE;

/*-------------------*/
PROCEDURE			Quit_portal;									/*  Quit_portal  */
FPAR				p				PortalId, 						/*-------------------*/
				reason		Reason;

DCL			result			Result,
				dp				DataPriority;
	START
	COMMENT	'If necessary, notify the controlling user.
				Quiesce the portal processes.
				';
	DECISION p in pUsed;
	 (False):		RETURN;
	ELSE:ENDDECISION;
	DECISION portal(p)!notify;
	 (True):		DECISION portal(p)!callSide = Called or portal(p)!opened;
				 (True):	OUTPUT	MCS.Disconnect.Provider.indication
									(portal(p)!mcId, reason);
				 (False):	DECISION reason;
							 (RN_domain_not_hierarchical):
									TASK	result := RT_domain_not_hierarchical;
							 (RN_parameters_unacceptable):
									TASK	result := RT_parameters_unacceptable;
							ELSE:
									TASK	result := RT_unspecified_failure;
							ENDDECISION;
							OUTPUT	MCS.Connect.Provider.confirm
									(portal(p)!label, result, 0, nullParms, NullString);
				ENDDECISION;
	ELSE:ENDDECISION;
	TASK		portal(p)!notify := False,
				dp := 0;
	 1b :			/* for dp = 0..3 */
	DECISION dp < 4;
	 (True):		DECISION portal(p)!proc(dp);
				(Receiving, Responding, Engaged):
							OUTPUT	Quit TO portal(p)!pids(dp);
							TASK		portal(p)!proc(dp) := Quitting;
				ELSE:ENDDECISION;
				TASK		dp := dp + 1;
				JOIN 1b;

ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
																/*---------*/
PROCEDURE			Quit;											/*  Quit  */
																/*---------*/
	DCL			p				PortalId,
				pSet			PortalIdSet,
				dp				DataPriority,
				ds				DomainSelector;
	START
	COMMENT	'Process a Quit input signal.
				When all processes are quiesced, it is safe
				to shut this portal on the domain.
				If an upward portal, quiesce all others too.
				';
	CALL		Identify_sender(p, dp);
	DECISION p in pUsed;
	 (False):		RETURN;
	ELSE:ENDDECISION;
	TASK		portal(p)!proc(dp) := Quit;
	CALL		Quit_portal(p, RN_unspecified);
	TASK		dp := 0;
	 1b :			/* for dp = 0..3 */
	DECISION dp < 4;
	 (True):		DECISION portal(p)!proc(dp);
				(Receiving, Responding, Engaged, Quitting):
							RETURN;
				ELSE:ENDDECISION;
				TASK		dp := dp + 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	DECISION portal(p)!opened;
	 (False):		CALL		Exit_portal(p);
				RETURN;
	ELSE:ENDDECISION;
	TASK		ds := portal(p)!domain,
				domain(ds)!portals := domain(ds)!portals - 1;
	OUTPUT		Shut.portal(p) TO domain(ds)!pid;
	DECISION portal(p)!kind = Uplink;
	 (True):		TASK	domain(ds)!upward := 0,
						pSet := pUsed;
		 2b :				/* for p in pSet */
		DECISION pSet = Empty;
		 (False):			TASK	p := Pick(pSet),
								pSet := Del(p, pSet);
						DECISION portal(p)!opened and portal(p)!domain = ds;
						 (True):	CALL	Quit_portal(p, RN_domain_disconnected);
						ELSE:ENDDECISION;
						JOIN 2b;
		ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
																/*--------------------*/
PROCEDURE			Shut_portal;									/*  Shut_portal  */
FPAR				p				PortalId;						/*--------------------*/

DCL			ds				DomainSelector,
				pSet			PortalIdSet;
	START
	COMMENT	'Process a Shut.portal input signal.
				It is now safe to stop the portal processes.
				If this was the last portal, the domain process stops too
				so that it can be recreated with different parameters.
				';

TASK		ds := portal(p)!domain;
	CALL		Exit_portal(p);
	DECISION domain(ds)!portals = 0;
	 (True):		TASK		domain(ds)!pid := Null,
							pSet := pUsed;
				 1b :		/* for p in pSet */
				DECISION pSet = Empty;
				 (False):	TASK		p := Pick(pSet),
									pSet := Del(p, pSet);
							DECISION portal(p)!domain = ds;
							 (True):	CALL	Open_portal(p);
							ELSE:ENDDECISION;
							JOIN 1b;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
																/*------------------*/
PROCEDURE			Exit_portal;									/*  Exit_portal  */
FPAR				p				PortalId;						/*------------------*/
	DCL			dp				DataPriority;
	START
	COMMENT	'Release the portal and stop its processes.
				';
	TASK		pUsed := Del(p, pUsed),
				pFree := Incl(p, pFree),
				dp := 0;
	 1b :			/* for dp = 0..3 */
	DECISION dp < 4;
	 (True):		DECISION portal(p)!proc(dp);
				 (Quit):
							OUTPUT	Exit TO portal(p)!pids(dp);
							TASK		portal(p)!proc(dp) := Nil;
				ELSE:ENDDECISION;
				TASK		dp := dp + 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/* Input transitions */
	DCL			p					PortalId,
				dp					DataPriority,
				pdu				PDUStruct,
				mcId				MCSConnectionId,
				ccId				Natural,
				tcId				TCEndpointId,
				reason			Reason,
				result				Result,
				diagnostic		Diagnostic,
				label				Natural,
				localTSAP		TSAPAddress,
				localDomain		DomainSelector,
				remoteTSAP		TSAPAddress,
				remoteDomain		DomainSelector,
				upward			Boolean,
				targetParms		DomainParameters,
				minParms		DomainParameters,
				maxParms		DomainParameters,
				parameters		DomainParameters,
				targetQOSByPri		TransportQOSByPri,
				minQOSByPri		TransportQOSByPri,
				offeredQOS		TransportQOS,
				minQOS			TransportQOS,
				userData			UserData;

START
	COMMENT	'The state machine contains a single state.
				';
	CALL		Initialize_resources;
	NEXTSTATE ~;

STATE ~;INPUT		MCS.Connect.Provider.request(label, localTSAP, localDomain,
				remoteTSAP, remoteDomain, upward, targetParms, minParms, maxParms,
				targetQOSByPri, minQOSByPri, userData);
	CALL		MCS_Connect_Provider_request(label, localTSAP, localDomain,
				remoteTSAP, remoteDomain, upward, targetParms, minParms, maxParms,
				targetQOSByPri, minQOSByPri, userData);
	NEXTSTATE -;

STATE ~;INPUT		MCS.Connect.Provider.response(mcId, result, parameters, userData);
	CALL		MCS_Connect_Provider_response(mcId, result, parameters, userData);
	NEXTSTATE -;

STATE ~;INPUT		MCS.Disconnect.Provider.request(mcId);
	CALL		MCS_Disconnect_Provider_request(mcId);
	NEXTSTATE -;

STATE ~;INPUT		MCS.Attach.User.request(label, localDomain);
CALL				MCS_Attach_User_request(label, localDomain);
	NEXTSTATE -;

STATE ~;INPUT		T.Connect.indication(tcId, remoteTSAP, localTSAP, offeredQOS, minQOS);
	CALL		T_Connect_indication(tcId, remoteTSAP, localTSAP, offeredQOS, minQOS);
	NEXTSTATE -;

STATE ~;INPUT		Connect.Initial(remoteDomain, localDomain, upward,
				targetParms, minParms, maxParms, userData);
	CALL		Connect_Initial(remoteDomain, localDomain, upward,
				targetParms, minParms, maxParms, userData);
	NEXTSTATE -;

STATE ~;INPUT		Connect.Response(result, ccId, parameters, userData);
	CALL		Connect_Response(result, ccId, parameters, userData);
	NEXTSTATE -;

STATE ~;INPUT		Connect.Additional(ccId, dp);
	CALL		Connect_Additional(ccId, dp);
	NEXTSTATE -;

STATE ~;INPUT		Connect.Result(result);
	CALL		Connect_Result(result);
	NEXTSTATE -;

STATE ~;INPUT		Drop.portal(p, reason);
	CALL		Drop_portal(p, reason);
	NEXTSTATE -;

STATE ~;INPUT		Report.portal(p, diagnostic);
	CALL		Report_portal(p, diagnostic);
	NEXTSTATE -;

STATE ~;INPUT		Shut.portal(p);
	CALL		Shut_portal(p);
	NEXTSTATE -;

STATE ~;INPUT		RJum(pdu);
	CALL		RJum(pdu);
	NEXTSTATE -;

STATE ~;INPUT		Quit;
	CALL		Quit;
	NEXTSTATE -;

ENDPROCESS;

Appendix  IV

SDL specification of the Domain processXE "SDL specification of the Domain process"§

(This appendix does not form an integral part of this Recommendation)

PROCESS	Domain;
FPAR		parameters			DomainParameters;				/* values established in domain */

SYNONYM	maxBufferIds		Natural = EXTERNAL;		/* an implementation limit */

TIMER		Time.portal(PortalId);							/* at most one timer per portal */

/* Type definitions */

NEWTYPE		ChannelStruct
STRUCT
	kind						ChannelKind;				/* (Static,UserId,Private,Assigned) */
	joined					PortalIdSet;				/* directions where channel is joined */
	portal					PortalId;					/* if (UserId): the direction to it */
	manager					UserId;					/* if (Private): channel's manager */
	admitted					UserIdSet;					/* if (Private): zero or more users */
	uMerge					UserIdSet;					/* if (Private): still to be merged */
ENDNEWTYPE;

NEWTYPE		Chan						Array(ChannelId, ChannelStruct);
ENDNEWTYPE;

NEWTYPE		TokenStruct
STRUCT
	kind						TokenKind;					/* (Grabbed,Inhibited,Giving,Ungivable,Given) */
	grabber					UserId;					/* if (Grabbed,Giving,Ungivable): user */
	recipient					UserId;					/* if (Giving,Given): an intended user */
	inhibitors				UserIdSet;					/* if (Inhibited): one or more users */
	uMerge					UserIdSet;					/* if (Inhibited): still to be merged */
ENDNEWTYPE;

NEWTYPE		Token						Array(TokenId, TokenStruct);
ENDNEWTYPE;

NEWTYPE		BooleanByPri				Array(DataPriority, Boolean);
ENDNEWTYPE;
NEWTYPE		NaturalByPri				Array(DataPriority, Natural);
ENDNEWTYPE;
NEWTYPE		BufferIdByPri				Array(DataPriority, BufferId);
ENDNEWTYPE;
NEWTYPE		BufferIdQueueByPri			Array(DataPriority, BufferIdQueue);
ENDNEWTYPE;
NEWTYPE		PortalIdSetByPri			Array(DataPriority, PortalIdSet);
ENDNEWTYPE;

NEWTYPE		NaturalByPriByKind			Array(PortalKind, NaturalByPri);
ENDNEWTYPE;

NEWTYPE		PortalStruct
STRUCT
	kind						PortalKind;						/* (Attached,Downlink,Uplink) */
	pids						PIdByPri;						/* processes constituting a portal */
	inCredit					NaturalByPri;					/* permission to allocate inBuffer */
	inBuffer					BufferIdByPri;					/* PDU input coming from a process */
	outReady				BooleanByPri;					/* True if process allows an output */
	outQueue				BufferIdQueueByPri;			/* PDUs awaiting output to process */
	outCount				Natural;						/* number queued for all priorities */
	outFlow					Natural;						/* number output since timer was set */
	elapsed					Duration;						/* interval set for portal timer */
	interval					Duration;						/* new interval to set for timer */
	subHeight				Natural;						/* subordinate's height or zero */
	subInterval				Duration;						/* subordinate's interval or zero */
ENDNEWTYPE;

NEWTYPE	Portal							Array(PortalId, PortalStruct);
ENDNEWTYPE;

NEWTYPE		PortalIdQueue				Queue(PortalId);
ENDNEWTYPE;

SYNTYPE		BufferId						= Integer CONSTANTS 0:maxBufferIds
ENDSYNTYPE;

NEWTYPE		BufferIdSet					SetOf(BufferId);
ENDNEWTYPE;

NEWTYPE		BufferStruct
STRUCT
	receiver				PortalId;				/* source of inCredit and input PDU */
	dataPriority			DataPriority;			/* index into inBuffer and outQueue */
	portals				Natural;				/* number of outQueues buffer is in */
	pdu					PDUStruct;			/* the content of one domain MCSPDU */
ENDNEWTYPE;

NEWTYPE		Buffer						Array(BufferId, BufferStruct);
ENDNEWTYPE;

NEWTYPE		BufferIdQueue				Queue(BufferId);
ENDNEWTYPE;

GENERATOR		Queue		(TYPE ItemType)			/* a first-in first-out queue */
LITERALS
	EmptyQueue;
OPERATORS
	Push:	ItemType,  Queue	->  Queue;				/* appends next item */
	Next:	Queue				->  ItemType;				/* reveals first item */
	Pull:	Queue				->  Queue;				/* deletes first item */
AXIOMS
	Next(EmptyQueue) == ERROR!;
	Pull(EmptyQueue) == ERROR!;
	FOR ALL q IN Queue (
	FOR ALL item IN ItemType
	(
			Next(Push(item,q)) == IF q = EmptyQueue THEN item
										ELSE Next(q) FI;
			Pull(Push(item,q)) == IF q = EmptyQueue THEN q
										ELSE Push(item,Pull(q)) FI;
	));
DEFAULT
	EmptyQueue;

ENDGENERATOR;

/* Data declarations */

DCL	control					PId;						/* the Control process */

DCL	upward					PortalId;					/* unique MCS Connection upward or */
													/* zero if this provider is the top */

DCL	merging					Boolean,					/* True if domain is merging upward */
	pdSend					Boolean,					/* True if PDin to be sent downward */
	edSend					Boolean;					/* True if EDrq to be sent upward */

DCL	uMerge					UserIdSet,					/* users still to be merged */
	uConfirm				UserIdSet,					/* users with merge confirmed */
	cMerge					ChannelIdSet,				/* channels still to be merged */
	cConfirm				ChannelIdSet,				/* channels with merge confirmed */
	tMerge					TokenIdSet,				/* tokens still to be merged */
	tConfirm					TokenIdSet,				/* tokens with merge confirmed */

DCL	mcrqQueue				PortalIdQueue,			/* origin of each pending MCrq */
	mtrqQueue				PortalIdQueue,			/* origin of each pending MTrq */
	aurqQueue				PortalIdQueue;			/* origin of each pending AUrq */

DCL	pDrop					PortalIdSet,				/* dropped portals needing DPum */
	uDetach					UserIdSet,					/* disconnected users needing DUrq */
	uRevoke					UserIdSet,					/* revoked token users needing DUrq */
	cLeave					ChannelIdSet,				/* unjoined channels needing CLrq */
	cDisband				ChannelIdSet,				/* unmanaged channels needing CDin */
	tReject					TokenIdSet;				/* ungivable tokens needing TVcf */

DCL	pBufferWait				PortalIdSetByPri,			/* portals requiring an inBuffer */
	pInputWait				PortalIdSetByPri;			/* inputs suspended during merge */

DCL	chan					Chan,						/* resource arrays */
	token					Token,
	portal					Portal,
	buffer					Buffer;

/* Note: The fields of a chan, token, portal, or buffer
	array element are undefined if the corresponding index
	is not in cUsed, tUsed, pUsed, or bUsed, respectively. */

DCL	cUsed					ChannelIdSet,				/* indexes used */
	tUsed					TokenIdSet,
	pUsed					PortalIdSet,
	bUsed					BufferIdSet;

DCL	cFree					ChannelIdSet,				/* indexes free */
	tFree					TokenIdSet,
	pFree					PortalIdSet,
	bFree					BufferIdSet;

DCL	uUsed					UserIdSet;					/* subset of cUsed */

DCL	numChannelIds			Natural,					/* number in use */
	numUserIds				Natural,
	numTokenIds				Natural;

DCL	height					Natural,					/* current height of provider */
	interval					Duration,					/* min throughput of one MCSPDU */
	maxInterval				Duration;					/* maximum of portal intervals */

DCL	inCredit					NaturalByPriByKind;		/* initializer */

/* Procedure decomposition */

/*		Initialize_resources
			Take_user				(c, diagnostic)
			Take_channel			(c, diagnostic)
			Take_token			(t, diagnostic)
			New_user				(u, result)
			New_channel			(c, result)
			Open_portal			(p, pKind, pids)
			Time_portal			(p)
			Drop_portal			(p, reason)
			Shut_portal			(p)
			Clean_queue			(p, queue)
			Erect_domain
			Identify_sender		(p, dp)
			PDU_ready				(dp)
			Input_PDU				(pdu)
			Allocate_buffer		(b)
			Cast_buffer			(b, p)
			Release_buffer		(b)
			Route_user			(u, p)
			Multicast_buffer		(b, uSet)
			Broadcast_buffer		(b)
			Crank_engine
			Drop_portals
			Merge_users
			Merge_channels
			Merge_tokens
			Detach_users
			Leave_channels
			Disband_channels
			Reject_tokens
			Process_PDU			(r, dp)
			Validate_input			(r, b, diagnostic)
			Top_provider			(r, b)
			Apply_PDU				(r, b, diagnostic)
			Token_status			(pdu)
			Token_route			(u, x, p)
			Delete_user			(u)
			Delete_channel		(c)
			Delete_token			(t)
			Purge_users			(uSet)
			Purge_channels		(cSet)
			Purge_tokens			(tSet)
			Output_buffer			(b, p)  */

/*-------------------------------*/
PROCEDURE		Initialize_resources;									/*  Initialize_resources  */
																/*-------------------------------*/
	DCL		c				ChannelId,
			t				TokenId,
			p				PortalId,
			b				BufferId,
			n				NaturalByPri;
	START
	COMMENT	'Initialize data structures during process start-up
				before accepting the first input signal.
				Note that each SetOf automatically defaults to Empty
				and each Queue to EmptyQueue.
				Fixed buffer credits are an example; other values could
				be computed from maxPortalIds and maxBufferIds.
				';
	TASK		control := PARENT,
				upward := 0,
				merging := False,
				pdSend := False,
				edSend := False,
				numChannelIds := 0,

numUserIds := 0,
				numTokenIds := 0,
				height := 1,
				interval := IF parameters!minThroughput = 0 THEN 0 ELSE oneSecond *
					(Float(parameters!maxMCSPDUsize) / Float(parameters!minThroughput)) FI,
				maxInterval := 0,
				c := 65535,
				t := 65535,
				p := maxPortalIds,
				b := maxBufferIds;
	 1b :			/* for c = ?..1 */
	DECISION c > 0;
	 (True):		TASK		cFree := Incl(c, cFree),
							c := c - 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	 2b :			/* for t = ?..1 */
	TASK		tFree := Incl(t, tFree);
	DECISION t > 1;
	 (True):		TASK		t := t - 1;
				JOIN 2b;
	ELSE:ENDDECISION;
	 3b :			/* for p = ?..1 */
	DECISION p > 0;
	 (True):		TASK		pFree := Incl(p, pFree),
							p := p - 1;
				JOIN 3b;
	ELSE:ENDDECISION;
	 4b :			/* for b = ?..1 */
	DECISION b > 0;
	 (True):		TASK		bFree := Incl(b, bFree),
							b := b - 1;
				JOIN 4b;
	ELSE:ENDDECISION;
	TASK		n(0) := 2, n(1) := 1, n(2) := 1, n(3) := 1,
				inCredit(Attached) := n,
				n(0) := 3, n(1) := 2, n(2) := 1, n(3) := 1,
				inCredit(Downlink) := n,
				n(0) := 4, n(1) := 3, n(2) := 2, n(3) := 1,
				inCredit(Uplink) := n;
	RETURN;
	ENDPROCEDURE;

/*------------------*/
PROCEDURE			Take_user;									/*  Take_user  */
FPAR				c				ChannelId,						/*------------------*/
	IN/OUT		diagnostic	Diagnostic;

DCL			u				UserId;
	START
	COMMENT	'Put the free channel id to use as a user id.
				';
	DECISION numUserIds < parameters!maxUserIds;
	 (False):		TASK		diagnostic := DC_too_many_users;
				RETURN;
	ELSE:ENDDECISION;
	CALL		Take_channel(c, diagnostic);
	DECISION diagnostic = DC_OK;
	 (True):		TASK		u := UserId(c),
							numUserIds := numUserIds + 1,
							uUsed := Incl(u, uUsed),
							chan(c)!kind := UserId;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/
PROCEDURE			Take_channel;									/*  Take_channel  */
FPAR				c				ChannelId,						/*-----------------------*/
	IN/OUT		diagnostic	Diagnostic;

START
	COMMENT	'Put the free channel id to unspecified use.
				';
	DECISION c in cFree;
	 (False):		TASK		diagnostic := DC_channel_id_conflict;
				RETURN;
	ELSE:ENDDECISION;
	DECISION numChannelIds < parameters!maxChannelIds;
	 (False):		TASK		diagnostic := DC_too_many_channels;
				RETURN;
	ELSE:ENDDECISION;
	TASK		diagnostic := DC_OK,
				numChannelIds := numChannelIds + 1,
				cFree := Del(c, cFree),
				cUsed := Incl(c, cUsed),
				chan(c)!joined := Empty,
				chan(c)!admitted := Empty;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Take_token;									/*  Take_token  */
FPAR				t				TokenId,						/*--------------------*/
	IN/OUT		diagnostic	Diagnostic;

START
	COMMENT	'Put the free token id to unspecified use.
				';
	DECISION t in tFree;
	 (False):		TASK		diagnostic := DC_token_id_conflict;
				RETURN;
	ELSE:ENDDECISION;
	DECISION numTokenIds < parameters!maxTokenIds;
	 (False):		TASK		diagnostic := DC_too_many_tokens;
				RETURN;
	ELSE:ENDDECISION;
	TASK		diagnostic := DC_OK,
				numTokenIds := numTokenIds + 1,
				tFree := Del(t, tFree),
				tUsed := Incl(t, tUsed),
				token(t)!inhibitors := Empty;
	RETURN;
	ENDPROCEDURE;

/*------------------*/
PROCEDURE			New_user;									/*  New_user  */
FPAR	IN/OUT		u				UserId,							/*------------------*/
	IN/OUT		result			Result;

DCL			c				ChannelId;
	START
	COMMENT	'Allocate a new user id or fail and return 0.
				';
	TASK		u := 0;
	DECISION numUserIds < parameters!maxUserIds;
	 (False):		TASK		result := RT_too_many_users;
				RETURN;
	ELSE:ENDDECISION;
	CALL		New_channel(c, result);
	DECISION result = RT_successful;
	 (True):		TASK		u := UserId(c),
							numUserIds := numUserIds + 1,

uUsed := Incl(u, uUsed),
							chan(c)!kind := UserId;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*----------------------*/
PROCEDURE			New_channel;									/*  New_channel  */
FPAR	IN/OUT		c				ChannelId,						/*----------------------*/
	IN/OUT		result			Result;

DCL			diagnostic	Diagnostic;
	START
	COMMENT	'Allocate a new channel id or fail and return 0.
				';
	TASK		c := 0;
	DECISION numChannelIds < parameters!maxChannelIds;
	 (False):		TASK		result := RT_too_many_channels;
				RETURN;
	ELSE:ENDDECISION;
	 1b :			/* keep trying */
	TASK		c := ANY(ChannelId);  /* randomize */
	DECISION c >= 1001 and c in cFree;
	 (False):		JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Take_channel(c, diagnostic);
	TASK		result := RT_successful;
	RETURN;
	ENDPROCEDURE;

/*---------------------*/
PROCEDURE			Open_portal;									/*  Open_portal  */
FPAR				p				PortalId,						/*---------------------*/
				pKind			PortalKind,
				pids			PIdByPri;

DCL			pid				PId,
				dp				DataPriority,
				c				ChannelId,
				cSet			ChannelIdSet,
				t				TokenId,
				tSet			TokenIdSet;
	START
	COMMENT	'Process an Open.portal input signal.
				Accept a new MCS connection or attachment to the domain.
				If this is an upward connection, prepare for merge.
				';
	DECISION pKind = Attached;
	 (True):		TASK		pid := pids(0),
							pids(1) := pid,
							pids(2) := pid,
							pids(3) := pid;
	ELSE:ENDDECISION;
	TASK		pFree := Del(p, pFree),
				pUsed := Incl(p, pUsed),
				portal(p)!kind := pKind,
				portal(p)!pids := pids,
				portal(p)!inCredit := inCredit(pKind),
				portal(p)!outCount := 0,
				portal(p)!interval := IF pKind = Uplink THEN 0 ELSE interval FI,
				portal(p)!subHeight := 0,
				portal(p)!subInterval := 0,
				dp := 0;
	 1b :			/* for dp = 0..? */

DECISION dp < parameters!numPriorities;
	 (True):		TASK		portal(p)!inBuffer(dp) := 0,
							portal(p)!outReady(dp) := False,
							portal(p)!outQueue(dp) := EmptyQueue,
							pBufferWait(dp) := Incl(p, pBufferWait(dp)),
							dp := dp + 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	DECISION portal(p)!kind = Uplink;
	 (True):		TASK		upward := p,
							merging := True,
							pdSend := True,
							edSend := True,
							uMerge := uUsed,
							uConfirm := Empty,
							cMerge := cUsed,
							cConfirm := Empty,
							tMerge := tUsed,
							tConfirm := Empty,
							cLeave := Empty,
							cDisband := Empty,
							tReject := Empty,
							cSet := cMerge;
				 2b :		/* for c in cSet */
				DECISION cSet = Empty;
				 (False):	TASK		c := Pick(cSet),
									cSet := Del(c, cSet),
									chan(c)!uMerge := chan(c)!admitted;
							JOIN 2b;
				ELSE:ENDDECISION;
				TASK		tSet := tMerge;
				 3b :		/* for t in tSet */
				DECISION tSet = Empty;
				 (False):	TASK		t := Pick(tSet),
									tSet := Del(t, tSet),
									token(t)!uMerge := token(t)!inhibitors;
							JOIN 3b;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	CALL		Erect_domain;
	CALL		Crank_engine;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Time_portal;									/*  Time_portal  */
FPAR				p				PortalId;						/*--------------------*/

START
	COMMENT	'Process a Time.portal input signal.
				Ensure that minimum throughput is maintained.
				Allow for the fact that outFlow takes integer steps.
				';
	DECISION portal(p)!elapsed > interval * (Float(portal(p)!outFlow) + 0.99);
	 (True):		OUTPUT	Report.portal(p, DC_throughput_inadequate) TO control;
	 (False):		TASK		portal(p)!outFlow := 0,
							portal(p)!elapsed := portal(p)!interval;
				SET(NOW + portal(p)!interval, Time.portal(p));
	ENDDECISION;
	CALL		Crank_engine;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Drop_portal;									/*  Drop_portal  */
FPAR				p				PortalId,						/*--------------------*/
				reason		Reason;

START
	COMMENT	'Process a Drop.portal input signal,
				knowing the only reason is user-requested.
				';
	DECISION p in pUsed and portal(p)!kind /= Attached;
	 (True):		TASK		pDrop := Incl(p, pDrop);
	ELSE:ENDDECISION;
	CALL		Crank_engine;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Shut_portal;									/*  Shut_portal  */
FPAR				p				PortalId;						/*--------------------*/

DCL			dp				DataPriority,
				b				BufferId,
				bSet			BufferIdSet,
				c				ChannelId,
				cSet			ChannelIdSet,
				u				UserId;
	START
	COMMENT	'Process a Shut.portal input signal.
				Remove the corresponding MCS connection or attachment.
				When the last one is gone, stop the domain process.
				';
	RESET(Time.portal(p));
	TASK		pUsed := Del(p, pUsed),
				pFree := Incl(p, pFree),
				pDrop := Del(p, pDrop),
				dp := 0;
	 1b :			/* for dp = 0..? */
	DECISION dp < parameters!numPriorities;
	 (True):		2b :		/* for b in outQueue(dp) */
				DECISION portal(p)!outQueue(dp) = EmptyQueue;
				 (False):	TASK		b := Next(portal(p)!outQueue(dp)),
									portal(p)!outQueue(dp) := Pull(portal(p)!outQueue(dp)),
									buffer(b)!portals := buffer(b)!portals - 1;
							CALL		Release_buffer(b);
							JOIN 2b;
				ELSE:ENDDECISION;
				TASK		b := portal(p)!inBuffer(dp),
							portal(p)!inBuffer(dp) := 0;
				CALL		Release_buffer(b);
				TASK		pBufferWait(dp) := Del(p, pBufferWait(dp)),
							pInputWait(dp) := Del(p, pInputWait(dp)),
							dp := dp + 1;
				JOIN 1b;
	ELSE:ENDDECISION;
	TASK		bSet := bUsed;
	 3b :			/* for b in bSet */
	DECISION bSet = Empty;
	 (False):		TASK		b := Pick(bSet),
							bSet := Del(b, bSet);
				DECISION buffer(b)!receiver = p;
				 (True):	TASK		buffer(b)!receiver := 0;
				ELSE:ENDDECISION;
				JOIN 3b;
	ELSE:ENDDECISION;
	TASK		cSet := cUsed;
	 4b :			/* for c in cSet */

DECISION cSet = Empty;
	 (False):		TASK		c := Pick(cSet),
							cSet := Del(c, cSet);
				DECISION p in chan(c)!joined;
				 (True):	TASK		chan(c)!joined := Del(p, chan(c)!joined);
							DECISION chan(c)!joined = Empty;
							 (True):	TASK	cLeave := Incl(c, cLeave);
							ELSE:ENDDECISION;
				ELSE:ENDDECISION;
				DECISION chan(c)!kind = UserId and chan(c)!portal = p;
				 (True):	TASK		chan(c)!portal := 0,
									u := UserId(c),
									uDetach := Incl(u, uDetach),
									cLeave := Del(c, cLeave);
				ELSE:ENDDECISION;
				JOIN 4b;
	ELSE:ENDDECISION;
	DECISION portal(p)!kind = Uplink;
	 (True):		TASK		upward := 0,
							merging := False,
							mcrqQueue := EmptyQueue,
							mtrqQueue := EmptyQueue,
							aurqQueue := EmptyQueue;
	 (False):		CALL		Clean_queue(p, mcrqQueue);
				CALL		Clean_queue(p, mtrqQueue);
				CALL		Clean_queue(p, aurqQueue);
	ENDDECISION;
	OUTPUT		Shut.portal(p) TO control;
	CALL		Erect_domain;
	CALL		Crank_engine;
	RETURN;
	ENDPROCEDURE;

/*----------------------*/
PROCEDURE			Clean_queue;									/*  Clean_queue  */
FPAR				p				PortalId,						/*----------------------*/
	IN/OUT		queue			PortalIdQueue;

DCL			x				PortalId,
				clean			PortalIdQueue;
	START
	COMMENT	'Remove a shut portal id from a queue.
				';
	TASK		clean := EmptyQueue;
	 1b :			/* for x in queue */
	DECISION queue = EmptyQueue;
	 (False):		TASK		x := Next(queue),
							queue := Pull(queue),
							x := IF x = p THEN 0 ELSE x FI,
							clean := Push(x, clean);
				JOIN 1b;
	ELSE:ENDDECISION;
	TASK		queue := clean;
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/
PROCEDURE			Erect_domain;									/*  Erect_domain  */
																/*-----------------------*/

DCL			p				PortalId,
				pSet			PortalIdSet,
				h				Natural,
				hmax			Natural,
				i				Duration,
				imax			Duration;

START
	COMMENT	'Recalculate the provider height and maxInterval.
				If either changed, communicate them upward.
				';
	TASK		hmax := 1,
				imax := 0,
				pSet := pUsed;
	 1b :			/* for p in pSet */
	DECISION pSet = Empty;
	 (False):		TASK		p := Pick(pSet),
							pSet := Del(p, pSet),
							h := portal(p)!subHeight + 1,
							hmax := IF h > hmax THEN h ELSE hmax FI,
							i := portal(p)!interval,
							imax := IF i > imax THEN i ELSE imax FI;
				JOIN 1b;
	ELSE:ENDDECISION;
	DECISION height = hmax and maxInterval = imax;
	 (False):		TASK		height := hmax,
							maxInterval := imax,
							edSend := True;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-------------------------*/
PROCEDURE			Identify_sender;									/*  Identify_sender  */
FPAR	IN/OUT		p				PortalId,						/*-------------------------*/
	IN/OUT		dp				DataPriority;

DCL			pSet			PortalIdSet;
	START
	COMMENT	'An alternative would be to carry this
				information explicitly in SDL signals.
				';
	TASK		pSet := pUsed;
	 1b :			/* for p in pSet */
	DECISION pSet = Empty;
	 (False):		TASK		p := Pick(pSet),
							pSet := Del(p, pSet),
							dp := 0;
				 2b :		/* for dp = 0..? */
				DECISION dp < parameters!numPriorities;
				 (True):	DECISION portal(p)!pids(dp) = SENDER;
							 (True):	RETURN;
							ELSE:ENDDECISION;
							TASK		dp := dp + 1;
							JOIN 2b;
				ELSE:ENDDECISION;
				JOIN 1b;
	ELSE:ENDDECISION;
	TASK		p := 0,
				dp := 0;
	RETURN;
	ENDPROCEDURE;

/*-------------------*/
PROCEDURE			PDU_ready;									/*  PDU_ready  */
FPAR				dp				DataPriority;						/*-------------------*/
	DCL			p				PortalId,
				x				DataPriority,
				b				BufferId;

START
	COMMENT	'Process a PDU.ready input signal.
				If a buffer is waiting, it can be output.
				';
	CALL		Identify_sender(p, x);
	DECISION p in pUsed;
	 (False):		RETURN;
	ELSE:ENDDECISION;
	DECISION portal(p)!outQueue(dp) = EmptyQueue;
	 (True):		TASK		portal(p)!outReady(dp) := True;
	 (False):		TASK		b := Next(portal(p)!outQueue(dp)),
							portal(p)!outQueue(dp) := Pull(portal(p)!outQueue(dp)),
							buffer(b)!portals := buffer(b)!portals - 1;
				CALL		Output_buffer(b, p);
				TASK		portal(p)!outReady(dp) := False;
				CALL		Release_buffer(b);
				TASK		portal(p)!outFlow := portal(p)!outFlow + 1,
							portal(p)!outCount := portal(p)!outCount - 1;
				DECISION portal(p)!outCount = 0;
				 (True):	RESET(Time.portal(p));
				ELSE:ENDDECISION;
	ENDDECISION;
	CALL		Crank_engine;
	RETURN;
	ENDPROCEDURE;

/*------------------*/
PROCEDURE			Input_PDU;									/*  Input_PDU  */
FPAR				pdu			PDUStruct;						/*------------------*/

DCL			p				PortalId,
				dp				DataPriority,
				b				BufferId;
	START
	COMMENT	'Process an MCSPDU input signal.
				If merging, requests and responses must wait.
				';
	CALL		Identify_sender(p, dp);
	DECISION p in pUsed;
	 (False):		RETURN;
	ELSE:ENDDECISION;
	DECISION portal(p)!kind = Attached;
	 (True):		DECISION pdu!kind;
				 (SDrq, SDin, USrq, USin):
						TASK	dp := pdu!dataPriority,
								dp := IF dp < parameters!numPriorities THEN dp
													ELSE parameters!numPriorities - 1 FI;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	TASK		b := portal(p)!inBuffer(dp),
				buffer(b)!pdu := pdu;
	DECISION p = upward or not merging;
	 (True):		CALL		Process_PDU(p, dp);
	 (False):		TASK		pInputWait(dp) := Incl(p, pInputWait(dp));
	ENDDECISION;
	CALL		Crank_engine;
	RETURN;
	ENDPROCEDURE;

/*-------------------------*/
PROCEDURE			Allocate_buffer;									/*  Allocate_buffer  */
FPAR	IN/OUT		b				BufferId;						/*-------------------------*/

START
	COMMENT	'Allocate a free buffer id or fail and return 0.
				';

DECISION b /= 0 and buffer(b)!portals = 0;
	 (True):		RETURN;
	ELSE:ENDDECISION;
	TASK		b := 0;
	DECISION bFree = Empty;
	 (False):		TASK		b := Pick(bFree),
							bFree := Del(b, bFree),
							bUsed := Incl(b, bUsed),
							buffer(b)!receiver := 0,
							buffer(b)!dataPriority := 0,
							buffer(b)!portals := 0;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Cast_buffer;									/*  Cast_buffer  */
FPAR				b				BufferId,						/*--------------------*/
				p				PortalId;

DCL			dp				DataPriority;
	START
	COMMENT	'Send buffer containing an MCSPDU to a single output.
				';
	DECISION p = 0;
	 (True):		RETURN;
	ELSE:ENDDECISION;
	 TASK		dp := buffer(b)!dataPriority;
	DECISION portal(p)!outReady(dp);
	 (True):		CALL		Output_buffer(b, p);
				TASK		portal(p)!outReady(dp) := False;
	 (False):		DECISION portal(p)!outCount = 0 and portal(p)!interval > 0;
				 (True):	TASK	portal(p)!outFlow := 0,
								portal(p)!elapsed := portal(p)!interval;
							SET(NOW + portal(p)!interval, Time.portal(p));
				ELSE:ENDDECISION;
				TASK		portal(p)!outQueue(dp) := Push(b, portal(p)!outQueue(dp)),
							buffer(b)!portals := buffer(b)!portals + 1,
							portal(p)!outCount := portal(p)!outCount + 1;
	ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-------------------------*/
PROCEDURE			Release_buffer;									/*  Release_buffer  */
FPAR				b				BufferId;						/*-------------------------*/

DCL			p				PortalId,
				dp				DataPriority;
	START
	COMMENT	'Release a buffer that is no longer needed.
				';
	DECISION b = 0 or buffer(b)!portals > 0;
	 (True):		RETURN;
	ELSE:ENDDECISION;
	TASK		bUsed := Del(b, bUsed),
				bFree := Incl(b, bFree),
				p := buffer(b)!receiver,
				dp := buffer(b)!dataPriority;
	DECISION p = 0;
	 (False):		TASK		portal(p)!inCredit(dp) := portal(p)!inCredit(dp) + 1;
				DECISION portal(p)!inBuffer(dp) = 0;
				 (True):	TASK	pBufferWait(dp) := Incl(p, pBufferWait(dp));
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/
PROCEDURE			Route_user;									/*  Route_user  */
FPAR				u				UserId,							/*--------------------*/
	IN/OUT		p				PortalId;

DCL			c				ChannelId;
	START
	COMMENT	'Return the portal id that leads toward a user.
				';
	TASK		p := 0;
	DECISION u in uUsed;
	 (True):		TASK		c := ChannelId(u),
							p := chan(c)!portal;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*--------------------------*/
PROCEDURE			Multicast_buffer;								/*  Multicast_buffer  */
FPAR				b				BufferId,						/*--------------------------*/
				uSet			UserIdSet;

DCL			u				UserId,
				p				PortalId,
				pSet			PortalIdSet;
	START
	COMMENT	'Send buffer containing an MCSPDU to multiple users.
				';
	TASK		pSet := Empty;
	 1b :			/* for u in uSet */
	DECISION uSet = Empty;
	 (False):		TASK		u := Pick(uSet),
							uSet := Del(u, uSet);
				CALL		Route_user(u, p);
				TASK		pSet := Incl(p, pSet);
				JOIN 1b;
	ELSE:ENDDECISION;
	 2b :			/* for p in pSet */
	DECISION pSet = Empty;
	 (False):		TASK		p := Pick(pSet),
							pSet := Del(p, pSet);
				CALL		Cast_buffer(b, p);
				JOIN 2b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*----------------------------*/
PROCEDURE			Broadcast_buffer;								/*  Broadcast_buffer  */
FPAR				b				BufferId;						/*----------------------------*/

DCL			pdu			PDUStruct,
				p				PortalId,
				pSet			PortalIdSet;
	START
	COMMENT	'Send buffer containing an MCSPDU to the whole subtree.
				';
	TASK		pdu := buffer(b)!pdu;
	DECISION (pdu!kind = PCin and pdu!detachUserIds = Empty
								and pdu!purgeChannelIds = Empty)
				or (pdu!kind = PTin and pdu!purgeTokenIds = Empty)
				or (pdu!kind = DUin and pdu!userIds = Empty);
	 (True):		RETURN;
	ELSE:ENDDECISION;
	TASK		pSet := pUsed;
	 1b :			/* for p in pSet */

DECISION pSet = Empty;
	 (False):		TASK		p := Pick(pSet),
							pSet := Del(p, pSet);
				DECISION portal(p)!kind;
				 (Attached):
							DECISION pdu!kind = PDin;
							 (False):CALL		Cast_buffer(b, p);
							ELSE:ENDDECISION;
				 (Downlink):
							CALL		Cast_buffer(b, p);
				ELSE:ENDDECISION;
				JOIN 1b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/
PROCEDURE			Crank_engine;									/*  Crank_engine  */
																/*-----------------------*/
	DCL			b				BufferId,
				p				PortalId,
				dp				DataPriority;
	START
	COMMENT	'After individual processing of each input signal,
				look for the most deserving work to do next.
				This advances incrementally through stages of a merge;
				else it cleans up users, channels, tokens left behind.
				Buffers are assigned to accept new inputs, with preference
				to PDUs flowing downward and to higher priorities first.
				';
	TASK		b := 0;
	DECISION pdSend;
	 (True):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		pdSend := False,
							buffer(b)!pdu!kind := PDin,
							buffer(b)!pdu!heightLimit := parameters!maxHeight - 1;
				CALL		Broadcast_buffer(b);
	ELSE:ENDDECISION;
	DECISION edSend;
	 (True):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		edSend := False,
							buffer(b)!pdu!kind := EDrq,
							buffer(b)!pdu!subHeight := height,
							buffer(b)!pdu!subInterval := maxInterval;
				CALL		Cast_buffer(b, upward);
	ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	TASK		dp := 0;
	 1b :			/* for dp = 0..? */
	DECISION dp < parameters!numPriorities;
	 (True):		DECISION upward in pBufferWait(dp);
				 (False):	TASK		dp := dp + 1;
							JOIN 1b;
				ELSE:ENDDECISION;
				TASK		p := upward,
							b := 0;
				CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;

TASK		pBufferWait(dp) := Del(p, pBufferWait(dp)),
							buffer(b)!receiver := p,
							buffer(b)!dataPriority := dp,
							portal(p)!inCredit(dp) := portal(p)!inCredit(dp) - 1,
							portal(p)!inBuffer(dp) := b;
				OUTPUT	PDU.ready(dp) TO portal(p)!pids(dp);
				JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Drop_portals;
	DECISION merging;
	 (True):		CALL		Merge_users;
				DECISION uConfirm = uUsed;
				 (True):	CALL		Merge_tokens;
							DECISION tConfirm = tUsed;
							 (True):	CALL  Merge_channels;
									DECISION cConfirm = cUsed;
									 (True):	TASK	merging := False;
									ELSE:ENDDECISION;
							ELSE:ENDDECISION;
				ELSE:ENDDECISION;
				DECISION merging;
				 (True):	RETURN;
				 (False):	TASK		dp := 0;
							 2b :		/* for dp = 0..? */
							DECISION dp < parameters!numPriorities;
							 (True):	/* for p in pInputWait(dp) */
									DECISION pInputWait(dp) = Empty;
									 (True):	TASK	dp := dp + 1;
											JOIN 2b;
									ELSE:ENDDECISION;
									TASK	p := Pick(pInputWait(dp)),
											pInputWait(dp) := Del(p, pInputWait(dp));
									CALL	Process_PDU(p, dp);
									JOIN 2b;
							ELSE:ENDDECISION;
				ENDDECISION;
	ELSE:ENDDECISION;
	CALL		Detach_users;
	CALL		Leave_channels;
	CALL		Disband_channels;
	CALL		Reject_tokens;
	TASK		dp := 0;
	 3b :			/* for dp = 0..? */
	DECISION dp < parameters!numPriorities;
	 (True):		/* for p in pBufferWait(dp) */
				DECISION pBufferWait(dp) = Empty;
				 (True):	TASK		dp := dp + 1;
							JOIN 3b;
				ELSE:ENDDECISION;
				TASK		p := Pick(pBufferWait(dp)),
							b := 0;
				CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		pBufferWait(dp) := Del(p, pBufferWait(dp)),
							buffer(b)!receiver := p,
							buffer(b)!dataPriority := dp,
							portal(p)!inCredit(dp) := portal(p)!inCredit(dp) - 1,
							portal(p)!inBuffer(dp) := b;
				OUTPUT	PDU.ready(dp) TO portal(p)!pids(dp);
				JOIN 3b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*----------------------*/
PROCEDURE			Drop_portals;									/*  Drop_portals  */
																/*----------------------*/
	DCL			b				BufferId,
				p				PortalId,
				pdu			PDUStruct;
	START
	COMMENT	'Generate DPum MCSPDUs requested by controller.
				';
	TASK		b := 0,
				pdu!kind := DPum,
				pdu!reason := RN_user_requested;
	 1b :			/* for p in pDrop */
	DECISION pDrop = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		p := Pick(pDrop),
							pDrop := Del(p, pDrop),
							buffer(b)!pdu := pdu;
				CALL		Cast_buffer(b, p);
				JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*----------------------*/
PROCEDURE			Merge_users;									/*  Merge_users  */
																/*----------------------*/
	DCL			b				BufferId,
				u				UserId,
				c				ChannelId,
				pdu			PDUStruct,
				cAttr			ChannelAttributes;
	START
	COMMENT	'Generate MCrq MCSPDUs to merge user ids upward.
				Fill them to maximum size that encoding allows.
				';
	TASK		b := 0,
				pdu!kind := MCrq,
				pdu!mergeChannels := Empty;
	 1b :			/* for u in uMerge */
	DECISION uMerge = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		u := Pick(uMerge),
							uMerge := Del(u, uMerge),
							c := ChannelId(u),
							cMerge := Del(c, cMerge),
							cAttr!channelId := c,
							cAttr!kind := UserId,
							cAttr!joined := (chan(c)!joined /= Empty),
							pdu!mergeChannels := Incl(cAttr, pdu!mergeChannels);
				DECISION'encoding of pdu + 9 <= maxMCSPDUsize';
				 ('True'):	JOIN 1b;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	DECISION pdu!mergeChannels = Empty;
	 (False):		TASK		buffer(b)!pdu := pdu,
							pdu!mergeChannels := Empty;
				CALL		Cast_buffer(b, upward);
				JOIN 1b;

ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*---------------------------*/
PROCEDURE			Merge_channels;								/*  Merge_channels  */
																/*---------------------------*/
	DCL			b				BufferId,
				c				ChannelId,
				u				UserId,
				pdu			PDUStruct,
				cAttr			ChannelAttributes;
	START
	COMMENT	'Generate MCrq MCSPDUs to merge channel ids upward.
				Fill them to maximum size that encoding allows.
				';
	TASK		b := 0,
				pdu!kind := MCrq,
				pdu!mergeChannels := Empty,
				pdu!purgeChannelIds := Empty;
	 1b :			/* for c in cMerge */
	DECISION cMerge = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		c := Pick(cMerge),
							cMerge := Del(c, cMerge),
							cAttr!channelId := c,
							cAttr!kind := chan(c)!kind,
							cAttr!manager := chan(c)!manager,
							cAttr!admitted := Empty,
							cAttr!joined := (chan(c)!joined /= Empty);
				DECISION (chan(c)!kind = Static or chan(c)!kind = Assigned)
							and chan(c)!joined = Empty;
				 (True):	CALL		Delete_channel(c);
							JOIN 1b;
				ELSE:ENDDECISION;
				DECISION chan(c)!kind = Private;
				 (True):	DECISION chan(c)!manager in uUsed;
							 (False):	TASK	pdu!purgeChannelIds :=
														Incl(c, pdu!purgeChannelIds);
									JOIN 3f;
							ELSE:ENDDECISION;
							 2b :		/* for u in chan(c)!uMerge */
							DECISION chan(c)!uMerge = Empty;
							 (False):	TASK	u := Pick(chan(c)!uMerge),
											chan(c)!uMerge := Del(u, chan(c)!uMerge),
											cAttr!admitted := Incl(u, cAttr!admitted),
											pdu!mergeChannels :=	/* try this many */
														Incl(cAttr, pdu!mergeChannels);
									DECISION'encoding of pdu + 4 <= maxMCSPDUsize';
									 ('True'):	TASK	pdu!mergeChannels := /* try another */
														Del(cAttr, pdu!mergeChannels);
											JOIN 2b;
									ELSE:ENDDECISION;
							ELSE:ENDDECISION;
				ELSE:ENDDECISION;
				TASK		pdu!mergeChannels := Incl(cAttr, pdu!mergeChannels);
				 3f :
				DECISION'encoding of pdu + 23 <= maxMCSPDUsize';
				 ('True'):	JOIN 1b;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;

DECISION pdu!mergeChannels = Empty and pdu!purgeChannelIds = Empty;
	 (False):		TASK		buffer(b)!pdu := pdu,
							pdu!mergeChannels := Empty,
							pdu!purgeChannelIds := Empty;
				CALL		Cast_buffer(b, upward);
				JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*------------------------*/
PROCEDURE			Merge_tokens;									/*  Merge_tokens  */
																/*------------------------*/
	DCL			b				BufferId,
				t				TokenId,
				u				UserId,
				pdu			PDUStruct,
				tAttr			TokenAttributes;
	START
	COMMENT	'Generate MTrq MCSPDUs to merge token ids upward.
				Fill them to maximum size that encoding allows.
				';
	TASK		b := 0,
				pdu!kind := MTrq,
				pdu!mergeTokens := Empty,
				pdu!purgeTokenIds := Empty,
	 1b :			/* for t in tMerge */
	DECISION tMerge = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		t := Pick(tMerge),
							tMerge := Del(t, tMerge),
							tAttr!tokenId := t,
							tAttr!kind := token(t)!kind,
							tAttr!grabber := token(t)!grabber,
							tAttr!recipient := token(t)!recipient,
							tAttr!inhibitors := Empty;
				DECISION token(t)!kind = Inhibited;
				 (True):	2b :		/* for u in token(t)!uMerge */
							DECISION token(t)!uMerge = Empty;
							 (False):	TASK	u := Pick(token(t)!uMerge),
											token(t)!uMerge := Del(u, token(t)!uMerge),
											tAttr!inhibitors := Incl(u, tAttr!inhibitors),
											pdu!mergeTokens :=		/* try this many */
															Incl(tAttr, pdu!mergeTokens);
									DECISION'encoding of pdu + 4 <= maxMCSPDUsize';
									 ('True'):	TASK	pdu!mergeTokens := /* try another */
															Del(tAttr, pdu!mergeTokens);
											JOIN 2b;
									ELSE:ENDDECISION;
							ELSE:ENDDECISION;
				ELSE:ENDDECISION;
				TASK		pdu!mergeTokens := Incl(tAttr, pdu!mergeTokens);
				DECISION'encoding of pdu + 16 <= maxMCSPDUsize';
				 ('True'):	JOIN 1b;
	ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	DECISION pdu!mergeTokens = Empty;
	 (False):		TASK		buffer(b)!pdu := pdu,
							pdu!mergeTokens := Empty;
				CALL		Cast_buffer(b, upward);
				JOIN 1b;

ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/
PROCEDURE			Detach_users;									/*  Detach_users  */
																/*-----------------------*/
	DCL			b				BufferId,
				u				UserId,
				pdu			PDUStruct,
				diagnostic	Diagnostic;
	START
	COMMENT	'Generate DUrq MCSPDUs to detach users.
				Fill them to maximum size that encoding allows.
				';
	TASK		b := 0,
				pdu!kind := DUrq,
				pdu!userIds := Empty;
	 1b :			/* for u in uDetach */
	DECISION uDetach = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		u := Pick(uDetach),
							uDetach := Del(u, uDetach),
							uRevoke := Del(u, uRevoke),
							pdu!reason := RN_domain_disconnected,
							pdu!userIds := Incl(u, pdu!userIds);
				DECISION'encoding of pdu + 4 <= maxMCSPDUsize';
				 ('True'):	JOIN 1b;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	DECISION pdu!userIds = Empty;
	 (False):		JOIN 3f;
	ELSE:ENDDECISION;
	 2b :			/* for u in uRevoke */
	DECISION uRevoke = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		u := Pick(uRevoke),
							uRevoke := Del(u, uRevoke),
							pdu!reason := RN_channel_purged,
							pdu!userIds := Incl(u, pdu!userIds);
				DECISION'encoding of pdu + 4 <= maxMCSPDUsize';
				 ('True'):	JOIN 1b;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	 3f :
	DECISION pdu!userIds = Empty;
	 (False):		TASK		buffer(b)!pdu := pdu,
							pdu!userIds := Empty;
				DECISION upward = 0;
				 (False):	CALL		Cast_buffer(b, upward);
				 (True):	TASK		buffer(b)!pdu!kind := DUin;
							CALL		Apply_PDU(0, b, diagnostic);
				ENDDECISION;
				JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*--------------------------*/
PROCEDURE			Leave_channels;								/*  Leave_channels  */
																/*--------------------------*/
	DCL			b				BufferId,
				c				ChannelId,
				pdu			PDUStruct;
	START
	COMMENT	'Generate CLrq MCSPDUs to leave channels.
				Fill them to maximum size that encoding allows.
				';
	TASK		b := 0,
				pdu!kind := CLrq,
				pdu!channelIds := Empty;
	 1b :			/* for c in cLeave */
	DECISION cLeave = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		c := Pick(cLeave),
							cLeave := Del(c, cLeave);
				DECISION chan(c)!kind;
				 (Static, Assigned):
							CALL		Delete_channel(c);
				ELSE:ENDDECISION;
				TASK		pdu!channelIds := Incl(c, pdu!channelIds);
				DECISION'encoding of pdu + 4 <= maxMCSPDUsize';
				 ('True'):	JOIN 1b;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	DECISION pdu!channelIds = Empty;
	 (False):		TASK		buffer(b)!pdu := pdu,
							pdu!channelIds := Empty;
				CALL		Cast_buffer(b, upward);
				JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*-----------------------------*/
PROCEDURE			Disband_channels;								/*  Disband_channels  */
																/*-----------------------------*/
	DCL			b				BufferId,
				c				ChannelId,
				pdu			PDUStruct,
				diagnostic	Diagnostic;
	START
	COMMENT	'Generate CDrq MCSPDUs to disband channels.
				';
	TASK		b := 0,
				pdu!kind := CDrq;
	 1b :			/* for c in cDisband */
	DECISION cDisband = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		c := Pick(cDisband),
							cDisband := Del(c, cDisband),
							pdu!channelId := c,
							buffer(b)!pdu := pdu;

DECISION upward = 0;
				 (False):	CALL		Cast_buffer(b, upward);
				 (True):	TASK		buffer(b)!pdu!kind := CDin;
							CALL		Apply_PDU(0, b, diagnostic);
				ENDDECISION;
				JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*------------------------*/
PROCEDURE			Reject_tokens;									/*  Reject_tokens  */
																/*------------------------*/
	DCL			b					BufferId,
				t					TokenId,
				u					UserId,
				p					PortalId,
				pdu				PDUStruct;
	START
	COMMENT	'Generate TVcf MCSPDUs to reject tokens.
				';
	TASK		b := 0,
				pdu!kind := TVcf,
				pdu!result := RT_no_such_user;
	 1b :			/* for t in tReject */
	DECISION tReject = Empty;
	 (False):		CALL		Allocate_buffer(b);
				DECISION b = 0;
				 (True):	RETURN;
				ELSE:ENDDECISION;
				TASK		t := Pick(tReject),
							tReject := Del(t, tReject),
							token(t)!kind := Grabbed,
							u := token(t)!grabber,
							pdu!initiator := u,
							pdu!tokenId := t;
				CALL		Token_status(pdu);
				TASK		buffer(b)!pdu := pdu;
				CALL		Route_user(u, p);
				CALL		Cast_buffer(b, p);
				JOIN 1b;
	ELSE:ENDDECISION;
	CALL		Release_buffer(b);
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/
PROCEDURE			Process_PDU;									/*  Process_PDU  */
FPAR				r				PortalId,						/*-----------------------*/
				dp				DataPriority;

DCL			b				BufferId,
				pdu			PDUStruct,
				diagnostic	Diagnostic;
	START
	COMMENT	'This is the main line of processing an MCSPDU.
				If its content is invalid, ignore or reject it.
				Else if this is the top provider, take the special
				actions of Top_provider.  Then whether or not this
				is the top provider, call Apply_PDU.
				';
	TASK		b := portal(r)!inBuffer(dp),
				pdu := buffer(b)!pdu;

DECISION pdu!kind = RJum;
	 (True):		TASK		diagnostic := pdu!diagnostic;
	 (False):		CALL		Validate_input(r, b, diagnostic);
				DECISION diagnostic = DC_OK;
				 (True):	DECISION buffer(b)!pdu!kind;
							 (MCrq):	TASK	mcrqQueue := Push(r, mcrqQueue);
							 (MTrq):	TASK	mtrqQueue := Push(r, mtrqQueue);
							 (AUrq):	TASK	aurqQueue := Push(r, aurqQueue);
							ELSE:ENDDECISION;
							DECISION upward = 0;
							 (True):	CALL	Top_provider(r, b);
							ELSE:ENDDECISION;
							CALL		Apply_PDU(r, b, diagnostic);
				ELSE:ENDDECISION;
	ENDDECISION;
	DECISION diagnostic;
	 (DC_OK, DC_ignore):
				/* no action */
	ELSE:
				OUTPUT	Report.portal(r, diagnostic) TO control;
				DECISION pdu!kind = RJum;
				 (False):	TASK'pdu!initialOctets := truncate pdu';
				ELSE:ENDDECISION;
				TASK		pdu!kind := RJum,
							pdu!diagnostic := diagnostic,
							buffer(b)!pdu := pdu,
							dp := buffer(b)!dataPriority,
							buffer(b)!dataPriority := 0;
				CALL		Cast_buffer(b, r);
				TASK		buffer(b)!dataPriority := dp;
	ENDDECISION;
	CALL		Release_buffer(b);
	TASK		portal(r)!inBuffer(dp) := 0;
	DECISION	portal(r)!inCredit(dp) > 0;
	 (True):		TASK		pBufferWait(dp) := Incl(r, pBufferWait(dp));
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/
PROCEDURE			Validate_input;									/*  Validate_input  */
FPAR				r				PortalId,						/*-----------------------*/
				b				BufferId,
	IN/OUT		diagnostic	Diagnostic;

DCL			pdu			PDUStruct,
				p				PortalId,
				dp				DataPriority,
				u				UserId,
				uSet			UserIdSet,
				c				ChannelId,
				cSet			ChannelIdSet,
				cAttr			ChannelAttributes,
				cAttrSet		ChannelAttributesSet,
				t				TokenId,
				tSet			TokenIdSet,
				tAttr			TokenAttributes,
				tAttrSet		TokenAttributesSet;
	START
	COMMENT	'Validate the direction and user ids of an MCSPDU
				and perform other checks, depending on its type.
				Top_provider and Apply_PDU trust this procedure
				to catch important anomalies and ease their logic.
				';
	TASK		pdu := buffer(b)!pdu,
				diagnostic := DC_OK;

DECISION	portal(r)!kind;
	 (Downlink):
				DECISION pdu!kind;
				 (EDrq, MCrq, MTrq, DPum, RJum,
				  AUrq, DUrq, CJrq, CLrq, CCrq,
				  CDrq, CArq, CErq, SDrq, USrq,
				  TGrq, TIrq, TVrq, TVrs, TPrq,
				  TRrq, TTrq):
							/* OK */
				ELSE:
							TASK		diagnostic := DC_forbidden_PDU_upward;
							RETURN;
				ENDDECISION;
	 (Uplink):
				DECISION pdu!kind;
				 (PDin, MCcf, PCin, MTcf, PTin,
				  DPum, RJum, AUcf, DUin, CJcf,
				  CCcf, CDin, CAin, CEin, SDin,
				  USin, TGcf, TIcf, TVin, TVcf,
				  TPin, TRcf, TTcf):
							/* OK */
				ELSE:
							TASK		diagnostic := DC_forbidden_PDU_downward;
							RETURN;
				ENDDECISION;
	ELSE:ENDDECISION;
	TASK		dp := 0;
	DECISION pdu!kind;
	 (SDrq, SDin, USrq, USin):
				TASK		dp := pdu!dataPriority,
							dp := IF dp < parameters!numPriorities THEN dp
											ELSE parameters!numPriorities - 1 FI;
	ELSE:ENDDECISION;
	DECISION buffer(b)!dataPriority = dp;
	 (False):		TASK		diagnostic := DC_wrong_transport_priority;
				RETURN;
	ELSE:ENDDECISION;
	DECISION pdu!kind;
	 (MCrq):		TASK		cAttrSet := pdu!mergeChannels,
							cSet := pdu!purgeChannelIds;
				 1b :		/* for cAttr in cAttrSet */
				DECISION cAttrSet = Empty;
				 (False):	TASK		cAttr := Pick(cAttrSet),
									cAttrSet := Del(cAttr, cAttrSet),
									c := cAttr!channelId;
							DECISION c in cSet;
							 (True):	TASK	diagnostic := DC_inconsistent_merge;
									RETURN;
							ELSE:ENDDECISION;
							TASK		cSet := Incl(c, cSet);
							DECISION cAttr!kind = Private;
							 (False):	JOIN 1b;
							ELSE:ENDDECISION;
							TASK		pdu!mergeChannels := Del(cAttr, pdu!mergeChannels);
							CALL		Route_user(cAttr!manager, p);
							DECISION p = r;
							 (False):	TASK	pdu!purgeChannelIds :=
															Incl(c, pdu!purgeChannelIds);
									JOIN 1b;
							ELSE:ENDDECISION;
							TASK		uSet := cAttr!admitted;
							 2b :		/* for u in uSet */
							DECISION uSet = Empty;
							 (False):	TASK	u := Pick(uSet),
											uSet := Del(u, uSet);
									CALL	Route_user(u, p);

DECISION p = r;
									 (False):	TASK	cAttr!admitted :=
															Del(u, cAttr!admitted);
									ELSE:ENDDECISION;
									JOIN 2b;
							ELSE:ENDDECISION;
							TASK		pdu!mergeChannels := Incl(cAttr, pdu!mergeChannels);
							JOIN 1b;
				ELSE:ENDDECISION;
	 (MTrq):		TASK		tAttrSet := pdu!mergeTokens,
							tSet := pdu!purgeTokenIds;
				 3b :		/* for tAttr in tAttrSet */
				DECISION tAttrSet = Empty;
				 (False):	TASK		tAttr := Pick(tAttrSet),
									tAttrSet := Del(tAttr, tAttrSet),
									t := tAttr!tokenId;
							DECISION t in tSet;
							 (True):	TASK	diagnostic := DC_inconsistent_merge;
									RETURN;
							ELSE:ENDDECISION;
							TASK		tSet := Incl(t, tSet),
									pdu!mergeTokens := Del(tAttr, pdu!mergeTokens);
							DECISION tAttr!kind;
							 (Grabbed, Ungivable):
									CALL	Route_user(tAttr!grabber, p);
									DECISION p = r;
									 (False):	JOIN 4f;
									ELSE:ENDDECISION;
							 (Given):
									CALL	Route_user(tAttr!recipient, p);
									DECISION p = r;
									 (False):	JOIN 4f;
									ELSE:ENDDECISION;
							 (Giving):
									CALL	Route_user(tAttr!recipient, p);
									DECISION p = r;
									 (True):	CALL  Route_user(tAttr!grabber, p);
											DECISION p = r;
											 (False):	TASK	tAttr!kind := Given;
											ELSE:ENDDECISION;
									 (False):	TASK	tAttr!kind := Ungivable;
											CALL	Route_user(tAttr!grabber, p);
											DECISION p = r;
											 (False): 4f :
													TASK	pdu!purgeTokenIds :=
																Incl(t, pdu!purgeTokenIds);
													JOIN 3b;
											ELSE:ENDDECISION;
									ENDDECISION;
							 (Inhibited):
									TASK	uSet := tAttr!inhibitors;
									 5b :	/* for u in uSet */
									DECISION uSet = Empty;
									 (False):	TASK	u := Pick(uSet),
													uSet := Del(u, uSet);
											CALL	Route_user(u, p);
											DECISION p = r;
											 (False):	TASK	tAttr!inhibitors :=
																Del(u, tAttr!inhibitors);
											ELSE:ENDDECISION;
											JOIN 5b;
									ELSE:ENDDECISION;
							ENDDECISION;
							TASK		pdu!mergeTokens := Incl(tAttr, pdu!mergeTokens);
							JOIN 3b;
				ELSE:ENDDECISION;

(DUrq):		TASK		uSet := pdu!userIds;
				 6b :		/* for u in uSet */
				DECISION uSet = Empty;
				 (False):	TASK		u := Pick(uSet),
									uSet := Del(u, uSet);
							CALL		Route_user(u, p);
							DECISION p = r;
							 (False):	TASK	pdu!userIds := Del(u, pdu!userIds);
							ELSE:ENDDECISION;
							JOIN 6b;
				ELSE:ENDDECISION;
				DECISION pdu!userIds = Empty;
				 (True):	TASK		diagnostic := DC_ignore;
							RETURN;
				ELSE:ENDDECISION;
	 (CJrq, CCrq, CDrq, CArq, CErq,
	  SDrq, USrq, TGrq, TIrq, TVrq,
	  TPrq, TRrq, TTrq):
				CALL		Route_user(pdu!initiator, p);
				DECISION p = r;
				 (False):	TASK		diagnostic := DC_ignore;
							RETURN;
				ELSE:ENDDECISION;
	 (TVin):		DECISION pdu!recipient in uUsed;
				 (False):	TASK		diagnostic := DC_misrouted_user;
							RETURN;
				ELSE:ENDDECISION;
	 (TVrs):		CALL		Route_user(pdu!recipient, p);
				DECISION p = r;
				 (False):	TASK		diagnostic := DC_ignore;
							RETURN;
				ELSE:ENDDECISION;
	 (CJcf, CCcf, TGcf, TIcf, TVcf,
	  TRcf, TTcf):
				DECISION pdu!initiator in uUsed;
				 (False):	TASK		diagnostic := DC_misrouted_user;
							RETURN;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	DECISION pdu!kind;
	 (CDrq, CArq, CErq):
				TASK		c := pdu!channelId;
				DECISION c in cUsed and chan(c)!kind = Private
							and pdu!initiator = chan(c)!manager;
				 (False):	TASK		diagnostic := DC_ignore;
							RETURN;
				ELSE:ENDDECISION;
	 (SDrq, USrq):
				TASK		c := pdu!channelId;
				DECISION c in cUsed and chan(c)!kind = Private;
				 (True):	DECISION pdu!initiator in chan(c)!admitted;
							 (False):	TASK	diagnostic := DC_ignore;
									RETURN;
							ELSE:ENDDECISION;
				ELSE:ENDDECISION;
	 (MCcf):		DECISION not merging and mcrqQueue = EmptyQueue;
				 (True):	TASK		diagnostic := DC_unrequested_confirm;
							RETURN;
				ELSE:ENDDECISION;
	 (MTcf):		DECISION not merging and mtrqQueue = EmptyQueue;
				 (True):	TASK		diagnostic := DC_unrequested_confirm;
							RETURN;
				ELSE:ENDDECISION;

(AUcf):		DECISION aurqQueue = EmptyQueue;
				 (True):	TASK		diagnostic := DC_unrequested_confirm;
							RETURN;
				ELSE:ENDDECISION;
	 (CJcf, CCcf, TGcf, TIcf, TVcf,
	  TRcf, TTcf):
				DECISION merging;
				 (True):	TASK		diagnostic := DC_unrequested_confirm;
							RETURN;
				ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	TASK		buffer(b)!pdu := pdu;
	RETURN
	COMMENT 	'Additional tests might be applied to check
				that MCSPDUs flowing downward are consistent
				with user or channel states already recorded.
				But stronger assertions could contain flaws,
				and such defenses are not necessary for the
				continued functioning of this MCS provider.
				The value of an MCS domain involves trust in
				the correct operation of superior providers.
				';
	ENDPROCEDURE;

/*----------------------*/
PROCEDURE			Top_provider;									/*  Top_provider  */
FPAR				r				PortalId,						/*----------------------*/
				b				BufferId;

DCL			pdu			PDUStruct,
				new			PDUStruct,
				u				UserId,
				c				ChannelId,
				cAttr			ChannelAttributes,
				t				TokenId,
				tAttr			TokenAttributes,
				result			Result,
				diagnostic	Diagnostic;
	START
	COMMENT	'The buffer containing an MCSPDU will be processed next
				by Apply_PDU.  Its contents may first be modified here.
				Changing pdu!kind results in "turning the corner".
				';
	TASK		pdu := buffer(b)!pdu;
	DECISION pdu!kind;
	 (EDrq):		DECISION pdu!subHeight < parameters!maxHeight;
				 (False):	TASK		pdSend := True;
				ELSE:ENDDECISION;
	 (MCrq):		TASK		new!kind := MCcf,
							new!mergeChannels := Empty,
							new!purgeChannelIds := pdu!purgeChannelIds;
				 1b :		/* for cAttr in mergeChannels */
				DECISION pdu!mergeChannels = Empty;
				 (False):	TASK		cAttr := Pick(pdu!mergeChannels),
									pdu!mergeChannels := Del(cAttr, pdu!mergeChannels),
									c := cAttr!channelId;
							DECISION c in cUsed;
							 (True):	DECISION chan(c)!kind;
									 (Static):
											JOIN 2f;
									 (Private):
											DECISION cAttr!kind = Private
													and cAttr!manager = chan(c)!manager;
											 (True):	JOIN 2f;
											ELSE:ENDDECISION;
									ELSE:ENDDECISION;

(False):	DECISION cAttr!kind = UserId;
									 (True):	CALL	Take_user(c, diagnostic);
									 (False):	CALL	Take_channel(c, diagnostic);
									ENDDECISION;
									DECISION diagnostic = DC_OK;
									 (True):	TASK	chan(c)!kind := cAttr!kind;
											 2f :
											TASK	new!mergeChannels :=
															Incl(cAttr, new!mergeChannels);
											JOIN 1b;
									ELSE:ENDDECISION;
							ENDDECISION;
							TASK		new!purgeChannelIds := Incl(c, new!purgeChannelIds);
							JOIN 1b;
				ELSE:ENDDECISION;
				TASK		pdu := new;
	 (MTrq):		TASK		new!kind := MTcf,
							new!mergeTokens := Empty,
							new!purgeTokenIds := pdu!purgeTokenIds;
				 3b :		/* for tAttr in mergeTokens */
				DECISION pdu!mergeTokens = Empty;
				 (False):	TASK		tAttr := Pick(pdu!mergeTokens),
									pdu!mergeTokens := Del(tAttr, pdu!mergeTokens),
									t := tAttr!tokenId;
							DECISION t in tUsed;
							 (True):	DECISION token(t)!kind;
									 (Inhibited):
											DECISION tAttr!kind = Inhibited;
											 (True):	JOIN 4f;
											ELSE:ENDDECISION;
									ELSE:ENDDECISION;
							 (False):	CALL	Take_token(t, diagnostic);
									DECISION diagnostic = DC_OK;
									 (True):	 4f :
											TASK	new!mergeTokens :=
															Incl(tAttr, new!mergeTokens);
											JOIN 3b;
									ELSE:ENDDECISION;
							ENDDECISION;
							TASK		new!purgeTokenIds  :=  Incl(t, new!purgeTokenIds);
							JOIN 3b;
				ELSE:ENDDECISION;
				TASK		pdu  :=  new;
	 (AUrq):		CALL		New_user(u, result);
				TASK		pdu!kind  :=  AUcf,
							pdu!result  :=  result,
							pdu!initiator  :=  u;
	 (DUrq):		TASK		pdu!kind  :=  DUin;
	 (CJrq):		TASK		pdu!kind  :=  CJcf,
							c  :=  pdu!channelId,
							pdu!requested  :=  c,
							result  :=  RT_successful;
				DECISION c = 0;
				 (True):	CALL		New_channel(c, result);
							DECISION result = RT_successful;
							 (True): 	TASK	chan(c)!kind  :=  Assigned;
							ELSE:ENDDECISION;
							TASK		pdu!channelId  :=  c;
				 (False):	DECISION c in cUsed;
							 (False):	DECISION c < 1001;
									 (False):	TASK	result  :=  RT_no_such_channel;
									 (True):	CALL	Take_chanel(c, diagnostic);
											DECISION diagnostic = DC_OK;
											 (False):	TASK	result  :=  RT_too_many_channels;
											 (True):	TASK	chan(c)!kind  :=  Static;
												ENDDECISION;
									ENDDECISION;

Recommendation T.125     (04/94)

(True):	DECISION chan(c)!kind;
				(UserId):
					TASK    u  :=  UserId(c);
					DECISION pdu!initiator = u;
					(False):TASK    result  :=  RT_other_user_id;
					ELSE:ENDDECISION;
				(Private):
					DECISION pdu!initiator in chan(c)!admitted;
					(False):TASK    result  :=  RT_not_admitted;
					ELSE:ENDDECISION;
				ELSE:ENDDECISION;
			ENDDECISION;
			DECISION result = RT_successful;
			(False):TASK    pdu!channelId  :=  0;
			ELSE:ENDDECISION;
		ENDDECISION;
		TASK	pdu!result  :=  result;
	(CLrq): 	/* no special action */
	(CCrq): 	CALL	New_channel(c, result);
		TASK	pdu!kind  :=  CCcf,
			pdu!result  :=  result,
			pdu!channelId  :=  c;
	(CDrq):	TASK	pdu!kind  :=  CDin;
	(CArq):	TASK	pdu!kind  :=  CAin;
	(CErq):	TASK	pdu!kind  :=  CEin;
	(SDrq):	TASK	pdu!kind  :=  SDin;
	(USrq):	TASK	pdu!kind  :=  USin;
	(TGrq):	TASK	pdu!kind  :=  TGcf,
			pdu!result  :=  RT_successful,
			t  :=  pdu!tokenId,
			u  :=  pdu!initiator;
		DECISION t in tUsed;
		(False):	CALL	Take_token(t, diagnostic);
			DECISION diagnostic = DC_OK;
			(False):	TASK	pdu!result  :=  RT_too_many_tokens;
			(True): 	TASK	token(t)!kind  :=  Grabbed,
					token(t)!grabber  :=  u;
			ENDDECISION;
		(True):	DECISION token(t)!kind = Inhibited
				and token(t)!inhibitors = Incl(u, Empty);
			(False):TASK	pdu!result  :=  RT_token_not_available;
			(True): TASK	token(t)!kind  :=  Grabbed,
					token(t)!grabber  :=  u;
			ENDDECISION;
		ENDDECISION;
		CALL	Token_status(pdu);
	(TIrq): 	TASK    	pdu!kind  :=  TIcf,
			pdu!result  :=  RT_successful,
			t  :=  pdu!tokenId,
			u  :=  pdu!initiator;
		DECISION t in tUsed;
		(False):	CALL    Take_token(t, diagnostic);
			DECISION diagnostic = DC_OK;
			(False):TASK	pdu!result  :=  RT_too_many_tokens;
			(True): TASK	token(t)!kind  :=  Inhibited,
					token(t)!inhibitors  :=  Incl(u, Empty);
			ENDDECISION;
		(True):	DECISION token(t)!kind = Grabbed and token(t)!grabber = u;
			(True): TASK	token(t)!kind  :=  Inhibited,
					token(t)!inhibitors  :=  Incl(u, Empty);
			ELSE:ENDDECISION;
			DECISION token(t)!kind = Inhibited;
			(False):TASK    pdu!result  :=  RT_token_not_available;
			(True): TASK    token(t)!inhibitors :=
					              Incl(u, token(t)!inhibitors);
			ENDDECISION;

ENDDECISION;
		CALL	Token_status(pdu);
	(TVrq):	TASK	pdu!kind  :=  TVcf,
			pdu!result  :=  RT_token_not_possessed,
			t  :=  pdu!tokenId,
			u  :=  pdu!initiator;
		DECISION t in tUsed and token(t)!kind = Grabbed and token(t)!grabber = u;
		(True):	DECISION pdu!recipient in uUsed;
			(False):TASK	pdu!result  :=  RT_no_such_user;
			(True): TASK	pdu!kind  :=  TVin,
					token(t)!kind  :=  Giving,
					token(t)!recipient  :=  pdu!recipient;
			ENDDECISION;
		ELSE:ENDDECISION;
		CALL	Token_status(pdu);
	(TPrq):	TASK	pdu!kind  :=  TPin;
	(TRrq):	TASK	pdu!kind  :=  TRcf,
			pdu!result  :=  RT_token_not_possessed,
			t  :=  pdu!tokenId,
			u  :=  pdu!initiator;
		DECISION t in tUsed;
		(True):	DECISION token(t)!kind;
			(Grabbed, Ungivable):
				DECISION token(t)!grabber = u;
				(True): TASK    pdu!result  :=  RT_successful;
					CALL    Delete_token(t);
				ELSE:ENDDECISION;
			(Giving):
				DECISION token(t)!grabber = u;
				(True): TASK    pdu!result  :=  RT_successful,
					               token(t)!kind  :=  Given;
				ELSE:ENDDECISION;
			(Inhibited):
				DECISION u in token(t)!inhibitors;
				(True): TASK    pdu!result  :=  RT_successful,
					               token(t)!inhibitors :=
					                              Del(u, token(t)!inhibitors);
				ELSE:ENDDECISION;
				DECISION token(t)!inhibitors = Empty;
				(True): CALL    Delete_token(t);
				ELSE:ENDDECISION;
			ENDDECISION;
		ELSE:ENDDECISION;
		CALL	Token_status(pdu);
	(TTrq):	TASK	pdu!kind  :=  TTcf;
		CALL	Token_status(pdu);
	(TVrs):	TASK	t  :=  pdu!tokenId,
			u  :=  pdu!recipient;
		DECISION t in tUsed and token(t)!recipient = u;
		(True):	DECISION token(t)!kind;
			(Giving):
				TASK	token(t)!kind  :=  Grabbed,
					pdu!kind  :=  TVcf,
					pdu!initiator  :=  token(t)!grabber;
				DECISION pdu!result = RT_successful;
				(True): TASK    token(t)!grabber  :=  u;
				ELSE:ENDDECISION;
				CALL	Token_status(pdu);
			ELSE:ENDDECISION;
		ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	TASK    buffer(b)!pdu  :=  pdu;
	RETURN;
	ENDPROCEDURE;

/*--------------------*/

PROCEDURE		Apply_PDU;	/*  Apply_PDU  */

FPAR		r		PortalId,	/*--------------------*/
		b		BufferId,

IN/OUT	diagnostic	Diagnostic;

	DCL	pdu		PDUStruct,
		new		PDUStruct,
		p		PortalId,
		pSet		PortalIdSet,
		x		PortalId,
		u		UserId,
		uSet		UserIdSet,
		c		ChannelId,
		cSet		ChannelIdSet,
		cAttr		ChannelAttributes,
		cAttrSet		ChannelAttributesSet,
		t		TokenId,
		tSet		TokenIdSet,
		tAttr		TokenAttributes,
		tAttrSet		TokenAttributesSet,
		result		Result;
	START
	COMMENT'This is a large case selection based on MCSPDU kind.
		  These actions and updates to the information base
		  take place in both the top and subordinate providers.
		  ';
	TASK	pdu  :=  buffer(b)!pdu;
	DECISION pdu!kind;
	(MCrq, MTrq, AUrq, DUrq, CCrq,
	CDrq,   CArq,  CErq, USrq, TGrq,
	TIrq,     TVrq,  TPrq, TRrq,  TTrq):
		CALL	Cast_buffer(b, upward);
	(PDin):	DECISION pdu!heightLimit > 0;
		(True):TASK	buffer(b)!pdu!heightLimit  :=  pdu!heightLimit - 1;
		(False):OUTPUT	Report.portal(r, DC_height_limit_exceeded) TO control;
		ENDDECISION;
		CALL	Broadcast_buffer(b);
	(EDrq):	TASK	portal(r)!subHeight  :=  pdu!subHeight,
			portal(r)!subInterval  :=  pdu!subInterval,
			portal(r)!interval  :=  IF pdu!subInterval = 0 THEN interval
					ELSE oneSecond + (pdu!subInterval * 3) FI;
		CALL	Erect_domain;
	(MCcf):	DECISION merging;
		(False):TASK	p  :=  Next(mcrqQueue),
				mcrqQueue  :=  Pull(mcrqQueue);
		ELSE:ENDDECISION;
		TASK	cAttrSet  :=  pdu!mergeChannels;
		1b :	/* for cAttr in cAttrSet */
		DECISION cAttrSet = Empty;
		(False):	TASK	cAttr  :=  Pick(cAttrSet),
				cAttrSet  :=  Del(cAttr, cAttrSet),
				c  :=  cAttr!channelId;
			DECISION c in cUsed and chan(c)!kind = cAttr!kind;
			(False):	DECISION cAttr!kind = UserId;
				(True): CALL     Take_user(c, diagnostic);
				(False): CALL    Take_channel(c, diagnostic);
				ENDDECISION;
				DECISION diagnostic = DC_OK;
				(False):RETURN;
				ELSE:ENDDECISION;
				TASK	chan(c)!kind  :=  cAttr!kind;
			ELSE:ENDDECISION;
			DECISION merging;
			(True):	DECISION chan(c)!kind = UserId;
				(True): TASK     u  :=  UserId(c),
					                uConfirm  :=  Incl(u, uConfirm);
				ELSE:ENDDECISION;

DECISION chan(c)!kind = Private
					      and  chan(c)!uMerge /= Empty;
				(True): TASK    cMerge  :=  Incl(c, cMerge);
				(False):TASK    cConfirm  :=  Incl(c, cConfirm);
				ENDDECISION;
				JOIN 1b;
			ELSE:ENDDECISION;
				TASK    chan(c)!portal  :=  p;
				DECISION cAttr!kind = Static or cAttr!kind = Assigned
					or cAttr!joined;
				(True): DECISION p = 0;
					(False):TASK    chan(c)!joined :=
					                                           Incl(p, chan(c)!joined),
					                            cLeave  :=  Del(c, cLeave);
					(True): DECISION chan(c)!joined = Empty;
					             (True): TASK    cLeave  :=  Incl(c, cLeave);
					             ELSE:ENDDECISION;
					ENDDECISION;
				ELSE:ENDDECISION;
				DECISION cAttr!kind = UserId and p = 0;
				(True): TASK    u  :=  UserId(c),
					               uDetach  :=  Incl(u, uDetach),
					               cLeave  :=  Del(c, cLeave);
				ELSE:ENDDECISION;
				DECISION cAttr!kind = Private;
				(False):JOIN 1b;
				ELSE:ENDDECISION;
				CALL    Route_user(cAttr!manager, x);
				DECISION x = p;
				(False):TASK    chan(c)!manager  :=  0,
					                buffer(b)!pdu!mergeChannels :=
					                              Del(cAttr, buffer(b)!pdu!mergeChannels),
					                buffer(b)!pdu!purgeChannelIds :=
					                              Incl(c, buffer(b)!pdu!purgeChannelIds),
					                cDisband  :=  Incl(c, cDisband);
				(True): TASK     chan(c)!manager  :=  cAttr!manager,
					                uSet  :=  cAttr!admitted and uUsed;
					2b :         /* for u in uSet */
					DECISION uSet = Empty;

(False):TASK	u  :=  Pick(uSet),
						uSet  :=  Del(u, uSet);

CALL        Route_user(u, x);
					              DECISION x = p;
					              (True): TASK    chan(c)!admitted :=
					                                                       Incl(u, chan(c)!admitted);
					              ELSE:ENDDECISION;
					              JOIN 2b;
					ELSE:ENDDECISION;
				ENDDECISION;
				JOIN 1b;
			ELSE:ENDDECISION;
			DECISION merging;
			(False):	CALL	Cast_buffer(b, p);
			(True): 	TASK	new!kind  :=  PCin,
					new!detachUserIds  :=  Empty,
					new!purgeChannelIds  :=  Empty,
					cSet  :=  pdu!purgeChannelIds and cUsed;
				3b :	/* for c in cSet */
				DECISION cSet = Empty;
			(False):	TASK	c  :=  Pick(cSet),
					cSet  :=  Del(c, cSet);
				DECISION chan(c)!kind;
				(UserId):
					TASK    u  :=  UserId(c),
					              new!detachUserIds :=
					                               Incl(u, new!detachUserIds);

(Static, Assigned, Private):
					TASK    new!purgeChannelIds :=
					                               Incl(c, new!purgeChannelIds);
				ENDDECISION;
				JOIN 3b;
			ELSE:ENDDECISION;
			CALL	Purge_users(new!detachUserIds);
			CALL	Purge_channels(new!purgeChannelIds);
			TASK	buffer(b)!pdu  :=  new;
			CALL	Broadcast_buffer(b);
		ENDDECISION;
	(PCin):	TASK	cSet  :=  pdu!purgeChannelIds and cUsed,
			buffer(b)!pdu!purgeChannelIds  :=  cSet;
		DECISION merging;
		(True):	TASK	buffer(b)!pdu!detachUserIds  :=  pdu!detachUserIds
					                                                           and uConfirm,
				buffer(b)!pdu!purgeChannelIds  :=  cSet and cConfirm;
			4b :	/* for c in cSet */
			DECISION cSet = Empty;
			(False):	TASK	c  :=  Pick(cSet),
					cSet  :=  Del(c, cSet);
				DECISION c in cConfirm;
				(False):TASK    chan(c)!uMerge  :=  chan(c)!admitted;
				ELSE:ENDDECISION;
				JOIN 4b;
			ELSE:ENDDECISION;
		ELSE:ENDDECISION;
		CALL	Purge_users(buffer(b)!pdu!detachUserIds);
		CALL	Purge_channels(buffer(b)!pdu!purgeChannelIds);
		CALL	Broadcast_buffer(b);
	(MTcf):	DECISION merging;
		(False):TASK	p  :=  Next(mtrqQueue),
				mtrqQueue  :=  Pull(mtrqQueue);
		ELSE:ENDDECISION;
		TASK	tAttrSet  :=  pdu!mergeTokens;
		   5b :	/* for tAttr in tAttrSet */
		DECISION tAttrSet = Empty;
		(False):	TASK	tAttr  :=  Pick(tAttrSet),
				tAttrSet  :=  Del(tAttr, tAttrSet),
				t  :=  tAttr!tokenId;
			DECISION t in tUsed;
			(False):	CALL	Take_token(t, diagnostic);
				DECISION diagnostic = DC_OK;
				(False):RETURN;
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			TASK	token(t)!kind  :=  tAttr!kind,
				token(t)!grabber  :=  tAttr!grabber,
				token(t)!recipient  :=  tAttr!recipient;
			DECISION merging;
			(True): 	DECISION token(t)!kind = Inhibited
					and token(t)!uMerge /= Empty;
				(True): TASK    tMerge  :=  Incl(t, tMerge);
				(False):TASK    tConfirm  :=  Incl(t, tConfirm);
					 DECISION token(t)!kind = Inhibited
					            and token(t)!inhibitors = Empty;
					 (True): CALL    Delete_token(t);
					 ELSE:ENDDECISION;
				ENDDECISION;
				JOIN 5b;
			ELSE:ENDDECISION;
			DECISION upward = 0 and token(t)!kind = Ungivable;
			(True): TASK    tReject  :=  Incl(t, tReject);
			ELSE:ENDDECISION;
			DECISION tAttr!kind;

(Grabbed, Ungivable):
				CALL	 Token_route(tAttr!grabber, x, p);
				DECISION x = p;
				(False):JOIN 6f;
				ELSE:ENDDECISION;
			(Given):
				CALL	 Token_route(tAttr!recipient, x, p);
				DECISION x = p;
				(False):JOIN 6f;
				ELSE:ENDDECISION;
			(Giving):
				CALL    Token_route(tAttr!recipient, x, p);
				DECISION x = p;
				(True): CALL    Token_route(tAttr!grabber, x, p);
					DECISION x = p;
					(False):TASK    token(t)!kind  :=  Given;
					ELSE:ENDDECISION;
				(False):CALL    Token_route(tAttr!grabber, x, p);
					DECISION x = p;
					(True): TASK    token(t)!kind  :=  Ungivable;
					(False): 6f :
					              TASK    buffer(b)!pdu!mergeTokens :=
					                     Del(tAttr, buffer(b)!pdu!mergeTokens),
					                              buffer(b)!pdu!purgeTokenIds :=
					                     Incl(t, buffer(b)!pdu!purgeTokenIds);
					              CALL    Delete_token(t);
					ENDDECISION;
				ENDDECISION;
			(Inhibited):
				TASK    uSet  :=  tAttr!inhibitors and uUsed;
				   7b :    /* for u in uSet */
				DECISION uSet = Empty;
					(False):TASK    u  :=  Pick(uSet),
					                            uSet  :=  Del(u, uSet);
					              CALL    Token_route(u, x, p);
					              DECISION x = p;
					              (True): TASK    token(t)!inhibitors :=
					                                               Incl(u, token(t)!inhibitors);
					              ELSE:ENDDECISION;
					              JOIN 7b;
					ELSE:ENDDECISION;
					DECISION token(t)!inhibitors = Empty;
					(True): CALL    Delete_token(t);
					ELSE:ENDDECISION;
				ENDDECISION;
				JOIN 5b;
			ELSE:ENDDECISION;
			DECISION merging;
			(False):	CALL    Cast_buffer(b, p);
			(True):	CALL    Purge_tokens(pdu!purgeTokenIds);
				TASK    buffer(b)!pdu!kind  :=  PTin;
				CALL    Broadcast_buffer(b);
			ENDDECISION;
		(PTin):	TASK	tSet  :=  pdu!purgeTokenIds and tUsed,
				buffer(b)!pdu!purgeTokenIds  :=  tSet;
			DECISION merging;
			(True):	TASK    buffer(b)!pdu!purgeTokenIds  :=  tSet and tConfirm;
				   8b :    /* for t in tSet */
				DECISION tSet = Empty;
				(False):TASK    t  :=  Pick(tSet),
					               tSet  :=  Del(t, tSet);
					DECISION t in tConfirm;
					(False):TASK    token(t)!uMerge  :=  token(t)!inhibitors;
					ELSE:ENDDECISION;
					JOIN 8b;
				ELSE:ENDDECISION;

ELSE:ENDDECISION;
			CALL	Purge_tokens(buffer(b)!pdu!purgeTokenIds);
			CALL	Broadcast_buffer(b);
		(DPum): OUTPUT  Drop.portal(r, pdu!reason) TO control;
		(AUcf):	TASK	p  :=  Next(aurqQueue),
				aurqQueue  :=  Pull(aurqQueue),
				c  :=  ChannelId(u),
				u  :=  pdu!initiator;
			DECISION pdu!result = RT_successful;
			(True):	DECISION upward = 0;
				(False):CALL    Take_user(c, diagnostic);
					 DECISION diagnostic = DC_OK;
					 (False):RETURN;
					 ELSE:ENDDECISION;
				ELSE:ENDDECISION;
				TASK    chan(c)!portal  :=  p;
				DECISION p = 0;
				(True): TASK    uDetach  :=  Incl(u, uDetach),
					               cLeave  :=  Del(c, cLeave);
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			CALL	Cast_buffer(b, p);
		(DUin):	DECISION merging;
			(True):	TASK    buffer(b)!pdu!userIds  :=  pdu!userIds and uConfirm;
			ELSE:ENDDECISION;
			CALL	Purge_users(buffer(b)!pdu!userIds);
			CALL	Broadcast_buffer(b);
		(CJrq):	TASK	c  :=  pdu!channelId,
				result  :=  RT_successful,
				p  :=  upward;
			DECISION c in cUsed;
			(True):	DECISION chan(c)!kind;
				(UserId):
					TASK    u  :=  UserId(c);
					DECISION pdu!initiator = u;
					(False):TASK    result  :=  RT_other_user_id;
					ELSE:ENDDECISION;
				(Private):
					DECISION pdu!initiator in chan(c)!admitted;
					(False):TASK    result  :=  RT_not_admitted;
					ELSE:ENDDECISION;
				ELSE:ENDDECISION;
				DECISION result = RT_successful;
				(False):TASK    buffer(b)!pdu!kind  :=  CJcf,
					buffer(b)!pdu!requested  :=  c,
					buffer(b)!pdu!result  :=  result,
					buffer(b)!pdu!channelId  :=  0,
					p  :=  r;
			(True):	DECISION chan(c)!joined /= Empty or c in cLeave;
				(True): TASK    chan(c)!joined :=
					                              Incl(r, chan(c)!joined),
					               cLeave  :=  Del(c, cLeave),
					               buffer(b)!pdu!kind  :=  CJcf,
					               buffer(b)!pdu!requested  :=  c,
					               buffer(b)!pdu!result  :=  result,
					               p  :=  r;
				ELSE:ENDDECISION;
			ENDDECISION;
		ELSE:ENDDECISION;
		CALL	Cast_buffer(b, p);
	(CJcf):	TASK	c  :=  pdu!channelId,
			u  :=  pdu!initiator;
		CALL	Route_user(u, p);
		DECISION pdu!result = RT_successful;
		(True):	DECISION c in cUsed;
			(False):	CALL    Take_channel(c, diagnostic);

DECISION diagnostic = DC_OK;
				(False):RETURN;
				ELSE:ENDDECISION;
				TASK    chan(c)!kind  :=  IF c < 1001 THEN Static
					                                   ELSE Assigned FI;
			ELSE:ENDDECISION;
			DECISION p = 0;
			(False):	TASK    chan(c)!joined  :=  Incl(p, chan(c)!joined),
					  cLeave  :=  Del(c, cLeave);
			(True):	DECISION chan(c)!joined = Empty;
				(True): TASK    cLeave  :=  Incl(c, cLeave);
				ELSE:ENDDECISION;
			ENDDECISION;
		ELSE:ENDDECISION;
		CALL    Cast_buffer(b, p);
	(CLrq):	TASK    cSet  :=  pdu!channelIds and cUsed;
		   9b :    /* for c in cSet */
		DECISION cSet = Empty;
		(False):	TASK    c  :=  Pick(cSet),
			            cSet  :=  Del(c, cSet);
			DECISION r in chan(c)!joined;
			(True):	TASK    chan(c)!joined  :=  Del(r, chan(c)!joined);
				DECISION chan(c)!joined = Empty;
				(True): TASK    cLeave  :=  Incl(c, cLeave);
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			JOIN 9b;
		ELSE:ENDDECISION;
	(CCcf):	TASK	c  :=  pdu!channelId,
			u  :=  pdu!initiator;
		DECISION pdu!result = RT_successful;
		(True):	DECISION upward = 0;
			(False):	CALL    Take_channel(c, diagnostic);
				DECISION diagnostic = DC_OK;
				(False):RETURN;
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			TASK	chan(c)!kind  :=  Private,
				chan(c)!manager  :=  u,
				chan(c)!admitted  :=  Incl(u, Empty);
		ELSE:ENDDECISION;
		CALL    Route_user(u, p);
		CALL    Cast_buffer(b, p);
	(CDin):	TASK    c  :=  pdu!channelId;
		DECISION c in cUsed;
		(False):	RETURN;
		(True): 	DECISION merging and not c in cConfirm;
			(True):	TASK    chan(c)!uMerge  :=  chan(c)!admitted;
				RETURN;
			ELSE:ENDDECISION;
			DECISION chan(c)!kind = Private;
			(False):	TASK    diagnostic  :=  DC_channel_id_conflict;
				RETURN;
			ELSE:ENDDECISION;
		ENDDECISION;
		TASK	uSet  :=  Incl(chan(c)!manager, chan(c)!admitted);
		CALL	Delete_channel(c);
		CALL	Multicast_buffer(b, uSet);
	(CAin):	TASK	c  :=  pdu!channelId,
			uSet  :=  pdu!userIds and uUsed,
			buffer(b)!pdu!userIds  :=  uSet;
		DECISION merging;
		(True):	TASK     uSet  :=  uSet and uConfirm,
				buffer(b)!pdu!userIds  :=  uSet;
			  10b :    /* for u in uSet */
			DECISION uSet = Empty;

(False):	TASK    u  :=  Pick(uSet),
					uSet  :=  Del(u, uSet),
					c  :=  ChannelId(u),
					chan(c)!portal = 0;
				JOIN 10b;
			ELSE:ENDDECISION;
			TASK     buffer(b)!pdu!kind  :=  DUrq,
				buffer(b)!pdu!reason  :=  RN_channel_purged;
			CALL     Cast_buffer(b, r);
			RETURN;
		ELSE:ENDDECISION;
		DECISION uSet = Empty;
		(False):	DECISION c in cUsed and chan(c)!kind = Private;
			(False):	CALL    Take_channel(c, diagnostic);
				DECISION diagnostic = DC_OK;
				(False):RETURN;
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			TASK	chan(c)!kind  :=  Private,
				chan(c)!manager  :=  pdu!initiator,
				chan(c)!admitted  :=  chan(c)!admitted or uSet;
			CALL	Multicast_buffer(b, uSet);
		ELSE:ENDDECISION;
	(CEin): 	TASK    c  :=  pdu!channelId;
		DECISION c in cUsed;
		(False):	RETURN;
		(True): 	DECISION merging and not c in cConfirm;
			(True):	TASK    chan(c)!uMerge  :=  chan(c)!admitted;
				RETURN;
			ELSE:ENDDECISION;
			DECISION chan(c)!kind = Private;
			(False):	TASK    diagnostic  :=  DC_channel_id_conflict;
				RETURN;
			ELSE:ENDDECISION;
		ENDDECISION;
		TASK     uSet  :=  chan(c)!admitted,
			pSet  :=  Empty;
		  11b :    /* for u in uSet */
		DECISION uSet = Empty;
		(False):	TASK     u  :=  Pick(uSet),
				uSet  :=  Del(u, uSet);
			DECISION u in pdu!userIds;
			(False):	CALL    Route_user(u, p);
				TASK    pSet  :=  Incl(p, pSet);
			ELSE:ENDDECISION;
			JOIN 11b;
		ELSE:ENDDECISION;
		TASK	uSet  :=  pdu!userIds and chan(c)!admitted,
			buffer(b)!pdu!userIds  :=  uSet;
		  12b :     /* for u in uSet */
		DECISION uSet = Empty;
		(False):	TASK     u  :=  Pick(uSet),
				uSet  :=  Del(u, uSet),
				chan(c)!admitted  :=  Del(u, chan(c)!admitted);
			CALL	Route_user(u, p);
			DECISION p in chan(c)!joined and not p in pSet;
			(False):	TASK    chan(c)!joined  :=  Del(p, chan(c)!joined);
				DECISION chan(c)!joined = Empty;
				(True): TASK    cLeave  :=  Incl(c, cLeave);
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			JOIN 12b;
		ELSE:ENDDECISION;
		DECISION chan(c)!admitted /= Empty or chan(c)!manager in uUsed;
		(False):	CALL    Delete_channel(c);
		ELSE:ENDDECISION;

TASK	uSet  :=  buffer(b)!pdu!userIds;
		CALL	Multicast_buffer(b, uSet);
	(SDrq): 	TASK      buffer(b)!pdu!kind  :=  SDin,
			pSet  :=  Incl(upward, Empty);
		JOIN 13f;
	(SDin):	TASK    pSet  :=  Empty;
		JOIN 13f;
	(USin):	TASK    pSet  :=  Empty;
		   13f :
		TASK    c  :=  pdu!channelId;
		DECISION c in cUsed and not merging;
		(True):	TASK    pSet  :=  pSet or chan(c)!joined;
			DECISION chan(c)!kind = UserId;
			(True):	TASK    pSet  :=  Del(upward, pSet);
			ELSE:ENDDECISION;
		ELSE:ENDDECISION;
		DECISION buffer(b)!pdu!kind = SDin;
		(True):	TASK    pSet  :=  Del(r, pSet);
		ELSE:ENDDECISION;
		   14b :    /* for p in pSet */
		DECISION pSet = Empty;
		(False):	TASK     p  :=  Pick(pSet),
				pSet  :=  Del(p, pSet);
			CALL     Cast_buffer(b, p);
			JOIN 14b;
		ELSE:ENDDECISION;
	(TGcf, TIcf, TVcf, TRcf, TTcf):
		TASK    t  :=  pdu!tokenId,
		              u  :=  pdu!initiator;
		DECISION pdu!tokenStatus;
		(SelfGrabbed):
			DECISION t in tUsed;
			(False):	CALL    Take_token(t, diagnostic);
				DECISION diagnostic = DC_OK;
				(False):RETURN;
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			TASK	token(t)!kind  :=  Grabbed,
				token(t)!grabber  :=  u;
		(SelfInhibited):
			DECISION t in tUsed;
			(False):	CALL    Take_token(t, diagnostic);
				DECISION diagnostic = DC_OK;
				(False):RETURN;
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			TASK	token(t)!kind  :=  Inhibited,
				token(t)!inhibitors  :=  Incl(u, token(t)!inhibitors);
		(SelfRecipient):
			TASK	token(t)!recipient  :=  u;
		(SelfGiving):
			TASK	token(t)!grabber  :=  u;
		(NotInUse):
			CALL	Delete_token(t);
		ELSE:
			DECISION token(t)!kind;
			(Grabbed, Ungivable):
				DECISION token(t)!grabber = u;
				(True): CALL    Delete_token(t);
				ELSE:ENDDECISION;
			(Giving, Given):
				DECISION token(t)!grabber = u;
				(True): TASK    token(t)!kind  :=  Given;
				ELSE:ENDDECISION;
				DECISION token(t)!recipient = u;
				(True): CALL    Delete_token(t);

ELSE:ENDDECISION;
			(Inhibited):
				TASK    token(t)!inhibitors :=
					                Del(u, token(t)!inhibitors);
				DECISION token(t)!inhibitors = Empty;
				(True): CALL    Delete_token(t);
				ELSE:ENDDECISION;
			ENDDECISION;
		ENDDECISION;
		CALL    Route_user(u, p);
		CALL    Cast_buffer(b, p);
	(TVin):	TASK    t  :=  pdu!tokenId,
		              u  :=  pdu!recipient;
		DECISION merging;
		(True):	TASK     buffer(b)!pdu!kind  :=  TVrs,
				buffer(b)!pdu!result  :=  RT_domain_merging;
			CALL     Cast_buffer(b, r);
			RETURN;
		ELSE:ENDDECISION;
		DECISION t in tUsed;
		(False):	CALL    Take_token(t, diagnostic);
			DECISION diagnostic = DC_OK;
			(False):RETURN;
			ELSE:ENDDECISION;
		ELSE:ENDDECISION;
		TASK	token(t)!kind  :=  Giving,
			token(t)!grabber  :=  pdu!initiator,
			token(t)!recipient  :=  u;
		CALL	Route_user(u, p);
		CALL	Cast_buffer(b, p);
	(TVrs):	TASK	t  :=  pdu!tokenId,
			u  :=  pdu!recipient;
		DECISION t in tUsed and token(t)!recipient = u;
		(True):	DECISION token(t)!kind;
			(Giving):
				TASK    token(t)!kind  :=  Grabbed;
				DECISION pdu!result = RT_successful;
				(True): TASK    token(t)!grabber  :=  u;
				(False):DECISION token(t)!grabber in uUsed;
					(False):CALL    Delete_token(t);
					ELSE:ENDDECISION;
				ENDDECISION;
				CALL    Cast_buffer(b, upward);
			(Given):
				TASK    token(t)!kind  :=  Grabbed,
					  token(t)!grabber  :=  u;
				DECISION pdu!result = RT_successful;
				(False):CALL    Delete_token(t);
				ELSE:ENDDECISION;
				CALL    Cast_buffer(b, upward);
			ELSE:ENDDECISION;
		ELSE:ENDDECISION;
	(TPin):	TASK    t  :=  pdu!tokenId;
		DECISION t in tUsed and not merging;
		(True):	DECISION token(t)!kind;
			(Grabbed, Ungivable):
				CALL    Route_user(token(t)!grabber, p);
				CALL    Cast_buffer(b, p);
			(Given):
				CALL    Route_user(token(t)!recipient, p);
				CALL    Cast_buffer(b, p);
			(Giving):
				TASK    uSet  :=  Incl(token(t)!grabber, Empty);
				TASK    uSet  :=  Incl(token(t)!recipient, uSet);
				CALL    Multicast_buffer(b, uSet);

(Inhibited):
				TASK    uSet  :=  token(t)!inhibitors;
				CALL    Multicast_buffer(b, uSet);
			ENDDECISION;
		ELSE:ENDDECISION;
	ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/

PROCEDURE	Token_status;			/*  Token_status  */
FPAR	IN/OUT	pdu		PDUStruct;	/*----------------------*/

	DCL	t	TokenId,
		u	UserId,
		status	TokenStatus;
	START
	COMMENT'Calculate for an MCSPDU the relationship between
		  the initiator user id and token id it contains.
		  ';
	TASK	t  :=  pdu!tokenId,
		u  :=  pdu!initiator;
	DECISION t in tUsed;
	(False):TASK    status  :=  NotInUse;
	(True): DECISION token(t)!kind;
		(Grabbed):
			DECISION token(t)!grabber = u;
			(True): TASK    status  :=  SelfGrabbed;
			(False):TASK    status  :=  OtherGrabbed;
			ENDDECISION;
		(Ungivable):
			DECISION token(t)!grabber = u;
			(True): TASK    status  :=  SelfGiving;
			(False):TASK    status  :=  OtherGiving;
			ENDDECISION;
		(Given):
			DECISION token(t)!recipient = u;
			(True): TASK    status  :=  SelfRecipient;
			(False):TASK    status  :=  OtherGiving;
			ENDDECISION;
		(Giving):
			DECISION token(t)!recipient = u;
			(True): TASK    status  :=  SelfRecipient;
			(False):DECISION token(t)!grabber = u;
				(True): TASK    status  :=  SelfGiving;
				(False):TASK    status  :=  OtherGiving;
				ENDDECISION;
			ENDDECISION;
		(Inhibited):
			DECISION u in token(t)!inhibitors;
			(True): TASK    status  :=  SelfInhibited;
			(False):TASK    status  :=  OtherInhibited;
			ENDDECISION;
		ENDDECISION;
	ENDDECISION;
	TASK    pdu!tokenStatus  :=  status;
	RETURN;
	ENDPROCEDURE;

/*----------------------*/

PROCEDURE	Token_route;			/*  Token_route  */
FPAR		u		UserId,	/*---------------------*/
	IN/OUT	x		PortalId,
		p		PortalId;

START
	COMMENT'Revoke token user if its route x is no longer via p.
		';
	CALL	Route_user(u, x);

DECISION x = p;
	(False): 	DECISION u in uUsed;
		(True): TASK    uRevoke  :=  Incl(u, uRevoke);
		ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
						/*---------------------*/
PROCEDURE	Delete_user;			/*  Delete_user  */
FPAR		u		UserId;	/*--------------------*/

	DCL	p		PortalId,
		c		ChannelId,
		cSet		ChannelIdSet,
		a		UserId,
		aSet		UserIdSet,
		q		PortalId,
		t		TokenId,
		tSet		TokenIdSet;
	START
	COMMENT'Update the information base to delete a user id.
		  This has consequences for its channels and tokens.
		  Any data still in transit is left undisturbed.
		  ';
	DECISION u in uUsed;
	(False):RETURN;
	ELSE:ENDDECISION;
	TASK	c  :=  ChannelId(u),
		p  :=  chan(c)!portal,
		uUsed  :=  Del(u, uUsed),
		numUserIds  :=  numUserIds - 1,
		cUsed  :=  Del(c, cUsed),
		cFree  :=  Incl(c, cFree),
		numChannelIds  :=  numChannelIds - 1,
		uConfirm  :=  Del(u, uConfirm),
		cConfirm  :=  Del(c, cConfirm),
		uDetach  :=  Del(u, uDetach),
		uRevoke  :=  Del(u, uRevoke),
		cLeave  :=  Del(c, cLeave);
	TASK	cSet  :=  cUsed;
	   1b :	/* for c in cSet */
	DECISION cSet = Empty;
	(False):	TASK	c  :=  Pick(cSet),
			cSet  :=  Del(c, cSet);
		DECISION portal(p)!kind = Attached and p in chan(c)!joined;
		(True):	TASK    chan(c)!joined  :=  Del(p, chan(c)!joined);
			DECISION chan(c)!joined = Empty;
			(True): TASK    cLeave  :=  Incl(c, cLeave);
			ELSE:ENDDECISION;
		ELSE:ENDDECISION;
		DECISION chan(c)!kind = Private;
		(True):	DECISION chan(c)!manager = u;
			(True):	TASK    chan(c)!manager  :=  0;
				DECISION upward = 0;
				(True): TASK    cDisband  :=  Incl(c, cDisband);
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
			TASK     chan(c)!admitted  :=  Del(u, chan(c)!admitted),
				chan(c)!uMerge  :=  Del(u, chan(c)!uMerge);
			DECISION chan(c)!admitted /= Empty or chan(c)!manager in uUsed;
			(False):	DECISION not merging or c in cConfirm;
				(True): CALL    Delete_channel(c);
				ELSE:ENDDECISION;
			(True):	DECISION p in chan(c)!joined;
				(True): TASK    aSet  :=  chan(c)!admitted;
					   2b :   /* for a in aSet */

DECISION aSet = Empty;
					(False):TASK    a  :=  Pick(aSet),
					                            aSet  :=  Del(a, aSet);
					              CALL    Route_user(a, q);
					              DECISION q = p;
					              (False):JOIN 2b;
					              (True): JOIN 1b;
					              ENDDECISION;
					ELSE:ENDDECISION;
					TASK    chan(c)!joined  :=  Del(p, chan(c)!joined);
					DECISION chan(c)!joined = Empty;
					(True): TASK    cLeave  :=  Incl(c, cLeave);
					ELSE:ENDDECISION;
				ELSE:ENDDECISION;
			ENDDECISION;
		ELSE:ENDDECISION;
		JOIN 1b;
	ELSE:ENDDECISION;
	TASK	tSet  :=  tUsed;
	   3b :	/* for t in tSet */
	DECISION tSet = Empty;
	(False):	TASK	t  :=  Pick(tSet),
			tSet  :=  Del(t, tSet);
		DECISION token(t)!kind;
		(Grabbed, Ungivable):
			DECISION token(t)!grabber = u;
			(True): JOIN 3f;
			ELSE:ENDDECISION;
		(Given):
			DECISION token(t)!recipient = u;
			(True): JOIN 3f;
			ELSE:ENDDECISION;
		(Giving):
			DECISION token(t)!recipient = u;
			(True):	TASK    token(t)!kind  :=  Ungivable;
				DECISION token(t)!grabber in uUsed;
				(False):JOIN 3f;
				(True): DECISION upward = 0;
					(True): TASK    tReject  :=  Incl(t, tReject);
					ELSE:ENDDECISION;
				ENDDECISION;
			(False):	DECISION token(t)!grabber = u;
				(True): TASK    token(t)!kind  :=  Given;
				ELSE:ENDDECISION;
			ELSE:ENDDECISION;
		(Inhibited):
			TASK	token(t)!inhibitors  :=  Del(u, token(t)!inhibitors),
				token(t)!uMerge  :=  Del(u, token(t)!uMerge);
			DECISION token(t)!inhibitors = Empty;
			(True):    3f :
				DECISION not merging or t in tConfirm;
				(True): CALL    Delete_token(t);
				(False):TASK    token(t)!kind  :=  Inhibited,
					               token(t)!inhibitors  :=  Empty;
				ENDDECISION;
			ELSE:ENDDECISION;
		ENDDECISION;
		JOIN 3b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
						/*--------------------------*/
PROCEDURE	Delete_channel;			/*  Delete_channel  */
FPAR		c		ChannelId;	/*-------------------------*/

DCL	u		UserId;
	START
	COMMENT'Update the information base to delete a channel id.
		  ';
	DECISION c in cUsed;
	(False):RETURN;
	ELSE:ENDDECISION;
	DECISION chan(c)!kind = UserId;
	(True):	TASK	u  :=  UserId(c);
		CALL	Delete_user(u);
	(False):	TASK	cUsed  :=  Del(c, cUsed),
			cFree  :=  Incl(c, cFree),
			numChannelIds  :=  numChannelIds - 1,
			cConfirm  :=  Del(c, cConfirm),
			cLeave  :=  Del(c, cLeave),
			cDisband  :=  Del(c, cDisband);
	ENDDECISION;
	RETURN;
	ENDPROCEDURE;
						/*----------------------*/
PROCEDURE	Delete_token;			/*  Delete_token  */
FPAR		t		TokenId;	/*----------------------*/

START
	COMMENT'Update the information base to delete a token id.
		  ';
	DECISION t in tUsed;
	(False):RETURN;
	ELSE:ENDDECISION;
	TASK	tUsed  :=  Del(t, tUsed),
		tFree  :=  Incl(t, tFree),
		numTokenIds  :=  numTokenIds - 1,
		tConfirm  :=  Del(t, tConfirm),
		tReject  :=  Del(t, tReject);
	RETURN;
	ENDPROCEDURE;
						/*----------------------*/
PROCEDURE	Purge_users;			/*  Purge_users  */
FPAR		uSet		UserIdSet;	/*---------------------*/

	DCL	u		UserId;
	START
	COMMENT'Delete a set of user ids.
		  ';
	   1b :	  /* for u in uSet */
	DECISION uSet = Empty;
	(False):	TASK	u  :=  Pick(uSet),
			uSet  :=  Del(u, uSet);
		CALL	Delete_user(u);
		JOIN 1b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
						/*---------------------------*/
PROCEDURE	Purge_channels;		/*  Purge_channels  */
FPAR		cSet		ChannelIdSet;	/*--------------------------*/

	DCL	c		ChannelId;
	START
	COMMENT'Delete a set of channel ids.
		  ';
	   1b :	  /* for c in cSet */
	DECISION cSet = Empty;
	(False):	TASK     c  :=  Pick(cSet),
			cSet  :=  Del(c, cSet);
		CALL	Delete_channel(c);

JOIN 1b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
						/*-----------------------*/
PROCEDURE	Purge_tokens;			/*  Purge_tokens  */
FPAR		tSet		TokenIdSet;	/*-----------------------*/

	DCL	t		TokenId;
	START
	COMMENT'Delete a set of token ids.
		  ';
	   1b :	  /* for t in tSet */
	DECISION tSet = Empty;
	(False):	TASK	t  :=  Pick(tSet),
			tSet  :=  Del(t, tSet);
		CALL    Delete_token(t);
		JOIN 1b;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;
						/*-----------------------*/
PROCEDURE	Output_buffer;			/*  Output_buffer  */
FPAR		b		BufferId,	/*-----------------------*/
		p		PortalId;

	DCL	dp		DataPriority,
		pdu		PDUStruct,
		pid		PId;
	START
	COMMENT'Send the output signal representing an MCSPDU.
		  The next must wait until PDU.ready is received.
		  ';
	TASK	dp  :=  buffer(b)!dataPriority,
		pdu  :=  buffer(b)!pdu,
		pid  :=  portal(p)!pids(dp);
	DECISION p = upward and pdu!kind = SDin;
	(True):	TASK    pdu!kind  :=  SDrq;
	ELSE:ENDDECISION;
	DECISION pdu!kind;

(PDin)	:	OUTPUT	PDin(pdu)	TO	pid;
	(EDrq)	:	OUTPUT	EDrq(pdu)	TO	pid;
	(MCrq)	:	OUTPUT	MCrq(pdu)	TO pid;
	(MCcf)	:	OUTPUT	MCcf(pdu)	TO	pid;
	(PCin)	:	OUTPUT	PCin(pdu)	TO	pid;
	(MTrq)	:	OUTPUT	MTrq(pdu)	TO	pid;
	(MTcf)	:	OUTPUT	MTcf(pdu)	TO	pid;
	(PTin)	:	OUTPUT	PTin(pdu)	TO	pid;
	(DPum)	:	OUTPUT	DPum(pdu)TO	pid;
	(RJum)	:	OUTPUT	RJum(pdu)	TO	pid;
	(AUrq)	:	OUTPUT	AUrq(pdu)	TO	pid;
	(AUcf)	:	OUTPUT	AUcf(pdu)	TO	pid;
	(DUrq)	:	OUTPUT	DUrq(pdu)	TO	pid;
	(DUin)	:	OUTPUT	DUin(pdu)	TO	pid;
	(CJrq)	:	OUTPUT	CJrq(pdu)	TO	pid;
	(CJcf)	:	OUTPUT	CJcf(pdu)	TO	pid;
	(CLrq)	:	OUTPUT	CLrq(pdu)	TO	pid;
	(CCrq)	:	OUTPUT	CCrq(pdu)	TO	pid;
	(CCcf)	:	OUTPUT	CCcf(pdu)	TO	pid;
	(CDrq)	:	OUTPUT	CDrq(pdu)	TO	pid;
	(CDin)	:	OUTPUT	CDin(pdu)	TO	pid;
	(CArq)	:	OUTPUT	CArq(pdu)	TO	pid;
	(CAin)	:	OUTPUT	CAin(pdu)	TO	pid;
	(CErq)	:	OUTPUT	CErq(pdu)	TO	pid;
	(CEin)	:	OUTPUT	CEin(pdu)	TO	pid;
	(SDrq)	:	OUTPUT	SDrq(pdu)	TO	pid;

(SDin)	:	OUTPUT	SDin(pdu)	TO	pid;
	(USrq)	:	OUTPUT	USrq(pdu)	TO	pid;
	(USin)	:	OUTPUT	USin(pdu)	TO	pid;
	(TGrq)	:	OUTPUT	TGrq(pdu)	TO	pid;
	(TGcf)	:	OUTPUT	TGcf(pdu)	TO	pid;
	(TIrq)	:	OUTPUT	TIrq(pdu)	TO	pid;
	(TIcf)	:	OUTPUT	TIcf(pdu)	TO	pid;
	(TVrq)	:	OUTPUT	TVrq(pdu)	TO	pid;
	(TVin)	:	OUTPUT	TVin(pdu)	TO	pid;
	(TVrs)	:	OUTPUT	TVrs(pdu)	TO	pid;
	(TVcf)	:	OUTPUT	TVcf(pdu)	TO	pid;
	(TPrq)	:	OUTPUT	TPrq(pdu)	TO	pid;
	(TPin)	:	OUTPUT	TPin(pdu)	TO	pid;
	(TRrq)	:	OUTPUT	TRrq(pdu)	TO	pid;
	(TRcf)	:	OUTPUT	TRcf(pdu)	TO	pid;
	(TTrq)	:	OUTPUT	TTrq(pdu)	TO	pid;
	(TTcf)	:	OUTPUT	TTcf(pdu)	TO	pid;

ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/* Input transitions */

DCL	p		PortalId,

dp	DataPriority,
		pdu	PDUStruct,
		pKind	PortalKind,
		pids	PIdByPri,
		reason	Reason;
	START
	COMMENT'The state machine contains a single state.
		  ';
	CALL	Initialize_resources;
	NEXTSTATE ~;

STATE ~;	INPUT	Open.portal(p, pKind, pids);

CALL	Open_portal(p, pKind, pids);
	NEXTSTATE -;

STATE ~;	INPUT	Time.portal(p);

CALL	Time_portal(p);
	NEXTSTATE -;

STATE ~;	INPUT	Drop.portal(p, reason);

CALL	Drop_portal(p, reason);
	NEXTSTATE -;

STATE ~;	INPUT	Shut.portal(p);

CALL	Shut_portal(p);
	DECISION pUsed = Empty;
	(False):NEXTSTATE -;
	(True): STOP;
	ENDDECISION;

STATE ~;	INPUT	PDU.ready(dp);

CALL	PDU_ready(dp);

NEXTSTATE -;

STATE ~;	INPUT	PDin(pdu);	TASK pdu!kind	:=  PDin;	CALL Input_PDU(pdu);	NEXTSTATE –;

STATE ~;	INPUT	EDrq(pdu);	TASK pdu!kind	:=  EDrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	MCrq(pdu); TASK pdu!kind	:=  MCrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	MCcf(pdu);	TASK pdu!kind	:=  MCcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	PCin(pdu);	TASK pdu!kind	:=  PCin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	MTrq(pdu);	TASK pdu!kind	:=  MTrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	MTcf(pdu);	TASK pdu!kind	:=  MTcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	PTin(pdu);	TASK pdu!kind	:=  PTin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	DPum(pdu); TASK pdu!kind   :=  DPum; CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	RJum(pdu); TASK pdu!kind	:=  RJum;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	AUrq(pdu);	TASK pdu!kind	:=  AUrq;	CALL Input_PDU(pdu);	NEXTSTATE –;

STATE ~;	INPUT	AUcf(pdu);	TASK pdu!kind	:=  AUcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	DUrq(pdu);	TASK pdu!kind	:=  DUrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	DUin(pdu);	TASK pdu!kind	:=  DUin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CJrq(pdu);	TASK pdu!kind	:=  CJrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CJcf(pdu);	TASK pdu!kind	:=  CJcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CLrq(pdu);	TASK pdu!kind	:=  CLrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CCrq(pdu);	TASK pdu!kind	:=  CCrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CCcf(pdu);	TASK pdu!kind	:=  CCcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CDrq(pdu);	TASK pdu!kind	:=  CDrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CDin(pdu);	TASK pdu!kind	:=  CDin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CArq(pdu);	TASK pdu!kind	:=  CArq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CAin(pdu);	TASK pdu!kind	:=  CAin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CErq(pdu);	TASK pdu!kind	:=  CErq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	CEin(pdu);	TASK pdu!kind	:=  CEin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	SDrq(pdu);	TASK pdu!kind	:=  SDrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	SDin(pdu);	TASK pdu!kind	:=  SDin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	USrq(pdu);	TASK pdu!kind	:=  USrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	USin(pdu);	TASK pdu!kind	:=  USin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TGrq(pdu);	TASK pdu!kind	:=  TGrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TGcf(pdu);	TASK pdu!kind	:=  TGcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TIrq(pdu);	TASK pdu!kind	:=  TIrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TIcf(pdu);	TASK pdu!kind	:=  TIcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TVrq(pdu);	TASK pdu!kind	:=  TVrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TVin(pdu);	TASK pdu!kind	:=  TVin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TVrs(pdu);	TASK pdu!kind	:=  TVrs;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TVcf(pdu);	TASK pdu!kind	:=  TVcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TPrq(pdu);	TASK pdu!kind	:=  TPrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TPin(pdu);	TASK pdu!kind	:=  TPin;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TRrq(pdu);	TASK pdu!kind	:=  TRrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TRcf(pdu);	TASK pdu!kind	:=  TRcf;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TTrq(pdu);	TASK pdu!kind	:=  TTrq;	CALL Input_PDU(pdu);	NEXTSTATE –;
STATE ~;	INPUT	TTcf(pdu);	TASK pdu!kind	:=  TTcf;	CALL Input_PDU(pdu);	NEXTSTATE –;

ENDPROCESS;

Appendix V

SDL specification of the Endpoint processXE "SDL specification of the Endpoint process"§

(This appendix does not form an integral part of this Recommendation)

PROCESS Endpoint;

FPAR	tcId		TCEndpointId,		/* incoming TC if not Null */
	localTSAP	TSAPAddress,		/* own transport address */
	remoteTSAP	TSAPAddress,		/* other transport address */
	givenQOS	TransportQOS,		/* target or offered QOS */
	minQOS	TransportQOS,		/* minimum acceptable QOS */
	parameters	DomainParameters;	/* values established or null */

/* Data declarations */

DCL	control		PId,			/* single Control process */

domain		PId;			/* selected Domain process */

/* Input transitions */

DCL	localDomain	DomainSelector,

remoteDomain	DomainSelector,
		upward	Boolean,
		targetParms	DomainParameters,
		minParms	DomainParameters,
		maxParms	DomainParameters,
		userData	UserData,
		result		Result,
		ccId		Natural,
		label		Natural,
		tsdu		TSDU,
		dp		DataPriority,
		pdu		PDUStruct;

START
	COMMENT'State machine:				NEXTSTATE
			1   connecting		*	.	2	.	.	.	6
			2   connbusy		*	.	2	3	.	5	6
			3   connready			.	2	3	.	5	6
			4   busy				.	.	.	4	5	6
			5   ready				.	.	.	4	5	6
			6   disconnected		.	.	.	.	.	6
		';
	TASK	control  :=  PARENT,
		label  :=  0;
	DECISION tcId = Null;
	(True):	OUTPUT  T.Connect.request(label, localTSAP, remoteTSAP,
						givenQOS, minQOS);
		NEXTSTATE connecting;
	(False):	OUTPUT  T.Connect.response(tcId, givenQOS);
		OUTPUT  T.ready(tcId);
		NEXTSTATE connbusy;
	ENDDECISION;

STATE *;

INPUT	Exit;
	STOP;

STATE *;

INPUT	T.Disconnect.indication(label, tcId);
	OUTPUT	Quit TO control;
	NEXTSTATE disconnected;

STATE connecting;

INPUT	T.Connect.confirm(label, tcId, givenQOS);
	OUTPUT	T.ready(tcId);
	NEXTSTATE connbusy;

STATE connecting;

SAVE	*;  /* await outcome of TC */

STATE connbusy, connready, busy, ready;

INPUT	*;
	OUTPUT	T.Disconnect.request(tcId);
	OUTPUT	Quit TO control;
	NEXTSTATE disconnected;

STATE connbusy;

INPUT	T.ready(tcId);
	NEXTSTATE connready;

STATE connbusy;

SAVE	Connect.Initial, Connect.Response, Connect.Additional, Connect.Result;

STATE connready;

INPUT	Connect.Initial(localDomain, remoteDomain, upward,
		targetParms, minParms, maxParms, userData);
	TASK'tsdu  :=  encode Connect-Initial using BER';
	   1b :
	OUTPUT	T.Data.request(tcId, tsdu);
	NEXTSTATE connbusy;

STATE connready;

INPUT	Connect.Response(result, ccId, parameters, userData);
	TASK'tsdu  :=  encode Connect-Response using BER';
	JOIN 1b;

STATE connready;

INPUT	Connect.Additional(ccId, dp);
	TASK'tsdu  :=  encode Connect-Additional using BER';
	JOIN 1b;

STATE connready;

INPUT	Connect.Result(result);
	TASK'tsdu  :=  encode Connect-Result using BER';
	JOIN 1b;

STATE connbusy, connready;

INPUT	T.Data.indication(tcId, tsdu);
	TASK'connect MCSPDU  :=  decode tsdu using BER';
	DECISION'connect MCSPDU';
	('Connect-Initial'):
		OUTPUT	Connect.Initial(localDomain, remoteDomain, upward,
				targetParms, minParms, maxParms, userData) TO control;
		NEXTSTATE -;
	('Connect-Response'):
		OUTPUT	Connect.Response(result, ccId, parameters, userData) TO control;
		NEXTSTATE -;
	('Connect-Additional'):
		OUTPUT	Connect.Additional(ccId, dp) TO control;
		NEXTSTATE -;
	('Connect-Result'):
		OUTPUT	Connect.Result(result) TO control;
		NEXTSTATE -;
	ELSE:
		OUTPUT	T.Disconnect.request(tcId);
		OUTPUT	Quit TO control;
		NEXTSTATE disconnected;
	ENDDECISION;

STATE connbusy;

INPUT	PDU.ready(dp);
	TASK	domain  :=  SENDER;
	OUTPUT	T.ready(tcId);
	NEXTSTATE ready;

STATE connready;

INPUT	PDU.ready(dp);
	TASK	domain  :=  SENDER;
	OUTPUT	PDU.ready(dp) TO domain;
	OUTPUT	T.ready(tcId);
	NEXTSTATE ready;

STATE busy;

INPUT	PDU.ready(dp);
	OUTPUT	T.ready(tcId);
	NEXTSTATE ready;

STATE busy, ready;

INPUT	T.ready(tcId);
	OUTPUT	PDU.ready(dp) TO domain;
	NEXTSTATE -;

STATE busy, ready;

INPUT	PDin(pdu);	TASK pdu!kind	:=	PDin;	 JOIN	2f;
	INPUT	EDrq(pdu);	TASK pdu!kind	:=	EDrq;	 JOIN	2f;
	INPUT	MCrq(pdu); TASK pdu!kind	:=	MCrq;	 JOIN	2f;
	INPUT	MCcf(pdu);	TASK pdu!kind	:=	MCcf;	 JOIN	2f;
	INPUT	PCin(pdu);	TASK pdu!kind	:=	PCin;	 JOIN	2f;
	INPUT	MTrq(pdu);	TASK pdu!kind	:=	MTrq;	 JOIN	2f;
	INPUT	MTcf(pdu);	TASK pdu!kind	:=	MTcf;	 JOIN	2f;
	INPUT	PTin(pdu);	TASK pdu!kind	:=	PTin;	 JOIN	2f;
	INPUT	DPum(pdu);TASK pdu!kind	:=	DPum; JOIN	2f;
	INPUT	RJum(pdu);TASK pdu!kind	:=	RJum; JOIN	2f;
	INPUT	AUrq(pdu);	TASK pdu!kind	:=	AUrq;	JOIN	2f;
	INPUT	AUcf(pdu);	TASK pdu!kind	:=	AUcf;	JOIN	2f;
	INPUT	DUrq(pdu);	TASK pdu!kind	:=	DUrq;	JOIN	2f;
	INPUT	DUin(pdu);	TASK pdu!kind	:=	DUin;	JOIN	2f;
	INPUT	CJrq(pdu);	TASK pdu!kind	:=	CJrq;	JOIN	2f;
	INPUT	CJcf(pdu);	TASK pdu!kind	:=	CJcf;	JOIN	2f;
	INPUT	CLrq(pdu);	TASK pdu!kind	:=	CLrq;	JOIN	2f;
	INPUT	CCrq(pdu);	TASK pdu!kind	:=	CCrq;	JOIN	2f;
	INPUT	CCcf(pdu);	TASK pdu!kind	:=	CCcf;	JOIN	2f;
	INPUT	CDrq(pdu);	TASK pdu!kind	:=	CDrq;	JOIN	2f;
	INPUT	CDin(pdu);	TASK pdu!kind	:=	CDin;	JOIN	2f;

INPUT	CArq(pdu);	TASK pdu!kind	:=	CArq;	JOIN	2f;
	INPUT	CAin(pdu);	TASK pdu!kind	:=	CAin;	JOIN	2f;
	INPUT	CErq(pdu);	TASK pdu!kind	:=	CErq;	JOIN	2f;
	INPUT	CEin(pdu);	TASK pdu!kind	:=	CEin;	JOIN	2f;
	INPUT	SDrq(pdu);	TASK pdu!kind	:=	SDrq;	JOIN	2f;
	INPUT	SDin(pdu);	TASK pdu!kind	:=	SDin;	JOIN	2f;
	INPUT	USrq(pdu);	TASK pdu!kind	:=	USrq;	JOIN	2f;
	INPUT	USin(pdu);	TASK pdu!kind	:=	USin;	JOIN	2f;
	INPUT	TGrq(pdu);	TASK pdu!kind	:=	TGrq;	JOIN	2f;
	INPUT	TGcf(pdu);	TASK pdu!kind	:=	TGcf;	JOIN	2f;
	INPUT	TIrq(pdu);	TASK pdu!kind	:=	TIrq;	JOIN	2f;
	INPUT	TIcf(pdu);	TASK pdu!kind	:=	TIcf;	JOIN	2f;
	INPUT	TVrq(pdu);	TASK pdu!kind	:=	TVrq;	JOIN	2f;
	INPUT	TVin(pdu);	TASK pdu!kind	:=	TVin;	JOIN	2f;
	INPUT	TVrs(pdu);	TASK pdu!kind	:=	TVrs;	JOIN	2f;
	INPUT	TVcf(pdu);	TASK pdu!kind	:=	TVcf;	JOIN	2f;
	INPUT	TPrq(pdu);	TASK pdu!kind	:=	TPrq;	JOIN	2f;
	INPUT	TPin(pdu);	TASK pdu!kind	:=	TPin;	JOIN	2f;
	INPUT	TRrq(pdu);	TASK pdu!kind	:=	TRrq;	JOIN	2f;
	INPUT	TRcf(pdu);	TASK pdu!kind	:=	TRcf;	JOIN	2f;
	INPUT	TTrq(pdu);	TASK pdu!kind	:=	TTrq;	JOIN	2f;
	INPUT	TTcf(pdu);	TASK pdu!kind	:=	TTcf;

2f :
	TASK'tsdu  :=  encode pdu using BER or PER,
		  depending on parameters!protocolVersion';
	OUTPUT	T.Data.request(tcId, tsdu);
	NEXTSTATE -;

STATE ready;

INPUT	T.Data.indication(tcId, tsdu);
	TASK'pdu  :=  decode tsdu using BER or PER,
		  depending on parameters!protocolVersion';
	DECISION pdu!kind;
	(RJum):	OUTPUT  RJum(pdu) TO control;
			OUTPUT  T.ready(tcId);
			NEXTSTATE -;

(PDin):	OUTPUT	PDin(pdu)	TO domain;	NEXTSTATE	busy;
	(EDrq):	OUTPUT	EDrq(pdu)	TO domain;	NEXTSTATE	busy;
	(MCrq):	OUTPUT	MCrq(pdu)	TO domain;	NEXTSTATE	busy;
	(MCcf):	OUTPUT	MCcf(pdu)	TO domain;	NEXTSTATE	busy;
	(PCin):	OUTPUT	PCin(pdu)	TO domain;	NEXTSTATE	busy;
	(MTrq):	OUTPUT	MTrq(pdu)	TO domain;	NEXTSTATE	busy;
	(MTcf):	OUTPUT	MTcf(pdu)	TO domain;	NEXTSTATE	busy;
	(PTin):	OUTPUT	PTin(pdu)	TO domain;	NEXTSTATE	busy;
	(DPum):	OUTPUT	DPum(pdu) TO domain;	NEXTSTATE	busy;
	(AUrq):	OUTPUT	AUrq(pdu)	TO domain;	NEXTSTATE	busy;
	(AUcf):	OUTPUT	AUcf(pdu)	TO domain;	NEXTSTATE	busy;
	(DUrq):	OUTPUT	DUrq(pdu)	TO domain;	NEXTSTATE	busy;
	(DUin):	OUTPUT	DUin(pdu)	TO domain;	NEXTSTATE	busy;
	(CJrq):	OUTPUT	CJrq(pdu)	TO domain;	NEXTSTATE	busy;
	(CJcf):	OUTPUT	CJcf(pdu)	TO domain;	NEXTSTATE	busy;
	(CLrq):	OUTPUT	CLrq(pdu)	TO domain;	NEXTSTATE	busy;
	(CCrq):	OUTPUT	CCrq(pdu)	TO domain;	NEXTSTATE	busy;
	(CCcf):	OUTPUT	CCcf(pdu)	TO domain;	NEXTSTATE	busy;
	(CDrq):	OUTPUT	CDrq(pdu)	TO domain;	NEXTSTATE	busy;
	(CDin):	OUTPUT	CDin(pdu)	TO domain;	NEXTSTATE	busy;
	(CArq):	OUTPUT	CArq(pdu)	TO domain;	NEXTSTATE	busy;
	(CAin):	OUTPUT	CAin(pdu)	TO domain;	NEXTSTATE	busy;
	(CErq):	OUTPUT	CErq(pdu)	TO domain;	NEXTSTATE	busy;
	(CEin):	OUTPUT	CEin(pdu)	TO domain;	NEXTSTATE	busy;
	(SDrq):	OUTPUT	SDrq(pdu)	TO domain;	NEXTSTATE	busy;
	(SDin):	OUTPUT	SDin(pdu)	TO domain;	NEXTSTATE	busy;
	(USrq):	OUTPUT	USrq(pdu)	TO domain;	NEXTSTATE	busy;
	(USin):	OUTPUT	USin(pdu)	TO domain;	NEXTSTATE	busy;
	(TGrq):	OUTPUT	TGrq(pdu)	TO domain;	NEXTSTATE	busy;
	(TGcf):	OUTPUT	TGcf(pdu)	TO domain;	NEXTSTATE	busy;

(TIrq):	OUTPUT	TIrq(pdu)	TO domain;	NEXTSTATE	busy;
	(TIcf):	OUTPUT	TIcf(pdu)	TO domain;	NEXTSTATE	busy;
	(TVrq):	OUTPUT	TVrq(pdu)	TO domain;	NEXTSTATE	busy;
	(TVin):	OUTPUT	TVin(pdu)	TO domain;	NEXTSTATE	busy;
	(TVrs):	OUTPUT	TVrs(pdu)	TO domain;	NEXTSTATE	busy;
	(TVcf):	OUTPUT	TVcf(pdu)	TO domain;	NEXTSTATE	busy;
	(TPrq):	OUTPUT	TPrq(pdu)	TO domain;	NEXTSTATE	busy;
	(TPin):	OUTPUT	TPin(pdu)	TO domain;	NEXTSTATE	busy;
	(TRrq):	OUTPUT	TRrq(pdu)	TO domain;	NEXTSTATE	busy;
	(TRcf):	OUTPUT	TRcf(pdu)	TO domain;	NEXTSTATE	busy;
	(TTrq):	OUTPUT	TTrq(pdu)	TO domain;	NEXTSTATE	busy;
	(TTcf):	OUTPUT	TTcf(pdu)	TO domain;	NEXTSTATE	busy;
	ELSE:

TASK'pdu!initialOctets  :=  truncate tsdu';
			TASK'pdu!diagnostic  :=  DC_invalid_?ER_encoding';
			OUTPUT  RJum(pdu) TO domain;
			NEXTSTATE busy;
	ENDDECISION;

STATE disconnected;

INPUT   Quit;
	OUTPUT  Quit TO control;
	NEXTSTATE disconnected;

ENDPROCESS;

Appendix VI

SDL specification of the Attachment processXE "SDL specification of the Attachment process"§

(This appendix does not form an integral part of this Recommendation)

PROCESS Attachment;

FPAR	label		Natural,			/* for MCS.Attach.User.confirm */
	parameters	DomainParameters;	/* values established in domain */

/* Type definitions */

NEWTYPE		MCSRequest

STRUCT
	kind		PDUKind;		/* SDrq, USrq, CArq, CErq */
	channelId	ChannelId;		/* parameter of request */
	segmentation	Segmentation;	/* parameter of request */
	userData		UserData;		/* parameter of request */
	offset		Integer;			/* octets sent so far */
	userIds		UserIdSet;		/* remaining to affect */
ENDNEWTYPE;

NEWTYPE		PrioritySet	SetOf(DataPriority);

ENDNEWTYPE;

NEWTYPE		MCSRequestByPri	Array(DataPriority, MCSRequest);

ENDNEWTYPE;
NEWTYPE	PDUStructByPri		Array(DataPriority, PDUStruct);
ENDNEWTYPE;

/* Data declarations */

DCL	maId		MCSAttachmentId,	/* this Attachment process */

control		PId,			/* single Control process */
	domain		PId;			/* selected Domain process */

DCL	user		UserId;	/* unique id of attached user */

DCL	cJoined		ChannelIdSet,	/* channels the user has joined */

cConvened	ChannelIdSet,		/* channels the user has convened */
	cAdmitted	ChannelIdSet;		/* channels user was admitted to */

DCL	tPossessed	TokenIdSet,	/* tokens grabbed or inhibited */

tRecipient	TokenIdSet;		/* tokens given waiting response */

DCL	uPending		PrioritySet,	/* request is pending at 0..3 */

mcsreq		MCSRequestByPri,	/* content of segmented request */
	dReady		PrioritySet;		/* domain MCSPDU allowed 0..? */

DCL	dPending		PrioritySet,	/* SDin or USin pending at 0..3 */

mcspdu		PDUStructByPri,	/* content of the domain MCSPDU */
	uReady		PrioritySet;		/* data indication allowed 0..3 */

/* Procedure decomposition */

/*		Segment_request	(dp)

Indicate_data
		Track_token			(t, status)     */
							/*-----------------------------*/
PROCEDURE	Segment_request;			/*  Segment_request  */
FPAR		dp	DataPriority;			/*----------------------------*/

DCL	udp	DataPriority,

req	MCSRequest,
		pdu	PDUStruct,
		b	Boolean,
		m	Integer,
		n	Integer,
		u	UserId;
	START
	COMMENT'Feed domain process the next segment of user data
		  or the next subset of private channel user ids,
		  if ready, for the specified transport priority.
		';
	DECISION dp in dReady;
	(False):RETURN;
	ELSE:ENDDECISION;
	TASK	udp  :=  dp;
	   1b :	/* for udp = dp..? */
	DECISION udp = dp or (udp >= parameters!numPriorities and udp < 4);
	(False):	RETURN;
	(True): 	DECISION udp in uPending;
		(False):	TASK    udp  :=  udp + 1;
			JOIN 1b;
		ELSE:ENDDECISION;
	ENDDECISION;
	TASK	dReady  :=  Del(dp, dReady),
		dp  :=  udp,
		req  :=  mcsreq(dp),
		pdu!initiator  :=  user,
		pdu!channelId  :=  req!channelId;
	DECISION req!kind;
	(SDrq, USrq):
		TASK	pdu!dataPriority  :=  dp,
			b  :=  req!segmentation!begin,
			pdu!segmentation!begin  :=  IF req!offset = 0 THEN b ELSE False FI,
			m  :=  parameters!maxMCSPDUsize,
			m  :=  IF parameters!protocolVersion = 1 THEN m - 24 ELSE m - 8 FI,
			n  :=  Length(req!userData) - req!offset,
			n  :=  IF n > m THEN m ELSE n FI,
			pdu!userData  :=  Substring(req!userData, 1 + req!offset, n),
			req!offset  :=  req!offset + n,
			n  :=  Length(req!userData) - req!offset,
			b  :=  req!segmentation!end,
			pdu!segmentation!end  :=  IF n = 0 THEN b ELSE False FI,
			mcsreq(dp)!offset  :=  req!offset;
	(CArq, CErq):
		TASK	pdu!userIds  :=  Empty,
			n  :=  0,
			m  :=  parameters!maxMCSPDUsize,
			m  :=  IF parameters!protocolVersion = 1 THEN (m - 16) / 4
					    ELSE (m - 7) / 2 FI;
		   2b :	/* while n < m */
		DECISION req!userIds = Empty;
		(True):	TASK	n  :=  0;
		(False):	TASK	u  :=  Pick(req!userIds),
				req!userIds  :=  Del(u, req!userIds);
			DECISION u >= 1001;
			(True):	TASK    pdu!userIds  :=  Incl(u, pdu!userIds);
			ELSE:ENDDECISION;
			TASK	n  :=  n + 1;
			DECISION n < m;
			(True): JOIN 2b;
			ELSE:ENDDECISION;
		ENDDECISION;
	ENDDECISION;
	DECISION req!kind;
	(SDrq):	OUTPUT  SDrq(pdu) TO domain;
	(USrq): 	OUTPUT  USrq(pdu) TO domain;
	(CArq): 	OUTPUT  CArq(pdu) TO domain;
	(CErq): 	OUTPUT  CErq(pdu) TO domain;
	ENDDECISION;
	DECISION n = 0;
	(True): 	TASK	uPending  :=  Del(dp, uPending);
		DECISION req!kind;
		(SDrq, USrq):
			OUTPUT  MCS.ready(maId, dp);
		ELSE:ENDDECISION;
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/*-----------------------*/

PROCEDURE	Indicate_data;			/*  Indicate_data  */
						/*----------------------*/

DCL	dp	DataPriority,

pdu	PDUStruct;
	START
	COMMENT'Indicate the receipt of user data, if ready
		  and none pending at higher priorities.
		  ';
	TASK	dp  :=  0;
	   1b :	/* for dp = 0..3 */
	DECISION dp < 4 and (dp in uReady or not dp in dPending);
	(False):	RETURN;
	(True):	DECISION dp in dPending;
		(False):	TASK    dp  :=  dp + 1;
			JOIN 1b;
		ELSE:ENDDECISION;
	ENDDECISION;
	TASK	dPending  :=  Del(dp, dPending),
		pdu  :=  mcspdu(dp);
	DECISION pdu!kind;
	(SDin):	OUTPUT  MCS.Send.Data.indication(maId, pdu!channelId,
			  pdu!dataPriority, pdu!initiator, pdu!segmentation, pdu!userData);
	(USin):	OUTPUT  MCS.Uniform.Send.Data.indication(maId, pdu!channelId,
			  pdu!dataPriority, pdu!initiator, pdu!segmentation, pdu!userData);
	ENDDECISION;
	TASK	uReady  :=  Del(dp, uReady),
		dp  :=  IF dp < parameters!numPriorities THEN dp
				ELSE parameters!numPriorities - 1 FI;
	OUTPUT	PDU.ready(dp) TO domain;
	JOIN 1b;
	ENDPROCEDURE;

/*---------------------*/

PROCEDURE		Track_token;		/*  Track_token  */
FPAR			t	TokenId,	/*---------------------*/
			status  TokenStatus;

START

COMMENT'Condense status into possessed or recipient.
		';
	TASK	tPossessed  :=  Del(t, tPossessed),
		tRecipient  :=  Del(t, tRecipient);
	DECISION status;
	(SelfGrabbed, SelfInhibited, SelfGiving):
		TASK	tPossessed  :=  Incl(t, tPossessed);
	(SelfRecipient):
		TASK	tRecipient  :=  Incl(t, tRecipient);
	ELSE:ENDDECISION;
	RETURN;
	ENDPROCEDURE;

/* Input transitions */

DCL	dp		DataPriority,

pdu		PDUStruct,
		c		ChannelId,
		cSet		ChannelIdSet,
		t		TokenId,
		u		UserId,
		uSet		UserIdSet,
		segmentation	Segmentation,
		userData	UserData,
		result		Result,
		kind		PDUKind,
		reason		Reason;
	START

COMMENT'State machine:				NEXTSTATE

1   initial			*	.	2	.	.	.	6
			2   attaching			.	2	3	4	.	6
			3   busy				.	.	3	4	5	6
			4   ready				.	.	3	4	.	6
			5   detaching			.	.	.	.	5	6
			6   detached			.	.	.	.	.	6

';
	TASK	maId  :=  SELF,
		control  :=  PARENT,
		user  :=  0;
	NEXTSTATE initial;

STATE *;

INPUT	Exit;
	STOP;

STATE initial, attaching;

INPUT	*;
	OUTPUT	MCS.Attach.User.confirm(label, RT_unspecified_failure, maId, user);
	OUTPUT	Quit TO control;
	NEXTSTATE detached;

STATE initial, attaching;

INPUT	Quit;
	OUTPUT	MCS.Attach.User.confirm(label, RT_domain_disconnected, maId, user);
	OUTPUT	Quit TO control;
	NEXTSTATE detached;

STATE initial;

INPUT	PDU.ready(dp);
	TASK	domain  :=  SENDER;
	DECISION dp = 0;
	(False):	TASK	dReady  :=  Incl(dp, dReady);
		NEXTSTATE -;
	ELSE:ENDDECISION;
	OUTPUT	PDU.ready(0) TO domain;
	OUTPUT	AUrq(pdu) TO domain;
	NEXTSTATE attaching;

STATE attaching;

INPUT	PDU.ready(dp);
	TASK	dReady  :=  Incl(dp, dReady);
	NEXTSTATE -;

STATE attaching;

INPUT	PCin(pdu), PTin(pdu), DUin(pdu);
	/* no action */
	NEXTSTATE -;

STATE attaching;

INPUT	AUcf(pdu);
	TASK	user  :=  pdu!initiator;
	OUTPUT	MCS.Attach.User.confirm(label, pdu!result, maId, user);
	DECISION pdu!result = RT_successful;
	(False):	OUTPUT  Quit TO control;
		NEXTSTATE detached;
	(True):	TASK	dp  :=  0;
		   1b :	/* for dp = 0..3 */
		DECISION dp < 4;
		(True):	OUTPUT  MCS.ready(maId, dp);
			DECISION dp < parameters!numPriorities;
			(True):	OUTPUT  PDU.ready(dp) TO domain;
			ELSE:ENDDECISION;
			TASK	dp  :=  dp + 1;
			JOIN 1b;
		ELSE:ENDDECISION;
		DECISION 0 in dReady;
		(False):	NEXTSTATE busy;
		(True):	NEXTSTATE ready;
		ENDDECISION;
	ENDDECISION;

STATE busy, ready;

INPUT	Quit;
	OUTPUT	MCS.Detach.User.indication(maId, user, RN_provider_initiated);
	OUTPUT	Quit TO control;
	NEXTSTATE detached;

STATE busy, ready;

INPUT	MCS.ready(maId, dp);
	TASK	uReady  :=  Incl(dp, uReady);
	CALL	Indicate_data;
	NEXTSTATE -;

STATE busy;

INPUT	MCS.Detach.User.request(maId);
	TASK	reason  :=  RN_user_requested;
	NEXTSTATE detaching;

STATE busy;

SAVE	*;  /* defer MCS other */

STATE ready;

INPUT	MCS.Detach.User.request(maId);
	TASK	pdu!reason  :=  RN_user_requested,
		pdu!userIds  :=  Incl(user, Empty);
	OUTPUT	DUrq(pdu) TO domain;
	NEXTSTATE detached;

STATE ready;

INPUT	MCS.Channel.Join.request(maId, c);
	TASK	pdu!initiator  :=  user,
		pdu!channelId  :=  c;
	OUTPUT	CJrq(pdu) TO domain;
	   2b :
	TASK	dReady  :=  Del(0, dReady);
	NEXTSTATE busy;

STATE ready;

INPUT	MCS.Channel.Leave.request(maId, c);
	TASK	cJoined  :=  Del(c, cJoined),
		pdu!channelIds  :=  Incl(c, Empty);
	OUTPUT	CLrq(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Channel.Convene.request(maId);
	TASK	pdu!initiator  :=  user;
	OUTPUT	CCrq(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Channel.Disband.request(maId, c);
	DECISION c in cConvened;
	(True):	TASK	cAdmitted  :=  Del(c, cAdmitted),
			cJoined  :=  Del(c, cJoined);
	ELSE:ENDDECISION;
	TASK	cConvened  :=  Del(c, cConvened),
		pdu!initiator  :=  user,
		pdu!channelId  :=  c;
	OUTPUT  CDrq(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Channel.Admit.request(maId, c, uSet);
	TASK	kind  :=  CArq;
	JOIN 3f;

STATE ready;

INPUT	MCS.Channel.Expel.request(maId, c, uSet);
	TASK	kind  :=  CErq;
	   3f :
	TASK	mcsreq(0)!kind  :=  kind,
		mcsreq(0)!channelId  :=  c,
		mcsreq(0)!userIds  :=  uSet,
		uPending  :=  Incl(0, uPending);
	CALL	Segment_request(0);
	DECISION 0 in dReady;
	(False):NEXTSTATE busy;
	(True): NEXTSTATE ready;
	ENDDECISION;

STATE ready;

INPUT	MCS.Send.Data.request(maId, c, dp, segmentation, userData);
	TASK	kind  :=  SDrq;
	JOIN 4f;

STATE ready;

INPUT	MCS.Uniform.Send.Data.request(maId, c, dp, segmentation, userData);
	TASK	kind  :=  USrq;
	   4f :
	DECISION dp >= 4 or dp in uPending;
	(True):	OUTPUT  MCS.Detach.User.indication(maId, user, RN_provider_initiated);
		TASK	pdu!reason  :=  RN_provider_initiated,
			pdu!userIds  :=  Incl(user, Empty);
		OUTPUT  DUrq(pdu) TO domain;
		NEXTSTATE detached;
	(False):	TASK	mcsreq(dp)!kind  :=  kind,
			mcsreq(dp)!channelId  :=  c,
			mcsreq(dp)!segmentation  :=  segmentation,
			mcsreq(dp)!userData  :=  userData,
			mcsreq(dp)!offset  :=  0,
			uPending  :=  Incl(dp, uPending),
			dp  :=  IF dp < parameters!numPriorities THEN dp
					ELSE parameters!numPriorities - 1 FI;
		CALL	Segment_request(dp);
		DECISION 0 in dReady;
		(False):	NEXTSTATE busy;

(True):	NEXTSTATE ready;
		ENDDECISION;
	ENDDECISION;

STATE ready;

INPUT	MCS.Token.Grab.request(maId, t);
	TASK	pdu!initiator  :=  user,
		pdu!tokenId  :=  t;
	OUTPUT	TGrq(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Token.Inhibit.request(maId, t);
	TASK	pdu!initiator  :=  user,
		pdu!tokenId  :=  t;
	OUTPUT	TIrq(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Token.Give.request(maId, t, u);
	DECISION u >= 1001;
	(False):	OUTPUT  MCS.Token.Give.confirm(maId, t, RT_no_such_user);
		NEXTSTATE -;

(True): 	TASK	pdu!initiator  :=  user,
			pdu!tokenId  :=  t,
			pdu!recipient  :=  u;
		OUTPUT	TVrq(pdu) TO domain;

JOIN 2b;
	ENDDECISION;

STATE ready;

INPUT	MCS.Token.Give.response(maId, t, result);
	DECISION result	= RT_successful;
	(False):TASK	result  :=  RT_user_rejected;
	ELSE:ENDDECISION;
	DECISION t in tRecipient and result = RT_successful;
	(True):	TASK	tPossessed  :=  Incl(t, tPossessed);
	ELSE:ENDDECISION;
	TASK	tRecipient  :=  Del(t, tRecipient),
		pdu!result  :=  result,
		pdu!recipient  :=  user,
		pdu!tokenId  :=  t;
	OUTPUT	TVrs(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Token.Please.request(maId, t);
	TASK	pdu!initiator  :=  user,
		pdu!tokenId  :=  t;
	OUTPUT	TPrq(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Token.Release.request(maId, t);
	TASK	tPossessed  :=  Del(t, tPossessed),
		pdu!initiator  :=  user,
		pdu!tokenId  :=  t;
	OUTPUT	TRrq(pdu) TO domain;
	JOIN 2b;

STATE ready;

INPUT	MCS.Token.Test.request(maId, t);
	TASK	pdu!initiator  :=  user,
		pdu!tokenId  :=  t;
	OUTPUT  TTrq(pdu) TO domain;
	JOIN 2b;

STATE busy, ready;

INPUT	PDU.ready(dp);
	TASK	dReady  :=  Incl(dp, dReady);
	CALL	Segment_request(dp);
	DECISION 0 in dReady;
	(False):	NEXTSTATE busy;
	(True):	NEXTSTATE ready;
	ENDDECISION;

STATE busy, ready;

INPUT	PCin(pdu);
	TASK	reason  :=  RN_channel_purged;
	DECISION user in pdu!detachUserIds;
	(True):	OUTPUT  MCS.Detach.User.indication(maId, user, reason);
		OUTPUT  Quit TO control;
		NEXTSTATE detached;
	(False):	TASK	uSet  :=  pdu!detachUserIds;
		   5b :	/* for u in uSet */
		DECISION uSet = Empty;
		(False):	TASK	u  :=  Pick(uSet),
				uSet  :=  Del(u, uSet);
			OUTPUT  MCS.Detach.User.indication(maId, u, reason);
			JOIN 5b;
		ELSE:ENDDECISION;
		TASK	cSet  :=  pdu!purgeChannelIds;
		   6b :	/* for c in cSet */
		DECISION cSet = Empty;
		(False):	TASK	c  :=  Pick(cSet),
				cSet  :=  Del(c, cSet);
			DECISION c in cConvened;
			(True):	TASK	cConvened  :=  Del(c, cConvened),
					cAdmitted  :=  Del(c, cAdmitted),
					cJoined  :=  Del(c, cJoined);
				OUTPUT  MCS.Channel.Disband.indication(maId, c, reason);
			ELSE:ENDDECISION;
			DECISION c in cAdmitted;
			(True):	TASK	cAdmitted  :=  Del(c, cAdmitted),
					cJoined  :=  Del(c, cJoined);
				OUTPUT  MCS.Channel.Expel.indication(maId, c, reason);
			ELSE:ENDDECISION;
			DECISION c in cJoined;
			(True):	TASK	cJoined  :=  Del(c, cJoined);
				OUTPUT  MCS.Channel.Leave.indication(maId, c, reason);
			ELSE:ENDDECISION;
			JOIN 6b;
		ELSE:ENDDECISION;
		OUTPUT  PDU.ready(0) TO domain;
		NEXTSTATE -;
	ENDDECISION;

STATE busy;

INPUT	PTin(pdu);
	DECISION (pdu!purgeTokenIds and (tPossessed or tRecipient)) = Empty;
	(False):	OUTPUT  MCS.Detach.User.indication(maId, user, RN_token_purged);
		TASK	reason  :=  RN_token_purged;
		NEXTSTATE detaching;
	(True):	OUTPUT  PDU.ready(0) TO domain;
		NEXTSTATE -;
	ENDDECISION;

STATE ready;

INPUT	PTin(pdu);
	DECISION (pdu!purgeTokenIds and (tPossessed or tRecipient)) = Empty;
	(False):	OUTPUT  MCS.Detach.User.indication(maId, user, RN_token_purged);
		TASK	pdu!reason  :=  RN_token_purged,
			pdu!userIds  :=  Incl(user, Empty);
		OUTPUT  DUrq(pdu) TO domain;
		NEXTSTATE detached;
	(True):	OUTPUT  PDU.ready(0) TO domain;
		NEXTSTATE -;
	ENDDECISION;

STATE busy, ready;

INPUT	AUcf(pdu);
	OUTPUT	MCS.Detach.User.indication(maId, user, RN_unspecified);
	OUTPUT	Quit TO control;
	NEXTSTATE detached;

STATE busy, ready;

INPUT	DUin(pdu);
	DECISION user in pdu!userIds;
	(True):	OUTPUT  MCS.Detach.User.indication(maId, user, pdu!reason);
		OUTPUT  Quit TO control;
	NEXTSTATE detached;
	(False):	TASK	uSet  :=  pdu!userIds;
		   7b :	/* for u in uSet */
		DECISION uSet = Empty;
		(False):	TASK	u  :=  Pick(uSet),
				uSet  :=  Del(u, uSet);
			OUTPUT  MCS.Detach.User.indication(maId, u, pdu!reason);
			JOIN 7b;
		ELSE:ENDDECISION;
		OUTPUT  PDU.ready(0) TO domain;
		NEXTSTATE -;
	ENDDECISION;

STATE busy, ready;

INPUT	CJcf(pdu);
	TASK	c  :=  pdu!channelId;
	DECISION pdu!result = RT_successful;
	(True):	TASK    cJoined  :=  Incl(c, cJoined);
	ELSE:ENDDECISION;
	OUTPUT	MCS.Channel.Join.confirm(maId, pdu!requested, pdu!result, c);
	   8b :
	OUTPUT	PDU.ready(0) TO domain;
	NEXTSTATE -;

STATE busy, ready;

INPUT	CCcf(pdu);
	TASK	c  :=  pdu!channelId;
	DECISION pdu!result = RT_successful;
	(True): 	TASK	cConvened  :=  Incl(c, cConvened),
			cAdmitted  :=  Incl(c, cAdmitted);
	ELSE:ENDDECISION;
	OUTPUT	MCS.Channel.Convene.confirm(maId, pdu!result, c);
	JOIN 8b;

STATE busy, ready;

INPUT	CDin(pdu);
	TASK	c  :=  pdu!channelId,
		reason  :=  RN_channel_disbanded;
	DECISION c in cConvened;
	(True):	TASK	cConvened  :=  Del(c, cConvened),
			cAdmitted  :=  Del(c, cAdmitted),
			cJoined  :=  Del(c, cJoined);
		OUTPUT  MCS.Channel.Disband.indication(maId, c, reason);
	ELSE:ENDDECISION;
	DECISION c in cAdmitted;
	(True):	TASK	cAdmitted  :=  Del(c, cAdmitted),
			cJoined  :=  Del(c, cJoined);
		OUTPUT  MCS.Channel.Expel.indication(maId, c, reason);
	ELSE:ENDDECISION;
	JOIN 8b;

STATE busy, ready;

INPUT	CAin(pdu);
	TASK	c  :=  pdu!channelId,
		cAdmitted  :=  Incl(c, cAdmitted);
	OUTPUT	MCS.Channel.Admit.indication(maId, c, pdu!initiator);
	JOIN 8b;

STATE busy, ready;

INPUT	CEin(pdu);
	TASK	c  :=  pdu!channelId,
		reason  :=  RN_user_requested;
	DECISION c in cAdmitted;
	(True):	TASK	cAdmitted  :=  Del(c, cAdmitted),
			cJoined  :=  Del(c, cJoined);
		OUTPUT  MCS.Channel.Expel.indication(maId, c, reason);
	ELSE:ENDDECISION;
	JOIN 8b;

STATE busy, ready;

INPUT	SDin(pdu);
	TASK	pdu!kind  :=  SDin;
	JOIN 9f;

STATE busy, ready;

INPUT	USin(pdu);
	TASK	pdu!kind  :=  USin;
	   9f :
	TASK	c  :=  pdu!channelId,
		dp  :=  pdu!dataPriority;
	DECISION c in cJoined;
	(True):	TASK	mcspdu(dp)  :=  pdu,
			dPending  :=  Incl(dp, dPending);
		CALL	Indicate_data;
	(False):	TASK	dp  :=  IF dp < parameters!numPriorities THEN dp
					ELSE parameters!numPriorities - 1 FI;
		OUTPUT  PDU.ready(dp) TO domain;
	ENDDECISION;
	NEXTSTATE -;

STATE busy, ready;

INPUT	TGcf(pdu);
	CALL	Track_token(pdu!tokenId, pdu!tokenStatus);
	OUTPUT	MCS.Token.Grab.confirm(maId, pdu!tokenId, pdu!result);
	JOIN 8b;

STATE busy, ready;

INPUT	TIcf(pdu);
	CALL	Track_token(pdu!tokenId, pdu!tokenStatus);
	OUTPUT	MCS.Token.Inhibit.confirm(maId, pdu!tokenId, pdu!result);
	JOIN 8b;

STATE busy, ready;

INPUT	TVin(pdu);
	TASK	tRecipient  :=  Incl(pdu!tokenId, tRecipient);
	OUTPUT	MCS.Token.Give.indication(maId, pdu!tokenId, pdu!initiator);
	JOIN 8b;

STATE busy, ready;

INPUT	TVcf(pdu);
	CALL	Track_token(pdu!tokenId, pdu!tokenStatus);
	OUTPUT	MCS.Token.Give.confirm(maId, pdu!tokenId, pdu!result);
	JOIN 8b;

STATE busy, ready;

INPUT	TPin(pdu);
	TASK	t  :=  pdu!tokenId;
	DECISION t in tPossessed or t in tRecipient;
	(True):	OUTPUT  MCS.Token.Please.indication(maId, t, pdu!initiator);
	ELSE:ENDDECISION;
	JOIN 8b;

STATE busy, ready;

INPUT	TRcf(pdu);
	CALL	Track_token(pdu!tokenId, pdu!tokenStatus);
	OUTPUT	MCS.Token.Release.confirm(maId, pdu!tokenId, pdu!result);
	JOIN 8b;

STATE busy, ready;

INPUT	TTcf(pdu);
	OUTPUT	MCS.Token.Test.confirm(maId, pdu!tokenId, pdu!tokenStatus);
	JOIN 8b;

STATE detaching, detached;

INPUT	*;
	NEXTSTATE -;

STATE detaching, detached;

INPUT	Quit;
	OUTPUT	Quit TO control;
	NEXTSTATE detached;

STATE detaching;

INPUT	PDU.ready(dp);
	DECISION dp = 0;
	(False):	NEXTSTATE -;
	(True):	TASK	pdu!reason  :=  reason,
			pdu!userIds  :=  Incl(user, Empty);
		OUTPUT  DUrq(pdu) TO domain;
		NEXTSTATE detached;
	ENDDECISION;

STATE detaching, detached;

INPUT	PCin(pdu);
	DECISION user in pdu!detachUserIds;
	(False):	NEXTSTATE -;
	(True):	OUTPUT  Quit TO control;
		NEXTSTATE detached;
	ENDDECISION;

STATE detaching, detached;

INPUT	DUin(pdu);
	DECISION user in pdu!userIds;
	(False):	NEXTSTATE -;
	(True):	OUTPUT  Quit TO control;
		NEXTSTATE detached;
	ENDDECISION;

ENDPROCESS;

Appendix VII

Characteristics of the reference implementation

(This appendix does not form an integral part of this Recommendation)

## VII.1	SDL decomposition

Figure II.1 depicts an MCS provider in the context of a system, with three channels binding it to its environment: Control.MCSAP to a single controller application, MCSAPs to zero or more attached users, and TSAPs to zero or more transport service providers. In a decomposition of the provider, these external channels connect with signal routes to and from component processes.

Each octagonal element represents a process type and gives bounds (minimum, maximum) on its number of instances. One Control process exists permanently. Dashed lines show that it creates instances of the remaining three processes. Attachment, Domain, and Endpoint do not exist initially, but their potential number is unlimited.

Signal routes to and from component processes convey SDL signals. The set of signals transmitted in a given direction is shown near the corresponding arrow head. Identifiers in parentheses are lists of related signals. The expansion of these lists is to long to include in the figure. Details appear in the text of Appendix II.

## VII.2	Service definitions

Signals over the external channels represent uses of MCS and the transport services defined abstractly in ITU-T Recommendation T.122 and CCITT Recommendation X.214. Modeling these in SDL requires further assumptions about details of the interactions. Two aspects are worth special note: a use of endpoints identifiers to specify the context of a service primitive and a use of flow control ready signals to invite data transfer.

The three identifiers involved are: MCSConnectionId, MCSAttachmentId, and TCEndpointId. These parameters are implicit (not depicted) in the service definition documents. The model here assumes that they are assigned by the responsible provider. For example, TCEndpointId appears in T.Connect.indication and T.Connect.confirm. To enable the user of a service to relate a confirm to a preceding request, the user may specify an arbitrary label to be echoed back.

Flow control is modeled here by a window-of-one mechanism. A ready signal must be sent in one direction before a data transfer signal may be sent in the opposite direction. Before the next transfer, another ready must be received. In a practical implementation, ready might take the form of moving a pointer or incrementing a counter. Casting ready as a signal does not imply it must be as ponderous as data transfer. There are three ready signals: T.ready, PDU.ready, and MCS.ready. These allow the transfer of, respectively, one TSDU, one domain MCSPDU, and one MCS service data unit. Flow control applies independently to the two directions of a signal route.

Transport quality of service on a TC is modeled here with parameters for throughput, transit delay, and data priority. This is an amalgam of standards which are not consistent among themselves. In practice, the details will surely differ. The specification of QOS is a burden that controller applications must bear.

## VII.3	Portals onto a domain

The Domain process is central to this decomposition of an MCS provider. Different instances represent different domains hosted by the same provider. A Domain process is created with a set of parameters that remain frozen for its lifetime. These domain parameters are provided by the Control process. The Control process decides which domain selectors are valid and how the corresponding Domain processes shall be configured. If there is room for parameter negotiation during MCS.Connect.Provider, the Control process supervises the interaction.

To keep the Domain process as simple as possible, the MCS attachments and MCS connections it interfaces to are abstracted to appear as portals with a number of common features. A portal is either a single Attachment process or a set of Endpoint processes, one for each constituent TC of an MCS connection. The Control process assigns portal ids and opens and shuts their associations to a Domain process. It hides details of building up and tearing down processes and hides the exchange of connect MCSPDUs.

To the Domain process, a portal looks like a set of signal routes, one for each data priority implemented. Each conveys MCSPDUs subject to PDU.ready flow control. Where distinctions in processing cannot be avoided, they are invoked by classifying a portal as one of three kinds: Attached, Uplink, or Downlink.

Figures VII.1 through VII.8 illustrate signal flows for typical operations. They assume a single user attached locally to a domain with one MCS connection to another provider. Endpoint1 and Endpoint2 arise from an assumption that two data priorities are implemented in the domain.

The signaling to shut a portal is rather lengthy. It defends against the possibility that one of the processes involved might send a signal to another that has already stopped, which would be a run-time error in SDL. Attachment and Endpoint processes make Quit the last signal they send, and they stop on receipt of Exit. Domain sends no further signals to a portal after replying Shut.portal to Control, and it stops after indicating thus that the last portal is shut.

Of remaining miscellaneous signals, Drop.portal is bidirectional and corresponds to DPum. Report.portal alerts Control that a diagnostic was issued towards another MCS provider. An RJum that originates in Endpoint is sent through Domain to get onto the initial TC. An RJum received externally is dispatched directly from Endpoint to Control.

Note that MCS.Attach.User.request and T.Connect.indication are directed to Control, since they are stimuli to create corresponding portals. Control can decline with a negative MCS.Attach.User.confirm or T.Disconnect.request.

The present model does not limit the number of MCS attachments or MCS connections per domain, except through a local limit (maxPortalIds) on the number of portals across all domains hosted by the provider. Many aspects of domain configuration must be managed locally until further standardization is undertaken.

## VII.4	Alignment of MCSPDUsXE "Alignment of MCSPDUs"§

Clarity requires that the MCSPDUs defined in clause 7 be represented individually by SDL signals. But simplicity of the Domain process depends on a more unified treatment. The resolution here is to define a PDUStruct data type, with components that can be selected to match any one of the domain MCSPDUs. Connect MCSPDUs, which are handled outside the Domain process, continue each to have its own structure.

A key component of PDUStruct is kind, which identifies the intended domain MCSPDU. Based on kind, a subset of other fields then become relevant: those listed in ASN.1 as components of the MCSPDU. The SDL-defined data types match as closely as possible the ASN.1-defined data types of clause 7. The intention should be clear that a PDUStruct is the internal, decoded representation of a domain MCSPDU.

Three MCSPDUs have optional components, AUcf, CJcf, and CCcf. In SDL their absence is indicated by coding zero for the corresponding field. This is an illegal value for a static or dynamic channel id.

The signals sent between Attachment, Domain, and Endpoint processes, with the exception of PDU.ready, take a PDUStruct as their single parameter. In a practical implementation, this kind of communication might be as simple as moving a buffer pointer.

## VII.5	Method of using SDL

The reference implementation uses the textual representation of SDL as a programming language whose power is enhanced by the ability to define abstract data types. In Control and Domain, the objects managed are too complex and numerous to benefit from the kind of finite state machine that SDL supports. These processes remain in a single state processing input signals. Attachment and Endpoint, with smaller scope, can use a handful of states to better advantage.

Appendix II contains a definition of the SetOf generator, whose scope is the entire provider. This takes any other defined type, such as channel ids, and formalizes the concept of subsets of values of that type. SetOf extends the built-in generator Powerset by adding a new operator that chooses an arbitrary element from a non-empty subset. The meaning of this Pick operator is defined through simple axioms.

Sets are used heavily throughout the implementation. Part of the information base, for example, is to record which subset of channel ids are in use and which subset of portal ids have joined a given channel.

In SDL, arrays are deployed over the full range of the indexing data type. Channel structures, for example, exist for each channel id from 0 to 65535. A practical implementation must deal in much sparser arrays. To this end, separate id sets are kept to record which channels and other resources are in use at a given time. A principle of the design is that array values need not be stored in the information base for ids that are not explicitly marked as in use.

Many iterations follow a familiar pattern: create a candidate set of interest, then pick and delete one member of it at a time, until no more remain. Such iterations are built out of decisions and joins. Comments show the higher-level control structure intended. Labels are numbered sequentially within a procedure, with one letter appended to indicate whether branches to the label are backward or forward.

A table of contents, which shows calling sequences, occurs before the first procedure definition in each process. SDL procedures may return results through formal parameters that are declared to be in/out.

A few constants appear dispersed throughout the code, like the maximum number of data priorities (4) and the dividing line between static and dynamic channel ids (1001). These are not parameters whose value can be redefined.

It is an important idea in MCS that user ids are a subset of channel ids. But the contexts in which each is used differ, so two separate types are appropriate. Appendix II formalizes a pair of operators for casting between them.

The processes of Appendices III through VI cooperate in the framework specified by Appendix II. Signals are constrained to the specified routes, and misbehavior across internal interfaces is not allowed. Defensive coding focuses its attention on controller and user applications residing in the environment outside the provider and on MCSPDUs received externally from peer providers.

The reference implementation allows input signals to be scheduled in arbitrary sequence if more than one are pending. It makes no assumption about the relative priority of processes. But it does rely on individual input transitions executing to completion without preemption.

## VII.6	Remarks on the Domain process

Significant use is made of the Queue generator in addition to SetOf. Buffers are queued in user-transmitted sequence to an output portal, and portal ids are queued in the order of unanswered MCrq, MTrq, and AUrq MCSPDUs.

To avoid copying user data, MCSPDUs are manipulated in buffers, which may be output to multiple ports. The number of buffers available to a Domain process is modeled as a fixed external parameter. This could easily be changed to vary by configuration from domain to domain. The implementation degrades gracefully, through global flow control, if the number of buffers provided is low. It applies buffers, as they become free, to the highest priority data flow.

When an MCSPDU arrives as input, the Domain process needs to know the portal of origin and data priority. The procedure Identify_sender does an exhaustive search of opened portals based on the process id reported by SDL. The needed information could be defined to be part of the signal parameter set, except that the intuitive meaning of signals would then be obscured. In a practical implementation this modeling is not a significant issue.

The heart of the Domain process is the procedure Process_PDU, which calls in sequence Validate_input, Top_provider, and Apply_PDU. Each of these is a large case selection based on the kind of PDU at hand. The details for any given kind are mostly straightforward. A good way to approach these procedures may be to cut across them horizontally, following the course of especially important MCSPDUs, like SDrq, one by one.

The encoding of MCSPDUs, using either BER or PER depending on the protocol version, is a detail left mostly to the Endpoint process. All Domain needs to know is how large it can make MCSPDUs with variable content, like DUrq with its multiple user ids. Such considerations are expressed through decision statements with informal text, using worst-case constants for the BER encoding. In practice, some limits on variable content may be precalculated in terms that Domain understands, like maximum numbers of ids. Others may require more direct knowledge of the encoding.