# AVC PART 3 Conference Services

Audio Video Conference Services  define  a number of supplementary conference services; each  encapsulating all the functionality required to provide a speciific type of service in a complete and comprehensive maner. These services can be regarded as a toolkit from which functionality may be selected as appropriate for any given conference requirement. All of these services utilise the AVC infrastructure services defined in T.132(RTLM) in order to deliver there functionality. All AVC resources including QoS are managed by the AVC Resource Manager. All the services must negotiate and ultimatly compete with each  other for the use  of available resources.

Services defined in this recommendation are:

·   Remote device control.	·   Audio and video source selection.

·   Video switching	·   Video processing (including support for continuous presence)

·   Audio mixing 	·   Audio and video source identification

·   On Air Indication 	·   Floor control

## Summary of Conference Services

AVC Primitives.

Table 1/T.133.

| AVC Functional Class | Primitives | Terminal Requirement | MCU Requirement |
| --- | --- | --- | --- |
|  | AVC-Device-Control-Options-request AVC-Device-Control-Options-indication AVC-Device-Control-Options-response AVC-Device-Control-Options-confirm |  |  |
|  | AVC-Device-Control-Commands-request AVC-Device-Control-Commands-indication AVC-Device-Control-Commands-response AVC-Device-Control-Commands-confirm |  |  |
|  | AVC-Service-Activate-request AVC-Service-Activate-indication AVC-Service-Activate-response AVC-Service-Activate-confirm |  |  |
|  | AVC-Service-Video-Route-request AVC-Service-Video-Route-indication AVC-Service-Video-Route-response AVC-Service-Video-Route-confirm |  |  |
|  | AVC-Service-Video-Identification-request AVC-Service-Video-Identification-indication AVC-Service-Video-Identification-confirm |  |  |
|  | AVC-Service-On-Air-Indication-request AVC-Service-On-Air-Indication-indication |  |  |
|  | AVC-Service-Audio-Mixer-Control-request AVC-Service-Audio-Mixer-Control-indication AVC-Service-Audio-Mixer-Control-response AVC-Service-Audio-Mixer-Control-confirm |  |  |
|  | AVC-Service-Floor-Control-request AVC-Service-Floor-Control-indication AVC-Service-Floor-Control-response AVC-Service-Floor-Control-confirm |  |  |

## Conference Services

This recommendation defines a number of common services which are provided by MCU's, to the conference.  Each service is assigned a standard  identifier which is used to represent it in the application roster and  advertises the capability for that service to the conference. Each service may consist of a mandatory part and optional enhancements. The use of enhancements is negotiated using the GCC capability exchange mechanism. The standard services provided by AVC are listed below:-

·	Video Switching 				·	Video Identification

·	On Air Indication				·	Floor Control

·	Audio Mixing				·	Continuous Presence -Multiplex Mode

·	Continuous Presence-Array Mode		·	Continuous Presence - Transcoding Mode

The capability to support a service is determined by GCC's application Roster exchange mechanism. The application roster entry for each service indicates if a node wishes to activate that service. If the service is available at two or more nodes and one or more nodes has requested activation on start-up then the roster exchange will cause the service to be launched. Once launched the service will continue until the end of the session or until terminated. Each individual service defines  any restrictions that apply to the launch or termination of a  service.

The conference convenor may chose to require, allow or to constrain the use of some or all of the defined services and there associated primitives. An authorised node may send an AVC-Activate-Service primitive to the MCU Communication Channel. All MCU's receive the request and respond by launching the specified service.

AVC-Activate-Service Primitive

Table 2/T.133.

|  | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Node Identifier(AVC User ID) | M | M= | M= | M= |
| Service Identifier(s) | M | M= | M= | M= |
| Mode | O | O= | O= | O= |
| Service-State-Flag | M | M= | M | M= |

Node Identifier	- This identifies the node that initiates or terminates a service.

Service Identifier - This identifies the AVC service of interest.

Mode		- The modes, if any are specified by the individual services and only have 			meaning within that context.

Service State Flag - This specifies if the service is to be activated or deactivated.

Figure 1/T.133

AVC-Activate-Service  -Sequence of primitives

### Video Switching Service (VSS).

#### Description

The video switching service  utilises the standardised  Switch Device and its set of standard functions to define a video switching service. Within a session there may be a number of physical devices in different nodes that actually provide the video switching functionality. These devices may communicate with each other using the MCU control channel. Terminals needing to communicate with the video service do so, by using the MCU communications channel which provides a common interface independent of the conference configuration and  network components.

In a multipoint conference  the connection topology should always be arranged such that the Top Provider for the conference shall be an MCU. In conference configurations where more than one MCU is present, it shall be the top provider MCU that decides if any given external request is to acted upon. Through  inter-MCU communication the top provider can assign privileges to it client MCU's to act independently, carrying out designated functions.

In a conducted conference  video switching mode selection  may be restricted or only available to the conductor or a conference organiser.

#### Video Switching - Operation

A number of 'participant' roles are defined for the video switching service. These roles are used to provide a focal point for the switching algorithms. Roles may be pre-assigned or dynamically allocated,  they may also be re-allocated at will during a session. The selected video switching mode  determines if and how the roles are used to determine video routing. The video switching service defines the following standard Roles:-

New-Speaker			Previous-Speaker			Overider

Designated Presenter(s)		Conference Conductor		Designated Stream

Conference Convenor		Viewer(s)			Unassigned

#### Role Definitions

New-Speaker Role

The "New-Speaker" role is a role where the holder changes dynamically according to some automatic algorithm which detects some type of designated activity e.g. audio, data or even video. The main purpose of this role is to support  Automatic Voice Switched mode where the algorithm is provided by the Audio Mixing service and role maps directly onto the corresponding Audio Mixer role. There can only be one New-Speaker at any given time.

Previous Speaker

The "Previous-Speaker" role is also a role where the holder changes dynamically according to some automatic algorithm as above, but the previous -Speaker will always be the last "New-Speaker". The main purpose of this role is also to support  Automatic Voice Switched mode where the algorithm is provided by the Audio Mixing service and role maps directly onto the corresponding Audio Mixer role.  There can only be one Previous-Speaker at any given time.

Presenter(s)

The Presenter Role is defined  to give the video switching service a representation of a presenter presenting to a conference session. A session may have more than one presenter. The role holder(s) are normally known in advance and pre-assigned and ordered.  There can only be one Presenter active at any given time. Note need to say how presenters are assigned.

Conductor

The conductor role is defined in recommendation T.124 which also provides the means to assign the role. The video switching service makes this role available as a point of focus.

Convenor

The Convenor role is defined in recommendation T.124, which also provides the means to assign the role. The video switching service makes this role available as a point of focus.

Designated Node(s)

This role is provided to allow any node to be specified as a point of focus in the switching service allowing access to the input and output options.

Viewer(s)

This role allows a sub group of session participants to be identified and handled in a specified way by the video switch. The viewer role is mainly  that of passive observation.

Overider(s)

The Stream Routing Service provides support for nodes to make specific video routing requests to it, in support of functions such as remote control. If these requests are permitted the video Switching Service(VSS) designates the requester a Overide role holder. Overiding requests take precedence over other settings but may also work alongside other selections where channel capacity permits.

#### Role Holder Input Options

There are a number of algorithms that can be selected from the video switching service to determine what a Role holder receives from the video switch. These are listed below:-

Rotate	- This causes the video switch to provide an output to the role holder which is a rotation of all other video streams.

Random	- This causes the video switch to provide an output to the role holder which is a random selection of all other video streams.

Loop back	- This causes the video switch to loop back the video from the role holder.

Role	- This causes the video switch to provide an output to the role holder which is the videeo from an identified role holder. Note:- In the case of there being more than one holder of a given role then one may be selected or a rotation of the role holder may be done.

#### Role Holder's Output Options

The algorithm in use, also determines how and if the video switching service routes  the video originating from the role holder. The following choices are available:-

Broadcast to All	- This causes the video received from the role holder to be broadcast to all nodes in the conference.

Broadcast to others	- This causes the video received from the role holder to be broadcast to all nodes in the conference, except the originating node.

Role		- This causes the video received from the role holder to be directed to the specific role holder identified, if the role holder identified is the same the video will be looped back to the originator.

Specific destination Stream(s) Group	- This allows a list of one or more receiving nodes to be specified (in terms of their AVC User ID's and stream ID's)

Authors Note 1:- MCUs are going to have to negotiate some parameters in order to support what is being proposed.

Authors Note 2:- In the case of a role holder having multiple streams this will have to be supported to.

#### Video Switching Matrix

In Table 12/T.133. below the video switch's switching options are presented as a matrix. A number of standard operating modes are defined using this matrix. The video switching service also provides an option that allows the matrix to be accessed directly, however in this mode care must be taken to avoid conflict between the input and output settings.

Video Service Control Matrix

Table 3/T.133

| VIDEO SWITCH | VIDEO OUTPUT TO ROLE HOLDER |  |  |  |  |  | VIDEO OUTPUT FROM ROLE HOLDER |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MODE ROLE | Rotation | Random | Loop back | Role | Specified Input | Broadcast to all |  | Broadcast to others | Role | Specified Outputs |
| New Speaker |  |  |  |  |  |  |  |  |  |  |
| Previous Speaker |  |  |  |  |  |  |  |  |  |  |
| Designated Presenter(s) |  |  |  |  |  |  |  |  |  |  |
| Conductor |  |  |  |  |  |  |  |  |  |  |
| Organiser |  |  |  |  |  |  |  |  |  |  |
| Designated Stream |  |  |  |  |  |  |  |  |  |  |
| Viewer(s) |  |  |  |  |  |  |  |  |  |  |
| Overider |  |  |  |  |  |  |  |  |  |  |

#### Standard Operating Modes

##### Voice Activated Video Switching Mode.

Automatic voice switching utilises the concept of a "New Speaker" and a "Previous Speaker" that are determined by the audio mixer(s) using some suitable algorithm (which is beyond the scope of this recommendation) to detect the currently active speaker. Once the active speaker has been determined, it is designated the "New-Speaker" site  and the previous site to be the "New-Speaker" is designated the "Previous-Speaker" site.  The Speakers are in fact Audio Mixer roles.

The video switch uses these parameters in its Voice Activated Video Switching Mode as follows:-

The Role holders will be dynamically changing as determined by the audio mixer, 	but on each switching operation the "New-Speaker"  will be broadcast to all other 	sites in the conference.

The "Previous-Speaker" video is only seen by  the "New-Speaker".

Video switching Service -Voice Switched Mode

Table 4/T.133.

| VIDEO SWITCH | VIDEO OUTPUT SEEN BY ROLE HOLDER | VIDEO OUTPUT FROM ROLE HOLDER TO:- |
| --- | --- | --- |
| ROLE HOLDER |  |  |
| Current-Speaker | Previous-Speaker | Broadcast To Others |
| Previous-Speaker | Broadcast from New-Speaker | New-Speaker |

##### Presenter Mode

The Presenter Mode utilises the two roles, the "Presenter"  who is assigned the role, and the "New-Speaker" as determined by the audio mixer. This mode only allows one presenter to be defined at a time. To change the presenter, the mode may be re-entered with a different Presenter identity assigned.

In this mode the operation is such that, the selected Presenter's location is always broadcast to all other sites in the conference. By default the Presenter sees the latest New-Speaker other than himself, if there is no other speaker or a detected New-Speaker is inactive for a period then the presenters will see a rotation of video from other nodes.

In conducted mode only the conductor or presenter may make mode changes.

Video switching Service -Presenter Mode

Table 5/T.133

| VIDEO SWITCH | VIDEO OUTPUT TO ROLE HOLDER | VIDEO OUTPUT FROM ROLE HOLDER |
| --- | --- | --- |
| ROLE |  |  |
| Presenter | New-Speaker/Rotation | Broadcast To Others |
| New Speaker | Broadcast from Presenter | New-Speaker |

##### External Control Mode

This mode allows  sites participating in the session to issue stream routing primitives in order to request specific routings of the video stream. In this mode the routing requests if permitted are also selected using the VSS control matrix, requesters are designated "Overider" roles and their incoming video is selected as a specified input. This mode may be used in conjunction with other modes, however it will in most cases effect the operation of other modes when requests are in effect.

In conducted mode these requests should be sent to the conductor's channel for authorisation and only requests from the conductor's node shall be processed by the  Video Switch.

##### Manual Control Mode.

This mode effectively provides a bypass of the VSS control matrix, only allowing the Video switch Device to be used directly.

Video-Service-Switching Primitive

Table 6/T.133.

|  | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Site Identifier(AVC User ID) | M | M= | M= | M= |
| Video Switching Service Mode | O | O= | O= | O= |
| Role Identifier |  |  | O= | O= |
| Role Holders Incoming Stream Identifier List | O | O= | O= | O= |
| Role Holders Outgoing Stream Identifier list | O | O= | O= | O= |
| Service-Status Report |  |  | O | O= |

Site identifier	- Site originating the service mode selection request

Video Switching Service Mode - This parameter is used to select one of the standard modes.

Role Identifier	- This parameter identifies the role used as a point of focus for video switching.

Role Holders Incoming Stream identifier List	- This specifies which of the services options for  determining the input to the role holders node should be used.

Role Holders Outgoing Stream identifier List	- This specifies which of the services options for  determining where the output from the role holders node should go, is to  be used.

Service Status Report	- This provides feedback to requester under certain circumstances.

AVC-Video Service-Switching  - Sequence of primitives

Figure 2/T.133.

### Video Service - Identification.

This service exploits the T.120 capabilities to offer an enhanced functionality to the  Video conference. It provides the capability for the origin and content of a video stream to be communicated.

Each node in the conference is uniquely identified by its GCC's User Id( or represented by a proxy service). If a node is only capable of originating a single video stream then its User Id is all that will be needed to uniquely identify it as the originating node for that stream. Where a node is capable of originating more than one video stream then this recommendation supports the unique identification of streams. The AVC database provides the mapping of stream to pipelines. Stream identifiers may correspond to a tag embedded within the stream or be used to map to some logical ordering of transmission or positioning relating of the pipes relative to each other.

In a multipoint conference; when an AVC aware node provides or changes the source of its video stream, it must send a video identification message to the MCU Communication channel. If no identifier is received the MCU(s) will assume the default stream and face camera and extract a name if available from the GCC Conference Roster entry for that site. The identification message will allow the Video pipeline, the stream source device  and a string identifier to be defined to replace the default determination.

A Video identification message can be associated with each  received video image by an MCU or other network element. It is the MCU's responsibility to relayed the identification onward to correspond to the routing applied to the  video stream. If the MCU carries out any manipulation of the video stream it must generate a new Video Identification message to correlate with the changes.

The information will be used to identify; to the receiver, who or what is being seen and also from what location the image originates. It may be used to generate a caption for displaying with the image. MCU's must send Video Identification messages individually link by link to the  AVC User ID for any given node.

Video-Service-Identification Primitive

Table 7/T.133.

|  | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Source AVC User ID | M | M= | O= |
| Source Device Id | O | O= | O= |
| Pipe Identification Num (n) | O | O= | O= |
| Video Image Title String (n) | O | O= |  |
| Reason Code | O | O= |  |
| Status |  |  | O= |

Source AVC User ID - Node identifier

Source Device Id - The device identifier as defined in the T.MNS capabilities advertised in the application roster.

Pipe Identification Num (n)

Video Image Title String (n)

Reason Code - This code tell receiving site why image has changed

00 - Mode change

00 - Stream control assigned to node

Status -

AVC-Video Service-Identification  - Sequence of primitives

Figure 3/T.133

Terminal Action.

In order for a  terminal to join the Video Identification service it must send out its own video identification message to the AVC Broadcast channel. Once this has been received it will  be included in the service. When ever the source it provides to the conference changes it must send a new identification with the source device and identifier string. If the optional fields are not completed the service will assume the source is the default  main camera and will use the user name string provided by the terminals 'GCC_Announce_Presence' message.

MCU Action

Each MCU must be able to associate each video stream it receives with a video identifier. When ever an MCU provides a new video source to any of its ports, it must relay on the associated identifier(s). Each MCU needs to know the User ID of the terminals or MCUs directly connected to its ports. If a connection is to another MCU it must also know the user ID of all other terminals and ports that can be reach via that MCU. This is  required because the MCU will have to provide a physical routing of messages given using the User Id in the message.

### "On Air" Indication Service

In a multipoint conference MCU(s) provide control and routing of audio and video streams. When a stream is routed onwards by an MCU  to a terminal  capable of  interpreting the stream it must indicate to any peer MCU's that source is 'On Air' . The MCU directly connected to the source originator has the responsible for passing the 'On Air' indication to that terminal.

AVC-Service-On-Air-Indication  Primitive

Table 8/T.133.

|  | Request | Indication |
| --- | --- | --- |
| Source AVC User ID | M | M= |
| Stream Id | M | M= |
| Source Device (n) | O | O= |
| Status |  |  |

Source AVC User Id -	The GCC User Id of the terminal originating the stream

Stream Id - Audio(n) or Video(n)

Source Device - The  Device Id of the Device originating the source at the terminal

AVC-Video-Service-On-Air  - Sequence of primitives

Figure 4/T.133.

### Continuous Presence Video Service

The primary objective of Continuous Presence systems is to allow all of the participants to be seen all of the time.

Practical considerations may however restrict the ability to provide a true Continuous Presence system, particularly in large conference situations. The following factors will all impact on what is achievable:-

·	The throughput capacity of the video pipes to and from terminals and on any 	inter-MCU links

·	The  topology of the conference

·	 Number of participants on Video Equipped Terminals.

·	The video protocol and encoding in use

It is beyond the scope of this recommendation to define the protocols for specific CP systems, which may, in any case vary for different networks and bandwidth capabilities. Other recommendations will define the formats and protocols for the video aspects of such a service.

The T.120 Continuous Presence(CP) service has the role of providing control and indications, in  support of  multipoint video conferences where some form of CP system is operating.

For the purposes of this recommendation three methods of providing CP systems are identified. Support for these three methods is provided by selection of the appropriate operational mode.

The AVC recommendation inherently supports many of the concepts required in order to provide a CP service; however the requirements placed on MCU's and also on terminals to support CP may be significant.  The flexible approach to the provision of control adopted within this recommendation should not  constrain these or alternative methods from being used to provide such services. Nodes advertise their ability to support the various CP modes in the application roster. The AVC conference creator may predefine the mode used or an appropriate set of defaults may be defined.

#### Continuous Presence Multiplex Mode

In this mode video terminals provide  a single video stream on their 'up links' to an MCU. In each MCU a Time Division Multiplex(TDM) is used to combines the 'up link' streams received from participating video terminals to produce a multiplex of output  streams. Provided sufficient bandwidth is available, the down link will contain a multiplex of  video streams from all other  terminals. A video multiplexer device is defined in this recommendation to support the control of such a method. This method provides a simple solution for MCU's; however terminals require as many codec receivers as there are images being received. Also this mode is the most difficult of the modes to scale up to larger conferences.  Figure 5/AVC(Part 3) shows an example where a terminal sources one stream and receives four. This configuration would also apply to the other four terminals in the conference.

Continuous Presence Multiplex Mode Showing a terminal Sourcing  1 Video Stream and receives Four Video streams in return.

Figure 5/T.133.

#### Continuous Presence Transcoder

The transcoding method is based on a video transcoder in each MCU which decodes the incoming video received at the 'up links' from participants terminals, converting back the signal into its raw uncompressed format. This signal can then be re-coded to provide a resultant image combining any required combination of the input images in any desired position to any scale. However, considerations such image resolution, and  size of the resulting elements in a combined picture will impose practical constraints on what can be done.

This method provides the most flexibility to reprocess, resize and scale images, allowing  transcoders at MCUs the capability to adapt the images they provide, tailoring them to their terminals requirements or capabilities.

Continuous Presence Transcoding Mode - MCU Trancoded Image.

Figure 6/T.133.

The example of a transcoded image above shows how an MCU transcoder can take a number of video input streams and combine them in a flexible format providing a single output stream in a form compatible with the destination terminals requirements or capabilities.

Transcoding will impose greater delays as an extra decoding and encoding process are added.

#### Continuous Presence Array Processor

The Array method combines the individual 'up-link' pictures into a larger overall picture  frame or frames; these can be sent to receiving terminals and displayed as an array of pictures from different participants. This procedure can be directly applied to the picture coding hierarchy in ITU recommendation H.261/262 (see figure below), which defines a picture in terms of three Common Interface Formats(CIF). the smallest is  quarter CIF (QCIF), the next is CIF which is 4 times the size of QCIF. The largest format is Super-CIF (S-CIF) which is 4 times the size of CIF.  The array processor can combine  'Up link' images flexibly within the output stream(s) to provide a variety of display options.

CIF Image Sizes

Figure 7/T.133.

### Audio and Visual Floor Control Service

This service provides the mechanism that allow conference participants to make requests to speak in a conducted conference and for the conductor to enable and disable participants from speaking. It is only operational in a multipoint conference in conducted mode.  This service makes use of the audio mixer device(s)  in order to provide the service.

The floor control service has 3 roles that conference nodes can adopt:-

Conducted Terminal Role:-	When a terminal is running the Audio application and its local GCC provides indication that the conference has become conducted, its transmitted audio will be muted at its local mixer. The terminal must provide its user with the ability to issue "floor requests" (requests for permission to speak).

Conductor's Role:-	When a node  adopts the conductor's role, the AV application must adapt to allow the conductor to be presented with; and to handle floor requests.

MCU Node Role:-		All MCU nodes must have joined the MCU communications channel and as a consequence will receive Mixer Control  messages sent on the MCU channel. Each must validate that the requester is the conductor  and that the identified node user ID is directly connected. If it is then the MCU must respond to the request.

The floor control service is an optional (or should it be mandatory) service provided by AVC. The capability to support the service is advertised by a node in the GCC application capability  exchange communicated via GCC by means of the application roster. The floor control service is conditional on the "on-air service also being supported. The service can only be invoked in conducted mode.

It is not mandatory that all participating nodes be capable of supporting the service in order for it to be used. The conductor may decide to launch the floor control service regardless, this will still result in the desired control, however;  nodes not supporting the service will not be able to make floor requests in the conventional way and must rely on the GCC-permission-to-act PDU to communicate a request to the conductor.

When the floor control service is invoked by issuing a Activate-Service  primitive, the initiating conductor specifies if the service is to be used for audio only or for both audio and video together.

For audio control only, the initial condition is with all audio sources  muted at the mixer except that of the conductor, video will continue in the current operational mode. Nodes other than the conductor wishing to speak make a Floor-Request to the conductor. The conductor may decide to grant or refuse the request. Normally requests will  either be granted by the conductor or held in a queue in received order until the conductor is ready to add a new speaker. The conductor may also  effectively remove any speaker by muting their audio source at the mixer.

For Audio and video control the audio tracks the video regardless of previous modes in operation.

The conductor initiates the control by communicating with mixer(s) via the MCU communication channel. In voice switched mode the action of the video is linked to the output of the audio mixer and is therefore indirectly controlled as well.

Note We also need to provide service support for waiting for floor functionality.

#### Starting Floor Control

This primitive can only be issued by the conductor and is broadcast to all participants in the session as announcement that the service is about to be activated or terminated depending on the setting of the Service-State-Flag

#### Floor Control Primitive

Table 9/T.133.

AVC-Service-Floor-Control Primitive

|  | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Site Identifier(AVC User ID) | M | M= | M= | M= |
| Request Floor | O | O= |  |  |
| Grant Floor |  |  | C | C= |
| Status |  |  | O= | O= |
| User defined extensions |  |  | O | O= |

Site Identifier(AVC User ID)	- This is the AVC User ID of the requesting  Node, which may be any node in the session

Request Floor	- This is a flag which is set in the request to indicate the originator wishes to speak.

Grant Floor	- This is a flag which is used by the conductor to indicate if the request has failed.

Status 	- This allow the conductors node to provide feedback to the requester such as position in a queue to speak.

AVC-Video-Service-On-Air  - Sequence of primitives

Figure 8/T.133.

### Audio Mixing Service

The audio mixing service allow the way audio is mixed in the session to be determined. The audio mixing service supports a number of operational modes.

#### Voice Switched Mode

In Voice Switched mode each MCU mixer detects the current speaker by an algorithm which is outside the scope of this recommendation to define. The selected incoming stream is designated  the "New Speaker" role. The input stream which was previously designated that role is redesignated to be the "Previous-Speaker". While this mode is in use, these two roles are being continuously determined and assigned. The roles are used to determine the video stream  routing, when the video is set to track the audio.

### Conductor Controlled Mixing

In this mode the conductor has access to the audio mixer input selection matrix and can decide  which stream to include in the mix.

AVC-Service-Audio-Mixer-Control Primitive

Table 10/T.133.

|  | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Site Identifier(AVC User ID) | M | M= | M | M= |
| Mix Input - Enable List | O | O= |  |  |
| Mixer Input - Mute List | O | O= |  |  |
| Mixer Input - Remove List | O | O= |  |  |
| Request Status | O | O= |  |  |
| Mixer Status Report |  |  | O | O= |

AVC-Service-Audio-Mixer-Control  - Sequence of primitives

Figure 9/T.133

## Service Functions

Service functions are described seperatly from the conference services as they do not depend on network support to operate or to be useful

### Privacy Function

Privacy is a mode that any node may invoke. The  result of a terminal node invoking privacy is that it no longer sources audio or video  stream(s). A terminal invoking privacy shall locally remove its audio and video streams. It should replace the video stream with  an alternative signal in order that the change not be interpreted as a failure condition. The alternative signal may for example comprise of simply a blue screen, or it may be more informative with text overlays etc, however the format of any privacy image is a local matter outside the scope of this recommendation.

When ever a terminal enters or leaves privacy it shall broadcast that fact on the AVC Broadcast channel using the privacy primitive.

AVC-Function-Privacy Primitive

Table 11/T.133.

|  | Request | Indication |
| --- | --- | --- |
| Source AVC User ID | M | M= |
| Privacy Flag | M | M= |
| Time to return | O | O= |

Source AVC User ID: 	Node identifier originating the privacy announcement

Privacy Flag: 	Flag indicating if privacy is being invoked or terminated at the specified node.

Time to Return:	When entering privacy a node may provide an estimated ellapsed time away from conference

AVC-Function-Privacy - Sequence of primitives

Figure 10/T.133.

### Conducted Mode Behaviour

AVC is a conductor aware application protocol in that it has two modes of operation one for non-conducted mode and one for conducted mode, as described in this section.

Application become aware of a GCC Conference Conductor being assigned through receipt of a GCC-Conductor-Assign indication from its GCC provider. This includes the GCC User ID of the conducting node. Application must determine the identity of the AVC application protocol which inherits the conductors role.

Communications on the AVC-MCU-Control channel are not effected, nor are start-up or capability processes. No new use of functions other than privacy may be started without permission from the conductor.

#### Conductor Privilege Ask

The conductor may chose to assign privileges to particular nodes or to all nodes. These privileges may be for specific functionality, or for all functionality.  In conducted mode the conducted applications must request permission to initiate specific types of AVC actions. This request is made using the AVC-Privilege-Ask Primitive.

AVC-Conductor-Privilege-Ask Primitive

Table 12/T.133.

|  | Request | Indication | Response | Confirm |
| --- | --- | --- | --- | --- |
| Requester's AVC User ID | M | M= | M= | M= |
| Device Control Privileges | O | O= | O= | O= |
| Service Access Privileges | O | O= | O= | O= |
| Link Configuration Privileges | O | O= | O= | O= |

Requester's AVC User ID:	AVC User ID of node requesting privileges from conductor.

Device Control Privileges:	Privileges to use Remote device control in conducted mode.

Service Access Privileges:	Privileges for a Terminal to use MCU services  in conducted 				mode.

Link Configuration Privileges:	Privileges to change link configuration within agreed 					capability set.

AVC-Conductor-Privilege-Ask -Sequence of primitives

Figure 11/T.133.

#### Conductor Privilege Grant

This primitive is provided to allow a conductor to grant privileges to nodes.

AVC-Conductor-Privilege-Grant Primitive

Table 13/T.133.

|  | Request | Indication | Confirm |
| --- | --- | --- | --- |
| Requester's AVC User ID | M | M= | M= |
| Device Control Privileges | O | O= | O= |
| Service Access Privileges | O | O= | O= |
| Link Configuration Privileges | O | O= | O= |

Requester's AVC User ID:	AVC User ID of node requesting privileges from conductor.

Device Control Privileges:	Privileges to use Remote device control in conducted mode.

Service Access Privileges:	Privileges for a Terminal to use MCU services  in conducted 				mode.

Link Configuration Privileges:	Privileges to change link configuration within agreed 					capability set.

AVC-Conductor-Privilege-Grant -Sequence of primitives

Figure 12/T.133.

## Network and Management Device Defintions

This section defines the standard devices used by conference services.

Authors note it is the intention that the these device defintion be aligned with those in AVC Part 2  and that the definition be moved to that document.

### Switching Device Class

An MCU will have a number of physical Ports associated with it. These determine the number of direct connections that the MCU can support. When an MCU is participating in a session normally two or more of its ports are connected to other nodes. These connections are termed Links. A link  may have a video component which comprises one or more video pipes in each direction. It may also have an audio component which comprises one or more audio pipes in each direction.

A video switch is an example of a device which resides in an MCU. Its function is to route video streams. It  has as its input; one or more video pipelines from each of the incoming video links. Its output is also one or more video pipelines from each of its outgoing video links. It is able to route any incoming video stream to one or more of its outgoing video pipelines.

Table 14/T.133.below shows an example of a switching matrix  for a four port MCU. Each port provides an incoming and outgoing video link capable of carrying two independent video streams. The example shows link 1 pipeline 1   being broadcast to the other participants while link 1 pipeline 1 sees video from link 4 pipeline 1. Link 2 pipeline 2 is being simultaneously broadcast  to all participants.

The matrix may be scaled up or down to represent the particular session in progress. a similar matrix may also be used for audio routing.

Video Switching - control Matrix Example

Table 14/T.133.

|  |  | VIDEO OUT |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VIDEO IN |  |  | LINK 1 |  |  | LINK 2 |  |  | LINK 3 |  | LINK 4 |  |
|  |  |  | P1 | P2 | P1 |  | P2 | P1 |  | P2 | P1 | P2 |
| LINK 1 | Pipeline 1 |  |  |  | X |  |  | X |  |  | X |  |
|  | Pipeline 2 |  |  |  |  |  |  |  |  |  |  |  |
| LINK 2 | Pipeline 1 |  |  |  |  |  |  |  |  |  |  |  |
|  | Pipeline 2 |  |  | X |  |  | X |  |  | X |  | X |
| LINK 3 | Pipeline 1 |  |  |  |  |  |  |  |  |  |  |  |
|  | Pipeline 2 |  |  |  |  |  |  |  |  |  |  |  |
| LINK 4 | Pipeline 1 |  | X |  |  |  |  |  |  |  |  |  |
|  | Pipeline 2 |  |  |  |  |  |  |  |  |  |  |  |

The video switching service (defined in 13.1) provides an automatic means of determining the algorithm used to determine the  routing.

This switching function can be represented by a two dimensional matrix . The only constraint placed on its operation is that any video pipeline can only carry one video stream at any given time.

Switch Device -Class Definition

Table 15/T.133.

| DEVICE NUMBER:- 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DEVICE CLASS:- Switch |  |  |  |  |  |
| ATTRIBUTE | ACTIONS | UNITS | RANGE | OPTIONS | STATUS |
| Route | From Input (pipeline) to output (list). | integer | 1-max pipelines |  |  |
| Monitor | Enable Disable |  |  |  |  |

### Audio Mixer Device Class

Audio Mixer Device -Class Definition

Table 16/T.133.

| DEVICE NUMBER:- 4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DEVICE CLASS:- Audio Mixer |  |  |  |  |  |
| ATTRIBUTE | ACTIONS | UNITS | RANGE | OPTIONS | STATUS |
| Input | Select Mute Add Remove Level |  |  |  |  |
| Output | Level Control |  |  |  |  |
| Monitor | Enable Disable |  |  |  |  |

INPUT

Select	-Selects an input to the audio mixer to be included in the mix.

Mute	- Mute an input to the audio mixer temporarily

Add	- Include a new input in the audio mix

Remove

OUTPUT

Level	- Adjust mixed output level

Control	- ?

#### Video  Processor Device Class

Video Processor Device -Class Definition

Table 17/T.133.

| STANDARD DEVICE NUMBER:- 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DEVICE CLASS:- Video Processor |  |  |  |  |  |
| ATTRIBUTE | ACTIONS | UNITS | RANGE | OPTIONS | STATUS |
| Mode | Array Multiplex Transcode |  |  |  |  |
| Monitor | Enable Disable |  |  |  |  |

MODE

Array		- Select the Array processing mode for continuous presence

Multiplex	- Select the Multiplex  mode for continuous presence

Transcode	- Selects the Array transcoding method for continuous presence.

#### Video Encoder/Decoder Device Class.

Video Encoder/Decoder Device -Class Definition

Table 18/T.133.

| STANDARD DEVICE NUMBER:- 8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DEVICE CLASS:- Video Encoder/Decoder |  |  |  |  |  |
| ATTRIBUTE | ACTIONS | UNITS | RANGE | OPTIONS | STATUS |
| Video Protocol | H261 QCIF CIF S-CIF MPEG |  |  |  |  |
| Image Resolution |  | X * Y |  |  |  |
| Pixel Aspect Ratio |  | Ratio X:Y |  |  |  |
| Frame Rate |  | frames/sec |  |  |  |
| Monitor | Enable Disable |  |  |  |  |

## The Virtual MCU.

This concept is based on the philosophy that like  functions in separate MCUs can be configured and managed as  single virtual functions for any given conference session.

Terminals always communicate with MCU's  using the MCU broadcast channel. In this way terminals have a standard way to communicate with the network  and its MCU elements irrespective of the topology of any given conference connection. Terminals  should use priority 2 for  such communication.

MCU's  use the MCU control channel to communicate, exchanging parameters and arrive at common decisions in a way not feasible iun the past. Inter MCU functions critical to the conference performance are regarded as a special case and can use this channel at the highest data priority. Routine Inter-MCU communication should use priority 2.

In a multipoint conference one or more MCU's will be connected to the MCU broadcast channel. When information is sent to this  channel one of two modes of operation will need to be applied.

Mode 1:	All MCU's decide and act on the received information independently.

Mode 2:	The Top Provider must authorise the request.

Mode 2 will be the normal default but the top provider may assign privileges to other MCUs.

Individual MCU's User ID's appear in the roster and can be addressed directly, but all communication from terminals should normally be directed the (static) MCU broadcast channel or to the Top Provider MCU.

The conference services make use of the virtual MCU model shielding terminal nodes from the network topolgy and complexites.

AVC PROTOCOL SPECIFICATION

## AVCPDU Definitions

01	Video Switching

02	Video Identification

03	Continuous Presence -Multiplex Mode

04	Continuous Presence - Array Processing Mode

05	Continuous Presence - Transcoding Mode

06	On Air Indication

07	Floor Control

08	Audio Mixing

09 - 127	Reserved For Future Use

128 - 255 Available For User Defined Services.

The structure of GCCPDUs is specified as follows using  ASN.1 notation as defined in CCITT Recommendation X.680. All AVCPDUs shall be encoded for transmission by applying the packed Encoding rules specified in ITU Recommendation X.691 using the basic aligned variant.

AVC-PROTOCOL DEFINITION  AUTOMATIC TAGS ::=

IMPORTS

FROM GCC-PROTOCOL;

AVC PDU Messages

Note?	-Other services such as  ITU H Series Recommendations .