# AVC Part 2 - Remote Device Control

## Remote Device Control - Introduction

Remote Device Control(RDC) provides modelling and classification of devices for the purpose of allowing remote activation and control within a conference environment. This is acheived by defining a template of control and configuration attributes for a set of commonly used devices. RDC also provides support to allow nodes to utilise the RDC model to define and advertise proprietry devices using standardise attributes from the toolkit.

Devices are normally physical entities but they may also be software processes. In both cases the RDC control model requires that the host node is the default owner of a given device. Nodes may advertise devices that they will allow to be remotly accessed and controlled. Devices are identified by their host node GCC User ID, the device class identifier and the instance number. Each device instance has a state paramater associated with it. Devices may be in one of four states ‘Available’, ‘In Use’ ‘Restricted’or ‘Managed’. Available devices will accept requests for remote access and control. Devices  “In Use” have been given exclusive access by another node, any requests will be put into a queue until that device becomes available. Restricted devices are only available to nodes assigned privilages e.g the convenor or Conductor. Managed devices are being utilised by the AVC Conference Resource manager and are unavailable for use by participants.

Devices intended for use by conference participants are advertised using GCC non-collapsing capability mechanism. Devices that are exclusively intended for conference  management services are advertised in band using the MCU control channel. Devices are also classified by type, some devices can be regarded as sources (no input potential) and others as sinks (no output potential), while a third type can be regarded as processors and switches (having both input(s) and output(s) ). RDC provides mechanisms to support the remote  inter-connection of devices to form complex devices.

## Device Control

The remote device control  may be used to exert actions on a device at a remote location in a T.120 environment. The term device is used to describe a multitude of function providers within a conference environment. Devices can be sources or they can be receivers of  real time stream information. A device may be both a source and a receiver such as in the case of a video tape recorder. Remote device control provides the following facilities to the conference :-

- declaration of devices and their associated attributes to enable a conference participant to determine a profile of device attached to a remote node.

- a mechanism to allow a participant to request exclusive access to a remote device in order to perform device control or configuration.

- mechanisms to allow a participant to control, configure, receive event notification and obtain the status of a remote device.

- a mechanism to allow a participant to select the remote device or devices that source their outputs into an established  real time stream.

At each node, devices are assigned unique numeric identifiers. The device identifier taken together with the device owners RDC User ID provides a unique handle for each device within the conference.

Devices require no corresponding peers in order to be usable by the conference. A list of available devices are provided to GCC as non-collapsing capabilities, which GCC distributes unchanged, to all other RDC nodes in the conference. A node must interrogate a remote node to obtain the device details before control or configuration of that device commences.

Remote Device Control defines a number of standard device classes. These aim to address mandatory or commonly required functionality for use in support of multipoint multimedia communication and in particular conferencing. For each standard device class there is a library of standard device attributes. The remote node declares which of these attributes it supports for an attached device.

Figure 2 shows the components that are involved in controlling a remote device. An RDC application can perform the following facilities :-

1. Generate requests to control a device attached to a remote node.

2. Receive requests from a remote RDC application to control a device attached to this node.

An RDC application protocol entity may be able to perform one or both of the above facilities.

Figure 1/T.133

Remote device control is defined for point to point control. (The provision of multipoint control affecting  devices attached to several nodes at once is for further study although it is though to be of little use.)

## The Remote Device Control Application

### Application Enrolment

A device is generally a physical entity that may be controlled or configured by remote users.  Since there can only be one instance of a physical device, it should only be a member of one session at one time. If this were not the case, and a device was a member of several sessions, a user changing the configuration of that device from within one particular session would also affect other simultaneous sessions. Therefore only a static session is allowed.

#### Static Mode

The RDC  entity shall first obtain an MCS User ID. It shall then join the predefined static RDC-CHANNEL by issuing an MCS-Channel-Join request specifying the RDC-CHANNEL as the channel to join. Once positive confirmation of joining this channel has been received, the RDC APE shall enrol by issuing a GCC-Application-Enroll request to the GCC provider.  The Active/Inactive flag shall be set to Active, the Session ID shall be specified as part of the Session Key, the Start-Up Channel shall be specified as Static and the full application capabilities and non-collapsing capability lists shall be provided.

#### Capabilities

Capability negotiation for the Audiovisual Control application is performed by GCC (as defined in ITU recommendation T.124). The RDC application passes its application capabilities to GCC as part of the application enrolment mechanism. GCC distributes the capabilities to the conference in the application Roster.

These capabilities may be unique to the node and unlike other capabilities, the information value will be lost if any of GCC's capability collapsing rules are applied.  The remote control capabilities are therefore provided as non-collapsing capabilities.

Each node will have its own list of devices that are available for remote control or configuration. Each device identifier will only be unique at its parent node and to achieve conference wide uniqueness it should be referenced with its parent node's RDC User Id.

A list of devices are provided to the conference via the non-collapsing capabilities. An RDC application will be required to interrogate the node that owns the device, to obtain the full device characteristics before it can perform remote control.

The remote control Capability list provides the GCC application Roster entry for the remote control Service.  Each device entry in the list has the following format.

| Parameter | Description |
| --- | --- |
| Device Class | This is one of the standard classes defined in this document or a non-standard user class. |
| Device Number | This is a number ranging from 0 to 127 which identifies a particular device of this class that is connected to this node. |

The Device Class allows selection of  a class of devices, (e.g. Cameras). The Device Number identifies a particular device in that class. These fields together provide a unique identification key for any given device at a specific node.

The list provides enough information which allows device attribute information to be retrieved.

#### RDC Communication Channel

The RDC application utilises 1 MCS static channel to provide management and control services :-

·	RDC Broadcast Channel

The RDC broadcast channel is joined by all nodes supporting RDC capabilities. It is used for broadcasting information to all nodes that support the RDC application.

General device control will be performed on a point to point client server basis. An  RDC application will send PDUs to a remote RDC application on a remote node in order to control a device owned by the remote node. The sending RDC application will specify the User ID of the remote RDC application as the channel ID of the PDU.

## Determining Attributes for a Remote Device

A list of available devices can be obtained from the roster entry for the RDC application. An RDC application wishing to control, configure, obtain the status or receive events from a specific device at another node will first be required to obtain the device attributes using the Remote-Device-Attribute-Request PDU. Device attributes are divided into three main types - control attributes,  status attributes and event attributes.

A positive reply to this request will contain a list of the attributes that are supported by the device. The device can only advertise an attribute if it is capable of supporting all the parameters that are defined for that attribute.

The request may be carried out as soon as the remote application enters the conference, just before control of a remote device is to commence or when a remote device has indicated that its attribute profile has changed. The device is identified from the key made up from the device class and the device number.

### Requesting Device Attributes

The RDC-Remote-Device-Attribute-Request PDU is issued by a node wishing to obtain the attribute profile for a specified device. The request is sent with the channel ID of the PDU set to the user ID of the intended recipient RDC application.

RDC-Remote-Device-Attribute-Request PDU

| Parameter | Description |
| --- | --- |
| Requesting RDC User ID | This identifies the User ID of the RDC application making the request. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number between 0 and 127 which identifies a particular device of this type that is under the control of the remote RDC application. |

The RDC-Remote-Device-Attribute-Response PDU is issued by an RDC application in reply to receiving the RDC-Remote-Device-Attribute PDU. A successful response contains the requested device attributes. The response is sent with the channel ID of the PDU set to the user ID of the RDC application that made the original request.

RDC-Remote-Device-Attribute-Response PDU

| Parameter | Description |
| --- | --- |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number ranging from 0 to 127 which identifies a particular device. |
| Device name | This is an octet string containing the name of the device. If this device has no name then this will be NULL. |
| Device attribute list | List of device attributes. This parameter will be NULL if the request is unsuccessful. |
| Request Result | This is set to one of the following result codes Successful - The request is successful. Invalid device number - The device number does not reference a known device. Invalid device class - The device class does not reference a known class. Request refused - The attribute request was refused. |

The format of each attribute record is as follows:-

| Parameter | Description |
| --- | --- |
| Device Attribute | This is an attribute for this device class. |
| Attribute type | This parameter takes one of the following values :- Control attribute Status attribute Event attribute |

Control attributes may be used to control a remote device and must be sent using the  Remote-Device-Control PDU.

Status attributes may be used to obtain the status of  a remote device and must be sent using the Remote-Device-Status PDU.

Event attributes may be used to request notification of events from a remote device.

## Remote Device Control

A node wishing to control a remote device should first ensure that the device is available to it, this may be achieved by locking the remote device using the RDC-Remote-Device-Lock-Request PDU. If the remote node grants the lock request,  attempts from other RDC applications at device control will be refused.  This will ensure that the controlling node has access to the device and that the device is also locked on to any associated stream. Having obtained the device, the controlling node then requests a stream routing. If the MCU(s) provide this routing, then control may commence. The controlling node must free both the stream and the device immediately it has completed the activity or if either of the resources is lost and control cannot be  continued.

If an RDC application locks a remote device and then subsequently terminates and is removed from the application roster, all locks requested by that application that are still in force should be automatically terminated.

A node can attempt to remove a lock that has been placed on one of its devices by interrogating the RDC application that requested the device lock to determine if it is still required. If the remote node indicates that the device may be unlocked, then it may be unlocked immediately. If no reply is received within an suitable- time period, the node can unlock the device.

### Exclusive Remote Device Access

- An RDC application can request a lock be placed on a device in order to obtain exclusive control over that device using the RDC-Device-Lock-Request PDU.

The request may not be granted. If this is the case then the requesting application will need to attempt another lock request at a later time.

The RDC-Device-Lock-Request PDU is sent with the channel ID of the PDU set to the user ID of the intended recipient RDC application.

RDC-Device-Lock-Request PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the application requesting the device lock. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number between 0 and 127 which identifies a particular device. |
| Lock request | This parameter can take one of the following values: Release the current lock previously placed by this RDC application. Lock device. |

The RDC-Device-Lock-Response PDU is issued by an RDC application in reply to the RDC-Device-Lock-Request PDU. The response is sent with the channel ID of the PDU set to the user ID of the RDC application that made the original request.

RDC-Device-Lock-Response PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the application that is returning this response. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number between 0 and 127 which identifies a particular device. |
| Result | This is set to one of the following result codes Successful - The request is successful. Device busy - The referenced device is busy. Device is locked - The referenced device has already been locked. Device out of service - The referenced device is no longer communicating with this node. Invalid device number - The device number does not reference a known device. Invalid device class - The device class does not reference a known class. Request refused - The device locking request was refused. |

If a node requires a lock to be removed from one of its devices it may interrogate the remote node that placed the lock to determine if it is still required. This can be performed using the RDC-Application-Unlock-Device-Request PDU. This is sent with the channel ID of the PDU set to the user ID of the RDC application that placed the device lock. If no response is received after a suitable time period then the lock may be removed.

RDC-Application-Unlock-Device-Request PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the application that owns the locked device and is making the request for it to be unlocked. |
| Device class | This is the class of the locked device. |
| Device number | This is a number between 0 and 127 which identifies the locked device. |

The RDC-Application-Unlock-Device-Response PDU is issued by an RDC application in reply to the RDC-Application-Unlock-Device-Request PDU. The response is sent with the channel ID of the PDU set to the user ID of the RDC application that made the original application unlock request. The result should be set to indicate if the referenced device can be unlocked.

RDC-Application-Unlock-Device-Response PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the application that is returning this response. |
| Device class | This is the device class that was contained in the request PDU. |
| Device number | This is the device number that was contained in the request PDU. |
| Result | This is set to one of the following result codes Device may be unlocked Device may not be unlocked - Exclusive access is still required. |

### Requesting Remote Device Control

The RDC-Remote-Device-Control PDU is issued by a node wishing to control a device on a remote node. The request is sent with the channel ID of the PDU being set to the user ID of the remote RDC application.

It is the responsibility of the sending application to not list opposing control attributes in this PDU. For example the attribute list should not contain an attribute to start panning a camera left followed by an attribute to start panning right. It is a local matter as to how a receiving application will respond to this.

The receiving application will ignore the following :-

- Any unrecognised control attributes. This includes attributes that are supported but were declared as configuration or status attributes.

- Any control attributes will invalid parameters.

RDC-Device-Control-Request PDU

| Parameter | Description |
| --- | --- |
| Requesting RDC User ID | This identifies the User ID of the RDC application making this request. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number ranging from 0 to 127 which identifies a particular device. |
| User string | This is an octet string with a maximum length of 255. Its use is a local matter. If this field is not used then it may be set to null. |
| List of control attributes. | List of device control attributes. The allowable values for these list members are described in the relevant device specification. |

The use of the user string parameter is a local matter. An example of a possible use is when it provides a minimum means of  user authentication where the requesting application is required to insert a predefined string into this field which the receiving end expects to be present before allowing the control request to proceed.

The RDC-Device-Control-Response PDU is issued by an RDC application in reply to the RDC-Device-Control-Request PDU. The response is sent with the channel ID of the PDU set to the user ID of the RDC application that made the original request.

RDC-Device-Control-Response PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the application that is returning this response. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number between 0 and 127 which identifies a particular device. |
| Result | This is set to one of the following result codes Successful - The request is successful. Device busy - The referenced device is busy and can not be controlled. Device is locked - The referenced device has been locked and can not be controlled. Device out of service - The referenced device is no longer communicating with this node. Invalid device number - The device number does not reference a known device. Invalid device class - The device class does not reference a known class. Request refused - The device control request was refused. |

A positive acknowledgement to the Remote Control Command PDU only implies that the receiving RDC application has accepted the PDU. It does not imply that the remote device has carried out any of the actions contained in that PDU. However the device should have actions sent to it by the RDC application in the same order as the reception of PDUs.

It is a local matter whether the receiving RDC application has a means to determine when a device is out of service and reject incoming actions destined for that device.

## Remote Device Status

An RDC application can interrogate a remote device for its status using the Remote-Device-Status-Request PDU. The request is sent with the channel ID of the PDU being set to the user ID of the remote RDC application.

This PDU contains a list of the status attributes for which the status is required. The response contains a list of these same status attributes with accompanying parameters indicating their status value.

### Requesting Remote Device Status

The RDC-Device-Status-Request PDU is issued by a node wishing to obtain the status of a specific device. The request contains a list of status attributes that are being requested. A positive response contains the status of these attributes at the time that the request was received.

RDC-Device-Status-Request PDU

| Parameter | Description |
| --- | --- |
| Requesting RDC User ID | This identifies the User ID of the RDC application making this request. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number ranging from 0 to 127 which identifies a particular device. |
| List of status attributes. | List of status attributes. The allowable values for list members are described in the relevant device specification |

The RDC-Device-Status-Response PDU is issued by an RDC application in reply to the RDC-Device-Status-Request PDU. The response is sent with the channel ID of the PDU set to the user ID of the RDC application that made the original request.

RDC-Device-Status-Response PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the RDC application that is returning this response. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number between 0 and 127 which identifies a particular device. |
| List of status attributes and associated parameters. | This is a list of the status attributes contained in the request with the accompanying parameters describing the attribute status. See the relevant device specification for a list of allowed attributes. If the status can not be obtained then this list will be null. |
| Request Result | The result of a request which can be one of the following :- Successful - The request is successful. Device busy - The referenced device is busy. Device out of service - The referenced device is no longer communicating with this node. Invalid device number - The device number does not reference a known device. Invalid device class - The device class does not reference a known class. Request refused - The device status request was refused. |

## Remote Device Events

An RDC application can request that it is notified of event changes occurring on a remote device in the form of an unsolicited  PDU. A remote device can declare support for particular event attributes in the attribute list contained in the RDC-Remote-Device-Attribute-Response PDU. The RDC application can then request notification for specific events. For example an application can request to be notified when the VCR tape has stopped playing or recording or a camera has panned to its limit.

### Requesting Remote Device Notification

An RDC application can register an interest in event notifications from a specific remote device. The RDC application sends a RDC-Device-Event-Notify-Request PDU to the remote node specifying the remote device and including a list of event attributes specifying the events for which notification is required. If the remote node accepts this request then it will send  a  RDC-Device-Event-Indication PDU to the node whenever a specified event occurs. The requesting RDC application may use the RDC-Device-Event-Notify-Request PDU to cancel all previously requested notifications by specifying a null list of attributes.

RDC-Device-Event-Notify-Request PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the RDC application making this request. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number ranging from 0 to 127 which identifies a particular device. |
| List of event attributes. | List of event attributes that are to be registered for notification. All previously registered attributes originating from the requesting RDC application are cancelled and are not appended to this list. The allowable values for list members are described in the relevant device specification. If event notification is to be cancelled then this parameter list is set to NULL. |

The RDC-Device-Event-Notify-Response PDU is issued by an RDC application in reply to the RDC-Device-Event-Notify-Request PDU. The response is sent with the channel ID of the PDU set to the user ID of the RDC application that made the original request.

RDC-Device-Event-Notify-Response PDU

| Parameter | Description |
| --- | --- |
| RDC application User ID | This identifies the User ID of the application that is returning this response. |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number ranging from 0 to 127 which identifies a particular device. |
| Result | This is set to one of the following result codes Successful - The request is successful. Device busy - The referenced device is busy. Device out of service - The referenced device is no longer communicating with this node. Invalid device number - The device number does not reference a known device. Invalid device class - The device class does not reference a known class. Request refused - The event notification request was refused. |

The RDC application will send an RDC-Device-Event-Indication to a remote RDC application that has previously requested event notification when ever an event that requires notification occurs.

RDC-Device-Event-Indication PDU

| Parameter | Description |
| --- | --- |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number ranging from 0 to 127 which identifies a particular device. |
| List of event attributes and associated parameters. | List of event attributes with associated parameters indicating which events have occurred. |

### Device Attribute Changes

The RDC-Device-Attribute-Change-Indication PDU is issued by a node when the attribute profile of a  device owned by that node is changed. This indication is broadcast on the static RDC-CHANNEL and will be received by all peer applications. It is then up to the receiving application to update itself by re-issuing a device attribute request.

RDC-Device-Attribute-Change-Indication PDU

| Parameter | Description |
| --- | --- |
| Device class | This is one of the standard classes or is a non-standard user class. |
| Device number | This is a number ranging from 0 to 127 which identifies a particular device. |

## Source Selection

Remote device control provides a facility to select the device or devices that source information into a previously established real time stream. The means of establishing the real time stream is outside of the scope of this recommendation.

The RDC application will declare a source selection device of the relevant type for each real time stream. This device will contain profiles that represent the selection combinations that are permitted for that real time stream.  In the case of  several devices being mixed together to produce a single output, the profile may allow the set of devices that will be combined to be configured.

Video profile

| Profile member | Description |
| --- | --- |
| Matrix size (horizontal x vertical) | This matrix describes how many devices are to be combined to produce the source for the video stream and the positioning of these devices on the video image. It can be read by a remote RDC application. |
| Set of available source devices | This lists the devices that are capable of sourcing an output into this profile. The ordering of the devices in this list matches the matrix as shown in the diagram. It can be read by a remote RDC application. |
| Available image types | This lists the image types the resultant output may take when it is sent over the video stream. The available image types that may be listed are live image and graphic image. It can be read by a remote RDC application. |
| Image type to send | This member may be set by a remote RDC application with an entry taken from the Available image types list. It sets the resultant output type that is to be sent over the video stream. |
| Set of source devices | This list may be configured with the set of devices that source an output into this profile. This list allows a remote end to select the combination and ordering of devices that are mixed to produce the real time video stream. Only those devices that appear in the available devices list may be included. The ordering of the devices in this list matches with the matrix as shown in the diagram. This configurable member is not available in all profiles in which case only a fixed set of devices with fixed positioning is allowed as shown in the available devices list. |
| Profile label | This is a text label that indicates the name of this profile. It may be read by a remote RDC application. |

Matrix and device numbering starts at the top left hand corner and moves towards the right and then down the matrix. For example, a profile may declare a 2 by 2 matrix with an available source device list containing 4 devices.

Device matrix = 2 x 2

Device list = camera 1, camera 2, VCR, camera 5

The matrix defines how the combined sources will be ordered in the video stream as shown in the diagram. The exact format used to display these devices at the remote end will depend upon the remote terminal.

| camera 1 | camera 2 |
| --- | --- |
| VCR | camera 5 |

If there are less entries in the device list than matrix positions, the remaining positions are undefined. If there are more entries in the device list than matrix positions, the extra devices are not used as part of the video stream.

Audio profile

| Profile member | Description |
| --- | --- |
| Number of devices | This number describes how many devices are to be combined to produce the source for the audio stream |
| Set of available source devices | This lists the devices that are capable of sourcing an output into this profile. |
| Set of source devices | This list may be configured with the set of devices that source an output into this profile. This list allows a remote end to select the combination of devices that are mixed to produce the real time audio stream. Only those devices that appear in the available devices list may be included. This configurable member is not available in all profiles in which case only a fixed set of devices is allowed as shown in the available devices list. |
| Profile label | This is a text label that indicates the name of this profile. It may be read by a remote RDC application. |

For example, for a terminal that provides one video and one audio output stream, there will be two  source selection devices - an audio source selection and a video source selection device. The video source selection device may contain the following profiles :-

Profile 1

| Device Matrix = 1 X 1 | Only one device be used to source the video stream. |
| --- | --- |
| Available image types = live image, graphic image | The resultant device output can be sent as a live image or as a graphic image. |
| Set of available source devices = camera 1, camera 2, camera 3. | These are the devices available to this profile. |
| Configuration of the set of source devices is available in this profile. | A remote RDC application can configure this device set. The set only contains 1 device and may be configured to be camera 1 or camera 2 or camera 3. |

Profile 2

| Device Matrix = 1 X 2 | Two devices will used to source the video stream with one device stacked above the other. |
| --- | --- |
| Available image types = live image | The resultant video output can only be sent as a live image. |
| Set of available source devices = camera 1, camera 2 | These are the devices available to this profile. |
| Configuration of the set of source devices is not available in this profile. | A remote RDC application can not configure this device set. Therefore this profile can only send a combination of camera 1 and camera 2 with camera 1 stacked above camera 2. |

The audio source selection device may contain the following profiles :-

Profile 1

| Number of devices = 1 | Only one device be used to source the video stream. |
| --- | --- |
| Set of available source devices = microphone 1, microphone 2 | These are the devices available to this profile. |
| Configuration of the set of source devices is available in this profile. | A remote RDC application can configure this device set. The set only contains 1 device and may be configured to be microphone 1 or microphone 2. |

Profile 2

| Number of devices = 2 | Two devices will be combined to source the audio stream. |
| --- | --- |
| Set of available source devices = microphone 1, microphone 2, microphone 3 | This is a set of devices that are available for combining together for connection to the real time stream. |
| Configuration of the set of source devices is available in this profile. | A remote RDC application can configure this device set. The set contains 2 devices and may be configured to be :- microphone 1 and microphone 2 or microphone 1 and microphone 3 or microphone 2 and microphone 3. |

A profile may also contain device presets which encompass one or more devices. These presets are separate from the presets that are contained within the individual device classes. For example, a preset may be available in a two camera video source selection profile which covers both the two cameras that are combined to produce the real time stream. If this preset is selected then both cameras will move to their preset positions.  These preset positions will only be available through this profile and can not be accessed via the individual camera devices.  A preset may encompass only a subset of the devices that are combined to produce the real time stream. This area is for further study.

When more than 1 output video stream or 1 audio stream is available, a remote terminal will need some means of linking the source selection device with a particular real time stream. This may require remote device control interacting with RTLM. In the case of 1 output  video stream and 1 output audio stream, it is obvious which stream the source selection device is connected to. In this case no interaction with RTLM is required.

## Stream Routing

The video or the audio stream will often be the only means by which the controlling node gets feedback from the remote control commands it issues. In many situations Multipoint bridges will be carrying out automatic manipulation of these streams under the control of some pre-defined algorithm. In such cases it may be necessary for the node requiring remote device control to request a specific stream routing. This service will be provided by accompanying recommendations that are part of the Real-time Multimedia Conferencing(RMC) set of standards .

## Conducted Mode Behaviour

RDC is a conductor aware application protocol in that it has two modes of operation one for non-conducted mode and one for conducted mode, as described in this section.

Applications become aware of a GCC Conference Conductor being assigned through receipt of a GCC-Conductor-Assign indication from its GCC provider. This includes the GCC User ID of the conducting node. Applications must determine the identity of the RDC application protocol which inherits the conductors role.

The start-up or capability processes are unaffected. No new use of functions may be started without permission from the conductor.

### Conductor Privilege Request

The conductor may chose to assign privileges to particular nodes or to all nodes. These privileges may be for specific functionality, or for all functionality.  In conducted mode the conducted applications must request permission to initiate specific types of RDC operations. This request is made using the RDC-Privilege-Request PDU.

RDC-Privilege-Request PDU

| Parameter | Description |
| --- | --- |
| Device control privilege | This flag indicates whether the requesting RDC application wants permission to control remote devices. |

### Conductor Privilege Assign

On receipt of an RDC-Privilege-Request PDU, the RDC conductor may respond by broadcasting an RDC-Privilege-Assign PDU on the RDC-CHANNEL.  The PDU contains a list of privileges granted and the User ID of the requesting RDC application and enables other RDC applications to determine which privileges have been assigned to their peers.  The RDC conductor does not need to send an RDC-Privilege-Assign if it does not wish to change the privileges of the requesting application.

RDC-Privilege-Assign PDU

| Parameter | Description |
| --- | --- |
| Privilege List | This contains a list of one or more RDC User IDs and the privileges assigned to them. These privileges are identified below. |
| Device control privileges | This flag indicates whether the requesting RDC application has been granted permission to control remote devices. |

## Standard RDC Devices

### Device Control Specifications

A device class definition is only intended to include functionality that may be appropriate for remote access and control.  A node having functionality that it is willing to allow to be remotely accessed and controlled, advertises this functionality in terms of the device classes and the associated attributes that it supports from the class.

A device owner advertising support for an attribute must be able to support the protocol and parameters associated with it. A device definition comprises a list of attributes, each  of which may have parameters associated with it. Some parameters indicate a value within a range. Parameter ranges are defined by this document in order to reduce complexity. The RDC application must map this range to the actual range that is supported by the physical device.

It is not always possible to quantify actions, and in such cases, a continuous change mode may be appropriate (e.g. Start panning the camera left). This however is not the preferred method of use and should be avoided where ever possible.

The result of many of the actions will be detectable by the controlling node by monitoring the audio or video stream received from the device owner. This feedback will have a communication delay associated with it, which will have some dependency on the number of communication links in the transmission path. Once the Stop message is sent over the T.120 channel, there is a further communication delay before it is received and acted upon. The problem may be compounded  by high T.120 link traffic, also T.120  supports many different networks and  therefore no general assumptions can be made about available throughput or the consistency of throughput capability across the session connections.

### Standard Device Classes

Device Class TABLE

| Device Class Number | Device Class Name |
| --- | --- |
| 00 | Camera |
| 01 | Microphone |
| 02 | VCR |
| 03 | Slide Projector |
| 04 | Light Source |
| 05 | Document scanner |
| 06 | Video Source Selection |
| 07 | Audio Source Selection |
| 08 .. 255 | Reserved |
| 256..511 | Available for user-defined classes |

Device Number

00 -127  available for nodes to allocate locally

### Control Action Ranges

Some control attributes will require an associated  parameter or parameters. An attribute parameter will have a predefined range allocated where possible by this specification. For example the range for the camera pan to absolute position action is from -32767 to +32767. The maximum +32767 represents the furthest right hand pan position, the minimum -32767 represents the furthest left hand pan position and 0 represents the mid point position. The receiving application will need to translate the pan parameter into the correct range for the actual camera being panned.

### Standard Device Control Specifications

Device class identifiers are assigned by this document. Each node may support up to 128 devices from any class, identified by a numeric index (0 to 127).

#### Camera Device Class

For a videoconferencing environment camera's are normally the principle source of  video. The camera device class therefore provides a comprehensive remote control capability allowing control over the camera position and viewing angle as well as the ability to manipulate  parameters relating to the image.

Camera control parameters may be specified relative to the current position or as an absolute value.

Certain attributes defined in this class have a fixed co-ordinate system into which the RDC application must map the actual co-ordinate system used by the camera. For example, the pan attributes have a fixed co-ordinate system -32767 to +32767. Pan position +32767 represents the maximum pan to the right. Pan position -32767 is the maximum pan to the left and pan 0 represents the mid point.

The following table show the control attributes. It also shows the parameter values that are used when these attributes are part of an RDC-Device-Control-Request PDU.

| Device Control Attribute |  | Request Parameter |  | Description |
| --- | --- | --- | --- | --- |
| PanPostionAbsolute | -32767.. +32767 |  | Moves the camera to an absolute horizontal pan position. The value -32767 represents the maximum pan to the left. |  |
| TiltPositionAbsolute | -32767.. +32767 |  | Moves the camera to an absolute vertical tilt position. The value -32767 represents the maximum tilt downwards. |  |
| PanPostionDegreesAbsolute See Note 1 | -180°..+180° |  | Moves the camera to an absolute horizontal pan position. The parameter value is in degrees where -180 represents the maximum pan to the left. |  |
| TiltPositionDegreesAbsolute | -180°..+180° |  | Moves the camera to an absolute vertical tilt position. The parameter value is in degrees where -180 represents the maximum tilt downwards. |  |
| ZoomPositionAbsolute | -127..+127 |  | Moves the camera to an absolute zoom position. Value -127 represents the unzoomed position, value +127 represents maximum zoom. |  |
| FocusPositionAbsolute | -127..+127 |  | Moves the camera to an absolute focus position. Value -127 represents the near focus position, value +127 represents the far focus position. |  |
| IrisPositionAbsolute | -127..+127 |  | Moves the camera iris to an absolute position. Value -127 represents the darkest position, value +127 represents the lightest position. |  |
| PanPostionRelative | -32767 .. +32767 |  | Moves the camera to a horizontal pan position relative to the current position. |  |
| TiltPositionRelative | -32767 .. +32767 |  | Moves the camera to a vertical tilt position relative to the current position. |  |
| ZoomPositionRelative | -127..+127 |  | Moves the camera to a zoom position relative to the current position. |  |
| FocusPositionRelative | -127..+127 |  | Moves the camera to a focus position relative to the current position. |  |
| IrisPositionRelative | -127..+127 |  | Sets the camera iris to a position relative to the current position. |  |
| PanContinuous See Note 2 | Left with timeout 100..1000ms, Right with timeout 100..1000ms, Stop |  | Starts or stops a continuous horizontal panning movement with a maximum timeout. |  |
| TiltContinuous See Note 2 | Up with timeout 100..1000ms, Down with timeout 100..1000ms, Stop |  | Starts or stops a continuous vertical tilting movement with a maximum timeout. |  |
| ZoomContinuous | Tele with timeout 100..1000ms, Wide with timeout 100..1000ms, Stop |  | Starts or stops a continuous zoom movement with a maximum timeout. |  |
| FocusContinuous | Near with timeout 100..1000ms, Far with timeout 100..1000ms, Stop |  | Starts or stops a continuous focus movement with a maximum timeout. |  |
| IrisContinuous | Darker with timeout 100..1000ms, Lighter with timeout 100..1000ms, Stop |  | Starts or stops a continuous iris control for making the image darker or lighter with a maximum timeout. |  |
| IrisMode | Set Manual mode, Set Auto mode |  | Sets the iris mode to be automatic or manual. |  |
| FocusMode | Set Manual mode, Set Auto mode |  | Sets the focus mode to be automatic or manual. |  |
| GoToPresetPosition | 0..255 |  | This attribute will command the camera to go to a preset position. The preset number parameter should refer to an existing preset otherwise this will be ignored. |  |
| GotoHomePosition | No parameter |  | Moves the camera to its default home position |  |
| ExternalCameraLightState | On with light identifier (0..255), Off with light identifier (0..255) |  | Sets the specified external light to the specified state. |  |
| SelectFilter | 0.255 |  | Instructs the camera to use the selected filter. |  |
| SelectLens | 0.255 |  | Instructs the camera to use the selected lens. |  |
| ClearLens | No parameter |  | This instructs the camera to clear the lens. For example in a security application an external camera may have a wiper fitted. |  |
| SetPanSpeedAbsolute | 0..1024 |  | Configures the camera panning speed. |  |
| SetTiltSpeedAbsolute | 0..1024 |  | Configures the camera tilting speed. |  |
| SetPresetPosition | 0..255 |  | Sets a specified preset using the current camera position. |  |
| SetCameraBackLight | 0..255 |  | Configures the back light setting. |  |
| SetWhiteBalance | 0..255 |  | Configures the white balance alignment. |  |

Note 1: These parameter values express the angle in degrees to which the camera is to be positioned assuming that the mid point of the camera pan and tilt range is 0°. The range -180° to +180° is not mapped to the camera range but is interpreted absolutely.

Note 2: Those attributes that start a continuous movement have a maximum time out as part of the attribute parameter. The continuous movement is started in the selected direction until the timeout expires. If another control PDU is received before the timeout expires, the movement will be stopped if the PDU contains the same attribute with a stop parameter. If the PDU contains the same attribute with a new timeout, the timer will be immediately restarted with the new timeout.

The following table show the status attributes. It also shows two parameter columns. The Request parameter column shows the values that required when this attribute is sent as part of a RDC-Device-Status-Request PDU when is used specify the exact status required. The Response parameter column shows the values that are returned with the attributes as part of a RDC-Device-Status-Response PDU.

| Status Attributes | Request Parameter | Response Parameter | Description |
| --- | --- | --- | --- |
| ReadAbsolutePanPosition | None | -32767.. +32767 | Returns the absolute pan position. |
| ReadAbsoluteTiltPosition | None | -32767.. +32767 | Returns the absolute tilt position. |
| ReadAbsoluteDegressPanPosition | None | -180°..+180° | Returns the absolute pan position in degrees. |
| ReadAbsoluteDegressTiltPosition | None | -180°..+180° | Returns the absolute tilt position in degrees. |
| ReadAbsoluteZoomPosition | None | -127..+127 | Returns the absolute zoom position. |
| ReadAbsoluteIrisPosition | None | -127..+127 | Returns the absolute iris position. |
| ReadFocusMode | None | Manual, Automatic | Returns the current focus mode. |
| ReadIrisMode | None | Manual, Automatic | Returns the current iris mode. |
| ReadAbsolutePanSpeed | None | 0..1024 | Returns the current panning speed. |
| ReadAbsoluteTiltSpeed | None | 0..1024 | Returns the current tilting speed. |
| ReadNumberof Presets | None | 0..255 | Returns the current number of programmed camera presets. |
| ReadPresetLabel | 0..255 | Octet string | Returns the text label associated with a particular preset. |
| ReadActivePreset | None | 0..255 or 256 if no preset active | Returns which preset has been selected if any. |
| ReadExternalCameraLightState | 0..255 | On, Off | Returns the state of a particular external camera light. |
| ReadCameraBackLight | None | 0..255 | Returns the current value of the camera backlight setting. |
| ReadCameraWhiteBalance | None | 0..255 | Returns the current value of the camera white balance setting. |
| ReadNumberOfLenses | None | 0..255 | Returns the current number of lenses available for selection. |
| ReadNumberOfFilters | None | 0..255 | Returns the current number of filters available for selection. |
| ReadPanPointsPerDegree | None | 0..32767 | Returns the number of points that represent 1 degree when panning the camera. |
| ReadTiltPointsPerDegree | None | 0..32767 | Returns the number of points that represent 1 degree when tilting the camera. |

The following table show the event attributes. It also shows the parameter values that are used when these attributes are part of an RDC-Device-Event-Notify-Indication PDU.

| Device Event Attributes |  | Parameter |  | Description |
| --- | --- | --- | --- | --- |
| CameraPannedtoLimit | Left, Right |  | This event is triggered when a camera has been panned to its limit. |  |
| CameraTiltedtoLimit | Up, Down |  | This event is triggered when a camera has been tilted to its limit. |  |
| CameraZoomedtoLimit | Tele, Wide |  | This event is triggered when a camera has been zoomed to its limit. |  |
| CameraFocusedtoLimit | Far, Near |  | This event is triggered when a camera has been focused to its limit. |  |
| DeviceLockStateChange | Locked, Unlocked |  | This event is triggered when the camera lock state changes. The parameter indicates the new state of the device lock. |  |
| DeviceServiceChange | OutOfService, DeviceOK |  | This event is triggered when the camera is detected as being faulty or the camera is now in service after being previously being faulty. |  |

#### Microphone Device Class

The microphone device class provides a means for controlling and collecting the status of a remote microphone.

Microphone Device -Class Definition

| Device Control Attributes |  | Request Parameters |  | Description |
| --- | --- | --- | --- | --- |
| SetMicrophoneState | Active, Muted |  | Active turns the microphone on, muted turns it off. |  |

| Status Attributes | Request Parameter | Response Parameter | Description |
| --- | --- | --- | --- |
| readMicrophoneState | None | Active, Muted | Returns the current microphone state. |

#### VCR  Device Class

The VCR device class provides a means for controlling, configuring and collecting the status of  a remote Video Cassette Recorder.

VCR Device -Class Definition

| Device Control Attributes | Request Parameters | Description |
| --- | --- | --- |
| SelectTape | 0..255 | This selects a particular tape when more than one tape is available. |
| Play | Start, Stop | Starts or stops the VCR playing the current tape. |
| Rewind | Start, Stop | Starts or stops the VCR rewinding the current tape. |
| Fast Forward | Start, Stop | Starts or stops the VCR fast forwarding through the current tape. |
| Search Forward | Start, Stop | Starts or stops the VCR forwarding at greater than playing speed through the tape whilst still showing the output. |
| Search Backwards | Start, Stop | Starts or stops the VCR reversing at greater than playing speed through the tape whilst still showing the output. |
| Pause | On, Off | Sets or releases the VCR pause function. |
| Record | Start, Stop | Starts or stops the VCR recording an incoming video stream. |
| ResetCounter | None | Resets the VCR counter |
| SetAudioOutputState | Active, Muted | Activates or mutes the audio output from the VCR without affecting the Video output. |
| GoToPresetPosition | 0..255 | This attribute will command the VCR to go to a preset tape position. The preset number parameter should refer to an existing preset otherwise this will be ignored. |
| SetDate | Date which consists of day (1..31) month (1..12) year (4 digits) | Sets the current date on the VCR. |
| SetTime | Time which consists of hour (0..23) minute (0..59) | Sets the current time on the VCR. |
| SetContinousPlayMode | On, Off | Sets the continuous play mode where the tape is played continuously. |

| Status Attributes | Request Parameter | Response Parameter | Description |
| --- | --- | --- | --- |
| ReadTapeRecordProtected | None | True, False | Returns true if the tape has been protected against being used for recording. |
| ReadContinousPlayMode | None | On, Off | Returns the state of the continuous play mode. |
| ReadCounter | None | 0..65536 | Returns the current value of the VCR tape counter. |
| ReadNumberof Presets | None | 0..255 | Returns the number of programmed VCR tape presets. |
| ReadPresetLabel | 0..255 | Octet string | Returns the text label associated with a particular preset. |
| ReadVCRState | None | Playing, Recording, Paused, Paused record, Paused play, Rewinding, FastForwarding, SearchingForwards, SearchingBackwards, Stopped, Tape Ejected, Not ready | Returns the current state of the VCR. |
| ReadNumberOfTapes | None | 0..255 | Returns the number of tapes that the VCR is capable of loading. |

| Device Event Attributes |  | Parameter |  | Description |
| --- | --- | --- | --- | --- |
| TapeHasStopped | None |  | This indicates that the VCR has terminated the current playing or recording operation. |  |
| DeviceLockStateChange | Locked, Unlocked |  | This event is triggered when the VCR lock state changes. The parameter indicates the new state of the device lock. |  |
| DeviceServiceChange | OutOfService, DeviceOK |  | This event is triggered when the VCR is detected as being faulty or the VCR is now in service after being previously being faulty. |  |

#### Slide Presenter Device Class.

The slide presenter device class provides a means for controlling, configuring and collecting the status of  a remote slide presenter.

Slide Presenter Device -Class Definition

| Device Control Attributes | Command Parameters | Description |
| --- | --- | --- |
| SelectSlideAbsolute | 0..255 | Selects a particular slide for showing. |
| SelectSlideRelative | -127..+127 | Selects a particular slide relative to the current position. |
| SlideMode | Automatic, Manual | Selects the slide show mode |
| SlideShow | Start, Pause, Stop | Used to start, stop or pause a slide show. |
| FocusContinuous | Near with timeout 100..1000ms, Far with timeout 100..1000ms, Stop | Starts or stops a continuous focus movement with a maximum timeout. |
| IrisContinuous | Darker with timeout 100..1000ms, Lighter with timeout 100..1000ms, Stop | Starts or stops a continuous iris control for making the image darker or lighter with a maximum timeout. |
| ZoomContinuous | Tele with timeout 100..1000ms, Wide with timeout 100..1000ms, Stop | Starts or stops a continuous zoom movement with a maximum timeout. |
| SetSlideDisplayTime | 0..255 | Configures the time period in seconds for which a slide is displayed when an automatic slide show is in progress. |

| Status Attributes | Request Parameter | Response Parameter | Description |
| --- | --- | --- | --- |
| ReadSlideDisplayTime | None | 0..255 | Returns the time period in seconds for which a slide is displayed when an automatic slide show is in progress. |

#### Light Source Device Class.

The light source device class provides a means for controlling and collecting the status of  a remote light source that can be considered a device in its own right and is not part of  another device. An example of when this device class may be used is when  a light source is part of an external security installation. The light source will need to be remotely activated when the natural light fades and removed when the natural light returns.

Light Source Device -Class Definition

| Device Control Attributes | Command Parameters | Description |
| --- | --- | --- |
| SetLightSourceState | Active, Inactive | Used to turn the light source on or off. |

| Status Attributes | Request Parameter | Response Parameter | Description |
| --- | --- | --- | --- |
| ReadLightSourceState | None | Active, Inactive | Returns the state of the light source. |

#### Document Scanner Device Class

The document scanner device class provides a means for controlling, configuring and collecting the status of  a remote document scanner.

This section is for further study.

#### Video Source Selection Device Class

The video source selection device class provides a means for selecting the video output of one or more devices to become the source of an established real time video stream. A single source selection class exists for each video output stream.

This section is for further study.

#### Audio Source Selection Device Class

The audio source selection device class provides a means for selecting the audio output of one or more devices to become the source of an established real time audio stream. A single source selection class exists for each audio output stream.

This section is for further study.