# The OPT v1.23 Quick Start User’s Guide

Last Updated: March 10, 2004

Please send comments, suggestions or bug reports to opt_feedback@usb.org.

Introduction

This document describes revision history, and how to install and run the OPT test platform.

Revision History

March 10, 2004	OPT v1.23 released.  Modifications:

- Fixed TD4.6, B-OPT now correctly does VBus pulsing to start a second session

- OPT no longer requires VBus to be removed before running A tests

- Changed dialog message in TD.4.11 from “Device Not Supported” to “No Device Response”

- Fixed bug so OPT can now receive descriptors larger than 64 bytes

October 7, 2003	OPT v1.22 released.  Modifications:

- TD.5.1 modified so that a B-device now has 1 second to connect after Vbus goes above Va_sess_vld instead of 100 ms

- TD.4.8 modified to not log an error if session not suspended by host after resume signaling

- OPT modified to not log two consecutive setup packets as an error if first setup transaction is not completed

- Fixed GUI error where tests removed from Queued Tests would not go to Available Tests

August 20, 2003	OPT v1.21 released.  Modifications:

- TD.4.6 now discharges Vbus before doing an SRP after HNP

- TD.5.8 responds with unrecognized VID/PID

- Changed unrecognized VID/PID used to same VID, but different PID

- Modified OPT GUI directory browse method (Thanks go to DeVerl Stokes and Ray Asbury from Cypress)

- OPT can now remember values it queried users for between batch queues and OPT runs.  Purposely cleared at startup to prevent wrong assumptions at startup

July 26, 2003		OPT v1.2b unofficial release.  Modifications:

- Fixed enumeration timeout in SetFeature(), was broken in v1.2a only.

- Changed typo timing parameter in TD.5.6 from 200 us to 200 ms

- Changed number of interfaces in Config Descriptor to 1 instead of 0

- Fixed issue where OPT drives Vbus after calling <BusReset> until end of test

- Fixed typo in TD.5.8, OPT now responds with Vendor ID 0x1234 and Product ID 0x1234 instead of 0xBADD and 0x1A0A (dec 6666)

- Changed disconnect measurement threshold in TD.4.6 from 3 ms to 2 ms to compensate for OPT suspend detect error margin

July 18, 2003		OPT v1.2a -- BROKEN!  DON’T USE!

Unofficial release for plugfest.  Modifications:

- Added a bus reset at end of TD.4.4 so that UUT transitions fully to a_peripheral

- Changed OPT device class to 0xFF (vendor-specific) instead of mass storage

- Changed TD.4.11 so that a suspend timeout is not an error

- Changed IN transaction timeout from 2 ms to 50 ms

July 15, 2003		OPT v1.2 released.  Modifications:

- Test TD.5.3 script will now measure for a 4 ms B-disconnect instead of 5 ms because of the error deviation of the interaction between OPT software and hardware

- Currently running test is now highlighted while running test

- Updated scripts to reflect OTG compliance document change so some error messages can be generated immediately as opposed to after 5 seconds

- Test TD.4.7a and TD.4.9 now prompt to see if UUT is insertion-based.  These tests should not be run with insertion-based devices.

- Test TD.5.2 will drive a reset after measuring disconnect to clear b_hnp_enable so that an UUT does not generate error messages because of failed HNP

- Installation program is correctly named

- First run of a new OPT version will reset path settings

- Fixed some handle leak issues

KNOWN ISSUES

- OPT will generate “cannot open script file” error after approximately 200 test have been run, used to be after 50.  Simply close the OPT and open it again.  Hope to have fixed soon!

July 10, 2003		OPT v1.1 released.  Modifications:

- Updated and added scripts to be current with OTG Compliance Document v1.0 RC 5

- Added functionality to prompt users after tests to gather error message information

- Fixed memory leak passing messages from OPT DLL to OPT Main

- Added settings tab

- Added ability for users to select delay method between tests

- Added more support for Peripheral command to detect various host behavior and modify descriptors returned to host

- Fixed HostIn command.  OPT as host can now properly access all data read from peripheral from script

- Updated design manual

April 15, 2003		OPT v1.0 released

## Installation

The OPT uses third-party software from Venturcom (www.venturcom.com) to achieve real-time timing accuracy.  The RTX (Real-Time Extension) allows windows applications to achieve timing accuracies of 20 us.

RTX must be installed to run the OPT.  Follow the instructions below to install both RTX and the OPT software.

- Install RTX Runtime

  - Run RTX Runtime Setup, follow the instructions.  This will reboot the machine.

  - NOTE* RTX requires the “Advanced Configuration and Power Interface (ACPI) PC)” driver to be used as the Windows HAL.  If an error occurs during the RTX installation regarding the Windows HAL, you must change the windows HAL driver.  To do this:

    - Select Start->Settings->Control Panel->System

    - Select the Hardware tab

    - Click the Device Manager

    - Expand the Computer Tree

    - Double click the node below the Computer node

    - Select the Driver tab

    - Click “Update Driver…”

    - Click “Next”

    - Select “Display a list of the known drivers for this device…”

    - On the next dialog, click the “Show all hardware of this device class”

    - Select “Advanced Configuration and Power Interface (ACPI) PC”

    - This will reboot the machine

    - Run RTX setup after machine reboots

- While the machine is rebooting, insert the OPT board.

  - When Windows starts up and detects new hardware, click cancel.

- Add RTX driver for the OPT board (Optional, this will stop Windows from detecting new hardware on boot)

  - Click Start->Programs->Venturcom RTX->RTX->RTX Properties

  - Select the Plug & Play tab

  - Deselect the “Show Filtered List” checkbox

  - Find and right click on “Other PCI Bridge Device”, select “Add RTX INF Support”

  - When machine reboots and detects RTX support, select the default driver

- Install OPT software by running “Setup.exe”

## Before Running Tests

While testing an A-Device, the OPT will generate SRP at the beginning of every test.  The A-Device being tested should be returned to its initial state before every test is run.

Similarly, while testing a B-Device, the OPT will be waiting for SRP at the beginning of every test.  The user must do what is necessary to cause the device to generate SRP.

The test will timeout after 30 s if no response is detected.

## Testing an A or B Devices

To test either an A or B Device, click the appropriate button at the top of the OPT Test Selection Page.  Clicking “Test A-Device” or “Test B-Device” queues all the tests a device must pass, and selects the Test Results tab page.

When tests are running, the Test Summary tab page (see Test Summary Tab Page) will be selected.

After a test has run, a dialogue box pops-up signifying that the test has completed.  Before clicking “OK”, the UUT should be returned to its initial starting state.

## Test Summary Tab Page

The Test Queue Summary list box shows all the tests and their corresponding status.  The status of each test can be; Queued, Running, Success, or Error.

Highlighting a test marked as Success or Error will display the log summary information of the test in the Test Log Summary box. If the test is marked as Queued or Running, the log summary will not yet be available.

Once a test has been highlighted and the test summary is being displayed, clicking the Details button will display further information in the default HTML Viewer.  Readers are directed to section 8 for information on interpretting the log files.

## Running Individual Tests

To run an individual test, follow the following steps.

- Check the “Enable Advanced Test Selection” checkbox on the Test Selection Page.  This enables the controls in the Individual Test Selection frame.

- Click the “Browse” button next to the Current Directory box and locate the directory containing the desired script to run.  All of the test scripts in this directory will be displayed in the “Available Tests” list box.

- Highlight the test to run, then click the “->” button to move the test to the “Tests To Run” list box.

- Repeat steps 2 and 3 until all desired test scripts have been selected

- Click “Run Test(s)” to run the selected tests

### Running Debug Tests

Several tests run the same sequence of events with varying timing parameters.  Users can run these tests with custom timing parameters by running the Debug version of the test script.  Debug versions of XML scripts have filenames such as “TD.4.2-Debug.xml”.  This will pop-up a dialog box prior to running the test prompting users to enter timing parameters.

## OPT Settings

On the Settings tab page there are various settings that the user can change.  These settings are preserved from session to session.

### -Currently Selected OPT Tester

The “Currently Selected OPT Tester” setting tells OPT Main which OPT DLL to use.  This setting is recommended only for advanced users.

### -Log File Destination Directory

This setting specifies the directory in which to store the log files.

### -Log Filename Prefix

This setting specifies a prefix to insert into the log filename.  This setting can be useful when testing a specific device.  For instance, if the prefix entered were “OTG Printer” the log filename would appear as:

01-15-03 12-30-34 OTG Printer – TD.4.1.xml

Where the first part of the filename specifies the date and time the file was run.  The second is the Log Filename Prefix entered in the Test Selection Tab.  The third is the test script that was run.

### -Delay / Prompt

When running several tests in a queue, the OPT can either delay a specified amount of time between tests or prompt the user before continuing with the next test.

### -Batch Summary

After the OPT has run a queue of tests, it can generate a .html batch summary summarizing all the tests run with hyperlinks to the individual test logs.

### -Persist Queries

Some tests prompt the user for information about the UUT.  The OPT will remember these settings throughout a batch of runs.  If this button is checked, the OPT will remember these settings until the OPT is closed, or this check box is cleared.

This checkbox is cleared when the OPT starts.

## Writing Custom Script Files

In addition to running the included tests, users can create their own XML script files.  Users are encouraged to look at the syntax in the Debug script files, and also refer to the design manual section 5 for script syntax definitions.